| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 15th Meeting: Busan, KR, 16-22 April, 2005 | Document: JVT-O072 Filename: input.doc NOTE – That first character after the dash up there in the document number is an "O", not a zero. |
| --- | --- |

| Title: | Verification of SVC CE7 – Enhancement Layer Intra Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Heiko Schwarz Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49-30-31002-226 heiko.schwarz@hhi.fhg.de |
| Source: | Fraunhofer Institute for Telecommunications – Heinrich Hertz Institute (HHI) |  |  |

_____________________________

### Summary

The Fraunhofer HHI has performed the verification of Thomson’s results for SVC Core Experiment 7 on Enhancement Layer Intra Prediction.

### Verification process

The supplied bit-streams have been decoded, and the PSNR’s and bit-rates have been calculated using the scripts and binaries provided by Thomson.

### Verification results

The following tables summarize the measured bit-rates and PSNR values. All values are identical to the results that have been reported by Thomson.

Rate-distortion plots comparing the coding efficiency of the original JSVM 1 and that of Thomson’s proposal are contained in the embedded Excel document.

| Sequence: BUS CIF 15Hz |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QP | JSVM 1 |  |  |  | Thomson-Proposal |  |  |  |
|  | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [bit/s] | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [bit/s] |
| 8 | 53.35471 | 53.28714 | 53.76466 | 10066691 | 53.91603 | 53.29041 | 53.70068 | 10038146 |
| 12 | 49.68526 | 50.43383 | 51.21938 | 7967539 | 50.16982 | 50.44195 | 51.18605 | 7993042 |
| 16 | 46.08129 | 47.72876 | 48.76156 | 6238811 | 46.45401 | 47.73545 | 48.78717 | 6270142 |
| 20 | 42.39906 | 45.26112 | 46.74742 | 4682966 | 42.67671 | 45.17272 | 46.6889 | 4712637 |
| 24 | 38.93044 | 42.74472 | 44.54806 | 3401920 | 39.16726 | 42.71693 | 44.48636 | 3429326 |
| 28 | 35.75944 | 40.69191 | 42.55931 | 2409114 | 35.84736 | 40.64978 | 42.4717 | 2432304 |
| 32 | 32.59281 | 39.3588 | 41.01185 | 1598627 | 32.63757 | 39.27065 | 40.89966 | 1620806 |
| 36 | 29.71436 | 38.00347 | 39.26132 | 1005270 | 29.78092 | 37.88831 | 39.19135 | 1026195 |

| Sequence: FOREMAN CIF 15Hz |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QP | JSVM 1 |  |  |  | Thomson-Proposal |  |  |  |
|  | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [bit/s] | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [bit/s] |
| 8 | 53.25519 | 53.11349 | 53.29791 | 8072400 | 54.01362 | 53.17517 | 53.33568 | 8119051 |
| 12 | 49.74081 | 50.25493 | 50.81068 | 6053364 | 50.29172 | 50.30259 | 50.83722 | 6120228 |
| 16 | 46.37988 | 47.55743 | 48.55738 | 4448860 | 46.72654 | 47.62101 | 48.6438 | 4495459 |
| 20 | 43.01161 | 45.17851 | 46.79441 | 3087807 | 43.21269 | 45.13583 | 46.76322 | 3126068 |
| 24 | 39.96416 | 42.74906 | 44.66224 | 2063726 | 40.15129 | 42.73105 | 44.65886 | 2096671 |
| 28 | 37.27932 | 40.80869 | 42.62961 | 1349647 | 37.33034 | 40.7715 | 42.56499 | 1373710 |
| 32 | 34.54451 | 39.61067 | 41.13949 | 840027,2 | 34.61726 | 39.53395 | 41.00981 | 864644,8 |
| 36 | 32.04231 | 38.37949 | 39.46902 | 508917,6 | 32.15209 | 38.19678 | 39.18479 | 528408,8 |

| Sequence: FOOTBALL CIF 15Hz |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QP | JSVM 1 |  |  |  | Thomson-Proposal |  |  |  |
|  | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [bit/s] | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [bit/s] |
| 8 | 53.10842 | 53.30441 | 53.4492 | 8130239 | 53.96274 | 53.35002 | 53.46494 | 8166818 |
| 12 | 49.71566 | 50.61236 | 50.85746 | 6166645 | 50.33238 | 50.65875 | 50.89125 | 6216215 |
| 16 | 46.48554 | 48.07503 | 48.39642 | 4602111 | 46.90454 | 48.14036 | 48.466 | 4637460 |
| 20 | 43.1649 | 45.91945 | 46.5054 | 3289866 | 43.41964 | 45.90134 | 46.46602 | 3324590 |
| 24 | 40.00694 | 43.48303 | 44.30616 | 2285710 | 40.23489 | 43.43175 | 44.27502 | 2323133 |
| 28 | 37.13959 | 41.10608 | 42.41041 | 1545222 | 37.09175 | 41.03736 | 42.32952 | 1578937 |
| 32 | 34.28897 | 39.17635 | 40.94395 | 970227,7 | 34.26434 | 39.04288 | 40.84135 | 1001894 |
| 36 | 31.75822 | 37.17083 | 39.12606 | 580372,6 | 31.84296 | 37.06421 | 39.0807 | 606290,8 |

| Sequence: MOBILE CIF 15Hz |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QP | JSVM 1 |  |  |  | Thomson-Proposal |  |  |  |
|  | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [bit/s] | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [bit/s] |
| 8 | 53.30091 | 52.94687 | 52.98981 | 13893625 | 53.97833 | 53.01082 | 53.04926 | 13941326 |
| 12 | 49.68657 | 49.60001 | 49.64548 | 11453662 | 50.25458 | 49.64629 | 49.69141 | 11547010 |
| 16 | 46.13588 | 46.27663 | 46.32692 | 9371392 | 46.62284 | 46.30771 | 46.35735 | 9474440 |
| 20 | 42.30808 | 42.90224 | 42.93386 | 7414438 | 42.76048 | 42.89696 | 42.93401 | 7519213 |
| 24 | 38.59406 | 39.71864 | 39.68749 | 5687832 | 39.00163 | 39.7181 | 39.69831 | 5791251 |
| 28 | 35.11256 | 36.87402 | 36.77538 | 4225586 | 35.43151 | 36.85247 | 36.7613 | 4320661 |
| 32 | 31.49986 | 34.818 | 34.62588 | 2974873 | 31.77882 | 34.79174 | 34.61991 | 3065409 |
| 36 | 28.18949 | 32.71748 | 32.47 | 1961767 | 28.42283 | 32.67748 | 32.44431 | 2046521 |

| Sequence: CITY 4CIF 30Hz |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QP | JSVM 1 |  |  |  | Thomson-Proposal |  |  |  |
|  | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [bit/s] | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [bit/s] |
| 8 | 53.07649 | 53.90677 | 54.52759 | 62526126 | 53.90447 | 53.87744 | 54.42891 | 63067442 |
| 12 | 49.60854 | 51.28742 | 52.02018 | 47726306 | 50.20368 | 51.31604 | 52.02718 | 48466648 |
| 16 | 46.2459 | 48.79091 | 49.6978 | 35704172 | 46.63133 | 48.8478 | 49.8022 | 36227764 |
| 20 | 42.7534 | 46.77975 | 48.12115 | 25232394 | 42.96823 | 46.7059 | 48.07576 | 25637426 |
| 24 | 39.52388 | 44.59908 | 46.20701 | 17262746 | 39.71167 | 44.60205 | 46.21311 | 17586737 |
| 28 | 36.65444 | 42.91618 | 44.80124 | 11519754 | 36.63829 | 42.8561 | 44.71931 | 11797639 |
| 32 | 33.75792 | 41.7021 | 43.7489 | 7178074 | 33.72396 | 41.54038 | 43.53219 | 7438051 |
| 36 | 31.11763 | 40.37209 | 42.3689 | 4286195 | 31.12345 | 40.15941 | 42.04296 | 4486582 |

| Sequence: CREW 4CIF 30Hz |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QP | JSVM 1 |  |  |  | Thomson-Proposal |  |  |  |
|  | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [bit/s] | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [bit/s] |
| 8 | 53.02437 | 53.21718 | 53.5437 | 47942284 | 54.08701 | 53.34431 | 53.64965 | 50333478 |
| 12 | 49.85232 | 50.43235 | 50.9492 | 33838730 | 50.47228 | 50.52624 | 51.0295 | 35627629 |
| 16 | 46.72493 | 47.58224 | 48.47013 | 23011418 | 46.99979 | 47.63602 | 48.5555 | 24137462 |
| 20 | 43.59793 | 45.18825 | 46.534 | 14388590 | 43.5417 | 45.12493 | 46.54206 | 15081621 |
| 24 | 40.829 | 43.11443 | 44.28188 | 8614994 | 40.88856 | 43.14101 | 44.32152 | 8986490 |
| 28 | 38.67077 | 41.68555 | 42.24687 | 5224919 | 38.55874 | 41.68996 | 42.22386 | 5440856 |
| 32 | 36.5261 | 40.6038 | 40.66828 | 3169082 | 36.50795 | 40.54395 | 40.55843 | 3333596 |
| 36 | 34.42862 | 39.28846 | 38.93298 | 1932329 | 34.57298 | 39.18862 | 38.73926 | 2052813 |

| Sequence: HARBOUR 4CIF 30Hz |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QP | JSVM 1 |  |  |  | Thomson-Proposal |  |  |  |
|  | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [bit/s] | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [bit/s] |
| 8 | 52.86714 | 53.49865 | 54.18245 | 62929421 | 53.72038 | 53.57662 | 54.20245 | 62597081 |
| 12 | 49.49748 | 50.88148 | 51.74082 | 48193938 | 50.0507 | 50.94788 | 51.78572 | 47736954 |
| 16 | 46.15161 | 48.35776 | 49.40812 | 35829962 | 46.50369 | 48.3998 | 49.51231 | 35223272 |
| 20 | 42.70691 | 46.24565 | 47.78789 | 25130215 | 42.93922 | 46.14296 | 47.70904 | 24640403 |
| 24 | 39.58435 | 44.01585 | 45.87143 | 17158154 | 39.78113 | 44.02084 | 45.86984 | 16929130 |
| 28 | 36.82467 | 42.38095 | 44.3112 | 11573810 | 36.82986 | 42.31869 | 44.2353 | 11564988 |
| 32 | 33.93517 | 41.22968 | 43.08085 | 7483446 | 33.90987 | 41.05588 | 42.85543 | 7647582 |
| 36 | 31.22229 | 39.86657 | 41.42841 | 4808291 | 31.20926 | 39.6354 | 41.24245 | 4978898 |

| Sequence: SOCCER 4CIF 30Hz |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QP | JSVM 1 |  |  |  | Thomson-Proposal |  |  |  |
|  | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [bit/s] | Y-PSNR [dB] | U-PSNR [dB] | V-PSNR [dB] | rate [bit/s] |
| 8 | 53.16096 | 54.15545 | 54.95288 | 48924874 | 53.99509 | 54.16607 | 54.9536 | 49783582 |
| 12 | 49.94324 | 51.45877 | 52.33733 | 35441126 | 50.48955 | 51.51181 | 52.38217 | 36292659 |
| 16 | 46.82397 | 48.90526 | 49.98187 | 25187917 | 47.12204 | 48.98979 | 50.10581 | 25767638 |
| 20 | 43.60018 | 47.05795 | 48.60557 | 16954026 | 43.66654 | 47.01584 | 48.57641 | 17425560 |
| 24 | 40.55308 | 45.17251 | 46.9444 | 11143004 | 40.66652 | 45.16186 | 46.94307 | 11516239 |
| 28 | 37.86347 | 43.51317 | 45.32567 | 7177650 | 37.71756 | 43.42415 | 45.17507 | 7473222 |
| 32 | 35.21388 | 42.18728 | 43.87839 | 4179919 | 35.10728 | 42.00494 | 43.57256 | 4397879 |
| 36 | 32.85631 | 40.63765 | 42.57629 | 2290999 | 32.91122 | 40.40001 | 41.95668 | 2425641 |