| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 15th Meeting: Busan, KR, 16-22 April, 2005 | Document: JVT-O073 Filename: input.doc NOTE – That first character after the dash up there in the document number is an "O", not a zero. |
| --- | --- |

| Title: | Improvement of the progressive refinement coding for 8x8 luma blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Heiko Schwarz, Detlev Marpe, and Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49-30-31002-226 / 619 / 617 heiko.schwarz@hhi.fhg.de detlev.marpe@hhi.fhg.de thomas.wiegand@hhi.fhg.de |
| Source: | Fraunhofer Institute for Telecommunications – Heinrich Hertz Institute (HHI) |  |  |

_____________________________

### Summary

A comparison of the progressive refinement (FGS) coding in JSVM 0 [1][2] and JSVM 1 [3][4] turned out that the coding efficiency at the end point of a progressive refinement (FGS) layer of JSVM 1 is worse than that of JSVM 0 when 8x8 transforms are used.  In this contribution we solved that problem by a straightforward extension of the cyclical block coding [5] as it is used for 4x4 blocks in the JSVM 1 to 8x8 luma blocks.  The experimental results show that the coding efficiency can be improved between 1% and 6% depending on the number of 8x8 blocks that are selected for a specific test sequence.

### Background

When comparing the coding efficiency of JSVM 0 and JSVM 1 using the provided configuration files, the problem arises that the quantization parameters specified in the configuration files do not fit to the modified quantizer selection in JSVM 1.  In order to verify the assumption that this difference is only a results of the modified quantizer selection, we performed a comparison between JSVM 0 and the JSVM 1 with the old (JSVM 0) quantizer selection algorithm and identical configuration files.  But even in this comparison, the coding efficiency of the JSVM 1 was worse at the high-rate points (for example, see Figure 1).

Consequently, we analyzed the new tools [5][6] in JSVM 1 and found that the difference results from an inefficiency of the progressive refinement coding (FGS) for 8x8 luma blocks.  When the base layer is compatible with the H.264/AVC Main Profile, and thus only the 4x4 transform is used for luma blocks, the end points of the progressive refinement layers are virtually identical for JSVM 0 and JSVM 1, and the extraction points in between show a higher reconstruction quality for the cyclical block coding in JSVM 1 as it is illustrated in Figure 2.  However, when the H.264/AVC High Profile is used for the base layer, and thus both the 4x4 and 8x8 transform can be chosen for luma blocks, the coding of the progressive refinement layers with JSVM 1 requires a higher rate (1%-6%) as the coding with JSVM 0 while the PSNR values are identical as it is shown in Figure 3 for the example of the FOOTBALL sequence.

We propose to modify the progressive refinement coding for 8x8 luma blocks in a straightforward way by using the same cyclical block coding algorithm as it is used for 4x4 luma blocks and for chroma blocks in the JSVM 1.

Figure 1: Comparison of JSVM 0 and JSVM 1 (without modified quantizer selection) for the coding conditions of Palma CE1 using identical configuration files.

Figure 2: Comparison of the progressive refinement coding (2 FGS layers) of JSVM 0 and JSVM 1 for H.264/AVC Main Profile compatible base layer (only 4x4 transform for luma blocks).

Figure 3: Comparison of the progressive refinement coding of JSVM 0 and JSVM 1 for H.264/AVC High Profile compatible base layer (4x4 and 8x8 transform for luma blocks).

### Proposed changes

We propose the following two changes of the progressive refinement coding:

- Usage of the cyclical block coding algorithm also for 8x8 luma blocks.  At this, in each cycle for 4x4 luma blocks, 4 cycles for the 8x8 luma blocks are coded.  For this modification, the same syntax elements and the same syntax structure as for 4x4 luma blocks is used, only the maximum index is increased to 64.  The deleted parts in the following syntax diagram are stroked out and marked blue, while the new parts are marked red.  We consider this proposal as a small bug-fix for the current progressive refinement coding.

- In the current JSVM 1 syntax, a last_significant_coeff_flag is always coded, when the scan index is greater than zero.  However, the last_significant_coeff_flag can only be non-zero, when the current coefficient is not the first non-refinement coefficient.  Here, a coefficient is called refinement coefficient, when the corresponding transform coefficient level in the base layer is non-zero.  Consequently, we propose to modify the syntax in a way that the last_significant_coeff_flag is only coded, when the scan index is greater than a variable scanIdxStart, where scanIdxStart is the scan index of the first non-refinement coefficient.  Although this modification has a very minor impact on the coding efficiency (< 0.5% rate savings), we believe that it represents a logical improvement of the syntax.  The required syntax changes are marked green in the following syntax diagrams.

The required syntax changes for the proposed modifications are shown in the following syntax diagrams.

| luma_coefficient( b4x4IdxY, b4x4IdxX, endCycles, sigBlk ) { | C | Descriptor |
| --- | --- | --- |
| CurrMbAddr = ( b4x4IdxY / 4 ) * PicWidthInMb + ( b4x4IdxX / 4 ) |  |  |
| i8x8 = 2 * ( ( b4x4IdxY % 4 ) / 2 ) + ( ( b4x4IdxX % 4 ) / 2 ) |  |  |
| i4x4 = 2 * ( b4x4IdxY % 2 ) + ( b4x4IdxX % 2 ) |  |  |
| scanIdx = 16 |  |  |
| for( idx = 0; idx < 16; idx++ ) { |  |  |
| idx8x8 = 4 * idx + 2 * ( b4x4IdxY % 2 ) + ( b4x4IdxX % 2 ) |  |  |
| if( scanIdx == 16 && ( ( ! TransformSize8x8Flag && LumaLevel[ 4 * i8x8 + i4x4 ][ idx ] = = NotCoded && LumaLevelBase[ 4 * i8x8 + i4x4 ][ idx ] = = 0 ) ¦ ¦ ( TransformSize8x8Flag && LumaLevel8x8[ i8x8 ][ idx8x8 ] = = NotCoded && LumaLevel8x8Base[ i8x8 ][ idx8x8 ] = = 0 ))) |  |  |
| scanIdx = idx |  |  |
| } |  |  |
| if( scanIdx != 16 ) |  |  |
| EndCycles = 0 |  |  |
| ScanIdx8x8 = 4 * scanIdx + 2 * ( b4x4IdxY % 2 ) + ( b4x4IdxX % 2 ) |  |  |
| if( mb_inside_base_quality_slice( CurrMbAddr ) && scanIdx != 16 ) { |  |  |
| if( moreCabacDataInNALUnit( ) && coded_block_pattern_bit[ i8x8 ] = = NotCoded ) { |  |  |
| coded_block_pattern_bit[ i8x8 ] | 3 ¦ 4 | ae(v) |
| if( coded_block_pattern_bit[ i8x8 ] = = 0 ) { |  |  |
| for( j = 0; j < 4; j++ ) |  |  |
| for( i = 0; i < 16; i++ ) |  |  |
| LumaLevel[ 4 * i8x8 + j ][ i ] = 0 |  |  |
| for( i = 0; i < 64; i++ ) |  |  |
| LumaLevel8x8[ i8x8 ][ i ] = 64 |  |  |
| } else { |  |  |
| if( moreCabacDataInNALUnit( ) && ! base_mb_sig( CurrMbAddr ) && mb_qp_delta = = NotCoded ) |  |  |
| mb_qp_delta | 2 | ae(v) |
| if( moreCabacDataInNALUnit( ) && transform_size_8x8_flag = = NotCoded ) { |  |  |
| transform_size_8x8_flag | 2 | ae(v) |
| TransformSize8x8Flag = transform_size_8x8_flag |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| if( coded_block_pattern_bit[ i8x8 ] = = 1 ) { |  |  |
| if( ! TranformSize8x8Flag ) { |  |  |
| if( moreCabacDataInNALUnit( ) && coded_block_flag_luma[ 4*i8x8 + i4x4 ] = = NotCoded ) { |  |  |
| coded_block_flag_luma[ 4*i8x8 + i4x4 ] | 3 ¦ 4 | ae(v) |
| if( coded_block_flag_luma[ 4 * i8x8 + i4x4 ] = = 0 ) |  |  |
| for( i = 0; i < 16; i++ ) |  |  |
| LumaLevel[ 4 * i8x8 + i4x4 ][ i ] = 0 |  |  |
| } |  |  |
| if( coded_block_flag_luma[ 4 * i8x8 + i4x4 ] = = 1 ) { |  |  |
| scanIdx = 16 |  |  |
| scanIdxStart = 16 |  |  |
| for( idx = 0; idx < 16; idx++ ) { |  |  |
| if( scanIdx = = 16 && LumaLevel[ 4 * i8x8 + i4x4 ][ idx ] = = NotCoded && base_luma_level( CurrMbAddr, i8x8, i4x4, idx ) = = 0 ) |  |  |
| scanIdx = idx |  |  |
| if( scanIdxStart = = 16 && base_luma_level( CurrMbAddr, i8x8, i4x4, idx ) = = 0 ) |  |  |
| scanIdxStart = idx |  |  |
| } |  |  |
| if( scanIdx != 16 ) |  |  |
| endCycles = 0 |  |  |
| if( scanIdx > 0 scanIdxStart && moreCabacDataInNALUnit( ) ) |  |  |
| last_significant_coeff_flag | 3 ¦ 4 | ae(v) |
| if( last_significant_coeff_flag ) { |  |  |
| for( i = scanIdx; i < 16; i++ ) |  |  |
| LumaLevel[ 4 * i8x8 + i4x4 ][ i ] = 0 |  |  |
| scanIdx = 16 |  |  |
| } |  |  |
| } |  |  |
| while( scanIdx < 16 ) { |  |  |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| significant_coeff_flag | 3 ¦ 4 | ae(v) |
| if( significant_coeff_flag ) { |  |  |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| coeff_sign_flag | 3 ¦ 4 | ae(v) |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| coeff_abs_level_minus1 | 3 ¦ 4 | ae(v) |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| LumaLevel[ 4 * i8x8 + i4x4 ][ scanIdx ] = |  |  |
| (coeff_abs_level_minus1 + 1) * (1 – 2 * coeff_sign_flag) |  |  |
| scanIdx = 16 |  |  |
| } else { |  |  |
| LumaLevel[ 4 * i8x8 + i4x4 ][ scanIdx ] = 0 |  |  |
| scanIdx++ |  |  |
| while( scanIdx < 16 && base_luma_level( CurrMbAddr, i8x8, i4x4, scanIdx ) ) { |  |  |
| scanIdx++ |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } else { |  |  |
| scanIdx8x8 = 64 |  |  |
| scanIdxStart8x8 = 64 |  |  |
| for( idx8x8 = 0; idx8x8 < 64; idx8x8++ ) { |  |  |
| if( scanIdx8x8 = = 64 && LumaLevel8x8[ i8x8 ][ idx8x8 ] = = NotCoded && base_luma_level8x8( CurrMbAddr, i8x8, idx8x8 ) = = 0 ) |  |  |
| scanIdx8x8 = idx8x8 |  |  |
| if( scanIdxStart8x8 = = 64 && base_luma_level8x8( CurrMbAddr, i8x8, idx8x8 ) = = 0 ) |  |  |
| scanIdxStart8x8 = idx8x8 |  |  |
| } |  |  |
| if( scanIdx8x8 > scanIdxStart8x8 && moreCabacDataInNALUnit( ) ) |  |  |
| last_significant_coeff_flag | 3 ¦ 4 | ae(v) |
| if( last_significant_coeff_flag ) { |  |  |
| for( i = scanIdx8x8; i < 64; i++ ) |  |  |
| LumaLevel8x8[ i8x8 ][ i ] = 0 |  |  |
| scanIdx8x8 = 64 |  |  |
| } |  |  |
| } |  |  |
| while( scanIdx8x8 < 64 ) { |  |  |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| significant_coeff_flag | 3 ¦ 4 | ae(v) |
| if( significant_coeff_flag ) { |  |  |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| coeff_sign_flag | 3 ¦ 4 | ae(v) |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| coeff_abs_level_minus1 | 3 ¦ 4 | ae(v) |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| LumaLevel[ i8x8 ][ scanIdx8x8 ] = |  |  |
| (coeff_abs_level_minus1 + 1) * (1 – 2 * coeff_sign_flag) |  |  |
| scanIdx = 64 |  |  |
| } else { |  |  |
| LumaLevel8x8[ i8x8 ][ scanIdx8x8 ] = 0 |  |  |
| scanIdx8x8++ |  |  |
| while( scanIdx8x8 < 64 && base_luma_level8x8( CurrMbAddr, i8x8, scanIdx8x8 ) ) { |  |  |
| scanIdx8x8++ |  |  |
| } |  |  |
| } |  |  |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| significant_coeff_flag | 3 ¦ 4 | ae(v) |
| if( significant_coeff_flag ) { |  |  |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| coeff_sign_flag | 3 ¦ 4 | ae(v) |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| coeff_abs_level_minus1 | 3 ¦ 4 | ae(v) |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| LumaLevel8x8[ i8x8 ][ scanIdx8x8 ] = ( coeff_abs_level_minus1 + 1 ) * ( 1 – 2 * coeff_sign_flag ) |  |  |
| } else |  |  |
| LumaLevel8x8[ i8x8 ][ scanIdx8x8 ] = 0 |  |  |
| if( significant_coeff_flag && moreCabacDataInNALUnit( ) && scanIdx < 63 ) { |  |  |
| last_significant_coeff_flag | 3 ¦ 4 | ae(v) |
| if( last_significant_coeff_flag ) |  |  |
| for( i = scanIdx8x8 + 1; i < 64; i++ ) |  |  |
| LumaLevel8x8[ i8x8 ][ i ] = 0 |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

| chromaDC_coefficient( iCbCr, mbIdxY, mbIdxX, endCycles, sigBlk ) { | C | Descriptor |
| --- | --- | --- |
| NumC8x8 = 4 / ( SubWidthC * SubHeightC ) |  |  |
| CurrMbAddr = mbIdxY / 4 * PicWidthInMb + mbIdxX |  |  |
| dcIdx = 4 |  |  |
| dcIdxStart = 4 |  |  |
| for( idx = 0; idx < 4; idx++ ) { |  |  |
| if( dcIdx == 4 && ChromaDCLevel[ iCbCr ][ idx ] = = NotCoded && base_chromaDC_level( CurrMbAddr, iCbCr, idx ) = = 0 ) |  |  |
| dcIdx = idx |  |  |
| if( dcIdxStart = = 4 && base_chromaDC_level( CurrMbAddr, iCbCr, idx ) = = 0 ) |  |  |
| dcIdxStart = idx |  |  |
| } |  |  |
| if( dcIdx != 4 ) |  |  |
| endCycles = 0 |  |  |
| if( MbInsideBaseLayerSlice( CurrMbAddr ) && BaseChromaDCBlockSignificance( CurrMbAddr, iCbCr ) = = sigBlk && dcIdx != 4 ) { |  |  |
| if( moreCabacDataInNALUnit( ) && coded_block_pattern_bit[ 4 ] = = NOT_CODED ) { |  |  |
| coded_block_pattern_bit[ 4 ] | 3 ¦ 4 | ae(v) |
| if( coded_block_pattern_bit[ 4 ] = = 0 ) { |  |  |
| for( j = 0; j < 2; j++ ) { |  |  |
| for( i = 0; i < 4 * NumC8x8; i++ ) |  |  |
| ChromaDCLevel[ j ][ i ] = 0 |  |  |
| for( i8x8 = 0; i8x8 < NumC8x8; i8x8++ ) |  |  |
| for( i4x4 = 0; i4x4 < 4; i4x4++ ) |  |  |
| for( scanIdx = 0; scanIdx < 15; scanIdx++ ) |  |  |
| ChromaACLevel[ j ][ 4 * i8x8 + i4x4 ][ scanIdx ] = 0 |  |  |
| } |  |  |
| } else { |  |  |
| if( moreCabacDataInNALUnit( ) && ! BaseLayerMacroblockSignificance( CurrMbAddr ) && mb_qp_delta = = NOT_CODED ) |  |  |
| mb_qp_delta | 2 | ae(v) |
| } |  |  |
| } |  |  |
| if( coded_block_pattern_bit[ 4 ] = = 1 ) { |  |  |
| if( moreCabacDataInNALUnit( ) && coded_block_flag_chromaDC[ iCbCr ] = = NOT_CODED ) { |  |  |
| coded_block_flag_chromaDC[ iCbCr ] | 3 ¦ 4 | ae(v) |
| if( coded_block_flag_chromaDC[ iCbCr ] == 0 ) |  |  |
| for( i = 0; i < 4 * NumC8x8; i++ ) |  |  |
| ChromaDCLevel[ iCbCr ][ i ] = 0 |  |  |
| } |  |  |
| if( coded_block_flag_chromaDC[ iCbCr ] = = 1 ) { |  |  |
| if( dcIdx > 0 dcIdxStart && moreCabacDataInNALUnit( ) ) { |  |  |
| last_significant_coeff_flag | 3 ¦ 4 | ae(v) |
| if( last_significant_coeff_flag ) |  |  |
| for( i = dcIdx; i < 4 * NumC8x8; i++ ) |  |  |
| ChromaDCLevel[ iCbCr ][ i ] = 0 |  |  |
| } |  |  |
| while( dcIdx < 4 ) { |  |  |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| significant_coeff_flag | 3 ¦ 4 | ae(v) |
| if( significant_coeff_flag ) { |  |  |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| coeff_sign_flag | 3 ¦ 4 | ae(v) |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| coeff_abs_level_minus1 | 3 ¦ 4 | ae(v) |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| ChromaDCLevel[ iCbCr ] [ dcIdx ] = |  |  |
| ( coeff_abs_level_minus1 + 1 ) * ( 1 – 2 * coeff_sign_flag ) |  |  |
| dcIdx++ |  |  |
| } else { |  |  |
| ChromaDCLevel[ iCbCr ][ dcIdx ] = 0 |  |  |
| dcIdx++ |  |  |
| while( dcIdx < 4 && ChromaDCLevelBase[ iCbCr ][ dcIdx ] = = 1 ) |  |  |
| dcIdx++ |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

| chromaAC_coefficient( iCbCr, bcIdxY, bcIdxX, endCycles, sigBlk ) { | C | Descriptor |
| --- | --- | --- |
| NumC8x8 = 4 / ( SubWidthC * SubHeightC ) |  |  |
| CurrMbAddr = ( b4x4IdxY / ( 2 * SubHeightC ) ) * PicWidthInMb + ( b4x4IdxX / ( 2 * SubHeightC ) ) |  |  |
| i8x8 = 2 * ( ( bcIdxY % ( 2 * SubHeightC ) ) / 2 ) + ( ( bxIdxX % ( 2 * SubWidthC ) ) / 2 ) |  |  |
| i4x4 = 2 * ( bcIdxY % 2 ) + ( bcIdxX % 2 ) |  |  |
| scanIdx = 16 |  |  |
| scanIdxStart = 16 |  |  |
| for( idx = 0; idx < 16; idx++ ) { |  |  |
| if( scanIdx == 16 && ChromaACLevel[iCbCr][4* i8x8 + i4x4][idx] == NotCoded && base_chromaAC_level( CurrMbAddr, iCbCr, i8x8, i4x4, idx ) = = 0) |  |  |
| scanIdx = idx |  |  |
| if( scanIdxStart == 16 && base_chromaAC_level( CurrMbAddr, iCbCr, i8x8, i4x4, idx ) = = 0) |  |  |
| scanIdxStart = idx |  |  |
| } |  |  |
| if( scanIdx != 16 ) |  |  |
| EndCycles = 0 |  |  |
| if( MbInsideBaseLayerSlice( CurrMbAddr ) && BaseChromaACBlkSignificance( CurrMbAddr, iCbCr, i8x8, i4x4 ) = = sigBlk && scanIdx != 16 ) { |  |  |
| if( moreCabacDataInNALUnit( ) && coded_block_pattern_bit[ 4 ] = = NOT_CODED ) { |  |  |
| coded_block_pattern_bit[ 4 ] | 3 ¦ 4 | ae(v) |
| if( coded_block_pattern_bit[ 4 ] = = 0 ) { |  |  |
| for( j = 0; j < 2; j++ ) { |  |  |
| for( i = 0; i < 4 * NumC8x8; i++ ) |  |  |
| ChromaDCLevel[ j ][ i ] = 0 |  |  |
| for( i = 0; i < NumC8x8; i++ ) |  |  |
| for( k = 0; k < 4; k++ ) |  |  |
| for( l = 0; l < 15; l++ ) |  |  |
| ChromaACLevel[ j ][ 4 * i + k ][ l ] = 0 |  |  |
| } |  |  |
| } else { |  |  |
| if( moreCabacDataInNALUnit( ) && ! BaseLayerMacroblockSignificance( CurrMbAddr ) && mb_qp_delta = = NOT_CODED ) |  |  |
| mb_qp_delta |  |  |
| } |  |  |
| } |  |  |
| if( coded_block_pattern_bit[ 4 ] = = 1 && moreCabacDataInNALUnit( ) && coded_block_pattern_bit[ 5 ] = = NOT_CODED ) { |  |  |
| coded_block_pattern_bit[ 5 ] | 3 ¦ 4 | ae(v) |
| if( coded_block_pattern_bit[ 5 ] = = 0 ) { |  |  |
| for( j = 0; j < 2; j++ ) |  |  |
| for( i = 0; i < NumC8x8; i++ ) |  |  |
| for( k = 0; k < 4; k++ ) |  |  |
| for( l = 0; l < 15; l++ ) |  |  |
| ChromaACLevel[ j ][ 4 * i + k ][ l ] = 0 |  |  |
| } |  |  |
| } |  |  |
| if( coded_block_pattern_bit[ 5 ] = = 1 ) { |  |  |
| if( moreCabacDataInNALUnit( ) && coded_block_flag_chromaAC[ iCbCr ][ 4 * i8x8 + i4x4] = = NOT_CODED ) { |  |  |
| coded_block_flag_chromaAC[ iCbCr ][ 4 * i8x8 + i4x4 ] | 3 ¦ 4 | ae(v) |
| if( coded_block_flag_chromaAC[ iCbCr ][ 4 * i8x8 + i4x4 ] = = 0 ) |  |  |
| for( k = 0; k < 15; k++ ) |  |  |
| ChromaACLevel[ iCbCr ][ 4 * i8x8 + i4x4 ][ k ] = 0 |  |  |
| } |  |  |
| if( coded_block_flag_chromaAC[ iCbCr ][ 4 i8x8 + i4x4 ] = = 1 ) { |  |  |
| if( scanIdx > 0 scanIdxStart && moreCabacDataInNALUnit( ) ) { |  |  |
| last_significant_coeff_flag | 3 ¦ 4 | ae(v) |
| if( last_significant_coeff_flag ) |  |  |
| for( i = scanIdx; i < 15; i++ ) |  |  |
| ChromaACLevel[ iCbCr ][ 4 * i8x8 + i4x4 ][ i ] = 0 |  |  |
| } |  |  |
| while( scanIdx < 16 ) { |  |  |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| significant_coeff_flag | 3 ¦ 4 | ae(v) |
| if( significant_coeff_flag ) { |  |  |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| coeff_sign_flag | 3 ¦ 4 | ae(v) |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| coeff_abs_level_minus1 | 3 ¦ 4 | ae(v) |
| if( moreCabacDataInNALUnit( ) ) |  |  |
| ChromaACLevel[ iCbCr ][ 4 * i8x8 + i4x4 ][ scanIdx ] = |  |  |
| (coeff_abs_level_minus1 + 1) * (1 – 2 * coeff_sign_flag) |  |  |
| scanIdx = 16 |  |  |
| } else { |  |  |
| ChromaACLevel[ iCbCr ][ 4 * i8x8 + i4x4 ][ scanIdx ] = 0 |  |  |
| scanIdx++ |  |  |
| while( scanIdx < 16 && ChromaACLevelBase[iCbCr][4 * i8x8 + i4x4][scanIdx] == 1 ) |  |  |
| scanIdx++ |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

### Experimental Results

For analyzing the coding efficiency improvement, we compared the JSVM 1 software with the proposed changes with the original JSVM 1 software.  For all 8 tested sequences, the base layer was coded using the H.264/AVC High Profile with 8x8 luma transform enabled, and on top of the base layer, 2 progressive refinement layers have been coded.  The base layer is always identical for both encoders, and furthermore, the PSNR’s of the end points of each progressive refinement layer are identical, since only the entropy coding was modified.  In Figure 4, rate-distortion curves for the sequence Football are depicted.  Rate-distortion curves for the other 7 test sequences can be found in the embedded Excel document.

With the proposed modifications, bit-rate savings between 1% and 6% are obtained in comparison to the original JSVM 1 depending on the number of macroblocks, for which an 8x8 transform is used. The rate savings for the end points of the progressive refinement layers are summarized in Table 1.

Figure 4: Comparison of the original JSVM 1 and the JSVM 1 with the proposed changes for H.264/AVC High Profile compatible base layer (4x4 and 8x8 transform for luma blocks) on the example of the FOOTBALL sequence.

Table 1: Bit-rate savings resulting from the proposed modifications for the end points of the PR layers.

| PR layer | Bus | Foreman | Football | Mobile | City | Crew | Harbour | Soccer |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.9% | 3.3% | 3.2% | 1.1% | 4.5% | 4.4% | 5.1% | 4.5% |
| 2 | 2.8% | 4.3% | 4.2% | 1.5% | 5.0% | 5.7% | 6.3% | 5.6% |

### References

- J. Reichel, H. Schwarz, M. Wien, eds., “Joint Scalable Video Model JSVM 0,” JVT, Doc. JVT-N021, Hong Kong, CN, Jan. 2005.

- JSVM 0 software, JVT, Doc. JVT-022, Hong Kong, CN, Jan. 2005.

- J. Reichel, H. Schwarz, M. Wien, eds., “Joint Scalable Video Model JSVM 1,” JVT, Doc. JVT-N023, Hong Kong, CN, Jan. 2005.

- JSVM 1 software, JVT, Doc. JVT-024, Hong Kong, CN, Jan. 2005.

- J. Ridge, Y. Bao, M. Karczewicz, X. Wang, “Cyclical block coding for FGS,” ISO/IEC JTC1/WG11, Doc. m11509, Hong Kong, CN, Jan. 2005.

- W.-J. Han, S.-C. Cha, H.-J. Ha, “Responses of SVC CE3: Inter-layer prediction,” ISO/IEC JTC1/WG11, Doc. m11585, Hong Kong, CN, Jan. 2005.

(Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Fraunhofer Institute for Telecommunications – Heinrich Hertz Institute |  |  |
| Mailing address | Einsteinufer 37 10587 Berlin, Germany |  |  |
| Country | Germany |  |  |
| Contact person | Heiko Schwarz |  |  |
| Telephone | +49-30-31002-226 |  |  |
| Fax | +49-30-3927200 |  |  |
| Email | heiko.schwarz@hhi.fhg.de |  |  |
| Place and date of submission | Berlin, April 12, 2005 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: