| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 15th Meeting: Busan, KR, 16-22 April, 2005 | Document: JVT-Oxxx Filename: input.doc NOTE – That first character after the dash up there in the document number is an "O", not a zero. |
| --- | --- |

| Title: | New downsampling and upsampling processes for chroma samples in SVC spatial scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT, 72nd MPEG meeting |  |  |
| Purpose: | Informatioin |  |  |
| Author(s) or Contact(s): | Chen Ying1 Zhai Jiefu1 Yin Peng2 Francois Edouard3 - Thomson Corporate Research Beijing, China - Thomson Corporate Research Princeton, USA - Thomson Corporate Research Rennes, France | Email: | ying.chen@thomson.net jiefu.zhai@thomson.net peng.yin@thomson.net Edouard.Francois@thomson.net |
| Source: |  |  |  |

Abstract

In this document, we proposed new downsampling and upsampling processes for chroma samples for SVC spatial scalability, based on two facts: the correlation of the chroma samples is low and the current downsampling and upsampling processes are not perfect for the sample format 4:2:0, defined in H.264.

New downsampling and upsampling processes

The 4:2:0 format is widely used in video coding standards [1][2][3][4].

Figure 1. YUV 4:2:0 sample positions (MPEG-1)

Historically, two kinds of 4:2:0 sample formats are supported in the video coding standards. MPEG-1 and H.263 use the format shown in Figure 1, named as type-1 4:2:0 format. MPEG-2, MPEG-4 and H.264/AVC supports the format shown in Figure 2, named as type-2 4:2:0 format.

Figure 2. YUV 4:2:0 sample positions (MPEG-2, MPEG-4 and H.264)

To provide spatial scalability, in scalable video coding scheme [7], MPEG-4 downsampling filter[5] is used to get the downsampled low resolution images and H.264 6 tap filter [4] is used to get the upsampled reconstructed picture[6]. The downsampling filter is h[n]=[2, 0,−4,−3, 5, 19, 26, 19, 5,−3,−4, 0, 2]/64, [1] which is an odd-length filter. The 6-tap upsampling filter is g[n]= [1, -5, 20, 20, -5, 1]/32[4]. Both filters are used for luma and chroma components. There is no phase shift problem for the Y component, which is crucial for the efficiency of some SVC modes, such as INTRA-BL.

However, there is phase difference problem for chroma samples based on the above solutions. Since the chroma samples have higher correlation than luma samples, shorter FIR filters will not reduce the coding efficiency and image quality while providing more effective solutions.

Chroma downsampling

2 tap filter f[n]=[1,1]/2 is a possible solution for downsampling both for horizontal or vertical directions. And to solve the possible phase shift problem, one solution is, for horizontal direction, to use 3 tap filter [1, 2, 1]/4, shown in figure 3 (downsampled samples are indicated in blue), and for vertical direction, to use 2 tap filter [3, 1]/4, which is actually a linear filter shown in figure 4 (downsampled samples are indicated in red).

Figure 3. Horizontal downsampling using 3 tap filter [1, 2, 1]/4

Figure 4. Vertical downsampling using 2 tap linear filter [3, 1]/4

Chroma upsampling

One solution is bi-linear interpolation, which is used in H.264/AVC upsampling for half/quarter pixel motion compensation.

Figure 4. Horizontal upsampling using 2 tap filter

Figure 5. Vertical upsampling using linear filters [5, 3]/8 and [1, 7]/8

Another solution is using 2 tap filters for the horizontal and vertical upsampling processes. However the weights of the filters are based on the distances between the locations of original samples (marked in black in figure 4; blue/black in figure 5) and that of the respected upsampled (marked in blue in figure 4 and red in figure 5) Vertically, for the odd pixels, we use a linear filter of [1, 7]/8 and for even pixels, we use a linear filter of [5, 3]/8.

A simpler solution could be utilized for low complexity applications, so the filters mentioned above can be approximate to [0, 8]/8 and [4, 4]/8, that is [0, 1]/1 and [1, 1]/2. So for chroma samples, let x[n] (n=0, 1, … N-1.) be the original column signal. Then suppose upsampled chroma samples are X[n] (n=0, 1, …, 2N-1). We have X[0]=x[0]; X[2*k+1]=(x[k]+x[k+1]+1)/2, X[2*k+2]=x[k+1] (k=0, 1, … N-2); X[2N-1]=x[N-1].

All the proposed downsampling and upsampling techniques are only for chroma components, for luma components, MPEG-4 downsampling filter and H.264 6-tap filter shall still be utilized.

For the cases that a filter encounters a border, we use border extension method just like JSVM.

References

- MPEG-1, ISO/IEC JTC 1, “Coding of moving pictures and associated audio for digital storage media at up to about 1.5 Mbit/s – Part 2: Video,” ISO/IEC 11172 (2), Nov. 1993.

- MPEG-2, ITU-T and ISO/IEC JTC 1, “Generic coding of moving pictures and associated audio information – Part 2: Video,” ITU-T Rec. H.262 and ISO/IEC 13818-2 (MPEG-2), Nov. 1994 (with several subsequent amendments and corrigenda).

- MPEG-4, ISO/IEC JTC 1, “Coding of audio-visual objects – Part 2: Visual,” ISO/IEC 14496-2 (MPEG-4 Part 2), Jan. 1999 (with several subsequent amendments and corrigenda).

- T. Wiegand, “Draft ITU-T Recommendation and Final Draft International Standard of Joint Video Specification (ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC)”, Mar 31, 2003.

- IS0/IEC JTC1/SC29/WG11 N3908, "MPEG-4 Video Verification Model Version 18.0," Jan. 2001.

- ISO/IEC JTC1/SC29/WG11 and ITU-T SG16, “Scalable Video Coding – Working Draft 1”, JVT-N020, 14th Meeting: Hong Kong, January, 2005

- ISO/IEC JTC1/SC29/WG11 and ITU-T SG16, “Joint Scalable Video Model JSVM 0,” JVT-N021, 14th Meeting: Hong Kong, January, 2005

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Thomson Beijing |  |  |
| Mailing address | Thomson Broadband R&D (Beijing) Co., Ltd. 8/F, Bld. A, Technology Fortune Center, 8 Xue Qing Road, Hai Dian District, Beijing 100085 |  |  |
| Country | China |  |  |
| Contact person | Chen Ying |  |  |
| Telephone | 0086-82730866-8114 |  |  |
| Fax | 0086-82730806 |  |  |
| Email |  |  |  |
| Place and date of submission | Beijing, April, 13, 2005 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | New downsampling and upsampling processes for chroma samples in SVC spatial scalability |  |
| Contribution number |  | JVT-O077 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| x |  |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: