| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 16th Meeting: Busan, Korea, April, 2005 | Document: JVT-O304 Filename: input.doc |
| --- | --- |

| Title: | CE4: Inter layer motion prediction |  |  |
| --- | --- | --- | --- |
| Status: | Output Document Approved by JVT |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Kyohyuk Lee Digital Media R&D Center, Samsung Electronics Co., Ltd., MaetanDong, YeongtongGu, Suwon, Korea | Tel: Email: | +82 31 277 8675 kyohyuk.lee@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Inter layer motion prediction

Coordinator: Kyohyuk Lee (Samsung)

| Proponents | Partner | Contact | Email |
| --- | --- | --- | --- |
| Samsung LG | NEC | Kyohyuk Lee Doe-Hyun Yoon | kyohyuk.lee@samsung.com dhyoon@lge.com |

## Functionality addressed

Improve the motion coding efficiency of the temporally enhanced picture.

## Description of tools

At the Busan meeting, an input on the motion prediction of temporally enhanced picture was contributed (JVT-O058).

In the current version of the JSVM, the motion prediction scheme is integrated for motion coding efficiency of the picture which has corresponding base layer picture. But the motion coding of temporally enhanced picture is processed independently. We can use temporal direct mode (TDM) for coding of the motion of temporally enhanced picture. Further more TDM can be used as motion predictor. We will provide motion prediction scheme with TDM predictor to temporally enhanced picture as like the motion prediction of a picture with base layer picture. The object of this CE is to improve the motion coding efficiency of temporally enhanced picture by motion prediction with TDM predictor.

### Anchor: TDM in open loop MCTF structure

The purpose of tool 0 is to make TDM available within MCTF structure.

JSVM 1.0 has open loop MCTF structure of enhancement layer. So, for enabling TDM within open loop MCTF, we will separate MCTF analysis into 2 stages similar to H.264 analysis in JSVM. First stage is the motion estimation without decomposition. Second stage is decomposition with pre-estimated motion in stage 1. This separation enables us to use TDM within open loop MCTF structure. We apply TDM to temporally enhanced picture.

In this 2 stage process, the combination of motion and update step is not optimal in view point of RD because the pre-estimated motion in stage 1 is not optimal with the update step. To overcome this draw back, some kind of closed loop type motion/update step combination should be studied. But the study on the closed loop processing is outside range of this CE. So, the anchor performance will come from 2 stage MCTF analysis with motion/update step mismatch.

### TOOL 1: Motion prediction from TDM with current layer motion

JSVM 1.0 supports the motion prediction scheme for motion coding of the picture with corresponding base layer picture. TDM of tool 0 is used as motion predictor of temporally enhanced picture. Motion prediction scheme is applied to the temporally enhanced picture with the motion predictor from TDM. This tool still has draw back of motion/update step mismatch in tool 0.

### TOOL 2: Motion prediction from TDM using base layer motion

In the multi-layer structure with MCTF, motion of TDM can be driven from the base layer motion for temporally enhanced picture under the consideration of spatial resolution difference. Base layer motion comes from the picture which is the nearest of the temporally enhanced picture. This tool does not require 2 stage process of MCTF analysis. So, there is no mismatch draw back of tool 1. This tool comes from JVT-O058

## Syntax and semantics modifications

- No additional syntax is required.

- Several semantics relevant to TDM and motion prediction should be changed.

- Several decoding processes relevant to TDM and motion prediction should be added.

## Expected JSVM modifications

For the test of tool 1, MCTF analysis process will be separated by 2 stages.

- Make TDM enabled in open loop MCTF structure.

- Motion prediction with TDM.

## Expected gains

We expect performance improvement relevant to motion coding of temporally enhanced picture.

## Core experiment conditions

#### Coding conditions

Add proposed feature to the updated JSVM software after 72nd MPEG meeting. Compare PSNRs with and without the feature.

#### Test set 1

Adjusted Munich configuration from RWTH (will be released within 2 weeks from Busan)

#### Test set 2

Adjusted Palma configuration from RWTH (will be released within 2 weeks from Busan)

#### Test set 3

Slight changes to test set 1. (Changes are indicated in yellow)

|  | QCIF@7.5Hz | QCIF@15Hz | CIF@15Hz | CIF@30Hz | 4CIF@30Hz | 4CIF@60Hz |
| --- | --- | --- | --- | --- | --- | --- |
| BUS | 64 | 96 | 192 384 | 512 | X | X |
| FOOTBALL | 128 | 192 | 384 512 | 1024 | X | X |
| FOREMAN | 32 | 48 | 96 192 | 256 | X | X |
| MOBILE | 48 | 64 | 128 256 | 384 | X | X |
| CITY | 64 | 128 | 256 | 512 | 1024 | 2048 |
| CREW | 96 | 192 | 384 | 750 | 1500 | 3000 |
| HARBOUR | 96 | 192 | 384 | 750 | 1500 | 3000 |
| SOCCER | 96 | 192 | 384 | 750 | 1500 | 3000 |

#### Evaluation criteria

All test set will provide the results with and without update step. Measure impact on bitrate/PSNR using provided data.

## Time lines

- Poznan -1W: Submissions of results and verifications.

- Poznan:         Report of the coordinator on the consistency of the submissions.