# Core Experiments on improved quantization for SVC (CE 9)

## Coordinator: France Telecom Division R&D

Contact: stephane.pateux@francetelecom.com

## List of participants

| Company | Name |  | Contact |  | Cross-check partner |  |
| --- | --- | --- | --- | --- | --- | --- |
| France Telecom Division R&D | Stephane Pateux |  | stephane.pateux@francetelecom.com |  | INRIA |  |
| Panasonic | Thomas Wedi |  | Thomas.Wedi@eu.panasonic.com |  | HHI |  |
| HHI | Heiko Schwarz |  | heiko.schwarz@hhi.fraunhofer.de |  |  |  |
| Sharp | Shijun Sun |  | ssun@sharplabs.com |  |  |  |
| Microsoft | Gary Sullivan |  | garysull@microsoft.com |  |  |  |
| Siemens | Peter Amon |  | p.amon@siemens.com |  | RWTH (Thomas Rusert rusert@ient.rwth.aachen.de) |  |
| Nokia | Yiliang Bao |  | Yiliang.Bao@nokia.com |  |  |  |

Any organization willing to participate to this CE is welcomed and should signal its intention to participate to the coordinator of the CE and on the reflector.

## Functionality addressed

This CE addresses the improvement of the performance of the quantization used in JSVM (FGS quantization as well as Base Layer quantization used in enhancement layers).

## Description

The aim of this CE is to evaluate quantization improvements for SVC.

In Busan meeting there have been some inputs on improving quantization used in AVC and in SVC (JVT-O066, JVT-O069, JVT-O060, JVT-O046).

Proposals JVT-O066, JVT-O069 and JVT-O060 propose to use a uniform threshold quantization with some deadzone size to tune the deadzone quantization parameter, and to introduce some offset for the reconstructed values. JVT-O066 presented results on AVC with possible extension to SVC progressive quantization. While JVT-O060 and JVT-O069 were proposing such techniques in the context of SVC.

Proposal JVT-O046 proposes a new progressive quantization technique for SVC based on a tree-binarization of the index of a fine quantization performed on the residue relative to a base layer.

## Tests condition

A common set of test conditions is defined in order to evaluate the relative performance of the proposed tools. Rate-distortion tables and curves should be provided for generated results.

Cross checking of the proposal results is needed in order to validate these results.

For SVC, testing conditions defined by the break-out group of Busan on SVC test conditions and RD performance will be used (see [8]). Configuration files will be the one provided by this break out group. Results will have to be provided for all sequences:

- Bus, Foreman, Football, Mobile for scenario 2 (CIF sequences)

- City, Crew, Harbour, Soccer for scenario 1 (4CIF sequences)

MGS and CGS performance will be assessed for all test conditions (i.e. including mono-resolution testing conditions).

SNR scalability scenario (see section 2.1 of [8]) and combined scalability scenario (see section 2.3 of [8]) will be considered for PSNR evaluation. The following table recalls the targeted bit-rates for the highest temporal resolution for each spatial layer.

| Sequence | Format | Bit rates (kbit/sec) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bus | QCIF 15Hz | 96 | 112 | 128 | 160 | 192 |
|  | CIF 30Hz | 384 | 448 | 512 | 640 | 768 |
| Football | QCIF 15Hz | 192 | 224 | 256 | 320 | 384 |
|  | CIF 30Hz | 768 | 896 | 1024 | 1280 | 1536 |
| Foreman | QCIF 15Hz | 48 | 56 | 64 | 80 | 96 |
|  | CIF 30Hz | 192 | 224 | 256 | 320 | 384 |
| Mobile | QCIF 15Hz | 64 | 80 | 96 | 112 | 128 |
|  | CIF 30Hz | 256 | 320 | 384 | 448 | 512 |
| City | QCIF 15Hz | 64 | 80 | 96 | 112 | 128 |
|  | CIF 30Hz | 256 | 320 | 384 | 448 | 512 |
|  | 4CIF 60Hz | 1024 | 1280 | 1536 | 1792 | 2048 |
| Crew, Harbour, Soccer | QCIF 15Hz | 96 | 112 | 128 | 160 | 192 |
|  | CIF 30Hz | 384 | 448 | 512 | 640 | 768 |
|  | 4CIF 60Hz | 1536 | 1780 | 2048 | 2560 | 3072 |

Further contents with Film Grain may optionally be considered. These additional sequences are optional sequences for the evaluation tests and are not mandatory for all participants. For these additional sequences viewing evaluation will also be performed.

These sequences are:

- Pouring Liquids, Oil on Fish, Rolling Tomatoes

These sequences are already used inside JVT for the FRExt standardization (See JVT-J-042 [7] for how to retrieve these sequences).

Following bit-rates will be considered for these additional sequences.

| Sequences | Format | Bit rates (Mbit/sec) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Pouring Liquids, Oil on Fish, RollingTomatoes | 1080p | 6 | 6 | 8 | 12 | 16 |

### Software

The latest JSVM 2.0 software available on the CVS server will be used by all the participants to this CE.

## Timeline

| Busan + 3 weeks | Finalization of CE description (on the reflector). |
| --- | --- |
| Poznan – 4 weeks | deadline for joining CE. List of participant will be fixed at that time. |
| Poznan – 1 week | Upload of contribution |

## References

[1]	ITU-T and ISO/IEC JTC1, “Scalable Video Coding - Working Draft 1”, JVT-N020, Jan 2005

[2]	"One-page at 12-point font on scalar quantization", G. Sullivan, JVT-O014, Apr. 2005

[3]	"Improved FGS for JSVM", V. Bottreau , JVT-O046, Apr. 2005

[4]	"SNR scalability for JSVM", I. Amonou, N. Cammas, S. Kervadec, S. Pateux , JVT-O060, Apr. 2005

[5]	"R-D estimation of quant offset params", T. Wedi and S. Wittman , JVT-O066, Apr. 2005

[6]	"Adaptive Quantization Offset", P. Amon, M. Kapralov , JVT-O069, Apr. 2005

[7]	"Film Originated Test Sequences", T. McMahon, JVT-J042, Dec. 2003

[8]	"AhG on Testing Conditions for Coding Efficiency and JSVM Performance Evaluation", M. Wien, H. Schwarz, JVT-Pxxx, July 2005