# CE12. Weighted prediction for FGS layer coding of close-loop P frames

Coordinator: Yiliang Bao

| Proponent | Partner | Contact | Email |
| --- | --- | --- | --- |
| Nokia | HHI | Yiliang Bao | yiliang.bao@nokia.com |

## Functionality addressed

In low-delay application, it is still important to encode the sequence in close-loop P frames. Currently the FGS layer of a close-loop P frame is encoded without temporal prediction, and the performance is not acceptable.

In this core experiment, the techniques for improving the FGS coding performance of the close-loop P frame will be tested.

JSVM1.0 also has the option of encoding the sequence in groups of pictures (GOP) with anchor frames as close-loop P frames and frames inside GOPs as open-loop P frames. If QP values are chosen to maximize the average RD performance, the large frame size variation makes such a coding structure not suitable for low-delay applications. But once the frame-level rate control is performed to smooth out the frame size variation, this coding structure does not give very good FGS performance either.

## Description of Approach

In JVT-O054, Nokia proposed to incorporate weighted temporal prediction in encoding FGS layer of close-loop P frames. More details can be found in the contribution JVT-O054 [1].

Preliminary results were presented and the effectiveness of the new techniques was clearly demonstrated.

In this core experiment, more tests on the technology will be performed. Further improvements will be explored.

## JSVM modifications

Minor syntax change to the slice header of the FGS slice is needed to signal the weighting factors used in generating the reference block. The decoding process will also be changed to incorporate the prediction in coding of the FGS layer of a close-loop P frame.

## Testing conditions

In the first set of tests, for the base layer the sequences will be encoded only in close-loop P frames except for the first frame that will be encoded as an intra frame. One FGS layer will be encoded on top of the base layer. The PSNR data and bit rates of bitstreams extracted at 10% rate increments will be presented. Following are the sequences and the base layer bit rates to be used in the tests.

| Sequence | Format | Base bit rate (kbits/s) |
| --- | --- | --- |
| Crew, Harbour, Soccer | CIF 30Hz | Base 256 kbps |
|  | 4CIF 30Hz | Base 1024 kbps |
| City | CIF 30Hz | Base 256 kbps |
|  | 4CIF 30Hz | Base 768 kbps |
| Football | QCIF 15Hz | Base 192 kbps |
|  | CIF 30Hz | Base 512 kbps |
| Mobile, Bus, Foreman | QCIF 15Hz | Base 64 kbps |
|  | CIF 30Hz | Base 256 kbps |

In additional to those tests, the coding performance of the modified JSVM2.0 will be compared with that of the unmodified JSVM2.0 at adjusted Palma test points.

## Evaluation criteria

Significant coding performance improvement compared to JSVM2.0 for FGS layer of closed loop P frame.

## Timelines

Busan  + 4W	: Provide formal description of the proposal.

Poznan - 1W	: Upload results and verification.

Poznan		: Report on the CE results.

## Reference

- Yiliang Bao, Marta Karczewicz, Justin Ridge, Xianglin Wang, JVT-O054, “Improvements to Fine Granularity Scalability for Low-Delay Applications”, April 2005, Busan, Korea.