| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 16th Meeting: Poznań, PL, 24-29 July, 2005 | Document: JVT-Pxxx Filename: input.doc |
| --- | --- |

| Title: | Meeting Report, Draft 1 |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Name Address line 1 Address line 2 Address line 3 | Tel: Email: |  |
| Source: | [Name of sourcing organization] |  |  |

_____________________________

# Abstract

The Joint Video Team (JVT) of ITU-T Q.6/16 and ISO/IEC JTC 1/SC 29/WG 11 held its 16th meeting during 24-29 July 2005 in Poznan, Poland. The JVT meeting was held under the chairmanship of Dr. Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany), and associate chairmanship of Dr. Thomas Wiegand (HHI/Germany) and Dr. Ajay Luthra (Motorola/USA). The JVT meetings opened at approximately 9:30 am on Sunday 24 July 2005 and closed at ??.?? pm on Friday 29 July 2005.  Approximately ??? people attended the JVT meetings (as recorded on a sign-in sheet passed at the meeting).  The meetings took place in a co-located fashion with a meeting of ISO/IEC JTC 1/SC 29/WG 11 (MPEG).  The subject matter of these activities consisted of work on video coding.

# Documents of the JVT meeting

## Pre-registered documents

JVT-P000 List of documents of Poznan meeting

JVT-P001 [G. J. Sullivan, J.-R. Ohm, A. Luthra, T. Wiegand] AHG Report: Proj mgmt and errata

JVT-P002 [T. Wiegand, K. Suehring, A. Tourapis, K. P. Lim] AHG Report: JM text and ref soft

JVT-P003 [T. Suzuki, L. Winger] AHG Report: Bitstreams & conformance

JVT-P004 [J. Ridge, U. Benzler] AHG Report: SVC core experiments

JVT-P005 [G. Cook] AHG Report: JSVM s/W and new func. integ.

JVT-P006 [J. Reichel, H. Schwarz, M. Wien] AHG Report: JSVM & WD text

JVT-P007 [G. J. Sullivan] AHG Report: Spatial scalability resampling

JVT-P008 [Y.-K. Wang] AHG Report: Err resil test cond's & apps

JVT-P009 [M. Wien, H. Schwarz] AHG Report: Coding eff & JSVM perf test cond

JVT-P010 [T. Suzuki] AHG Report: Study of 4:4:4 functionality

JVT-P011 [G. J. Sullivan, J.-R. Ohm, A. Luthra, T. Wiegand] Meeting report of Busan meeting

JVT-P012 [S. Sun] CE10Pt1: Direct interpolation for upsampling

JVT-P013 [S. Sun] CE02Pt2: Deblocking filter for I_BL Spatial SVC

JVT-P014 [H. Schwarz, D. Marpe, T. Wiegand] Hierarchical B Pictures

JVT-P015 [G. J. Sullivan, J. Lou, M.-T. Sun] Adaptive quantization yielding 10 dB gain

JVT-P016 [Y.-L. Lee] Lossless intra coding for improved 4:4:4

JVT-P017 [H. Yu, L. Liu] Advanced 4:4:4 profile

JVT-P018 [E. Francois, J. Vieron, S. Sun] CE10: Report of results

JVT-P019 [J. Vieron, E. Francois] CE10: Unified motion upsampling in extended spat scal

JVT-P020 [J. Vieron] CE10Pr1: Cross-verif of JVT-P012 direct upsamp

JVT-P021 [X. Yi, J. Zhang, N. Ling, W. Shang] Improved & simplified fast ME for JM

JVT-P022 [V. Bottreau] CE09: Cross-verif of FT JVT-P030 on SVC quantization

JVT-P023 [V. Bottreau] CABAC variant and options for bit error resilience

JVT-P024 [T. Suzuki] Editor's proposed draft FRExt conformance

JVT-P025 [T. Suzuki] Prop list of changes of AVC conformance

JVT-P026 [A. M. Tourapis, H.-Y. Cheong, P. Topiwala] Fast ME in the JM reference software

JVT-P027 [V. Bottreau] Prop. on FGS bitplane coding

JVT-P028 [J. Reichel, D. Santa Cruz, F. Ziliani] CE01: Improved residual picture buffer management

JVT-P029 [I. Amonou, N. Cammas, S. Kervadec, S. Pateux] (M12304) CE05: CE report on quality layers

JVT-P030 [I. Amonou, N. Cammas, S. Kervadec, S. Pateux] (M12305) CE09: CE report on improved quantization

JVT-P031 [I. Amonou, N. Cammas, S. Kervadec, S. Pateux] (M12306) Syntax for FGS pass fractioning

JVT-P032 [I. Amonou, N. Cammas, S. Kervadec, S. Pateux] (M12307) On high-level syntax for SVC

JVT-P033 [I. Amonou, N. Cammas, S. Kervadec, S. Pateux] (M12308) Withdrawn

JVT-P034 [I. Amonou, N. Cammas, S. Kervadec, S. Pateux] (M12309) CE02: Cross-check of Aachen prop on deblocking QQ

JVT-P035 [M.-H. Lee] CE01: Verification of VisioWave proposal for CE1 QQ

JVT-P036 [M.-H. Lee] CE06: Verification of MSRA proposal JVT-P047

JVT-P037 [M.-H. Lee] CE01: Reduced Memory MCTF

JVT-P038 [M.-H. Lee] CE01: Picture Duplication Process

JVT-P039 [M.-H. Lee] Withdrawn

JVT-P040 [M.-H. Lee] Progressive MCTF

JVT-P041 [L. Xiong] Reducing enh.-layer directional intra pred modes

JVT-P042 [T.-C. Thang, T.-M. Bae, Y.-J. Jung, Y.-M. Ro, J.-W. Kang, H.-C. Choi, J.-G. Kim, J.-W. Hong] CE08: Spatial scalability of multiple ROIs

JVT-P043 [T.-M. Bae, T.-C. Thang, D.-Y. Kim, Y.-M. Ro, J.-W. Kang, J.-G. Kim, J.-W. Hong] FMO implementation in JSVM

JVT-P044 [H.-C. Choi] Embedded quant. for improved SNR scalability

JVT-P045 [I.-H. Shin, H.-W. Park] Adapt. upsamp. for spat. scal.

JVT-P046 [J.-Z. Xu] Alternative to FGS quant. method of JVT-O054.

JVT-P047 [J.-Z. Xu] CE06: Proposed approach to CE06

JVT-P048 [W.-S. Kim+] Further experimental results on 4:4:4 coding

JVT-P049 [W.-S. Kim+] Study on advanced 4:4:4 coding

JVT-P050 [J. Jia] Polyphase downsamp.-based redundant slices

JVT-P051 [J. Jia] Imp. coding eff. by polyphase downsampling

JVT-P052 [X. Wang, Y. Bao, M. Karczewicz, J. Ridge] CE06: Simplified update step operation for MCTF

JVT-P053 [Y. Bao, M. Karczewicz, X. Wang, J. Ridge] CE09: Improved quantization for FGS coding

JVT-P054 [J. Ridge, M. Karczewicz, Y. Bao, X. Wang] CE11: Core experiment results for FGS reordering

JVT-P055 [Y. Bao, M. Karczewicz, X. Wang, J. Ridge] CE12: Weighted pred. FGS coding closed-loop P frames

JVT-P056 [J. Ridge, M. Karczewicz, Y. Bao, X. Wang] Variable length codes for SVC

JVT-P057 [X. Wang, Y. Bao, M. Karczewicz, J. Ridge] Implementation of close-loop coding in JSVM

JVT-P058 [Y. Bao, M. Karczewicz, X. Wang, J. Ridge] Withdrawn

JVT-P059 [H. Schwarz, D. Marpe, T. Wiegand] Comparison of MCTF and closed-loop hierarch. B pics.

JVT-P060 [X. Wang] CE06: Verif. of Samsung prop.

JVT-P061 [Y.-K. Wang, M. M. Hannuksela] Signaling of scalab. info.

JVT-P062 [Y.-K. Wang, Y.-L. Bao] Indication of non-required pics.

JVT-P063 [P. Yin, J. Boyce, P. Pandit] CE07: Proposal for CE07 enh. layer intra pred.

JVT-P064 [P. Yin, J. Boyce, P. Pandit] Weighted prediction for SVC

JVT-P065 [P. Yin, J. Boyce, P. Pandit] Weighted prediction for SVC MCTF update step

JVT-P066 [P. Pandit] CE07: Verif. Sharp proposal JVT-O010

JVT-P067 [W. Choi, J. Yang, B. Jeon] CE03: CE report coding eff. improvement of CABAC

JVT-P068 [W. Choi, J. Yang, B. Jeon] CE03: Verif. Samsung prop. QQ

JVT-P069 [A. Segall, S. Lei] CE07: Prop. for adaptive upsamp. spatial scalab.

JVT-P070 [A. Segall] CE07: Verif. of JVT-P063 enh. layer intra pred.

JVT-P071 [A. Segall] Verif. of JVT-P064 weighted pred.

JVT-P072 [S. Bandyopadhyay, Z. Wu, P. Pandit, J. Boyce] Frame loss concealment

JVT-P073 [withdrawn] Withdrawn

JVT-P074 [S.-W. Park, J.-H. Park, D.-H. Yoon, B.-M. Jeon] CE10: Verif. of Thomson proposal JVT-O041/O042

JVT-P075 [K.-H. Lee] CE04: Report of results of CE04

JVT-P076 [K.-H. Lee] Multi-layer weighted prediction

JVT-P077 [D.H. Yoon, B.-Y. Jeon, J.-H. Park, S.-W. Park] Complexity reduction by simplified chroma update step

JVT-P078 [D.H. Yoon, B.-Y. Jeon, J.-H. Park, S.-W. Park] Clarification of intra_base_flag

JVT-P079 [D.H. Yoon, B.-Y. Jeon, J.-H. Park, S.-W. Park] Key picture indication in AVC compatible base layer

JVT-P080 [S. Jeong, M. Park, G. Park, K. Kim] Flag bit in SEI for adaptive GOP structure

JVT-P081 [T. Kimoto] CE04: Verification of QQ

JVT-P082 [T. Kimoto] Multi-loop decoding for unrestricted temporal decomp.

JVT-P083 [Y. Yamada, S. Sekiguchi, Y. Isu, K. Asai, T. Murakami] Consideration on intra coding eff. of High 4:4:4 pr. (not Sunday afternoon)

JVT-P084 [W.-J. Han] CE06: Core experiment report on update step

JVT-P085 [W.-J. Han] Smoothed ref. pred. for single-loop decoding

JVT-P086 [W.-J. Han] Distribution-based inverse quantization

JVT-P087 [W.-J. Han] PFGS with bilinear interpolation filter

JVT-P088 [T. Wedi, S. Kondo, H. Saito] 4:4:4 Intra-only coding

JVT-P089 [B.-K. Lee] CE03: Report on symbol pred. of resid. pred. flag

JVT-P090 [B.-K. Lee] Adaptive FGS process

JVT-P091 [B.-K. Lee] CE03: Verification of SKKU proposal JVT-P067

JVT-P092 [M. Boltz, U. Benzler] Verif of HHI results JVT-P059 on closed-loop decode

JVT-P093 [M. Beermann, K. Hanke, M. Wien] CE02: Deblock. Filt. Adaptation for open-loop coding

JVT-P094 [M. Beermann, M. Wien] CE02: Cross-check of Sharp JVT-P013 on deblocking

JVT-P095 [T. Rusert, M. Wien] Locally adaptive quantiz.: concept and first results

JVT-P096 [W.-J. Han] CE06: Verif of Nokia proposal JVT-P052

JVT-P097 [G. H. Park, K. I. Lee] Advanced FMO for ROI in SVC

JVT-P098 [J. Reichel, D. Santa Cruz]  Verification of Panasonic contribution and encoder improvement

JVT-P099 [S. Cha] Withdrawn

JVT-P100 [P. Amon, A. Hutter, T. Rathgen] High-level syntax for SVC

JVT-P101 [P. Amon] Withdrawn

JVT-P102 [H. Yu] Artifacts seen in some Viper 4:4:4 sequences

JVT-P103 [A. Rodriguez] Sample aspect ratio support

JVT-P104 [J.-H. Park, D.H. Yoon, S.-W. Park, B.-M. Jeon] CABAC context model for base_mode_flag

JVT-P105 [J. Reichel, D. Santa Cruz, F. Ziliani] Is there really a need for a normative update process ?

JVT-P106 [Z. Lu] CE8 report: Perceptual Region-of-interest (ROI) based Scalable Video Coding

JVT-P107 [Z. Lu] CE8: Cross-check of Spatial scalability of multiple ROIs

JVT-P108 [J. Ridge] Verification of CE5 results for France Telecom

JVT-P109 [M. Zhou] Verification of JVT-P053

## Major output documents

(Dates listed are planned dates of availability.  Note that document JVT-O001 should also be considered output documents.)

JVT-P201/N???? Working Draft 3 of Scalable Video Coding [??/??/??]

JVT-P202/N???? Joint Scalable Video Model (JSVM) 3 [??/??/??]

JVT-P203/N???? JSVM 3 Software [??/??/??]

## SVC core experiment output documents

JVT-P301 – JVT-P3??

# JVT Administrative topics

## Administrative documents

##### JVT-P000 List of documents of Poznan meeting

##### JVT-P001 [G. J. Sullivan, J.-R. Ohm, A. Luthra, T. Wiegand] AHG Report: Proj mgmt and errata

##### JVT-P002 [T. Wiegand, K. Suehring, A. Tourapis, K. P. Lim] AHG Report: JM text and ref soft

##### JVT-P003 [T. Suzuki, L. Winger] AHG Report: Bitstreams & conformance

##### JVT-P004 [J. Ridge, U. Benzler] AHG Report: SVC core experiments

##### JVT-P005 [G. Cook] AHG Report: JSVM s/W and new func. integ.

##### JVT-P006 [J. Reichel, H. Schwarz, M. Wien] AHG Report: JSVM & WD text

New procedure: Use Annex S in JSVM for all normative elements. This can be copied into the WD right away.

##### JVT-P007 [G. J. Sullivan] AHG Report: Spatial scalability resampling

##### JVT-P008 [Y.-K. Wang] AHG Report: Err resil test cond's & apps

##### JVT-P009 [M. Wien, H. Schwarz] AHG Report: Coding eff & JSVM perf test cond

Detailed Results provided for JSVM2 single layer, spatial scal., SNR scal. and combined scalability under new testing rate points as proposed earlier on the reflector. Group agrees to adopt these testing conditions for future benchmarking of SVC development and core experiments.

##### JVT-P010 [T. Suzuki] AHG Report: Study of 4:4:4 functionality

##### JVT-P011 [G. J. Sullivan, J.-R. Ohm, A. Luthra, T. Wiegand] Meeting report of Busan meeting

## IPR policy reminder

Participants were reminded of the IPR policies established by the parent organizations of the JVT and were referred to the parent body web sites for further information.

Participants were particularly reminded of the need to supply a completed JVT IPR status reporting form in all technical proposals for normative standardization.

## Late documents

No objections / objections were voiced to the consideration of the late documents.  Documents listed in this report in italics and with a "-L" suffix to their document numbers were classified as late.

##### JVT-P017-L [H. Yu, L. Liu] Advanced 4:4:4 profile

##### JVT-P022-L [V. Bottreau] CE09: Cross-verif of FT JVT-P030 on SVC quantization

##### JVT-P024-L [T. Suzuki] Editor's proposed draft FRExt conformance

##### JVT-P027-L [V. Bottreau] Prop. on FGS bitplane coding

##### JVT-P033-L [I. Amonou, N. Cammas, S. Kervadec, S. Pateux] (M12308) CE05: Cross-check of Nokia prop on quality layers QQ

##### JVT-P039-L [M.-H. Lee] Issue with temporal scalability

##### JVT-P046-L [J.-Z. Xu] Alternative to FGS quant. method of JVT-O054.

##### JVT-P048-L [W.-S. Kim+] Further experimental results on 4:4:4 coding

##### JVT-P049-L [W.-S. Kim+] Study on advanced 4:4:4 coding

##### JVT-P053-L [Y. Bao, M. Karczewicz, X. Wang, J. Ridge] CE09: Improved quantization for FGS coding

##### JVT-P054-L [J. Ridge, M. Karczewicz, Y. Bao, X. Wang] CE11: Core experiment results for FGS reordering

##### JVT-P055-L [Y. Bao, M. Karczewicz, X. Wang, J. Ridge] CE12: Weighted pred. FGS coding closed-loop P frames

##### JVT-P056-L [J. Ridge, M. Karczewicz, Y. Bao, X. Wang] Variable length codes for SVC

##### JVT-P057-L [X. Wang, Y. Bao, M. Karczewicz, J. Ridge] Implementation of close-loop coding in JSVM

##### JVT-P058-L [Y. Bao, M. Karczewicz, X. Wang, J. Ridge] Entropy coding for Fine Granularity SNR Scalability

##### JVT-P063-L [P. Yin, J. Boyce, P. Pandit] CE07: Proposal for CE07 enh. layer intra pred.

##### JVT-P067-L [W. Choi, J. Yang, B. Jeon] CE03: CE report coding eff. improvement of CABAC

##### JVT-P070-L [A. Segall] CE07: Verif. of JVT-P063 enh. layer intra pred.

##### JVT-P099-L [S. Cha] CE07: Enhancement-layer intra prediction

##### JVT-P101-L [P. Amon] CE09: Result report

##### JVT-P102-L [H. Yu] Artifacts seen in some Viper 4:4:4 sequences

##### JVT-P105-L [J. Reichel, D. Santa Cruz, F. Ziliani] Is there really a need for a normative update process ?

##### JVT-P106-L [Z. Lu] CE8 report: Perceptual Region-of-interest (ROI) based Scalable Video Coding

##### JVT-P107-L [Z. Lu] CE8: Cross-check of Spatial scalability of multiple ROIs

##### JVT-P108-L [J. Ridge] Verification of CE5 results for France Telecom

# JVT Scalable video coding work

## CE1 & related docs: Memory management for MCTF process

##### JVT-P028 [J. Reichel, D. Santa Cruz, F. Ziliani] CE01: Improved residual picture buffer management

The special case where a single residual buffer is used is investigated. The decoding update process is modified as follows: Each update process using N reference pictures is split into N update processes using only a single input picture each. This algorithm is called the split-update. Using the algorithm reduces the decoding latency, and simplifies the decoding process. This contribution shows that this algorithm can be introduced into the JSVM without any degradation of the compression performances.

MCTF decoding requires storage of residual pictures until the inverse prediction step, number cannot be controlled by num_ref_frames, higher precision required. Solutions: Constraints on MCTF structure (proposed in JVT-O0009) or decode with fixed number of residual picture buffer as proposed here. Modification of the encoder could be done, but is not necessary (in some cases slight improvements, in some cases slight degradations as compared to JSVM, with 1%, when encoder is unchanged). Difference most probably caused by different effect of adaptive update.

##### JVT-P035 [M.-H. Lee] CE01: Verification of VisioWave proposal for CE1 QQ

Verification of JVT-P028. The binaries are compiled and built from the modified software provided through JVT-P028 and the results are generated using the same set of configurations provided through JVT-P028. The results obtained are similar to JVT-P028 results.

Cross-check 100% when run on same platform. Most probably a memory initialisation problem still existing in JSVM2 software.

##### JVT-P037 [M.-H. Lee] CE01: Reduced Memory MCTF

An approach called 8Bit_Separate is described to reduce memory requirement of storing a high-pass residual picture. It uses the MSB 8-bits of decoded residual of a high-pass picture for modified 8-bits inverse update operation. An alternative solution called 8Bit_Saturate is evaluated too. It uses the saturated 8-bits of decoded residual of a high-pass picture for inverse update operation by saturating decoded residual to 8-bits (-127 to 128).

Complexity reduction, minor loss occuring in some cases.

##### JVT-P038 [M.-H. Lee] CE01: Picture Duplication Process

For the current Joint Scalable Video Model, picture duplication is required by the decoder for the lowpass picture of the current GOP. To ensure conformance between encoded bitstream and a decoder having a finite amount of memory, the memory used by the duplicated picture has to be controlled, from the instance it is created, used and finally removed. The current methods used by the SVC WD and the JSVM software for handling the duplicated picture are examined. Some issues are caused by the original buffer and the duplicated picture buffer having the same index. It is proposed that the duplicated picture is assigned a long-term index, to differentiate it from the original picture. This is done using an MMCO command proposed previously, with an addition of marking the duplicated picture as “used for long-term reference” and assigned a long term index.

Problem in buffer management with SNR scalability (different decoding processes for key pictures) is recognized, but doubts are raised whether the proposed solution is general enough. Get together with the editors to discuss solution.

##### JVT-P098 [J. Reichel, D. Santa Cruz] CE01: Verif of Panasonic proposals

Verification of JVT-P037 contribution to CE1 on “Memory Management for MCTF process”. Proposes minor encoder modifications to avoid artifacts in the decoded pictures. JVT-P037 proposes two solutions to reduce the precision of the buffers used to store inter MBs to 8 bits: split the storage into two buffers or simply saturate (i.e. clip) the residuals to 8 bits. The fist proposal has been fully verified using source code and description provided by Panasonic. The second proposal is claimed to have the potential to create visible artifacts in the decoded pictures. In addition to verifying the results in JVT-P037, an encoder modification is proposed that completely avoids such artifacts with a small degradation of the rate-distortion performance. The proposed modification consists in constraining the mode decision process so as to forbid inter modes for the current macroblock that would lead to saturation of the residual signal at the decoder.

Verifies JVT-P037 and modifies the encoder by constrained mode decision such that artifacts are avoided. Loss in RD performance is up to 0.16 db (Mobile), 0.12 (Bus), negligible in other sequences. Encoder is becoming more complex.

##### JVT-P040 [M.-H. Lee] Progressive MCTF

An approach called progressive motion-compensated temporal filtering is described. It modifies the MCTF structure such that MCTF can be performed in a progressive and straightforward manner. It affects the order of processing the prediction and update steps and the choice of reference pictures for each step. The prediction and update steps are interleaved such that the low-pass pictures are partially updated immediately with each high-pass picture in a progressive manner and the partially updated low-pass pictures are used for subsequent prediction steps. The progressive MCTF is claimed to simplify the inverse MCTF process, eliminate irregular complexity peaks, lower decoding delay and lower memory requirements, compared to the conventional MCTF in the JSVM, without any penalty on coding efficiency.

Similar to JVT-P028 (in fact, the same. Interleave prediction&update progressively instead of waiting for all residual pictures that are involved in a specific update.

##### JVT-P057-L [X. Wang, Y. Bao, M. Karczewicz, J. Ridge] Implementation of closed-loop coding in JSVM

Information on closed-loop coding implementation in JSVM1.0. While the implementation of closed-loop for discrete layer coding is trivial in JSVM, it is reported that is takes some extra efforts to optimize the performance of the codec for combined scalabilities, especially the FGS scalability.  For the results presented in this contribution, how the FGS layer of an anchor frame is coded is not changed.  The focus of this contribution will be on close-loop FGS layer coding of high-pass frames.

Comparison: Open loop (original for ME and MC at encoder), "partial" closed loop with all-FGS for MC at encoder, "true" closed loop with partial-FGS or no-FGS for MC at encoder. Also non-originals for ME in latter cases. Impact on quantizer and entropy coder should be observed. Gain up to 0.5 dB for the case of "true" closed loop vs. open loop. No change of decoder, multiple loop at encoder.

##### JVT-P059 [H. Schwarz, D. Marpe, T. Wiegand] Comparison of MCTF and closed-loop hierarch. B pics.

The coding efficiency of MCTF-based coding and a simple closed-loop coding with hierarchical B pictures for single-layer coding as well as spatial, SNR, and combined scalability. For all test scenarios, the coding efficiency with hierarchical B pictures and a closed-loop control is claimed to be higher or similar to that of MCTF-based coding for nearly all test sequences and resolutions. Only for the “City” sequence MCTF-based coding yields a coding efficiency gain of up to 0.5 dB in comparison the closed-loop coding with hierarchical B pictures. A further analysis of these gains for “City” lead to the conclusion, that the gains for the “City” are a result of the temporal noise reduction property of the MCTF. It is claimed to be shown that this noise reduction can also be realized as a pre-processing step and efficiently combined with closed-loop hierarchical B picture coding. Based on the simulation results and the fact that the update steps at the decoder side dramatically increase the complexity, it is propose to remove the update steps from the SVC design.

Results indicate that we can have a comparison benchmark that in most cases is better than MCTF open-loop design. To be revisited in the context of discussions on CE6.

JVT-P092 [M. Boltz, U. Benzler] Verif of HHI results JVT-P059 on closed-loop decode

In JVT-P059, a comparison between the JSVM MCTF coding and closed-loop coding is described. The encoded bit streams from HHI have been received and successfully decoded them using the version of the JSVM described in JVT-P059 (CVS Tag JSVM_1_2_5). The results match exactly those published in JVT-P059.

##### JVT-P105-L [J. Reichel, D. Santa Cruz, F. Ziliani] Is there really a need for a normative update process ?

It is claimed that it is possible to remove the update process from the decoding process without reducing the compression performance of the JSVM. When the update process is performed on the encoder side but not on the decoder side, the decoded video is claimed to be of nearly of the same quality as JSVM decoding with update step for most test sequences. It should be noted that also removing the update process from the encoder will, in this case, reduce the coding performances. The experiments conducted in this contribution seem to show the update process is in fact necessary only on the encoder side, and could be made non-normative.

Omitting update step at decoder gives little loss (< 0.1 db) for CIF, but up to 0.4 dB for 4CIF (City, Harbour). Omitting update at both encoder and decoder gives more critical loss. Mainly PSNR in lowpass pictures is decreased. Difference becomes larger when the bitrate increases. Needs more investigation probably also for higher rates; there seems to be some tendency that encoder_update+decoder_noupdate comes below encoder_noupdate+decoder_noupdate.

## CE 2 & related docs: Improved deblocking filter settings

##### JVT-P013 [S. Sun] CE02Pt2: Deblocking filter for I_BL Spatial SVC

A simplification of the deblocking filter for macroblocks in I_BL mode in spatial scalable video coding is proposed.  By treating the I_BL blocks as inter blocks during boundary strength decision, the deblocking filter operations for I_BL blocks are claimed to be significantly reduced (34% on average) while acceptable visual quality is claimed.

##### JVT-P094 [M. Beermann, M. Wien] CE02: Cross-check of Sharp JVT-P013 on deblocking

Verification of JVT-P013. The source of the proposal as well as the bitstreams has been received. The verification has been performed with the compiled source. The RD results as well as the filter statistics reported in JVT-P013 are verified.

##### JVT-P093 [M. Beermann, K. Hanke, M. Wien] CE02: Deblock. Filt. Adaptation for open-loop coding

The H.264|AVC deblocking filter has been designed and tuned for closed-loop application. The SVC extension is claimed to contain an open-loop structure. This fundamental difference has not yet been considered in the necessary adaptations of the deblocking filter for its use in the scalable extension. The differences that evolve from the open-loop situation is discussed. Proposed modifications to the deblocking filter are compared to a very simple adaptation of the deblocking filter through available slice-header syntax. This comparison includes rate-distortion results as well as visual coding results.

##### JVT-P034 [I. Amonou, N. Cammas, S. Kervadec, S. Pateux] (M12309) CE02: Cross-check of Aachen prop on deblocking QQ

Verification of JVT-P093.

## CE 3 & related docs: Coding efficiency improvement of entropy coding

##### JVT-P067-L [W. Choi, J. Yang, B. Jeon] CE03: CE report coding eff. improvement of CABAC

This contribution proposes the adaptive entropy coding scheme by using inter-layer relationship for coding efficiency improvement in response to CE 3. A new context modeling process of CABAC is proposed based on a syntax element of the base layer macroblock. In simulation results, the overall coding gain of the proposed scheme is claimed to be up to 1% in terms of bit saving. The coding gain of the proposed scheme is reported to be increased as the slice size is reduced. In case of mobile application which has a limitation on packet size, the proposed scheme would have a benefit on coding efficiency.

Number of new models: unknown

Size of new binarization table: 23 symbols

Did not follow new test conditions.

Contribution has been noted.

##### JVT-P091 [B.-K. Lee] CE03: Verification of SKKU proposal JVT-P067

Verification of JVT-O067. Due to the tight deadline, the verification results for spatial test condition and Munich test configuration have not been finished. All other sequences are verified and there are no problems. The details are described in the attached excel file. (JVT-P091.xls) For the unfinished sequences, the results until the Poznan meeting will be provided.

##### JVT-P089 [B.-K. Lee] CE03: Report on symbol pred. of resid. pred. flag

According to the current JSVM2 implementation, residual_pred_flag is set to one if the base-layer residual is used for predicting the current inter residual. However, if the base-layer residual in the macroblock has no non-zero pixel, the coding of residual_pred_flag is not required. Thus we encode reversed residual prediction flag instead of the residual_pred_flag itself to increase number of '0'. It provides some performance gain when the VLC is used instead of CABAC. This process can be summarized as: 1) Skip coding of residual_pred_flag if base-layer residual has no non-zero pixels, 2) Code reversed residual prediction flag = 1-resiudal_pred_flag

Bit-rate savings around 1-1.4%.

Adopt, by adding a context model that is conditioned on the CBP value of the base layer.

##### JVT-P068 [W. Choi, J. Yang, B. Jeon] CE03: Verif. Samsung prop. QQ

Verification of JVT-P089 for CE 3. Source code based on the reference software JSVM 2.0 has been provided. It has been checked that the provided source code has no problem and the coded bitstreams are also verified. The proposed scheme is tested under modified Munich test condition and coding performance is checked in terms of bitrate. To obtain the bit reduction ratio of the proposed method, tests were conducted without rate control. The layers are decided by fixed QP value not bitrate.

##### JVT-P104 [J.-H. Park, D.H. Yoon, S.-W. Park, B.-M. Jeon] CABAC context model for base_mode_flag

CABAC can be divided into three modules such as, binarizer, context modeler and binary arithmetic coder. In the current JSVM 2.0, one of the most important modules, the context modeler uses the same syntax elements of neighboring maroblocks to calculate context index increment. For the single layer predictive coding scheme, using the same syntax of the neighboring macroblocks is reasonable. However, regarding the multi-layer scheme, syntax elements of upper layer can obtain the information from the lower layer. If the macroblock of spatial enhancement layer has a base macroblock and the two macroblocks have the same temporal location, their relationship is claimed to be closer than the relationship of neighboring macroblocks. Measurements on the current context model for base_mode_flag based on Palma conditions show the actual percentages. Expanded context models for base_mode_flag incorporating the effect of the macroblock modes of base layer are proposed.

Number of new models: 3

Did not follow new test conditions.

Further work is encouraged.

## CE 4 & related docs: Inter-layer motion prediction

##### JVT-P075 [K.-H. Lee] CE04: Report of results of CE04

Extension of JVT-O058. In JSVM 2.0 the motion prediction method of the picture which has a time synchronized base layer picture at lower temporal resolution (called synchronized picture) cannot be used because the analysis step of the MCTF process estimates the motion field starting from the highest temporal level proceeding through to lower temporal levels. The method proposed in JVT-O058 estimated a motion predictor from the base layer motion for the unsynchronized picture. If the MCTF analysis process can be separated into 2 stages, motion estimation and decomposition stages, then the motion predictor of the unsynchronized picture can be derived from the lower temporal level picture. This motion predictor estimation is similar to temporal direct mode.

Check of temporal direct mode. Test with JM decoder with only 1 sequence.

Combine with closed-loop coding.

Continue CE. Specify more optimal anchors.

##### JVT-P081 [T. Kimoto] CE04: Verification of QQ

Verification of JVT-P075. Only coding a performance check was carried out by running software.

##### JVT-P082 [T. Kimoto] Multi-loop decoding for unrestricted temporal decomp.

Inter-layer MC with multi-loop decoding (MLD) to improve coding performance for SVC is proposed.

Decouples MC structure between base and enhancement layer.

Proposes multiple loop decoding and using enhancement layer MVs to create prediction signals using MC in the Base layer.

Create CE.

##### JVT-P085 [W.-J. Han] Smoothed ref. pred. for single-loop decoding

Single-loop decoding is a very useful technique to reduce the decoding complexity in the multi-layer structure. It is claimed that the restrictions of the intra-base prediction sometimes degrades the performance especially for fast-motion sequences. A new prediction mode based on the residual prediction process is proposed to simulate the intra-base prediction by adding the smoothing function to the prediction signal with consideration of the residual prediction process. A performance improvement for the sequences Football, Crew, and Soccer is claimed.

Add to CE for JVT-P082.

## CE 5 & related docs: Quality layers

##### JVT-P029 [I. Amonou, N. Cammas, S. Kervadec, S. Pateux] (M12304) CE05: CE report on quality layers

Contribution on quality layers as described in JVT-O305. Rate-distortion optimized quality layers are associated to the SVC NAL headers. A gain of up to 0.4 dB is claimed using quality layers with respect to JSVM2.

##### JVT-P033-LL [I. Amonou, N. Cammas, S. Kervadec, S. Pateux] (M12308) CE05: Cross-check of Nokia prop on quality layers QQ

##### JVT-P108-LL [J. Ridge] Verification of CE5 results for France Telecom

## CE 6 & related docs: Improvement of update step

##### JVT-P047 [J.-Z. Xu] CE06: Proposed approach to CE06

An alternative update operator is presented. The update method proposed is based on the one proposed in JVT-O030, but further modifications are made to reduce the computational complexity and to eliminate the need of an extra frame buffer used to store the update components in the originally proposed method. The proposed method is implemented in the current SVC reference software JSVM2. Experiments are conducted according to the test conditions set for CE6. It is claimed that on most of the test sequences, the proposed method can achieve better or similar coding performance. The proposed update operator skips the step of inversing the motion field and that of computing update weights, which must be done in the current update operator in JSVM2. Moreover, the proposed method is claimed to significantly reduce the memory requirement.

##### JVT-P036 [M.-H. Lee] CE06: Verification of MSRA proposal JVT-P047

Verification of JVT-P047. The binaries are compiled and built from the modified software provided by MSRA and the results are generated using the same set of configurations provided by MSRA. The results obtained are similar to MSRA’s results. See attached excel file JVT-P036.xls.

##### JVT-P052 [X. Wang, Y. Bao, M. Karczewicz, J. Ridge] CE06: Simplified update step operation for MCTF

Follow-up of JVT-O015. New ideas are: 1) Update motion vector derivation process is omitted. The reverse direction of motion vectors used in prediction step is used directly as update motion vectors. 2) Outlier motion vectors are checked and excluded from the update step. 3) Energy based adaptive weighting on update signal is replaced with a simple block energy based capping operation to limit the maximum amplitude of update signal.

With all these new simplifications, there is no obvious degradation claimed on coding performance. For some test sequences, the coding performance is claimed to be slightly improved.

For 1) "Direct update" replaces derivation of update motion vector for each 4x4 block. For 2) Up to 50% of motion vectors excluded from update process, so in general less update is made. Minor impact on PSNR. Comparison made against the simplified version (with simple interpolation method). Interpolation is made on basis of 16x16 blocks in cases of 16x16 block mode.

##### JVT-P096 [W.-J. Han] CE06: Verif of Nokia proposal JVT-P052

Verification of JVT-P052. By using the provided binaries and configurations files, results in JVT-P052 can be reproduced. However, due to the tight deadline, the verification results for Crew, Harbour, and Crew sequences are nit finished. All other sequences are verified. The details are described in the attached excel file. For the unfinished sequences, results are promised for the Poznan meeting.

Verification has been finished, no problem found.

##### JVT-P084 [W.-J. Han] CE06: Core experiment report on update step

It is well-known that the open-loop MCTF with update-step structure provides a state-of-the art performance as well as the excellent scalability feature although most standardized video codecs utilize the closed-loop concept. In this contribution, the various properties between closed-loop and open-loop framework including the single-layer and multi-layer tests are investigated. Furthermore, a new scheme for utilizing both good properties of the closed-loop scheme and the update-step by combining the closed-loop prediction and the switched update-step technique is proposed. From the test results of combined scalability configuration, it can be concluded that the new scheme can combine the closed-loop prediction and the update-step successfully and improve the PSNR values up to 0.8 dB.

Up to 15% savings in bit rate by update step in open-loop. Tool1: Closed-loop prediction also gives gain if update step is used. In base layer, closed loop HB with closed-loop ME shows best performance. In enhancement layer, usage of update step gives additional gain (up to 0.44 dB). Tool2: Performing prediction step first in the decoder allows to avoid mismatch. Gives up to 3 dB gain at small QP (8) in the PSNR range of 40 dB and above. For higher QP value, update step can be skipped at the decoder (QP > 24: no PSNR drop). Would allow to apply complexity scalability at the decoder. Combination of the two tools gives typically 0.2-0.3 dB over JSVM2. Update step gives less PSNR fluctuations. For inter-layer motion prediction, closed loop ME gives sometimes worse results (less coherence between base and enhancement).

Further investigation in CE recommended to find out benefits of update at higher fidelity ranges.

##### JVT-P060 [X. Wang] CE06: Verif. of Samsung prop.

Verification of JVT-P084. Both the encoder and decoder (binary executable) were provided. Using the configuration files provided in JSVM2, sequences were re-encoded, decoded and PSNR values were computed against original at each target bitrate on all the test sequences. The PSNR values from the verification test match those provided in JVT-P084.

##### JVT-P077 [D.H. Yoon, B.-Y. Jeon, J.-H. Park, S.-W. Park] Complexity reduction by simplified chroma update step

The update step for the chroma signal is simplified by quantizing chroma update motion vector to integer-pel accuracy. It is claimed that this simplification does not introduce any noticeable degradation but the overall complexity of update step is reduced because fractional pixel interpolation can be avoided. Besides that, skipping of chroma update is also proposed, and this feature could be controlled by using a syntax element disable_chroma_update_flag.

Method 1: Simplification by using integer MV, only small loss in  PSNR (<0.1 dB); method 2: Skipping of chroma update entirely: loss of 0.5-1 dB. Comparable to other results reported on the entire omission of the update step, but questionable why it should be best strategy to omit it only for luma.  Further discuss in definition of CE with the other participants.

For further consideration of CE6 results, subjective comparison will be performed for

- JSVM2

- Hierarchical B frames, closed loop

- P047

- P052

- P084

## CE 7 & related docs: Enhancement layer intra prediction

##### JVT-P063-L [P. Yin, J. Boyce, P. Pandit] CE07: Proposal for CE07 enh. layer intra pred.

The technical description and experimental result in response to CE7 are provided, which as an update of JVT-O053. Unlike JVT-O053, the new mode I_BLS, which enables spatial intra prediction for the enhancement layer residue, is added as an additional macroblock prediction mode, so that I_BL mode and I_BLS mode can be selected on macroblock basis. Compared to JVT-O053, the new proposal is claimed that it can still maintain the gain at the higher bitrate, while also achieve some gain at lower bitrate.

##### JVT-P070-L [A. Segall] CE07: Verif. of JVT-P063 enh. layer intra pred.

Verification of JVT-P063.  Bit-streams and a binary decoder have been provided.  It was verified that the bit-streams are decodable and bit-rates and PSNR values were measured.  The rate and PSNR information is enclosed in a supporting document.

##### JVT-P069 [A. Segall, S. Lei] CE07: Prop. for adaptive upsamp. spatial scalab.

An approach to adaptive upsampling.  The description is based on JVT-O010, which proposes a technique to select filters on a block-by-block basis and utilizes a four-channel filterbank.  Filter selection is performed at the encoder and signaled to the decoder.  For the case of IntraBL coded blocks, a coding improvement of up to 0.6dB is claimed when compared to the current JSVM and encoded with a PSNR of 38dB.  Coding improvements of 1.7dB are claimed for high-rate conditions.

##### JVT-P066 [P. Pandit] CE07: Verif. Sharp proposal JVT-O010

Verification of JVT-P069. Related to the direct texture upsampling method.

##### JVT-P099-LL [S. Cha] CE07: Enhancement-layer intra prediction

##### JVT-P078 [D.H. Yoon, B.-Y. Jeon, J.-H. Park, S.-W. Park] Clarification of intra_base_flag

Proposal is about I_BL (intra base) macroblock, which uses the scaled and reconstructed base layer signal as the predictor. Two flags are used to indicate I_BL macroblock type; base_mode_flag and intra_base_flag. The usage of intra_base_flag in the current JSVM is claimed to be inappropriate for the single-loop decoding case. Contains a proposed clarification of the intra_base_flag syntax with regards to inter-layer prediction constraint.

##### JVT-P041 [L. Xiong] Reducing enh.-layer directional intra pred modes

In addition to H.264/MPEG4-AVC Intra prediction maccroblock types in the JSVM2 enhancement layer Intra prediction and the I_BL model is added. For I_BL, the corresponding encoded macroblock information in layer k can be used to process the Intra prediction of current macroblock in the layer k+1. By using I_BL, on some occasions, it is claimed that better efficiency of prediction and encoding can be achieved.

## CE 8 & related docs: Region-of-interest coding

##### JVT-P042 [T.-C. Thang, T.-M. Bae, Y.-J. Jung, Y.-M. Ro, J.-W. Kang, H.-C. Choi, J.-G. Kim, J.-W. Hong] CE08: Spatial scalability of multiple ROIs

The functionality of ROI spatial scalability is addressed.

##### JVT-P043 [T.-M. Bae, T.-C. Thang, D.-Y. Kim, Y.-M. Ro, J.-W. Kang, J.-G. Kim, J.-W. Hong] FMO implementation in JSVM

FMO(Flexible Macrblock Ordering) and ASO(Arbitrary Slice Order) are already existing tools. Currently, these tools are not supported by both encoder and decoder of reference software of SVC, JSVM2.0. To support these functionalities, FMO and ASO have been implemented in the JSVM 1.0 and the encoding procedure is described.

##### JVT-P097 [G. H. Park, K. I. Lee] Advanced FMO for ROI in SVC

An Advanced Flexible Macroblock Ordering (A-FMO) method is proposed with the aim of improved picture quality of the ROI region in which the cyclic-FGS coding method is used. The concept of the virtual rings on the FMO is proposed and the number of recursive scan frequency (RSF) for each virtual ring is controlled. By changing the RSF on each virtual ring, image information of the blocks of each virtual ring can be differently encoded and decoded according to the priority of the user interests, therefore the picture quality of the blocks assigned into the ROI region can be enhanced in comparison with the picture quality of the blocks located in the left-over region. The proposed method uses the box-out scan with introducing the virtual ring concept and applies the different number of scanning for coding to the blocks belong to virtual rings to accomplish the enhancement of the ROI without using the conventional bit-shift process.

##### JVT-P106-L [Z. Lu] CE8 report: Perceptual Region-of-interest (ROI) based Scalable Video Coding

Reports progress in CE8 test 1. Changing ROI are implemented and introduced in the report. Subjective and objective comparison between JSVM1 and perceptual ROI-based JSVM1 are to be presented in the report. Because the subjective viewing test is arranged to hold on Saturday (23rd July 2005) afternoon, a revised version with the comparison on subjective quality will be uploaded later.

##### JVT-P107-L [Z. Lu] CE8: Cross-check of Spatial scalability of multiple ROIs

Verification of CE8 test 1, JVT-P042. The source of the proposal as well as the executable code and configuration files has been received. The bitrate reported in JVT-P042 are verified.

## CE 9 & related docs: Improved quantization

##### JVT-P030 [I. Amonou, N. Cammas, S. Kervadec, S. Pateux] (M12305) CE09: CE report on improved quantization

An adaptive reconstruction offset has been tested as well as embedded quantization intervals as proposed in JVT-O060. Using new test conditions, some gain is claimed in SNR scenarios (up to 0.23dB on luminance and 0.31dB on chromas for higher rates). For some sequences such as Mobile no specific gain is observed. When considering combined scalability scenarios, gain is only observed at the lowest resolution. This gain is not passed on higher resolution.

##### JVT-P022-L [V. Bottreau] CE09: Cross-verif of FT JVT-P030 on SVC quantization

Verification of JVT-P030. The test conditions from JVT-P030 were used.

##### JVT-P053-L [Y. Bao, M. Karczewicz, X. Wang, J. Ridge] CE09: Improved quantization for FGS coding

No abstract provided!

##### JVT-P101-LL [P. Amon] CE09: Result report

##### JVT-P044 [H.-C. Choi] Embedded quant. for improved SNR scalability

An embedded quantization is presented. The significant DCT coefficients of the residues of each SNR enhancement layer are divided into 3 groups and quantized into three values, -1, 0, and 1. In order to quantize and de-quantize the DCT significant coefficients, the proposed method calculates quantization intervals and reconstructions values in the sense of R-D optimization. Then, DCT significant coefficients of each enhancement layer are quantized and de-quantized using the values obtained by the proposed method. This method computes the quantization intervals and reconstruction values in the sense of R-D optimization. Improvement of 0~1.4dB are claimed in the first SNR enhancement layer.

##### JVT-P086 [W.-J. Han] Distribution-based inverse quantization

The quantization and inverse quantization process defined in JSVM assumes the Laplacian distribution with fixed decoding points. In this contribution, the PDF of the coefficients at the decoder side is estimated using the histogram of the quantized levels. In addition, a simplified approximation of the coefficient histogram is proposed to minimize the complexity overhead. The reconstruction points are the derived by equally dividing the PDF areas in the given quantization interval. The PSNR gain is claimed to be up to 0.8 dB at the Qp value of 8 and 0.2 – 0.3 dB at the relatively low Qp values from 24 to 28.

##### JVT-P095 [T. Rusert, M. Wien] Locally adaptive quantiz.: concept and first results

A concept for bit rate allocation with locally adaptive quantization for the JSVM is presented. The error-propagation characteristics within the MCTF scheme are considered on a macroblock basis by modified computation of the distortion measure used for rate-distortion optimization. In an analytic approach, a performance gain of up to 0.3 dB is observed for four decomposition levels. In a model-based approach, a performance gain of up to 0.2 dB is observed for one decomposition level.

## CE 10 & related docs: Extended spatial scalability

##### JVT-P012 [S. Sun] CE10Pt1: Direct interpolation for upsampling

This contribution is a response to CE10 part 1.  It shows performance of the improved upsampling process applied to extended spatial scalability.  The proposed direct interpolation method is claimed to reduce computational complexity while slightly improving quality of the up-sampled pictures by up to 0.45 dB and 0.19 dB on average comparing to the JSVM2 implementation.

##### JVT-P018 [E. Francois, J. Vieron, S. Sun] CE10: Report of results

This document reports the results of CE10 related to Extended Spatial Scalability. The goal of this CE is to improve the upsampling processes applied in extended spatial scalability as described in the JSVM2. This basically addresses the intra texture, residual and motion upsampling.

##### JVT-P019 [J. Vieron, E. Francois] CE10: Unified motion upsampling in extended spat scal

This document relates to CE10, dealing with Extended Spatial Scalability. It corresponds to proposal 2, focused on motion upsampling, in case of cropping and non dyadic spatial scalability. The proposed solution aims at unifying the two tools of Extended Spatial Scalability, that is, spatial scalability with ratio 3/2 and MB aligned cropping, and spatial scalability with any ratio and any cropping.

##### JVT-P020 [J. Vieron] CE10Pr1: Cross-verif of JVT-P012 direct upsamp

Reports the cross-check result for the proposal 1 of CE10 described in document JVT-P012, related to direct texture upsampling method.

##### JVT-P074 [S.-W. Park, J.-H. Park, D.-H. Yoon, B.-M. Jeon] CE10: Verif. of Thomson proposal JVT-O041/O042

The main purpose of this document is to verify the performance of the CE 10 related to improving upsampling process applied in Extended Spatial Scalability (ESS) as described in the JSVM2. In this document, we focus on verifying the results of Thomson’s proposal (JVT-O041/O042) of CE 10. The proposal consists in providing a complexity reduction by fully unifying ESS_3_2 with a limited enhancement / base layer ratio of 3/2 and ESS_generic supporting any ratio. The three ratios of 3/4, 2/3 and 3/5 are considered and generic solution and unified solution are compared for each of those ratios.

## CE 11 & related docs: FGS ordering

##### JVT-P054-LL [J. Ridge, M. Karczewicz, Y. Bao, X. Wang] CE11: Core experiment results for FGS reordering

##### JVT-P027-LL [V. Bottreau] Prop. on FGS bitplane coding

##### JVT-P058-LL [Y. Bao, M. Karczewicz, X. Wang, J. Ridge] Entropy coding for Fine Granularity SNR Scalability

##### JVT-P090 [B.-K. Lee] Adaptive FGS process

No abstract available!

##### JVT-P046-L [J.-Z. Xu] Alternative to FGS quant. method of JVT-O054.

An alternative FGS coding method to JVT-O054 is presented. It is claimed that it can improve the coding performance compared to the proposed one in JVT-O054 and meanwhile effectively reduce the decoding complexity of motion compensation operation for enhancement macro-block. It is implemented based on Progressive FGS (PFGS) with macro-block based drifting control.

## CE 12 & related docs: Weighted prediction FGS coding of closed-loop P frames

##### JVT-P055-L [Y. Bao, M. Karczewicz, X. Wang, J. Ridge] CE12: Weighted pred. FGS coding closed-loop P frames

##### JVT-P087 [W.-J. Han] PFGS with bilinear interpolation filter

Compared to the current implementation of FGS technique in JSVM, progressive fine grain scalability (PFGS) uses the FGS layer as the reference frames to improve the coding efficiency. However, the computational complexity is the major burden of PFGS since motion compensation is needed for each FGS layer. In this contribution, the scheme for estimating the FGS residual in the PFGS scheme is modified. In addition, a less complex bi-linear interpolation filter in the FGS layer is used to reduce the overall complexity of PFGS-based schemes. From the experimental results, it can be shown that the use of a bi-linear interpolation filter does not degrade the PSNR values in the FGS layer, and the modified residual estimation process provides up to 7% bit-saving compared to the direct PFGS residual computation process.

## High-Level Syntax

##### JVT-P031 [I. Amonou, N. Cammas, S. Kervadec, S. Pateux] (M12306) Syntax for FGS pass fractioning

A syntax modification for allowing FGS pass fractioning is proposed. With that, an FGS enhancement pass may be split over several NAL units. This syntax modification is claimed to be justified for enabling dead sub-streams implementation and to help proposing Medium Grain Scalability at the NAL unit level.

##### JVT-P032 [I. Amonou, N. Cammas, S. Kervadec, S. Pateux] (M12307) On high-level syntax for SVC

Some discussions with examples and syntax improvements for the high level syntax of NAL units are proposed. Based on these discussions it is proposed to always use the extended NAL unit header (i.e. having simple_priority_id, dependency_id, temporal_level and quality_level) and thus removing the extension_flag introduced in the NAL unit header. Further the mapping between simple_priority_id and (dependency_id, temporal_level, quality_level) should be relaxed to get more flexibility.

##### JVT-P061 [Y.-K. Wang, M. M. Hannuksela] Signaling of scalab. info.

A mechanism for signaling of scalability information is proposed, targeting for easy and convenient system-level operations on scalable video streams. The concrete proposals enabling the signaling mechanism are 1) An improved syntax for the scalability information SEI message specified in the JSVM 2 (JVT-O202), 2) A new SEI message called sub-picture scalable layer information SEI message, 3) A restriction on the use of the three sub-sequence related SEI messages.

##### JVT-P062 [Y.-K. Wang, Y.-L. Bao] Indication of non-required pics.

A new SEI message is proposed for indication of non-required pictures before their decoding, such that the decoder can avoid decoding and buffering of the non-required picture while at the same time low delay is allowed.

##### JVT-P079 [D.H. Yoon, B.-Y. Jeon, J.-H. Park, S.-W. Park] Key picture indication in AVC compatible base layer

The JSVM2.0 key pictures use closed-loop motion compensation. The reference pictures for the key picture are the base representation without FGS enhancement. The reference pictures of non-key picture are the base representation + FGS enhancement. It is claimed that the way how to indicate key picture for slices conforming to AVC ( the base layer of SVC ) is not clear so far, since key_picture_flag is defined only in slice_header_in_scalable_extension. A solution using nal_ref_idc is proposed.

##### JVT-P080 [S. Jeong, M. Park, G. Park, K. Kim] Flag bit in SEI for adaptive GOP structure

Adaptive GOP Structure was adopted into JSVM as non-normative because it dose not require any modification for the decoder. It is claimed that it requires a modification on the extractor. In order to use Adaptive GOP structure in JSVM, the information whether Adaptive GOP Structure is used or not is required. Therefore, a flag bit in Scalable Information is proposed in order to notify whether Adaptive GOP Structure is used or not to the extractor. This flag bit is added into Scalable Information SEI, because it used only at the extractor.

##### JVT-P100 [P. Amon, A. Hutter, T. Rathgen] High-level syntax for SVC

Three different tools for the high-level syntax of SVC are proposed. The first topic addresses the scalability description in the NAL unit header. First, the current status of the discussion is briefly reviewed, which is a variable length design. Then two alternate solutions are presented, which basically introduce more flexibility in the usage of the NAL unit header. The second tool describes the concept of Super-NAL units. These container elements include all NAL units of one scalability level (the same spatial, temporal, and SNR ID). This design facilitates scalability operations and supports transport. The third tool deals with the design of FGS NAL units to allow medium and fine grain SNR scalability with the high-level syntax.

## VLC-based entropy coding

##### JVT-P056-L [J. Ridge, M. Karczewicz, Y. Bao, X. Wang] Variable length codes for SVC

As noted at the previous JVT meeting, context-adaptive variable-length codes (CAVLC) is a feature currently missing from the JSVM software. CAVLC is incorporated into the JSVM. Base-layer compatibility is verified with H.264/AVC by decoding test sequences using H.264/AVC JM 9.6. For enhancement layers, the performance is claimed to be in line with the single-layer case.

## Weighted prediction

##### JVT-P064 [P. Yin, J. Boyce, P. Pandit] Weighted prediction for SVC

JSVM2.0 uses the same pred_weight_table() syntax in the scalable extension as in AVC, which indicates that the enhancement layer can have a different pred_weight_table() from its previous layer. In this contribution, it is proposed to re-use the base layer weighting parameters for enhancement layer if the corresponding base layer exists, i.e., to remap the pred_weight_table() from the base (or previous) layer to pred_weight_table() in the current enhancement layer. It has the advantage of saving memory, complexity and can achieve a slight coding efficiency improvement at very low bit rates.

##### JVT-P071 [A. Segall] Verif. of JVT-P064 weighted pred.

Verification of JVT-P064.  Bit-streams and a binary decoder were provided.  It has been verified that the bit-streams are decodable and bit rates and PSNR values have been measured.  The bit rate and PSNR information is enclosed in a supporting document.

##### JVT-P065 [P. Yin, J. Boyce, P. Pandit] Weighted prediction for SVC MCTF update step

The update step is investigated when weighted prediction is used in prediction step. First, the weights for the MCTF update step are theoretically derived based on the weights from prediction step. Then simulations are conducted to study two issues using fading sequences. The first issue is if the MCTF update step can improve coding efficiency compared to MCTF without update step when weighted prediction is used in prediction step. The experiment shows that MCTF update can achieve only a little gain compared to MCTF without update for fading sequences when weighted prediction is used in prediction step. The bigger gain can only be achieved by the sequences of longer fades which mainly contain global motion with larger GOP Size. The second issue is if the update step should incorporate the weighting parameters used in the prediction step since JSVM2.0 applies adaptive update weights to control the update strength. The experiment shows that MCTF update with derived weights from prediction step does not achieve any gain compared to MCTF update without derived weights. But since MCTF update with derived weights from prediction step involves more computational complexity, it is simpler to perform MCTF update without derived weights.

##### JVT-P076 [K.-H. Lee] Multi-layer weighted prediction

MCTF is a temporal wavelet process with motion compensation. MCTF can be used in multi layer structure scalable video coding. It is proposed that weight for motion prediction in upper layer can be estimated from base layer information.

## Other subjects

##### JVT-P039-LL [M.-H. Lee] Issue with temporal scalability

##### JVT-P045 [I.-H. Shin, H.-W. Park] Adapt. upsamp. for spat. scal.

A new up-sampling method for SVC is proposed. A type-1 DCT up-sampling method is employed. An adaptive filtering method of luminance component in the type-1 DCT up-sampling is used, which applies different weighting parameters to DCT coefficients. The adaptation is claimed not to incur an overhead in computational complexity at the decoder side. The weighting parameters of the up-sampling matrix are transmitted as side information.

##### JVT-P050 [J. Jia] Polyphase downsamp.-based redundant slices

A polyphase down sampling (PD) based redundant slice coding method for SVC error resilience is proposed. The proposal is to implement PD in SVC redundant slice coding process, then encode and send a part of the DCT coefficients. In decoding process, the uncoded pixels in redundant slice are reconstructed by both motion compensation and spatial interpolation. Experiments based on JM9.6 are performed on different test sequences in error prone environment with average packet loss rate of 3%, 5%, 10% and 20%. It is claimed that the PD-based redundant slice coding method gives 1dB improvement on average PSNR and 0.5-5.2 dB improvement on frame by frame PSNR over the conventional redundant slice coding method based on the same bit rate.

##### JVT-P051 [J. Jia] Imp. coding eff. by polyphase downsampling

A method for improving SVC coding efficiency on the basis of polyphase down sampling (PD) is described. The presented method proposes to implement PD on the residual signal before integer transformation in the SVC enhancement layer. Experiments based on JSVM2.0 are performed with different test sequences on Palma test points. It is claimed that the presented method gives 0.08 – 0.2dB improvement over the tested set of sequences when comparing at the same bit rate. It also claimed that the presented method effectively improves the coding efficiency of FGS enhancement layer by 14% average bit rate reduction at a 0.07dB PSNR loss in enhancement layer.

# JVT Non-SVC normative modifications

## Improved 4:4:4 coding

##### JVT-P016 [Y.-L. Lee] Lossless intra coding for improved 4:4:4

A new lossless Intra coding method based on DPCM (Differential Pulse Code Modulation) is proposed. The High 4:4:4 profile uses a multi-directional spatial prediction method to reduce the spatial redundancy by using block boundary pixels. In the proposed lossless Intra coding method, the spatial prediction residuals are transformed to the residual DPCMs. Also, the proposed method based on DPCM is designed not to break the decoder pipeline processing. The proposed lossless Intra coding based on DPCM is applied after the existing H.264 horizontal and vertical prediction. The proposed lossless Intra coding method based on residual DPCM reduced the bit rate by approximately 12.53 % in comparison with the lossless Intra coding method included in the H.264 standard.

##### JVT-P017-L [H. Yu, L. Liu] Advanced 4:4:4 profile

The Advanced 4:4:4 Profile proposed in JVT-O013 has been implemented in the JM software. The presented test results are used to claim that the proposed Advanced 4:4:4 Profile outperforms the current High 4:4:4 Profile. The latest development of the proposed Advanced 4:4:4 Profile are discussed and new test results are presented.

##### JVT-P048-L [W.-S. Kim+] Further experimental results on 4:4:4 coding

Performance of coding tools using various images of different characteristics – film grain noise, thermal noise, sampling, and cropping, etc is discussed. After brief description about RCT coding method, additional experimental results on the 4:4:4 chroma format materials are shown aiming to verify the efficiency of the RCT coding over various kinds of image sequences.

##### JVT-P049-L [W.-S. Kim+] Study on advanced 4:4:4 coding

Coding methods such as inter-plane prediction and single mode prediction are proposed aiming at improving the coding efficiency of the current FRExt 4:4:4 video coding technology, especially for RGB coding using RCT. A new work item of advanced 4:4:4 video coding is suggested in addition to the current standard.

##### JVT-P083 [Y. Yamada, S. Sekiguchi, Y. Isu, K. Asai, T. Murakami] Consideration on intra coding eff. of High 4:4:4 pr. (not Sunday afternoon)

Discussion of intra coding efficiency of the High 4:4:4 profile. At the Busan meeting, there was a discussion on the need of advanced 4:4:4 profile due to a loss of coding efficiency at very high bitrate. There was a discussion focus on intra coding efficiency of the High 4:4:4 profile especially when the input source signal is not YUV basis. For the applications targeting very high quality and resolution, color conversion to YUV space may be a limitation for further coding gain due to an information loss derived from color matrix conversion. The intra coding performance of the High 4:4:4 profile for XYZ formatted content was tested, which is adopted by Digital Cinema Initiative (DCI) as the signal representation for digital cinema contents. It is claimed that the intra coding performance for 4:4:4 high quality contents could further be improved by careful design of the intra prediction scheme suitable for 4:4:4 contents.

##### JVT-P088 [T. Wedi, S. Kondo, H. Saito] 4:4:4 Intra-only coding

High 4:4:4 profile uses the same mode for all three color-components of a MB. Thus, the color components cannot be coded independently and the possibility of parallel processing is limited. This limitation is claimed to be a drawback, because parallelization is essential, especially for future professional cameras that record video content at incessant higher resolutions. An Intra-only method is proposed that allows independently encoding and decoding of each color component. Therefore, separate mode information is associated to each color component on MB basis. In order to restrict the memory consumption, it is further proposed to keep the interleaving of color components on MB-level. Thus, the possibility of parallel processing for Intra only applications is significantly enhanced. It is claimed that the maximum (worst-case) overhead due to assignment of separate modes for each color component is negligible for professional applications.

##### JVT-P102-L [H. Yu] Artifacts seen in some Viper 4:4:4 sequences

Some severe artifacts in some Viper 4:4:4 JVT test sequences are reported. These sequences are “breeze” and “kungfu”. These problems are discussed and some preliminary results from the investigation are presented.

## Improved Error resilience

##### JVT-P023 [V. Bottreau] CABAC variant and options for bit error resilience

CABAC variants are proposed as options to support error-resilient decoding. It also presents a CABAC decoding algorithm making use of these options. The decoding approach is based on a Maximum-A-Posteriori (MAP) estimation algorithm. The trade-off between compression efficiency is controlled in a straightforward manner with a single parameter. When this parameter is set to zero, the performance is identical to the current CABAC implementations. Similarly, the trade-off between error-resilience and complexity is controlled by another parameter exploited by pruning techniques in the decoder. The performances of the decoding algorithm in the coder/decoder JM software are assessed with different error detection method against those obtained with classical CABAC hard decoder.

# JVT Non-SVC non-normative aspects

## Encoder optimization techniques

##### JVT-P014 [H. Schwarz, D. Marpe, T. Wiegand] Hierarchical B Pictures

An investigation of H.264/MPEG4-AVC compatible encoding with hierarchical B pictures is presented. The memory requirements are analyzed, the modifications of the encoder control compared to the H.264/MPEG4-AVC Test Model are described which improve the coding efficiency for hierarchical B pictures, and present a comparison of the coding efficiency for different GOP sizes (number of hierarchy stages) and a wide range of test sequences. It is claiomed through simulation results that in comparison to the classical “IBBP…” coding, the coding efficiency can be improved by up to 1.5 dB just by modifying the coding structure. It is also claimed that the usage of hierarchical B pictures does not only improve the objective quality for the majority of test sequences, but also the subjective quality especially for sequences with fine-detailed slow/regular moving image regions.

##### JVT-P015 [G. J. Sullivan, J. Lou, M.-T. Sun] Adaptive quantization yielding 10 dB gain

Results obtained from experimentation using adaptive quantization in video encoding are described.  The results of a small number of tests are reported on the effect of adaptive quantization on luma and chroma fidelity with different adaptive-update weighting factors.  The amount of luma-component quality improvement that is obtained by use of this technique is claimed to be about 11 dB (10 dB at equivalent bit rates).  This substantially exceeds the 3.7 dB anticipated improvement as calculated using high-resolution quantization theory analysis described in the contribution.  The chroma fidelity difference for use of the technique is claimed to be in the 5 to 6 dB range.  Results are reported for one particular video sequence – however, one additional sequence is reported to have been checked, and it is asserted that the results are likely to be roughly similar for others.  The results are reported to have been approximately verified independently by Shijun Sun of Sharp.  Remarks on intra and lossless coding are also provided.

##### JVT-P021 [X. Yi, J. Zhang, N. Ling, W. Shang] Improved & simplified fast ME for JM

In order to reduce encoding time, the current JVT reference software JM adopted a fast motion estimation method called UMHexagonS. In this document, an improved and simplified fast motion estimation scheme is proposed that claims to further speed up the encoding process and enhance the rate-distortion performance. Simulation results are shown to underline the claim that with similar or even better rate-distortion performance, the proposed method reduces motion estimation time by about 55% and 94% on average when compared with those of UMHexagonS and Fast Full Search respectively. In addition, a bit rate reduction up to 18% is claimed when comparing with that of full search in low complexity mode.

##### JVT-P026 [A. M. Tourapis, H.-Y. Cheong, P. Topiwala] Fast ME in the JM reference software

The scheme presented in JVT-E023, with some minor enhancements is proposed.

##### JVT-P072 [S. Bandyopadhyay, Z. Wu, P. Pandit, J. Boyce] Frame loss concealment

The current JM 9.6 software has limited error concealment support. In particular, the reference decoder software cannot handle entire frame (or picture) losses. Two error concealment algorithms are proposed to be be added to the JM to handle frame loss – frame copy and motion vector copy.  The proposed error concealment algorithms were tested using the packet loss model of VCEG Q15-I-16r1.  The motion copy method outperforms the frame copy method by up to 4.6 dB.

## VUI

##### JVT-P103 [A. Rodriguez] Sample aspect ratio support

Three aspect_ratio_idc values are proposed for picture resolutions used in broadcast applications and HD DVD.

# Withdrawn JVT documents

##### JVT-P073 [withdrawn] Withdrawn

# Updating of JVT group documents

##### JVT-P024 [T. Suzuki] Editor's proposed draft FRExt conformance

##### JVT-P025 [T. Suzuki] Prop list of changes of AVC conformance

The conformance specification, including FRExt conformance, were consented in ITU-T and it is under trial publication. However, some minor bugs were reported after ITU-T’s publication. All problems and its fixes are summarized in the provided http file. CRC32 and time stamp is attached for each bitstream to identify the latest version. All those fixes have already been incorporated into the ISO version of the conformance specification during editing period before the ISO publication.

# Attendance

- Gary J. Sullivan (Microsoft)

- Thomas Wiegand (Fraunhofer HHI)

- Jens-Rainer Ohm (RWTH Aachen Univ.)

- Lee Men Huang (Panasonic)

- Yoshihiro Miyamoto (NEC)

- Jun Zhang (Santa Clara Univ.)

- Mike Nilsson (BT)

- Masato Shima (Texas Instruments Japan)

- Vieron Jérôme (Thomson)

- Edouard Francois (Thomson)

- Hsin-Hao Chen (OES/ITRI)

- Je-Woo Kim (KETI)

- Yong-Hwan Kim (KETI)

- Hyok Sung (KETI)

- Byeongho Choi (KETI)

- Diego Santa Cruz (Visiowave / G.E. Security)

- Francesco Ziliani (Visiowave / G.E. Security)

- Julien Reichel (Visiowave / G.E. Security)

- Shijun Sun (Sharp)

- Andrew Segall (Sharp)

- Doug Young Suh (KHU)

- Munchul Kim (Info. & Comm. Univ. KR)

- Jung Won Kang (ETRI)

- Zhongkang Lu (I2R)

- Keng Pang Lim (I2R)

- Leszek Cieplinski (Mitsubishi Electric)

- Yung-Lyul Lee (Sejong Univ.)

- Donggyu Sim (Kwangwoon Univ.)

- Woo-Shik Kim (Samsung AIT)

- Satoru Sakazume (Victor Company of Japan)

- Hiroya Nakamura (Victor Company of Japan)

- Jae-Gon Kim (ETRI)

- Mathias Wien (RWTH Aachen Univ.)

- Ulrich Benzler (Robert Bosch GmbH)

- Marco Boltz (Robert Bosch GmbH)

- Anthony Vetro (Mitsubishi)

- Emin Martinian (Mitsubishi)

- Ong Chin Phek (Panasonic)

- Dae-Yeon Kim (Sejong Univ.)

- Jae-Ho Hur (Sejong Univ.)

- Huifang Sun (Mitsubishi)

- Jizheng Xu (Microsoft)

- Per Fröjdh (Ericsson)

- Michael Horowitz (CoVi Technologies)

- Sebastien Brangoulo (ENST Paris)

- Jong-Lak Kim (Nextreaming)

- Il-Hong Shin (KAIST)

- Reha Civanlar (LMI)

- Ofer Shapiro (LMI)

- Alex Eleftheriadis (LMI / Columbia Univ.)

- Tokumichi Murakami (Mitsubishi Electric)

- Kohtaro Asai (Mitsubishi Electric)

- Viji Raveendran (Qualcomm.)

- Seon-Tae Kim (ETRI)

- Byung-Gyu Kim (ETRI)

- Markus Beermann (RWTH Aachen Univ.)

- Peter Amon (Siemens AG)

- Thomas Rathgen (Ilmenau Tech. Univ.)

- Seung-Wook Park (LG Electronics)

- Ji-Ho Park (LG Electronics)

- Doe Hyun Yoon (LG Electronics)

- Byeong-Moon Jeon (LG Electronics)

- Arnaud Bourge (Philips)

- Xavier Henocq (Canon)

- Fabrice Le Leannec (Canon)

- Yongju Cho (ETRI)

- Han Sae Song (Seoul National Univ.)

- Chanyul Park (Seoul National Univ.)

- Bae-Keun Lee (Samsung)

- Sangchang Cha (Samsung)

- Woo-jin Han (Samsung)

- Kyohyuk Lee (Samsung)

- Byeungwoo Jeon (Sung Kyun Kwan Univ.)

- Woong Il Choi (Sung Kyun Kwan Univ.)

- Steffen Wittman (Panasonic)

- Thomas Wedi (Panasonic)

- Seyoon Jeong (ETRI)

- Gwang Hoon Park (Kyung Hee Univ.)

- Ye-Kui Wang (Nokia)

- Min Woo Park (Kyung Hee Univ.)

- Kyung Il Lee (Kyung Hee Univ.)

- Tae Heon Bae (Info. & Comm. Univ.)

- Truong Cong Thang (Info. & Comm. Univ.)

- Justin Ridge (Nokia)

- Yiliang Bao (Nokia)

- Xianglin Wang (Nokia)

- K. K. Tu (ETRI)

- Heiko Schwarz (Fraunhofer HHI)

- Teruhiko Suzuki (Sony)

- Jan Lievens (Vrije Univ.)

- Stephan Pateux (France Telecom. R&D)

- Nathalie Cammas (France Telecom. R&D)

- Isabelle Amonou (France Telecom. R&D)

- Nam Hyeong Kim (Korea Univ.)

- Hyeong Min Nam (Korea Univ.)

- Haechul Chou (ETRI)

- Marta Karczewicz (Nokia)

- Lu Yu (Zhejiang Univ.)