| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 16th Meeting: Poznań, PL, 24-29 July, 2005 | Document: JVT-P007 Filename: input.doc |
| --- | --- |

| Title: | Spatial Scalability Filters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Shijun Sun | Email: | garysull@microsoft.com ssun@sharplabs.com |
| Source: | AHG on resampling |  |  |

_____________________________

As adopted to the JSVM2 in the Busan meeting, the H.264/AVC sub-sample luma motion-compensation interpolation filter was chosen for both texture and residual upsampling for Extended Spatial Scalability, and chroma phase shift has now also been factored in for the upsampling.

The AHG activities were mostly within the scope of CE10, as described in JVT-O310.  Related contributions to the Poznan meeting are the following.

- JVT-P018, CE10 report

- JVT-P012, response to CE10 part 1

- JVT-P020, cross-verification of JVT-P012.

During the AHG activities, several sets of downsampling filters were designed based on the Kaiser-windowed sinc function as studied in JVT-I018.  And new upsampling filters based on direct interpolation are tested and proposed in JVT-P012.

For the dyadic spatial resampling case, there is no new activity in the AHG study.  However, CE7 covers some of the texture upsampling process.

Following is a more detailed description of the resampling method in JSVM2.

1. Dyadic spatial resampling

1.1 downsampling (non-normative)

By default in JSVM software, the following filter is applied to all Y, U, and V channels and in both horizontal and vertical directions.

(0,  2,  0, -4, -3,  5, 19, 26, 19,  5, -3, -4,  0,  2,  0) / 64

The picture boundaries are padded by repeating the edge samples.

In general, there is no explicit consideration of the luma/chroma phase shift during the downsampling operations.

For sequences downsampled from the 4:2:0 4CIF resolution (City, Crew, 
Harbour, Ice, and Soccer), since the chroma sampling position is Type 2 in Figure E-1 of the H.264 specification (based on email discussion), the relative chroma sampling position remains the same after the downsampling.

For sequences downsampled from the 4:2:0 CIF resolution (Bus, Football, Foreman, and Mobile), the original chroma sampling position might not be Type 2; therefore, the downsampled version might be different in terms of relative chroma sampling position.

1.2 upsampling (normative)

The following 6-tap filter for half-sample luma interpolation in the H.264 specification (subsection 8.4.2.2.1 Luma sample interpolation process) is applied to generate the “missing” samples of all Y, U, and V channels.

(1, -5, 20, 20, -5,  1)/32

The detailed description is available in the Working Draft (subsection S.8.3.6.3 Upsampling process for Intra_Base prediction).

There is no explicit consideration of the luma/chroma phase shift during the downsampling operations.  On one hand, the non-normative downsampling process in JSVM software is still a good match to the normative upsampling procedure.  On the other hand, both of these can be further improved.

2. Non-dyadic spatial resampling

2.1 downsampling (non-normative)

The downsampling process is not normative part of the Working Draft, Thomson and Sharp have both provided their own software for JSVM2 implementation.

Thomson provided several sets of linear polyphase filters for several specific scaling ratios (3:2, 4:3, 5:3, 6:5, 9:8, and 10:9).  Sharp provided a solution with three sets of 16-phase filters based on Kaiser-windowed sinc functions (JVT-I018).

In general, the downsampling process is still an area deserving further attention.

2.2 upsampling (normative)

The quarter-sample luma interpolation procedure (subsection 8.4.2.2.1 of H.264 specification) is applied for the upsampling of all Y, U, and V channels.

Given a luma sample in the current picture at position (x, y) in units of single luma samples, its corresponding position in the base picture (px,L, py,L) in units of quarter luma samples of the base picture can be derived as

where RL = 4 (for quarter-sample resolution interpolation), (xorig, yorig) represents the position of the upper-left corner of the cropping window in the current picture in units of single luma samples of current picture, (wbase, hbase) is the resolution of the base picture in units of single luma samples of the base picture, (wextract, hextract) is the resolution of the cropping window in units of the single luma samples of current picture, and “//” is a simplified division operator defined in JSVM2.

Similarly, given a chroma sample in the current picture at position (xc, yc) in units of single chroma samples, its corresponding position in the base picture (px,c, py,c) in units of quarter chroma samples of the base picture can be derived as

where RC = 4, (xorig,c, yorig,c) represents the position of the upper-left corner of the cropping window in the current picture in units of single chroma samples of current picture, (wbase,c, hbase,c) is the resolution of the base picture in units of single chroma samples of the base picture, (wextract,c, hextract,c) is the resolution of the cropping window in units of the single chroma samples of current picture, (pbase,x, pbase,y) represents the relative chroma phase shift of the base picture in units of quarter chroma samples of the base picture, and (penh,x, penh,y) represents the relative chroma phase shift of the current picture in units of quarter chroma samples of the current picture.

Detailed description of texture (as well as residual) upsampling process is available in JSVM2.  Basically, to get a quarter-sample interpolation, it is required to go through the half-sample position first using the 6-tap filter, followed by a linear interpolation for the quarter-sample position.  One task of CE-10 defined in Busan is to replace this with a direct interpolation procedure.

2.3 downsampling again – consideration of chroma phase shift (non-normative)

Similar to the upsampling process, given a luma sample in the low-resolution picture at position (x, y) in units of single luma samples of the low-resolution picture, its corresponding position in the high-resolution picture (px,L, py,L) in units of one-sixteenth luma samples of the high-resolution picture can be derived as

Given a chroma sample in the low-resolution picture at position (xc, yc) in units of single chroma samples of the low-resolution picture, its corresponding position in the high-resolution picture (px,c, py,c) in units of one-sixteenth chroma samples of the high-resolution picture can be derived as

Based on sampling position, a filter can be selected from a pre-defined set of filters as mentioned briefly in Section 2.1 of this report.