| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 16th Meeting: Poznan, Poland, 25-29 July, 2005 | Document: JVT-P012 Filename: input.doc |
| --- | --- |

| Title: | Direct Interpolation for Upsampling in Extended Spatial Scalability – Response to CE10 Part 1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Source: | Shijun Sun Sharp Labs of America 5750 NW Pacific Rim Blvd. Camas, WA 98607, USA | Tel: Fax: Email: | ssun@sharplabs.com |
| Source: | Sharp |  |  |

Abstract – This contribution is a response to CE10 part 1.  It shows performance of the improved upsampling process applied to extended spatial scalability.  The proposed direct interpolation method reduces the computational complexity in the meanwhile slightly improves quality of the up-sampled pictures by up to 0.45 dB and 0.19 dB on average comparing to the JSVM2 implementation.

# 1. Introduction

In the current SVC design, the upsampling process is based on the quarter luma sample interpolation procedure specified in H.264 for inter prediction.  The method inherits two drawbacks when being applied to texture and residual upsampling for spatial scalable coding: (1) the interpolation resolution is limited to quarter samples and (2) we will have to go through the half sample interpolation in order to get to a quarter sample position.

The proposed direct interpolation method removes both of the drawbacks by (1) finer interpolation resolution and (2) direct interpolation.  As a result, the proposed method reduces the computational complexity in the meanwhile slightly improves quality of the up-sampled pictures.

The proposed method is based on direct interpolation with 16 filters.  The filter selection is according to the interpolation positions or phases, ranging from 0 to 15 in units of one-sixteenth picture samples.  The set of filters are designed to be backward compatible with the half sample interpolation process of SVC and the half sample luma inter prediction of H.264.  Therefore, the proposed process can be a very simple and natural extension of H.264 from hardware/software implementation point of view.  Basically, an interpolation filter will be applied for each corresponding interpolation phase shift.  Instead of the 1/4-pel interpolations, we propose 1/16-pel-interpolation accuracy.  To be consistent with the current design for the typical dyadic scalability case, filters derived from Lanczos3 will be applied for texture upsampling; and bilinear interpolation process will be applied to residual upsampling.

### 2. Proposed modification with respect to JSVM-2

1. The following in S.8.3.6 shall be modified

- The corresponding fractional-sample position ( xf, yf ) in base layer of ( xP , yP ) is derived as follows.

xf = ( 4  16 * ( xP – ScaledBaseLeftOffset ) * BasePicWidth + 2  8 * deltaX ) // ScaledBaseWidth
yf = ( 4  16 * ( yP – ScaledBaseTopOffset ) * BasePicHeight + 2  8 * deltaY ) // ScaledBaseHeight

- the full-sample position ( xB, yB ) is derived as follows.

xB = xf >> 2  4
yB = yf >> 2  4

- The corresponding fractional-sample position ( xf1, yf1 ) in base layer of ( xP + 15 , yP + 15 ) is derived as follows.

xf1 = ( 4  16 * ( xP + 15 – ScaledBaseLeftOffset ) * BasePicWidth + 2  8 * deltaX ) // ScaledBaseWidth
yf1 = ( 4  16 * ( yP + 15 – ScaledBaseTopOffset ) * BasePicHeight + 2  8* deltaY ) // ScaledBaseHeight

- the full-sample position ( xB1, yB1 ) is derived as follows.

xB1 = xf1 >> 2  4
yB1 = yf1 >> 2  4

2. The following in S.8.3.6 shall bemodified

- Let ( xfC, yfC ) be defined as follows

xfC = ( 4  16 * ( xC – ScaledBaseLeftOffsetC ) * BasePicWidthC + 4 * deltaXC ) // ScaledBaseWidthC
yfC = ( 4  16 * ( yC – ScaledBaseTopOffsetC ) * BasePicHeightC + 4 * deltaYC ) // ScaledBaseHeightC

- the full-sample position ( xCB, yCB ) is derived as follows.

xCB = xfC >> 2  4
yCB = yfC >> 2  4

- Let ( xfC1, yfC1 ) be defined as follows

xfC1 = ( 4  16 * ( xC + MbWidthC – 1 – ScaledBaseLeftOffsetC ) * BasePicWidthC + 4 * deltaXC ) // ScaledBaseWidthC
yfC1 = ( 4  16 * ( yC + MbHeightC – 1 – ScaledBaseTopOffsetC ) * BasePicHeightC + 4 * deltaYC ) // ScaledBaseHeightC

- the full-sample position ( xCB1, yCB1 ) is derived as follows.

xCB1 = xfC1 >> 2  4
yCB1 = yfC1 >> 2  4

3. The following in S.8.3.6.4

- Let deltaX and deltaY be defined as follows.

deltaX = BasePicWidth – ScaledBaseWidth

deltaY = BasePicHeight – ScaledBaseHeight

- The corresponding fractional-sample position ( xf, yf ) in base layer is derived as follows.

xf = ( 4 * ( x + mbIdxX * 16 ) * BasePicWidth + 2 * deltaX ) // ScaledBaseWidth

yf = ( 4 * ( y + mbIdxY * 16) * BasePicHeight + 2 * deltaY ) // ScaledBaseHeight

- Let ( xInt, yInt ) and ( xFrac, yFrac) be defined as follows

xInt = ( xf >> 2 ) - xB

yInt = ( yf >> 2 ) - yB

xFrac = xf % 4

yFrac = yf % 4

- subclause Error: Reference source not found is invoked with ( xInt, yInt ), ( xFrac, yFrac ) and baseL[ x, y ] as inpus corresponding respectively to ( xIntL, yIntL ), ( xFracL, yFracL ), and the luma sample array of the selected reference picture, and with output predPartLXL[ xL, yL ] is assigned to predL[ x, y ].

shall be replaced by

- Let tempL[ x, y ] with x = –2..( xB1 – xB + 3) and y =0..15 be a temporary luma sample array.

- Each tempL[ x, y ] with x = –2..( xB1 – xB + 3) and y = 0..15 is derived as follows

- Let deltaY be defined as follows.

deltaY = BasePicHeight – ScaledBaseHeight

- The corresponding fractional-sample position yf in base layer is derived as follows.

yf = ( 16 * ( y + mbIdxY * 16) * BasePicHeight + 8 * deltaY ) // ScaledBaseHeight

- Let yInt and yFrac be defined as follows

yInt = ( yf >> 4 ) – yB
yFrac = yf % 16

Table-Amd1.8.X 16-phase interpolation filter for Intra_Base prediction

| phase | (6-tap) interpolation filter coefficients |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | e[-2] | e[-1] | e[0] | e[1] | e[2] | e[3] |
| 0 | 0 | 0 | 32 | 0 | 0 | 0 |
| 1 | 0 | -2 | 32 | 2 | 0 | 0 |
| 2 | 1 | -3 | 31 | 4 | -1 | 0 |
| 3 | 1 | -4 | 30 | 7 | -2 | 0 |
| 4 | 1 | -4 | 28 | 9 | -2 | 0 |
| 5 | 1 | -5 | 27 | 11 | -3 | 1 |
| 6 | 1 | -5 | 25 | 14 | -3 | 0 |
| 7 | 1 | -5 | 22 | 17 | -4 | 1 |
| 8 | 1 | -5 | 20 | 20 | -5 | 1 |
| 9 | 1 | -4 | 17 | 22 | -5 | 1 |
| 10 | 0 | -3 | 14 | 25 | -5 | 1 |
| 11 | 1 | -3 | 11 | 27 | -5 | 1 |
| 12 | 0 | -2 | 9 | 28 | -4 | 1 |
| 13 | 0 | -2 | 7 | 30 | -4 | 1 |
| 14 | 0 | -1 | 4 | 31 | -3 | 1 |
| 15 | 0 | 0 | 2 | 32 | -2 | 0 |

- Select a six-tap filter e[j] with j = -2..3 from Table-Amd1.8.X using yFrac as phase, and derive tempL[ x, y ] as

tempL[ x, y ] = baseL[ x, yInt - 2 ]*e[-2] + baseL[ x, yInt - 1 ]*e[-1] + baseL[ x, yInt ]*e[0] +
                         baseL[ x, yInt + 1 ]*e[1] + baseL[ x, yInt + 2 ]*e[2] + baseL[ x, yInt + 3 ]*e[3]

- Each sample predL[ x, y ] with x = 0..15 and y = 0..15 is derived as follows.

- Let deltaX be defined as follows.

deltaX = BasePicWidth – ScaledBaseWidth

- The corresponding fractional-sample position xf in base layer is derived as follows.

xf = ( 16 * ( x + mbIdxX * 16) * BasePicWidth + 8 * deltaX ) // ScaledBaseWidth

- Let xInt and xFrac be defined as follows

xInt = ( xf >> 4 ) - xB
xFrac = xf % 16

- Select a six-tap filter e[j] with j = -2..3 from Table-Amd1.8.X using xFrac as phase, and derive predL[ x, y ] as

predL[ x, y ] = Clip1Y( ( tempL[ xInt - 2, y ]*e[-2] + tempL[ xInt - 1, y ]*e[-1] + tempL[ xInt, y ]*e[0] +
                            tempL[ xInt + 1, y ]*e[1] + tempL[ xInt + 2, y ]*e[2] + tempL[ xInt + 3, y ]*e[3] +
                            512 ) / 1024 )

4. The following in S.8.3.6.4

- Let deltaXC and deltaYC be defined as follows

deltaXC = ( 2 + BaseChromaPhaseX)*BasePicWidthC – ( 2 + ScaledBaseChromaPhaseX ) * ScaledBaseWidthC

deltaYC = ( 2 + BaseChromaPhaseY)*BasePicHeightC – ( 2 + ScaledBaseChromaPhaseY ) * ScaledBaseHeightC

- Let ( xfC, yfC ) be defined as follows

xfC = ( 4 * ( x + mbIdxX * MbWidthC) * BasePicWidthC + deltaXC ) // ScaledBaseWidthC

yfC = pyC + ( 4 * ( y + mbIdxY * MbHeightC) * BasePicHeightC + deltaYC ) // ScaledBaseHeightC

- Let ( xIntC, yIntC ) and ( xFracC, yFracC) be defined as follows

xIntC = ( xfC >> 2 ) - xCB

yIntC = ( yfC >> 2 ) - yCB

xFracC = xfC % 4

yFracC = yfC % 4

- subclause Error: Reference source not found is invoked with ( xIntC, yIntC ), ( xFracC, yFracC ) and baseC[ x, y ] as inputs corresponding respectively to ( xIntL, yIntL ), ( xFracL, yFracL ), and the luma sample array of the selected reference picture, and with output predPartLXL[ xL, yL ] is assigned to predC[ x, y ].

shall be replaced by

- Let tmp1Cb[ x, y ] and tmp1Cr[ x, y ] with x = -2.. ..( xCB1 – xCB + 3) and y = 0..MbHeightC - 1 be temporary chroma sample arrays.

- Each tempC[ x, y ] with C as Cb and Cr, x = –2..( xCB1 – xCB + 3), and y = 0..MbHeightC - 1 is derived as follows

- Let deltaYC be defined as follows.

deltaYC = ( 2 + BaseChromaPhaseY)*BasePicHeight – ( 2 + ScaledBaseChromaPhaseY ) * ScaledBaseHeight

- The corresponding fractional-sample position yfC in base layer is derived as follows.

yfC = pyC + (16 * ( y + mbIdxY * MbHeightC) * BasePicHeight + 4 * deltaYC ) // ScaledBaseHeight

- Let yIntC and yFracC be defined as follows

yIntC = ( yfC >> 4 ) - yCB
yFracC = yfC % 16

- Select a six-tap filter e[j] with j = -2..3 from Table-Amd1.8.X using yFracC as phase, and derive tempC[ x, y ] as

tempC[ x, y ] = baseC[ x, yIntC - 2 ]*e[-2] + baseC[ x, yIntC - 1 ]*e[-1] + baseC[ x, yIntC ]*e[0] +
                         baseC[ x, yIntC + 1 ]*e[1] + baseC[ x, yIntC + 2 ]*e[2] + baseC[ x, yIntC + 3 ]*e[3]

- Each sample predC[ x, y ] with C as Cb and Cr, x = 0..MbWidthC - 1 and y = 0..MbHeightC - 1 is derived as follows.

- Let deltaXC be defined as follows.

deltaXC = ( 2 + BaseChromaPhaseX)*BasePicWidth – ( 2 + ScaledBaseChromaPhaseX ) * ScaledBaseWidth

- The corresponding fractional-sample position xfC in base layer is derived as follows.

xfC = ( 16 * ( x + mbIdxX * MbWidthC) * BasePicWidth + 4 * deltaXC ) // ScaledBaseWidth

- Let xIntC and xFracC be defined as follows

xIntC = ( xfC >> 4 ) - xCB
xFracC = xfC % 16

- Select a six-tap filter e[j] with j = -2..3 from Table-Amd1.8.X using xFracC as phase, and derive predC[ x, y ] as

predC[ x, y ] = Clip1C( ( tempC[ xIntC - 2, y ]*e[-2] + tempC[ xIntC - 1, y ]*e[-1] + tempC[ xIntC, y ]*e[0] +
                                    tempC[ xIntC + 1, y ]*e[1] + tempC[ xIntC + 2, y ]*e[2] + tempC[ xIntC + 3, y ]*e[3] +
                                    512 ) / 1024 )

5. The following in S.8.5.14 shall be modified

- Let arrays basePosX4[x] and basePosY4[y] with x = 0..15 and y = 0..15 be derived as follows.

basePosX4[x] = ( 4  16 * ( x + xP – ScaledBaseLeftOffset ) * BasePicWidth + 2  8 * deltaX ) // ScaledBaseWidth
basePosY4[y] = ( 4  16 * ( y + yP – ScaledBaseTopOffset ) * BasePicHeight + 2  8 * deltaY ) // ScaledBaseHeight

- the full-sample position ( xB, yB ) is derived as follows.

xB = basePosX4[0] >> 2  4
yB = basePosY4[0] >> 2  4

- the full-sample position ( xB1, yB1 ) is derived as follows.

xB1 = (basePosX4[15] + 3  15 ) >> 2  4
yB1 = (basePosY4[15] + 3  15 ) >> 2  4

6. The following in S.8.5.14 shall be modified

- The upsampling process as specified in Error: Reference source not found is invoked with ( xB, yB ), ( 16, 16 ), basePosX4, basePosY4, ( xB1 – xB + 1, yB1 – yB + 1 ), TransBlkBase, BitDepthY, and r as input and resPredL[x, y] with x, y = 0..15 as output.

7. The following in S.8.5.14 shall be modified

- Let arrays basePosCX4[x] and basePosCY4[y] with x = 0..MbWidthC – 1 and y = 0..MbHeightC – 1 be derived as follows.

basePosCX4[x] = ( 4  16 * ( x + xC – ScaledBaseLeftOffsetC ) * BasePicWidthC + 4 * deltaXC ) // ScaledBaseWidthC
basePosCY4[y] = ( 4  16 * ( y + yC – ScaledBaseTopOffsetC ) * BasePicHeightC + 4 * deltaYC ) // ScaledBaseHeightC

- the full-sample position ( xCB, yCB ) is derived as follows.

xCB = basePosCX4[0] >> 2  4
yCB = basePosCY4[0] >> 2  4

- the full-sample position ( xCB1, yCB1 ) is derived as follows.

xCB1 = ( basePosCX4[ MbWidthC – 1 ] + 3  15) >> 2  4
yCB1 = ( basePosCY4[ MbHeightC –1 ] + 3  15) >> 2  4

8. The following in S.8.5.14 shall be modified

- The upsampling process as specified in Error: Reference source not found is invoked with ( xCB, yCB ), (MbWidthC, MbHeightC ), basePosCX4, basePosCY4, ( xCB1 – xCB + 1, yCB1 – yCB + 1 ), TransBlkBaseC, BitDepthC, and r as input and resPredC[x, y] with x = 0..MbWidthC – 1, y = 0..MbHeightC – 1 as output.

9. The subclause S.8.5.14.2 shall be replaced by

Input to this process are

- two variables ( x0, y0 ) represent an integer position offset

- two variables ( M, N ) representing the output block size

- an array posX[x] with x = 0..M – 1

- an array posY[y] with y = 0..N – 1

- two variables ( m, n ) representing the base block size

- an array TransBlkIdx[x, y] with x = 0..m – 1 and y = 0..n – 1

- an array resBase[x, y] with x = 0..m –1 and y = 0..n – 1.

Output of this process is an array resInterp[x, y] with x = 0..M – 1 and y = 0..N – 1.

Each output sample resInterp[ x, y ] with x = 0..M – 1 and y = 0..N – 1 is derived as follows.

- Let integer postions ( x1, y1 ) and ( x2, y2 ) be derived as

x1 = ( posX[ x ] >> 4 ) – x0
y1 = ( posY[ y ] >> 4 ) – y0

- if ( posX[ x ] % 16 ) is not 0

x2 = x1 + 1

- otherwise

x2 = x1

- if ( posY[ y ] % 16 ) is not 0

y2 = y1 + 1

- otherwise

y2 = y1

- Let temp1 be derived as

- if TransBlkIdx[x1, y1] is equal to TransBlkIdx[x2, y1]

temp1 = r[x1, y1] * ( 16 – ( posX[ x ] % 16 )  ) + r[x2, y1] * ( posX[ x ] % 16 )

- otherwise if ( posX[ x ] % 16 ) < 8

temp1 = r[x1, y1] * 16

- otherwise

temp1 = r[x2, y1] * 16

- Let temp2 be derived as

- if TransBlkIdx[x1, y2] is equal to TransBlkIdx[x2, y2]

temp2 = r[x1, y2] * ( 16 – ( posX[ x ] % 16 )  ) + r[x2, y2] * ( posX[ x ] % 16 )

- otherwise if ( posX[ x ] % 16 ) < 8

temp2 = r[x1, y2] * 16

- otherwise

temp2 = r[x2, y2] * 16

- Let resInterp be derived as

- if TransBlkIdx[x1, y1] is equal to TransBlkIdx[x1, y2]

resInterp[x, y] = ( temp1 * ( 16 – ( posY[ y ] % 16 )  ) + temp2 * ( posY[ y ] % 16 ) + 128 ) >> 8

- otherwise if ( posY[ x ] % 16 ) < 8

resInterp[x, y] = ( temp1 + 8 ) >> 4

- otherwise

resInterp[x, y] = ( temp2 + 8 ) >> 4

10. The modifications in subclause S.8.4.2.2.1 shall be removed if it is not used by any SVC tool.

3. Experiments

We compared the performance of the proposed method and the existing method in the JSVM 2 software.  The experiment follows the test condition specified in CE10 description (JVT-O310).

4. Conclusion

In summary, the proposed direct interpolation method is very effective in improving the coding quality and reducing the computational complexity.  Therefore, we propose that JVT adopt the method into the SVC extension.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Sharp |  |  |
| Mailing address | 5750 NW Pacific Rim Blvd Camas, WA 98607 USA |  |  |
| Country | USA |  |  |
| Contact person | Shijun Sun |  |  |
| Telephone | 1-360-817-7531 |  |  |
| Fax | 1-360-817-8436 |  |  |
| Email | ssun@sharplabs.com |  |  |
| Place and date of submission | Busan, Korea (April 2005) |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Extended Spatial Scalability with Picture-Level Adaptation |  |
| Contribution number |  | JVT-O008 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  | Provisional filing (Sharp) |  |
| Inventor(s)/Assignee(s) |  | S. Sun/Sharp |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: