| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 16th Meeting: Poznań, PL, 24-29 July, 2005 | Document: JVT-P018 Filename: JVT_P018_CE10_Report.doc |
| --- | --- |

| Title: | CE10: Report of results |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E.François, J.Vieron, N.Burdin Thomson Multimedia R&D France 1 avenue de belle fontaine 35511 Cesson Sevigne Cedex France Shijun Sun Sharp Labs of America 5750 NW Pacific Rim Blvd. Camas, WA 98607, USA | Tel: Email: | 33 2 99 27 32 62 edouard.francois@thomson.net 1-360-817-7531 ssun@sharplabs.com |
| Source: | Thomson |  |  |

_____________________________

# Introduction

This document reports the results of CE10 related to Extended Spatial Scalability, as described in document JVT-O310. The goal of this CE is to improve the upsampling processes applied in extended spatial scalability as described in the JSVM2. This basically addresses the intra texture, residual and motion upsampling.

Coordinator: Thomson

| Proponent | Partner | Contact | Email |
| --- | --- | --- | --- |
| Thomson | Sharp, LG | E.Francois | edouard.francois@thomson.net |
| Sharp | Thomson | S.Sun | ssun@sharplabs.com |

# Test conditions definition

The tests conditions of CE10 of Hong-Kong meeting, as described in document JVT-O039, have been used (cf following table). The downsampled test sequences have been generated using the filtering process described in document JVT-I018, that looks as a good compromise between aliasing and sharpness of the filtered texture.

| Ratio Low/High ¾ | bitrates | Low - 528*432@30HZ | High - 704*576@30HZ |
| --- | --- | --- | --- |
|  | City | 810 kbps | 1024 kbps |
|  | Crew, Harbour, Soccer | 1190 kbps | 1500 kbps |
| Ratio Low/High 2/3 | bitrates | Low - 448*384@30HZ | High - 672*576@30HZ |
|  | City | 720 kbps | 1000 kbps |
|  | Crew, Harbour, Soccer | 1050 kbps | 1470 kbps |
| Ratio Low/High 3/5 | bitrates | Low - 384*336@30HZ | High - 640*560@30HZ |
|  | City | 610 kbps | 980 kbps |
|  | Crew, Harbour, Soccer | 890 kbps | 1430 kbps |

# Results

## Proposal 1 – texture and residual upsampling

This proposal is described in document JVT-P012.  The proposed direct interpolation method is expected to reduce the computational complexity in the meanwhile slightly improves quality of the up-sampled pictures.

### Results

The simulations have been performed with the following motion parameters:

- Search mode 	4

- Search function full pel 	3

- Search function sub pel 	2

- Search range (Full Pel) 	1

- Maximum number of iterations for bi-prediction search 	1

- Search range for iterations (0: normal search range) 	1

## Proposal 2 – motion upsampling

This proposal is described in document JVT-P019. The proposed solution aims at unifying the two tools of Extended Spatial Scalability, that is, spatial scalability with ratio 3/2 and MB aligned cropping, and  is to unify spatial scalability with any ratio and any cropping.

The following tests have been performed:

- DYADIC : 	dyadic scalability (Spatial Scalability Type 1), with upsampling of decoded base layer pictures

- S32 : 	Spatial Scalability with ratio3/2 (Spatial Scalability Type 2), with upsampling of decoded base layer pictures

- GENERIC : 	JSVM2 – generic Spatial Scalability (Spatial Scalability Type 3)

- SIMULCAST : 	each layer is encoded using a single layer approach. The overall bitrate is distributed on each layer proportionally to the picture size. The same GOP structure is used as in the scalable case.

- UNIFIED : 	unified motion upsampling solution, as described in Proposal 2 of document JVT-P019.

The simulations have been performed with the following motion parameters:

- Search mode 	4

- Search function full pel 	3

- Search function sub pel 	2

- Search range (Full Pel) 	96

- Maximum number of iterations for bi-prediction search 	4

- Search range for iterations (0: normal search range) 	8

### Results for ratio 4/3

| Ratio | Sequence | Config | Layer | Rate% | DataRate | PSNRY | PSNRU | PSNRV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4_3 | City | DYADIC | layer0 | 0,90% | 810 | 33,01 | 45,02 | 46,79 |
| 4_3 | City | DYADIC | layer1 | 1,75% | 1834 | 36,17 | 44,06 | 46,04 |
| 4_3 | City | GENERIC | layer0 | 3,45% | 810 | 38,39 | 44,86 | 46,59 |
| 4_3 | City | GENERIC | layer1 | 0,90% | 1834 | 36,24 | 44,00 | 46,03 |
| 4_3 | City | S32 | layer0 | 1,71% | 810 | 38,25 | 45,14 | 46,84 |
| 4_3 | City | S32 | layer1 | 0,94% | 1834 | 36,19 | 44,01 | 46,06 |
| 4_3 | City | SIMULCAST | layer0 | 0,98% | 660 | 37,13 | 44,01 | 45,94 |
| 4_3 | City | SIMULCAST | layer1 | 0,16% | 1174 | 34,77 | 42,59 | 44,93 |
| 4_3 | City | UNIFIED | layer0 | 3,52% | 810 | 38,38 | 44,86 | 46,59 |
| 4_3 | City | UNIFIED | layer1 | 0,98% | 1834 | 36,20 | 44,00 | 46,02 |

| Ratio | Sequence | Config | Layer | Rate% | DataRate | PSNRY | PSNRU | PSNRV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4_3 | Crew | DYADIC | layer0 | 1,70% | 1190 | 37,00 | 42,07 | 41,45 |
| 4_3 | Crew | DYADIC | layer1 | 0,65% | 2690 | 37,48 | 41,63 | 42,47 |
| 4_3 | Crew | GENERIC | layer0 | 0,81% | 1190 | 37,46 | 41,82 | 41,37 |
| 4_3 | Crew | GENERIC | layer1 | 0,46% | 2690 | 37,19 | 41,47 | 42,18 |
| 4_3 | Crew | S32 | layer0 | 1,73% | 1190 | 37,58 | 41,72 | 41,31 |
| 4_3 | Crew | S32 | layer1 | 0,34% | 2690 | 37,34 | 41,57 | 42,34 |
| 4_3 | Crew | SIMULCAST | layer0 | 0,96% | 968 | 37,30 | 41,99 | 41,43 |
| 4_3 | Crew | SIMULCAST | layer1 | 0,87% | 1722 | 36,57 | 41,49 | 41,90 |
| 4_3 | Crew | UNIFIED | layer0 | 0,81% | 1190 | 37,46 | 41,82 | 41,37 |
| 4_3 | Crew | UNIFIED | layer1 | 0,45% | 2690 | 37,22 | 41,50 | 42,24 |

| Ratio | Sequence | Config | Layer | Rate% | DataRate | PSNRY | PSNRU | PSNRV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4_3 | Harbour | DYADIC | layer0 | 0,90% | 1190 | 30,95 | 42,44 | 44,19 |
| 4_3 | Harbour | DYADIC | layer1 | 0,06% | 2690 | 33,56 | 42,70 | 44,77 |
| 4_3 | Harbour | GENERIC | layer0 | 0,76% | 1190 | 33,05 | 42,20 | 43,89 |
| 4_3 | Harbour | GENERIC | layer1 | 1,49% | 2690 | 33,59 | 42,75 | 44,73 |
| 4_3 | Harbour | S32 | layer0 | 1,23% | 1190 | 33,20 | 42,24 | 43,90 |
| 4_3 | Harbour | S32 | layer1 | 0,01% | 2690 | 33,59 | 42,77 | 44,76 |
| 4_3 | Harbour | SIMULCAST | layer0 | 1,05% | 968 | 31,89 | 41,66 | 43,09 |
| 4_3 | Harbour | SIMULCAST | layer1 | 0,86% | 1722 | 32,13 | 41,56 | 43,32 |
| 4_3 | Harbour | UNIFIED | layer0 | 0,77% | 1190 | 33,05 | 42,20 | 43,89 |
| 4_3 | Harbour | UNIFIED | layer1 | 1,56% | 2690 | 33,56 | 42,76 | 44,73 |

| Ratio | Sequence | Config | Layer | Rate% | DataRate | PSNRY | PSNRU | PSNRV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4_3 | Soccer | DYADIC | layer0 | 0,13% | 1190 | 36,47 | 44,55 | 46,58 |
| 4_3 | Soccer | DYADIC | layer1 | 0,57% | 2690 | 37,61 | 44,25 | 46,17 |
| 4_3 | Soccer | GENERIC | layer0 | 0,47% | 1190 | 38,03 | 44,44 | 46,24 |
| 4_3 | Soccer | GENERIC | layer1 | 0,11% | 2690 | 37,50 | 44,20 | 46,12 |
| 4_3 | Soccer | S32 | layer0 | 1,49% | 1190 | 38,23 | 44,50 | 46,33 |
| 4_3 | Soccer | S32 | layer1 | 1,48% | 2690 | 37,67 | 44,29 | 46,22 |
| 4_3 | Soccer | SIMULCAST | layer0 | 1,29% | 968 | 37,33 | 43,98 | 45,90 |
| 4_3 | Soccer | SIMULCAST | layer1 | 1,00% | 1722 | 36,04 | 43,31 | 45,42 |
| 4_3 | Soccer | UNIFIED | layer0 | 0,49% | 1190 | 38,03 | 44,44 | 46,24 |
| 4_3 | Soccer | UNIFIED | layer1 | 0,40% | 2690 | 37,49 | 44,22 | 46,14 |

### Results for ratio 3/2

| Ratio | Sequence | Config | Layer | Rate% | DataRate | PSNRY | PSNRU | PSNRV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3_2 | City | DYADIC | layer0 | 0,93% | 720 | 32,71 | 45,64 | 47,24 |
| 3_2 | City | DYADIC | layer1 | 1,12% | 1720 | 36,11 | 45,82 | 47,43 |
| 3_2 | City | GENERIC | layer0 | 0,69% | 720 | 38,04 | 45,37 | 47,01 |
| 3_2 | City | GENERIC | layer1 | 0,53% | 1720 | 36,12 | 45,81 | 47,43 |
| 3_2 | City | SIMULCAST | layer0 | 3,73% | 529 | 36,40 | 44,17 | 46,02 |
| 3_2 | City | SIMULCAST | layer1 | 1,59% | 1191 | 34,81 | 44,44 | 46,37 |
| 3_2 | City | UNIFIED | layer0 | 0,69% | 720 | 38,04 | 45,37 | 47,01 |
| 3_2 | City | UNIFIED | layer1 | 1,00% | 1720 | 36,06 | 45,80 | 47,43 |

| Ratio | Sequence | Config | Layer | Rate% | DataRate | PSNRY | PSNRU | PSNRV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3_2 | Crew | DYADIC | layer0 | 0,29% | 1050 | 36,37 | 42,13 | 41,29 |
| 3_2 | Crew | DYADIC | layer1 | 1,70% | 2520 | 37,32 | 43,18 | 43,18 |
| 3_2 | Crew | GENERIC | layer0 | 0,51% | 1050 | 37,02 | 41,83 | 41,16 |
| 3_2 | Crew | GENERIC | layer1 | 1,53% | 2520 | 37,20 | 43,09 | 43,03 |
| 3_2 | Crew | SIMULCAST | layer0 | 0,58% | 775 | 36,45 | 41,79 | 40,95 |
| 3_2 | Crew | SIMULCAST | layer1 | 1,91% | 1745 | 36,56 | 43,16 | 42,90 |
| 3_2 | Crew | UNIFIED | layer0 | 0,51% | 1050 | 37,02 | 41,83 | 41,16 |
| 3_2 | Crew | UNIFIED | layer1 | 1,81% | 2520 | 37,22 | 43,13 | 43,08 |

| Ratio | Sequence | Config | Layer | Rate% | DataRate | PSNRY | PSNRU | PSNRV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3_2 | Harbour | DYADIC | layer0 | 0,84% | 1050 | 30,64 | 42,77 | 44,52 |
| 3_2 | Harbour | DYADIC | layer1 | 0,52% | 2520 | 33,56 | 43,71 | 45,52 |
| 3_2 | Harbour | GENERIC | layer0 | 0,57% | 1050 | 32,82 | 42,58 | 44,20 |
| 3_2 | Harbour | GENERIC | layer1 | 0,63% | 2520 | 33,58 | 43,79 | 45,51 |
| 3_2 | Harbour | SIMULCAST | layer0 | 1,45% | 775 | 31,44 | 41,72 | 43,18 |
| 3_2 | Harbour | SIMULCAST | layer1 | 0,22% | 1745 | 32,31 | 42,56 | 44,14 |
| 3_2 | Harbour | UNIFIED | layer0 | 0,57% | 1050 | 32,82 | 42,58 | 44,20 |
| 3_2 | Harbour | UNIFIED | layer1 | 0,13% | 2520 | 33,53 | 43,79 | 45,52 |

| Ratio | Sequence | Config | Layer | Rate% | DataRate | PSNRY | PSNRU | PSNRV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3_2 | Soccer | DYADIC | layer0 | 1,41% | 1050 | 35,98 | 44,12 | 46,16 |
| 3_2 | Soccer | DYADIC | layer1 | 0,78% | 2520 | 37,56 | 44,21 | 46,14 |
| 3_2 | Soccer | GENERIC | layer0 | 1,27% | 1050 | 37,84 | 44,23 | 46,04 |
| 3_2 | Soccer | GENERIC | layer1 | 1,35% | 2520 | 37,46 | 44,17 | 46,10 |
| 3_2 | Soccer | SIMULCAST | layer0 | 1,17% | 775 | 36,64 | 43,56 | 45,43 |
| 3_2 | Soccer | SIMULCAST | layer1 | 0,54% | 1745 | 36,17 | 43,37 | 45,54 |
| 3_2 | Soccer | UNIFIED | layer0 | 1,27% | 1050 | 37,84 | 44,23 | 46,04 |
| 3_2 | Soccer | UNIFIED | layer1 | 1,71% | 2520 | 37,43 | 44,17 | 46,10 |

### Results for ratio 5/3

| Ratio | Sequence | Config | Layer | Rate% | DataRate | PSNRY | PSNRU | PSNRV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5_3 | City | DYADIC | layer0 | 0,25% | 610 | 28,35 | 44,65 | 46,41 |
| 5_3 | City | DYADIC | layer1 | 1,87% | 1590 | 36,20 | 44,06 | 46,10 |
| 5_3 | City | GENERIC | layer0 | 0,66% | 610 | 37,43 | 44,51 | 46,27 |
| 5_3 | City | GENERIC | layer1 | 0,27% | 1590 | 36,13 | 43,97 | 46,06 |
| 5_3 | City | S32 | layer0 | 0,95% | 610 | 32,78 | 45,23 | 46,59 |
| 5_3 | City | S32 | layer1 | 0,94% | 1590 | 36,14 | 43,96 | 46,06 |
| 5_3 | City | SIMULCAST | layer0 | 1,40% | 420 | 35,31 | 42,99 | 44,94 |
| 5_3 | City | SIMULCAST | layer1 | 0,87% | 1170 | 34,95 | 42,61 | 44,99 |
| 5_3 | City | UNIFIED | layer0 | 0,66% | 610 | 37,43 | 44,51 | 46,27 |
| 5_3 | City | UNIFIED | layer1 | 0,14% | 1590 | 36,08 | 43,98 | 46,06 |

| Ratio | Sequence | Config | Layer | Rate% | DataRate | PSNRY | PSNRU | PSNRV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5_3 | Crew | GENERIC | layer0 | 0,92% | 890 | 36,64 | 40,89 | 40,23 |
| 5_3 | Crew | GENERIC | layer1 | 1,63% | 2320 | 36,91 | 41,18 | 41,85 |
| 5_3 | Crew | S32 | layer0 | 0,13% | 890 | 36,33 | 41,24 | 40,42 |
| 5_3 | Crew | S32 | layer1 | 0,74% | 2320 | 37,04 | 41,26 | 42,00 |
| 5_3 | Crew | UNIFIED | layer0 | 0,87% | 890 | 36,64 | 40,90 | 40,23 |
| 5_3 | Crew | UNIFIED | layer1 | 1,29% | 2320 | 36,93 | 41,20 | 41,89 |

| Ratio | Sequence | Config | Layer | Rate% | DataRate | PSNRY | PSNRU | PSNRV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5_3 | Crew | GENERIC | layer0 | 0,92% | 890 | 36,64 | 40,89 | 40,23 |
| 5_3 | Crew | GENERIC | layer1 | 1,63% | 2320 | 36,91 | 41,18 | 41,85 |
| 5_3 | Crew | S32 | layer0 | 0,13% | 890 | 36,33 | 41,24 | 40,42 |
| 5_3 | Crew | S32 | layer1 | 0,74% | 2320 | 37,04 | 41,26 | 42,00 |
| 5_3 | Crew | UNIFIED | layer0 | 0,87% | 890 | 36,64 | 40,90 | 40,23 |
| 5_3 | Crew | UNIFIED | layer1 | 1,29% | 2320 | 36,93 | 41,20 | 41,89 |

| Ratio | Sequence | Config | Layer | Rate% | DataRate | PSNRY | PSNRU | PSNRV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5_3 | Harbour | GENERIC | layer0 | 1,22% | 890 | 32,57 | 41,77 | 43,41 |
| 5_3 | Harbour | GENERIC | layer1 | 1,89% | 2320 | 33,41 | 42,64 | 44,61 |
| 5_3 | Harbour | S32 | layer0 | 0,82% | 890 | 31,59 | 42,46 | 44,09 |
| 5_3 | Harbour | S32 | layer1 | 0,20% | 2320 | 33,59 | 42,74 | 44,73 |
| 5_3 | Harbour | SIMULCAST | layer0 | 0,78% | 614 | 30,96 | 40,98 | 42,49 |
| 5_3 | Harbour | SIMULCAST | layer1 | 0,34% | 1706 | 32,45 | 41,65 | 43,32 |
| 5_3 | Harbour | UNIFIED | layer0 | 1,23% | 890 | 32,57 | 41,77 | 43,41 |
| 5_3 | Harbour | UNIFIED | layer1 | 1,67% | 2320 | 33,53 | 42,73 | 44,71 |

| Ratio | Sequence | Config | Layer | Rate% | DataRate | PSNRY | PSNRU | PSNRV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5_3 | Soccer | GENERIC | layer0 | 0,59% | 890 | 37,42 | 43,81 | 45,64 |
| 5_3 | Soccer | GENERIC | layer1 | 1,50% | 2320 | 37,43 | 44,12 | 46,03 |
| 5_3 | Soccer | S32 | layer0 | 0,28% | 890 | 34,91 | 44,26 | 45,90 |
| 5_3 | Soccer | S32 | layer1 | 1,10% | 2320 | 37,42 | 44,10 | 46,04 |
| 5_3 | Soccer | SIMULCAST | layer0 | 0,43% | 614 | 35,79 | 42,79 | 44,70 |
| 5_3 | Soccer | SIMULCAST | layer1 | 1,87% | 1706 | 36,32 | 43,43 | 45,58 |
| 5_3 | Soccer | UNIFIED | layer0 | 0,62% | 890 | 37,42 | 43,81 | 45,64 |
| 5_3 | Soccer | UNIFIED | layer1 | 1,46% | 2320 | 37,41 | 44,12 | 46,03 |

### Comments

- The Unified solution gives results very similar to the generic solution of JSVM2.

- The noticeable advantage of the Extended Spatial Scalability solution against the dyadic one, already seen in previous CE results (see document JVT-O039) is confirmed for the considered tests conditions.

- For the considered tests conditions, offering an important bitrate overlap between low and high layers, the simulcast solution is outperformed by the scalable solution.

# Conclusions

## Proposal 1 – texture and residual upsampling

The direct interpolation as proposal 1 for texture and residual upsampling reduces the computational complexity and in the meanwhile slightly improves the coding efficiency (by up to 0.45 dB and 0.19 dB on average comparing to the JSVM2 implementation).  Therefore, we recommend adopting this proposal into the SVC.

## Proposal 2 – motion upsampling

The unified solution proposed as proposal 2 gives similar results than the JSVM2, while unifying the different types of spatial scalability (1:dyadic with MB aligned cropping / 2:ratio 3/2 with MB aligned cropping / 3:any ratio with any cropping), and consequently simplifying both the textual description and the implementation. In particular, the number of divisions and multiplications is significantly reduced with this approach. Consequently we recommend to adopt this proposal for introduction into the JSVM.

# References

[JVT-I018]	Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG JVT-I018, “Color format downconversion for test sequence generation”, G.Sullivan.

[JVT-O039]	Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG JVT-O039, “Report on Core experiment 10 : Non Dyadic Spatial Scalability”, E. Francois, J. Viéron, G. Marquant, N. Burdin, P. Lopez.

[JVT-O310]	Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG JVT-O310, “Core Experiments on Extended Spatial Scalability (CE-10)”, E.Francois, S.Sun.

[JVT-P012]	Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG JVT-P012, “CE10 proposal 1: Direct interpolation for upsampling”, S.Sun.

[JVT-P019]	Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG JVT-P019, “Unified solution for Extended Spatial Scalability (CE-10)”, J.Vieron, E.Francois, N.Burdin.