| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 16th Meeting: Poznań, PL, 24-29 July, 2005 | Document: JVT-P045 Filename: JVT-P045.doc |
| --- | --- |

| Title: | Adaptive up-sampling method for spatial scalability in SVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ilhong Shin and HyunWook Park Dept. EE, KAIST, 373-1 Guseong-dong, Yuseong-gu, Daejeon 305-701, Korea | Tel: Email: | +82-42-869-5466 ssi@athena.kaist.ac.kr hwpark@athena.kaist.ac.kr |
| Source: | KAIST |  |  |

_____________________________

# 1. Introduction

In a scalable video coding, up-sampling of video sequences is an important process for spatial scalability. We proposed a new up-sampling method for SVC [1]. When no-phase shift is expected, type-1 DCT up-sampling method was employed. For performance improvement, we apply adaptive filtering method of luminance component in the type-1 DCT up-sampling, which applies different weighting parameters to DCT coefficients. When this method is applied, up-sampling operation in the decoder has no difference in the computational complexity. Since the decoder requires the weighting parameters of up-sampling matrix, we need to transmit the weighting parameters of horizontal and vertical up-sampling, only 88 bits are required to transmit the weighting parameters for intra frame. The proposed adaptive up-sampling method shows much improved PSNR in comparison with the recent JSVM 2.0 up-sampling method. The proposed adaptive filtering method and its experimental results are given in sections 3 and 4, respectively.

# 2.  Type-1 DCT up-sampling method

The type-1 DCT has no phase-shift in up-sampling [1]. The definition of type-1 DCT is given as follows,

eq. (1)

where  is the input signal and N is the input sample size.

The original type-1 DCT cannot provide the Nyquist interpolation kernel. To make the Nyquist interpolation kernel, the highest frequency coefficient in the transform domain should be halved. The signal length of type-1 DCT is N+1, and N zeros are appended in the high frequency region after type-1 DCT. Then, type-1 IDCT of 2N+1 samples is performed to obtain up-sampled data.

We can write the two-fold up-sampling process as follows:

eq. (2)

where T denotes 1D type-1 DCT kernel in eq. (1), and  and  are the original block of  and up-sampled block of , respectively.  represents left kernels of type-1 DCT from column 1 to , and the superscript t means transpose of the matrix. In eq. (2),  is the  zero matrix.  and  denote the vertical and horizontal up-sampling kernels, respectively. The transpose relationship of the vertical and horizontal up-sampling matrix holds as . The element of vertical up-sampling matrix is written as follows:

eq. (3)

The 2D up-sampling can be also separately performed using columnwise (vertical) up-sampling followed by row (horizontal) up-sampling. Also we employ overlapping method to reduce blocking artifact and to improve the PSNR performance. Detail things were provided in the JVT document [1].

# 3. Adaptive up-sampling method

We extend our up-sampling method by using adaptive filtering scheme. Adaptive filtering method of motion compensation was proposed at the MPEG BUSAN meeting, which used 6x6 non-separable filter whose coefficients can be obtained with least square method [2-3]. The proposed method is somewhat different approach. We employ the type-1 DCT up-sampling matrix in the adaptive filtering, which determines relative weighting of each frequency components of type-1 DCT. We also consider only luminance block for adaptive up-sampling. Figure 1 shows sample positions of two-fold up-sampling.

Fig. 1. Sample positions of two-fold up-sampling.

Position ‘a’ in Fig. 1 indicates even position after up-sampling, which is not modified from the down-sampled image. In the proposed method, the same scheme is employed, that do not change pixel value of ‘a’. Position ‘b’ and ‘c’ are the horizontal and vertical up-sampled image data, respectively. The horizontal and vertical up-sampling can be performed with  and  matrix, which are odd-row up-sampling kernel of eq. (3).  can be written as follows:

eq. (4)

The position ‘d’ in Fig. 1 shows diagonal up-sampled image data. For up-sampling of position ‘d’, we perform horizontal filtering after vertical filtering by using vertically up-sampled data such as position ‘c’.

For adaptive up-sampling, we derive adaptive up-sampling matrix as follows:

eq. (5)

eq. (6)

Eqs. (5) and (6) show adaptive up-sampling matrix of vertical and horizontal directions, respectively.  and  are frequency weighting parameters of up-sampling matrix, which consists of  parameters, respectively.  shows the odd-row transform kernel of type-1 DCT. For diagonal up-sampling, we use already determined adaptive vertical and horizontal up-sampling matrix as follows:

eq. (7)

Therefore, we need to transmit the vertical and horizontal frequency weighting parameters which are  parameters per intra frame. However, we know  and  are always zero in eq. (4). And we can set the DC weighting parameter as one, because the proposed adaptive filtering sheme only exploits the relative weighting of each frequency component. Therefore, the number of weighting parameters to be transmitted becomes . The proposed adaptive up-sampling matrix with the weighting parameters can be made as follows:

eq. (9)

The hozirontal up-sampling matrix can be written as similar form to the vertical up-sampling matrix in eq. (9) by using the horizontal weighting coefficients. We assume 32-bits integer operation in the decoder. Therefore, eq. (7) was changed to integer form by scaling each figuring terms. The quantization of weighting coefficients is required in the encoder side. We experimentally determine that 4-bits quantization of weighting parameters is suitable.

When the proposed adaptive up-sampling method is applied, the up-sampling matrices of  and  are computed for every frame by using the optimum weighting parameters of  and , respectively. The computational complexity of up-sampling matrix is same as that of all weighting parameters of one, which is the original type-1 DCT up-sampling matrix. In this proposal, we consider only luminance component of intra frames for adaptive up-sampling, and the high-pass frames can use original type-1 DCT up-sampling method, which has no need to transmit weighting parameters.

# 4. Obtaining weighting coefficients

Estimation of the weighting parameters is an encoder issue. However, we show the reference method of obtaining the weighting parameters. We used least-square minimization method. Let i denote NxN block index of the down-sampled image, which consists of width (W) and height (H). We perform vertical up-sampling process as follows:

eq. (10)

where  denotes i’th block of the down-sampled image. Then we obtain the optimum  that minimizes error between the original up-sampled image and the computed up-sampled image as follows:

eq. (11)

where  is i’th block of original up-sampled image, which are set of pixels such as position ‘c’ in Fig. (1), and NB is the number of blocks in the down-sampled image. We should translate the eq. (11) as solvable form to avoid mathematical difficulty. The definition of kronecker tensor product is shown as follows [4]:

eq. (12)

where s is the stack operator, which maps MxN matrix into an MNx1 vector. The stack of matrix  is the vector formed by stacking columns of  into an MNx1 vector. Eq. (10) can be translated into a kronecker tensor product form as follows:

eq. (13)

Also we can rewrite eq. (11) as the reduced form where the zero elements of diagonal matrix  are removed as follows:

eq. (14)

where Diag(X) maps diagonal components of matrix  into a vector and  denotes j’th column of  matrix. Therefore, we should minimize the following equation to obtain the optimum  as follows:

eq. (15)

Since we set the first frequency components  as one, eq. (15) should be changed as follows:

eq. (16)

The horizontal weighting parameters also can be obtained in a similar way to the vertical weighting parameters in eq. (16). The obtained weighting parameters should be quantized properly.

# 5. Experimental results

We implemented the proposed adaptive type-1 DCT up-sampling methods in the recent JSVM 2.0 video codec [5]. The size of block to be up-sampled is 8x8, and the overlap points are 5 [1]. It make the up-sampled block 16x16 from the overlapped 13x13 block. The required number of weighting parameters is 22 (2N-2=2x12-2=22), so the required number of bits for each intra frame is 88 bits for 4 bits quantization. Entropy coding of the weighting parameters will be helpful in the coding performance. We used matrix inverse method of eq. (16) which is numerically unstable operation in the encoder. Therefore, we determine the employment of adaptive up-sampling or original type-1 DCT up-sampling using RDO operation per frame. The computational complexity of up-sampling in the decoder is about three times higher than that of current JSVM [5]. However, the added complexity is only contributed to the up-sampling operation.

We used four CIF video sequences such as Bus, Football, Mobile, and Tempete which have a spatial resolution of 352288, frame rate of 15 Hz and two 4CIF video sequences such as City and Harbour which have a spatial resolution of 704576, and frame rate of 30 Hz. The base layer was encoded with 176144 (QCIF) in the CIF sequence and 352288 (CIF) in the 4CIF sequence after down-sampling by JSVM 2.0. The enhancement layer uses the CIF and 4CIF resolution with up-sampling of JSVM 2.0 and the proposed adaptive method for inter-layer prediction [4]. The proposed adaptive up-sampling method will be efficient for high-bitrate coding of base-layer, i.e., less quantization of base layer images gives more accurate up-sampling results. However, use of very low QP in the base-layer derives coding performance degradation of scalable video bitstream. Therefore, we use an moderate experiment condition, where QP of base-layer is set to 24 and 30. Also we show the experimental result of extreme QP condition of base layer, which have QP (1) and QP (45). The minimum QP as one shows maximum performance of the proposed method, and the very large QP of 45 shows the saturated performance of the proposed method, which shows the nearly same performance of the proposed method and JSVM 2.0 up-sampling method. The experimental condition is shown in Table 1.

Table 1. Experimental condition of the proposed method in JSVM 2.0

| Number of spatial layer | 2 |
| --- | --- |
| GOP size | 1 |
| Intra period | 1 |
| Base layer | AVC compatible |

Figure 2 shows the rate-psnr curves of the enhancement layer from the proposed adaptive up-sampling method and JSVM 2.0 method with base layer QP of 24 (high quality).

- Bus CIF (Base layer QP: 24)

(b) Football CIF (Base layer QP: 24)

(c) Mobile CIF (Base layer QP: 24)

(d) Tempete CIF (Base layer QP: 24)

(e) City 4CIF (Base layer QP: 24)

(f) Harbour 4CIF (Base layer QP: 24)

Fig. 2. Rate-PSNR curves of the enhancement layer from the proposed and JSVM 2.0 up-sampling (base layer QP=24).

We can see the proposed adaptive up-sampling method improved PSNR performance. The maximum improvements of PSNR are about 0.4 dB (a), 0.25 dB (b), 0.4 dB (c), 0.4 dB (d), 0.2 dB (e), and 0.5 dB (f). Figure 3 shows the rate-psnr curves of the enhancement layer from the proposed adaptive up-sampling method and JSVM 2.0 method with base layer QP of 30 (middle quality).

(a) Bus CIF (Base layer QP: 30)

(b) Football CIF (Base layer QP: 30)

(c) Mobile CIF (Base layer QP: 30)

(d) Tempete CIF (Base layer QP: 30)

(e) City 4CIF (Base layer QP: 30)

(f) Harbour 4CIF (Base layer QP: 30)

Fig. 3. Rate-PSNR curves of the enhancement layer from the proposed and JSVM 2.0 up-sampling (base layer QP=30).

We can see the proposed adaptive up-sampling method still provides more improved result with moderate base layer QP condition (middle quality). The maximum improvements of PSNR are 0.3 dB (a), 0.15 dB (b), 0.3 dB (c), 0.25 dB (d), 0.1 dB (e), and 0.35 dB (f). As base layer quality become worse, the performance of up-sampling will be degraded. However, SVC provides any condition of video quality, so the proposed adaptive up-sampling method will be efficient at the high and middle quality of base-layer for spatial scalability. Figure 4 shows the rate-psnr curves of the enhancement layer from the proposed adaptive up-sampling method and JSVM 2.0 method with extreme base layer QP of 1 (highest quality) and 45 (very low quality).

(a) Harbour 4CIF (Base layer QP: 1)

(b) Harbour 4CIF (Base layer QP: 45)

Fig. 4. Rate-PSNR curves of the enhancement layer from the proposed and JSVM 2.0 up-sampling

(Base layer QP=1 (a) and 45 (b)).

As shown in Fig. 4 (a), the proposed adaptive up-sampling method gives substantial PSNR gain than JSVM 2.0 up-sampling method. Even though, it is not proper in real environment, we can see the maximum performance of the proposed method. The maximum improvement of PSNR is about 0.7 dB (a). Fig. 4 (b) shows the saturated result of the proposed adaptive up-sampling method.

# 6.  Conclusion

In this contribution, we proposed an adaptive up-sampling method for performance improvement of spatial scalability in the SVC. By transmitting weighting parameters of type-1 DCT up-sampling kernel, we showed the improved result of the proposed adaptive up-sampling method in comparison with the JSVM 2.0 up-sampling method with proper assumption of base layer quality. The number of bits for the weighting parameters is 88 bits for each intra frame, when no entropy coding is applied, which is generally negligible. The computational complexity of the proposed adaptive up-sampling method is just same as the original type-1 DCT up-sampling method. The proposed contribution will help SVC spatial scalability be more realistic in many applications of video coding.

# 7.  References

[1] Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG, ‘Down/up-sampling methods for spatial scalability of SVC’, JVT-O24, Busan, Korea, April, 2005

[2] ITU-T VCEG, ‘Two-dimensional non-separable adaptive wiener interpolation filter for H.264/AVC’, VCEG-Z17, Busan, Korea, April, 2005

[3] T. Wedi, “Adaptive Interpolation Filter for Motion and Aliasing Compensated Prediction”, Proc. Electronic Imaging 2002: Visual Communications and Image Processing (VCIP 2002), San Jose, California USA, January 2002.

[4] A. K. Jain, ‘Fundamentals of Digital Image Processing’, Prentice-Hall, Englewood Cliffs, NJ (1989).

[5] Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG, ‘Joint Scalable Video Model JSVM-2’, JVT-O202, Busan, Korea, April, 2005

(Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Korea Advanced Institute of Science and Technology (KAIST) |  |  |
| Mailing address | KAIST, Dept. EE, 373-1 Guseong-dong, Yuseong-gu, Daejeon 305-701, Korea |  |  |
| Country | South Korea |  |  |
| Contact person | Professor, HyunWook Park |  |  |
| Telephone | +82-42-869-5466 |  |  |
| Fax | +82-42-869-8066 |  |  |
| Email | hwpark@athena.kaist.ac.kr |  |  |
| Place and date of submission | Busan Korea, April 13,2005. |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Adaptive up-sampling method for spatial scalability |  |
| Contribution number |  | JVT-P024 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| X | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  | Planned |  |
| Inventor(s)/Assignee(s) |  | Ilhong Shin, HyunWook Park |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: