| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 16th Meeting: Poznań, PL, 24-29 July, 2005 | Document: JVT-P050 Filename: JVT-P050 |
| --- | --- |

| Title: | Polyphase down sampling based redundant slice coding for SVC error resilience |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Jia, Hae-Kwang Kim Multimedia Framework Lab Sejong University Seoul, 143-747, KOREA Hae-Chul Choi, Jae-Gon Kim Broadcasting Media Research Group ETRI Daejeon, 305-350, KOREA | Tel: Email: Tel: Email: | +82 02 3408 3757 Joriejiejia@gmail.com hkkim@sejong.ac.kr +82-42-860-1808 choihc@etri.re.kr jgkim@etri.re.kr |
| Source: | Sejong Univ. and ETRI |  |  |

_____________________________

Abstract:

In this proposal, a polyphase down sampling (PD) based redundant slice coding method for SVC error resilience is described. The presented method proposes to implement PD in SVC redundant slice coding process, then encode and send part of the DCT coefficients. In decoding process, the uncoded pixels in redundant slice are reconstructed by both motion compensation and spatial interpolation. Experiments based on JM9.6 are performed on different test sequences in error prone environment with average packet loss rate of 3%, 5%, 10% and 20%. Experimental results show that the PD based redundant slice coding method gives 1dB improvement on average PSNR and 0.5-5.2 dB improvement on frame by frame PSNR over the conventional redundant slice coding method based on the same bit rate.

- Introduction

In current AVC error resilient tools, redundant slice coding is an effective tool for video communication being robust to packet-loss. It can be implemented by totally repeating slice or coding slice with different coding parameters, for instance, redundant slice can be coded with higher QP [1]. In this proposal, a polyphase down sampling (PD) based redundant slice coding method is described which achieves higher PSNR of the reconstruction picture than common redundant slice coding methods do with the same coding redundancy. Also, the combination of the PD based redundant slice coding and the current redundant slice coding methods is presented, which takes more flexibility to the current encoder when video data are transmitted in unstable network environments.

- PD based redundant slice coding for SVC error resilience

  - PD based redundant slice coding

The PD based video coding method as described in [2] is adopted in this proposal for redundant slice coding. The proposed method suggest to implement polyphase down sampling on each 8x8 inter residual block before DCT, where 4x4 transformation is adopted. After PD, N/4 of the residual signals can be flexible coded and transmitted according to the available bandwidth, where N can be any integer value between 1 and 4. In this way, the video encoder will generate different bit streams corresponding to different bit rates, respectively.

Besides working independently, the PD based video coding method can also be implemented in a cooperative way, where redundant slices are encoded not only with the PD based method but also with the other redundant slice coding methods as described in AVC standard to provide further bit-rate flexibility. For instance, redundant slices can be encoded with both the PD based method and the higher QP based method. In this case, after PD and DCT process, the transformed coefficients are encoded with higher QP values.

  - PD based redundant slice reconstruction

In error free environment, video data are reconstructed as described in [1]. When the decoded primary picture can not be correctly decoded due to the errors or losses in transmission of the sequence while a coded redundant slice can be correctly decoded, the decoder replace the samples of the decoded primary picture with the corresponding samples of the decoded redundant slice.

In PD based redundant slice coding method, when error occurred, redundant slices are first decoded from the encoded bit stream, after that, such pixels that are not sent in the encoder are reconstructed with the prediction values and their neighboring pixel values which are decoded from the bit stream.

Fig. 1 illustrated the method for reconstructing such pixels that are not sent in the encoder.

Fig. 1 Pixel reconstruction in PD based redundant slice coding method

In the above figure, the gray color pixels in the current frame represent the reconstructed pixels which are decoded from the bit stream, while the white color pixels represent pixels which are reconstructed by predicting from both the previous frame and their spatial neighboring pixels, as illustrated by the yellow color pixel. When the reconstruction process of the white color pixel is concerned, the prediction value from previous frame is first used as the preliminary reconstruction value, after that, the average value from its neighboring pixels is compared with the preliminary reconstruction value. If the difference between the two prediction values is less than the threshold, the preliminary value is used as the reconstruction value, or else, the average value is used as the reconstruction value.

- Experimental results

To evaluate the performance of the proposed method, experiments based on JM96 [3] are performed in error-prone environment with average packet-loss rate (PLR) equals to 3%, 5%, 10% and 20% respectively [4]. Redundant slice is coded for each slice. For simplicity, here, every picture without division is coded as one slice, and each slice is put into one packet independently. For the same reason, bit-stream repetition is adopted in each test sequence instead of sequence repetition. Also, 4000 contiguous coded pictures were used in the simulation to avoid the influence of error distribution in the patterns in the simulation results.

Experimental results for both the PD based and higher QP based redundant slice coding are shown in the following figures. Sequences are coded with the same target bit rates by both of the two methods. In the following figures, PD and QP are used to represent the experimental results of the PD based and the higher QP based redundant slice coding, respectively.

Experimental results show that, in error-prone environments, PD based redundant slice coding method constantly achieves higher reconstruction quality than the common method does. Especially when PLR is increased, the gap between the two methods becomes wider and wider. Around 1dB improvement in average Y-PSNR of the reconstruction pictures can be obtained by PD based method. From the curves for PSNR against time, it can also be observed that when packet loss occurred, PD based method provides better frame by frame performance over the common method.

Fig.2 PSNR versus PLR for PD based and higher QP based redundant slice coding.

Fig.3 PSNR versus Frame number for the PD based and higher QP based redundant slice coding.

Fig. 4 PSNR versus PLR for PD based and higher QP based redundant slice coding.

Fig. 5 PSNR versus Frame number for the PD based and higher QP based redundant slice coding

Fig. 6 PSNR versus PLR for PD based and higher QP based redundant slice coding

Fig. 7 PSNR versus Frame number for the PD based and higher QP based redundant slice coding

Fig. 8 PSNR versus PLR for PD based and higher QP based redundant slice coding

Fig. 9 PSNR versus Frame number for the PD based and higher QP based redundant slice coding

Fig. 10 PSNR versus PLR for PD based and higher QP based redundant slice coding

Fig. 11 PSNR versus Frame number for the PD based and higher QP based redundant slice coding

Fig. 12 PSNR versus PLR for PD based and higher QP based redundant slice coding

Fig. 13 PSNR versus Frame number for the PD based and higher QP based redundant slice coding

- Conclusion

The PD based redundant slice coding method improves the performance of the AVC codec in error-prone environment over the common redundant slice coding method. The improvement can be seen from the 1 dB enhancement on average PSNR and 0.5 dB to 5.2 dB increase in frame by frame PSNR based on the same bit rate.

We recommend that the redundant slice should be able to contain PD based low quality content for the SVC standard

- References

[1] “Draft ITU-T recommendation and final draft standard of joint video specification” (ITU-T Rec. H.264/ISO/IEC 14496-10 AVC) in Joint Video Team (JVT) of ISO/IEC MPEG and ITU-T VCEG, JVT G050, 2003;

[2] J. Jia, and H.K. Kim, “Error resilience for SVC base and enhanced layers,” in JVT-O048, Apr. 2005;

[3] JM96.zip, http://iphome.hhi.de/suehring/tml/download/, May, 2005;

[4] Q15-I-16r1, “Error Patterns for Internet Experiments,” Stephan Wenger;

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | ETRI |  |  |
| Mailing address | Broadcasting Media Research Group Digital Broadcasting Media Research Division ETRI(Electronics and Telecommunications Research Institute) 161 Gajeong-ding, Yuseong-gu, Daejeon, 305-350, Republic of KOREA |  |  |
| Country | Republic of KOREA |  |  |
| Contact person | Haechul Choi |  |  |
| Telephone | +82-42-860-1808 |  |  |
| Fax | +82-42-860-5479 |  |  |
| Email | choihc@etri.re.kr |  |  |
| Place and date of submission | Poznan, Poland, 24-29 July, 2005 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Polyphase down sampling based redundant slice coding for SVC error resilience |  |
| Contribution number |  | JVT-P050 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks:

In this proposal, the threshold is set to 10, further investigation on the threshold need to be done in the following study.