| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 16th Meeting: Poznań, PL, 24-29 July, 2005 | Document: JVT-P051 Filename: JVT-P051 |
| --- | --- |

| Title: | An improvement on coding efficiency for SVC enhancement layer by polyphase down sampling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie JIA, Hae-Kwang KIM Multimedia Framework Lab Sejong University Seoul, 143-747, KOREA Hae-Chul CHOI, Jae-Gon KIM Broadcasting Media Research Group ETRI Daejeon, 305-350, KOREA | Tel: Email: Tel: Email: | +82 02 3408 3757 Joriejiejia@gmail.com hkkim@sejong.ac.kr +82-42-860-1808 choihc@etri.re.kr jgkim@etri.re.kr |
| Source: | Sejong Univ. and ETRI |  |  |

_____________________________

Abstract:

In this proposal, a method for improving SVC coding efficiency on the basis of polyphase down sampling (PD) is described. The presented method proposes to implement PD on residual signal before integer transformation in SVC enhancement layer, especially for SNR enhancement layer. Experiments based on JSVM2.0 are performed with different test sequences on Palma test points. Experimental results show that the presented method gives 0.08 – 0.2dB improvement over the tested sequence based on the same bit rate. It can also be observed that the presented method effectively improves the coding efficiency of FGS enhancement layer by 14% average bit rate reduction with 0.07dB PSNR loss in enhancement layer.

- Introduction

This document presents a method using PD in JSVM FGS coding process for improving coding efficiency. In SVC, the input video signal of each layer after MCTF is decomposed into a set of temporal low-pass and high-pass pictures. For each picture, after MCTF, texture information and motion information are encoded, respectively. When fine grain SNR scalability (FGS) is enabled, the quality scalability is achieved by progressive SNR refinement texture coding. The presented method proposes to implement PD on residual signal before DCT transformation in the coding process of high-pass picture for each quality enhancement layer. The proposed method can be easily implemented with simple modification to the current JSVM coding structure. Furthermore, as PD is implemented on each 8x8 inter block where 4x4 transformation is adopted, hence only one 8x8x2 Octet memory block is required for the whole PD process.

- Implementation of PD in FGS layer

The PD operation on residual signal has been proposed in [3] for error resilient coding purpose. In this document, PD is adopted for improving coding efficiency in SVC enhancement layer. In SVC, MCTF is first implemented on each spatial layer, after that, the input video signal is decomposed into high-pass and low-pass signal, then, the texture information and motion information derived from the MCTF process are coded for each low-pass and high-pass picture. In this proposed, one PD module is introduced in the coding process of FGS enhancement layer. The SVC encoder structure with the proposed PD module is shown in Fig. 1.

Fig. 1 Example of the encoder structure using PD in SNR enhancement layer coding

As the figure shown, PD module is inserted into the coding process of each spatial layer before the progressive SNR refinement texture coding and implements down sampling on the texture information which will be coded in FGS layer. Commonly, texture information includes both Intra texture and Inter texture. In this proposal, PD operation is proposed to be implemented on Inter texture, which is the residual signal obtained from the inter prediction. However, the proposed method can also be extended to Intra texture information, that will be studied later.

When the encoding process is concerned, in each spatial layer, as shown in Fig.1, the PD method will not be applied to the base layer coding. In this proposal, the PD operation is proposed to be implemented in the FGS layer coding process on each 8x8 inter residual block where 4x4 integer transform is employed. Further information on the implementation of polyphase down sampling in each 8x8 block can be referred to [3].

Above description is the implementation of the proposed method in SVC encoder. When the decoder is concerned, the inverse operation needs to be implemented in FGS layer decoding process. Corresponding to the PD operation in encoder, the polyphase upsampling is performed in decoder to the reconstructed residual signal which is obtained from the inverse integer transform. Similar to the encoder part, the polyphase upsampling will not be applied to the base layer.

- Experimental results

To evaluate the performance of the proposed method, experiments based on JSVM2.0 are performed on Palma test points using the configuration files provided along with the software. In order to get the preliminary experimental results, PD is implemented on luma residual signals. The experimental results are shown in Fig. 2 – Fig. 8.

From the preliminary results, it can be observed that the proposed method which implements polyphase down sampling in FGS coding process, achieves 0.08 -- 0.2dB PSNR improvement at high bit-rate. As the proposed method is only implemented to FGS enhancement layer, so at low bit-rate, the proposed method will have similar performance as the original JSVM2.0. In addition, in order to evaluate the effectiveness of the proposed method, a simple simulation with 1 FGS layer on QCIF format base layer is made on foreman sequence. The test condition for that is illustrated in Table 1. The curves of per-frame bits reduction on FGS layer by the proposed method is given in Fig. 8. Where the ratio of bits reduction in FGS layer is obtained by (1):

r=RP−RR×100%		(1)

Where r is the ratio of the bits reduction, negative value represents a decrease in bit rate, while positive value indicates an increase in bit rate. Rpand R represent the bits used for coding each frame in FGS layer by the proposed method and the original JSVM2.0, respectively.

Experimental results in Fig. 8 clearly show that the proposed method significantly reduced the bits used for coding each frame in FGS layer. Around 14% average bit-rate reduction can be obtained by implementing the proposed method in FGS layer with the penalty of 0.07dB quality loss.

Fig. 2 Performance comparison between the proposed method and JSVM2.0 on Bus sequence

Fig. 3 Performance comparison between the proposed method and JSVM2.0 on Foreman sequence

Fig. 4 Performance comparison between the proposed method and JSVM2.0 on Mobile sequence

Fig. 5 Performance comparison between the proposed method and JSVM2.0 on Football sequence

Fig. 6 Performance comparison between the proposed method and JSVM2.0 on City sequence

Fig. 7 Performance comparison between the proposed method and JSVM2.0 on Soccer sequence

Table 1. Test condition for analysis on FGS enhancement layer bit-rate reduction

| Scalability | SNR |
| --- | --- |
| GOP Size | 32 |
| Intra period | Only first frame |
| Base layer resolution | QCIF |
| Base layer mode | AVC compatible |
| Intra mode usage | On |
| Number of FGS layers | 1 (Δ QP = 6) |

Fig. 8 Foreman sequence frame by frame performance on bits reduction in FGS enhancement layer

- Conclusion

Compared with the original FGS coding in JSVM2.0, the proposed method significantly reduced the bit rate on FGS layer by approximately 14% with polyphase down sampling. So, we recommend that the polyphase down sampling method could be included in the FGS coding process.

- Reference

[1] J. Reichel, H. Schwarz, and M. Wien, “Working Draft 2 of ISO/IEC 14496-10:2005/AMD1 Scalable Video Coding,” in ISO/IEC JTC 1/SC 29/WG11 N7086, Apr. 2005

[2] ISO/IEC JTC1, “JSVM1.0 Software,” ISO/IEC JTC 1/SC 29/WG 11 N6900, Jan. 2005

[3] J. Jia, and H.K. Kim, “Error resilience for SVC base and enhanced layers,” in JVT-O048, Apr. 2005

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Sejong Univ. |  |  |
| Mailing address | 98 Gunja-Dong, Gwangjin-Gu Seoul, 143-747 KOREA |  |  |
| Country | KOREA |  |  |
| Contact person | Hae-Kwang Kim, Jie JIA |  |  |
| Telephone | +82 02 3408 3757 |  |  |
| Fax |  |  |  |
| Email | hkkim@sejong.ac.kr ; Joriejiejia@gmail.com |  |  |
| Place and date of submission | Poznan, Poland, 24-29 July, 2005 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | An improvement on coding efficiency for SVC enhancement layer by polyphase down sampling |  |
| Contribution number |  | JVT-P051 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: