| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 16th Meeting: Poznan, PL, 25-30 July, 2005 | Document: JVT-P068 Filename: JVT-P068.doc |
| --- | --- |

| Title: | Verification of CE 3 Samsung proposal JVT-P089 |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Woong Il Choi School of ICE, Sungkyunkwan Univ. 300 Chunchun-dong, Suwon, Korea Jung Youp Yang School of ICE, Sungkyunkwan Univ. 300 Chunchun-dong, Suwon, Korea Byeungwoo Jeon School of ICE, Sungkyunkwan Univ. 300 Chunchun-dong, Suwon, Korea | Tel: Email: | +82-31-290-7186 creata@ece.skku.ac.kr +82-31-290-7186 binbak1@ece.skku.ac.kr +82-31-290-7144 bjeon@yurim.skku.ac.kr |
| Source: | Sungkyunkwan Univ. |  |  |

_____________________________

## Summary

Sungkyunkwan University has performed verification of Samsung’s proposal, JVT-P089 for JSVC CE 3. They provided the source code based on the reference software JSVM 2.0 where their proposed scheme, symbol prediction technique is implemented. We checked that the provided source code has no problem and also verified the coded bitstreams. We tested their proposed scheme under modified Munich test condition and checked coding performance of their proposal in terms of bitrate. To obtain the bit reduction ratio of the proposed method, we tested it without rate control. The layers are decided by fixed QP value not bitrate.

## Verification data

| BUS |  | PSNR | Reference | JVT-P089 | Bit-reduction ratio |
| --- | --- | --- | --- | --- | --- |
|  | QCIF@7.5 | 29.63 | 66.31 | 66.31 | 0.00% |
|  | QCIF@15 | 29.77 | 98.84 | 98.32 | 0.53% |
|  | CIF@7.5 | 27.51 | 199.08 | 197.12 | 0.98% |
|  | CIF@15 | 30.94 | 391.80 | 388.32 | 0.89% |
|  | CIF@30 | 31.04 | 524.91 | 518.45 | 1.23% |

| Football |  | PSNR | Reference | JVT-P089 | Bit-reduction ratio |
| --- | --- | --- | --- | --- | --- |
|  | QCIF@7.5 | 32.90 | 133.79 | 133.79 | 0.00% |
|  | QCIF@15 | 32.03 | 201.76 | 201.35 | 0.20% |
|  | CIF@7.5 | 30.71 | 399.44 | 396.51 | 0.73% |
|  | CIF@15 | 32.02 | 526.15 | 519.78 | 1.21% |
|  | CIF@30 | 33.88 | 1046.41 | 1036.32 | 0.96% |

| Foreman |  | PSNR | Reference | JVT-P089 | Bit-reduction ratio |
| --- | --- | --- | --- | --- | --- |
|  | QCIF@7.5 | 32.61 | 32.14 | 32.14 | 0.00% |
|  | QCIF@15 | 32.87 | 48.04 | 47.70 | 0.71% |
|  | CIF@7.5 | 31.99 | 96.97 | 95.62 | 1.39% |
|  | CIF@15 | 34.69 | 195.62 | 193.17 | 1.25% |
|  | CIF@30 | 34.87 | 259.67 | 255.22 | 1.71% |

| Mobile |  | PSNR | Reference | JVT-P089 | Bit-reduction ratio |
| --- | --- | --- | --- | --- | --- |
|  | QCIF@7.5 | 29.27 | 50.44 | 50.44 | 0.00% |
|  | QCIF@15 | 29.37 | 68.20 | 67.81 | 0.58% |
|  | CIF@7.5 | 25.64 | 134.60 | 133.32 | 0.95% |
|  | CIF@15 | 28.98 | 263.00 | 260.80 | 0.84% |
|  | CIF@30 | 29.82 | 391.24 | 387.20 | 1.03% |

| City |  | PSNR | Reference | JVT-P089 | Bit-reduction ratio |
| --- | --- | --- | --- | --- | --- |
|  | QCIF@7.5 | 35.07 | 64.39 | 64.39 | 0.00% |
|  | QCIF@15 | 38.74 | 132.03 | 131.42 | 0.46% |
|  | CIF@15 | 33.12 | 260.51 | 258.82 | 0.65% |
|  | CIF@30 | 37.42 | 518.79 | 515.62 | 0.61% |
|  | 4CIF@30 | 34.07 | 1021.83 | 1014.77 | 0.69% |
|  | 4CIF@60 | 36.14 | 2029.86 | 2016.07 | 0.68% |

| Crew |  | PSNR | Reference | JVT-P089 | Bit-reduction ratio |
| --- | --- | --- | --- | --- | --- |
|  | QCIF@7.5 | 32.75 | 97.90 | 97.88 | 0.02% |
|  | QCIF@15 | 36.12 | 192.96 | 192.12 | 0.43% |
|  | CIF@15 | 33.54 | 386.60 | 382.60 | 1.04% |
|  | CIF@30 | 36.21 | 765.57 | 757.20 | 1.09% |
|  | 4CIF@30 | 35.60 | 1541.87 | 1517.13 | 1.60% |
|  | 4CIF@60 | 36.55 | 3055.86 | 3008.40 | 1.55% |

| Harbour |  | PSNR | Reference | JVT-P089 | Bit-reduction ratio |
| --- | --- | --- | --- | --- | --- |
|  | QCIF@7.5 | 31.18 | 96.93 | 96.93 | 0.00% |
|  | QCIF@15 | 34.11 | 194.97 | 194.06 | 0.47% |
|  | CIF@15 | 29.18 | 386.49 | 383.23 | 0.84% |
|  | CIF@30 | 32.33 | 766.97 | 760.07 | 0.90% |
|  | 4CIF@30 | 31.51 | 1544.83 | 1525.14 | 1.27% |
|  | 4CIF@60 | 33.62 | 3068.74 | 3029.90 | 1.27% |

| Soccer |  | PSNR | Reference | JVT-P089 | Bit-reduction ratio |
| --- | --- | --- | --- | --- | --- |
|  | QCIF@7.5 | 36.68 | 151.58 | 151.56 | 0.01% |
|  | QCIF@15 | 38.44 | 219.14 | 218.04 | 0.50% |
|  | CIF@15 | 33.26 | 398.03 | 394.07 | 0.99% |
|  | CIF@30 | 35.86 | 669.06 | 660.39 | 1.30% |
|  | 4CIF@30 | 34.74 | 1337.23 | 1316.81 | 1.53% |
|  | 4CIF@60 | 37.17 | 3053.48 | 3011.90 | 1.36% |

## Conclusion

To validate the implementation, we checked the source code of proponent’s software and tested it under modified Munich test condition. The verification test results show that the implementation of Samsung’s proposal is valid and their scheme has clearly coding gain with compared to JSVM 2.0.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name |  |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: