| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 16th Meeting: Poznań, PL, 24-29 July, 2005 | Document: JVT-P078 Filename: input.doc |
| --- | --- |

| Title: | Clarification of intra_base_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Doe Hyun Yoon, Byeong-Moon Jeon, Ji-Ho Park, Seung-Wook Park DM Research Lab., LG Electronics Inc., 16 Woomyeon-dong, Seocho-gu Seoul, Korea | Email: Tel: Fax: | dhyoon@lge.com jeonbm@lge.com scottie@lge.com beatitude@lge.com +82-2-526-4369 +82-2-526-4283 |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

I_BL (intra base) macroblock, which uses the scaled and reconstructed base layer signal as the predictor, is used in the current working draft of SVC [1]. Two flags could be used to indicate I_BL macroblock type; base_mode_flag and intra_base_flag. However, the usage of intra_base_flag in the current JSVM seems to be inappropriate for the single-loop decoding case. This contribution clarifies the intra_base_flag syntax in regards to inter-layer prediction constraint.

- Macroblock syntax in JSVM

The macroblock header syntax structure of SVC, when base_id_plu1 is not 0 and adaptive_prediction_flag is not 0, is depicted in Fig. 1.

Fig. 1 Macroblock syntax of SVC

If base_mode_flag is 1, then the remaining syntaxes are skipped and the macroblock type, sub-macroblock type and motion information are inferred from the base layer. base_mode_refinement_flag indicates that the quarter-pel refinement of the base layer motion vector is employed, and is used only when SpatialScalabilityType is greater than 0 and the corresponding base layer macroblock mode is not intra.

In case that neither base_mode_flag nor base_mode_refinement_flag is 1, mb_type is encoded. And if mb_type is I_NxN, intra_base_flag is used to notify whether the current macroblock is I_BL macroblock.

Therefore, we have two ways to set I_BL macroblock; (1) set base_mode_flag to be equal to 1 or (2) set intra_base_flag to be equal to 1

The first method, using base_mode_flag, is applicable only when the corresponding base layer macroblock is intra coded. On the contrary, the second method could be used regardless of the base layer macroblock type. However, using this method, for which the base layer macroblock is intra, seems to be inappropriate because it could be done by simply setting base_mode_flag to 1.

In summary, I_BL macroblock should be encoded using base_mode_flag when the base layer macroblock is intra and should be encoded using intra_base_flag when the base layer macroblock is inter.

If the constrained inter-layer prediction (also known as single loop decoding) is assumed, I_BL macroblock is allowed only when the base layer macroblock is intra.

So, considering the inter-layer prediction constraint and intra_base_flag, intra_base_flag is meaningful only for multi-loop decoding case. However, the current syntax structure uses intra_base_flag regardless of inter-layer prediction constraint, which seems to be irrelevant.

- The proposed syntax for intra_base_flag

intra_base_flag in JSVM is used as follows.

if( mb_type == I_NxN && base_id_plus1 != 0 )

intra_base_flag

It is obvious that intra_base_flag should be used only when multi-loop decoding is assumed. Thus, in order to clean up macroblock syntax structure regarding intra_base_flag, we propose one additional condition for intra_base_flag.

So, it is suggested to be modified as follows.

if( mb_type == I_NxN && base_id_plus1 != 0 && Constrained_inter_layer_prediction == 0)

intra_base_flag

The variable Constrained_inter_layer_prediction for each picture is defined as follows.

- If multi-loop decoding is assumed, 
Constrained_inter_layer_prediction is set to be equal to 0

- Otherwise ( single-loop decoding is assumed), 
Constrained_inter_layer_prediction is set to be equal to 1

By adding this condition for intra_base_flag, it is used only when multi-loop decoding is employed so that the redundant usage of intra_base_flag for single-loop decoding case is avoided. In case that this flag is not present in bitstream, it shall be inferred to be equal to 0.

More detailed description for the proposed modification to JSVM [2] could be found at the end of this contribution document.

- Conclusion

We have added additional condition on intra_base_flag so that the ambiguity regarding this flag is clarified. Based on this discussion, we suggest that it should be included in macroblock level syntax of SVC.

Reference:

- JVT-O201, “Scalable Video Coding – Working Draft 2,” Busan, April, 2005.

- JVT-O202, “Joint Scalable Video Model JSVM-2,” Busan, April, 2005.

Appendix: the proposed modification to JSVM annex.S

- add the following function in sub clause S.7.2 Specification of syntax functions, categories, and descriptors

is_constrained_inter_layer_prediction( ) is specified as follows.

- a variable result is set to 
!( key_picture_flag = = 1 && ( constrained_intra_pred_flag in the PPS of the base layer slice ) = = 0 )

Note that it could be defined alternatively as follows when multi-loop for high-pass picture could be enabled by always_decode_base_layer flag in SPSE.
!( ( key_picture_flag = = 1 && ( constrained_intra_pred_flag in the PPS of the base layer slice ) = = 0 ) || 
  ( key_picture_flag = = 0 && always_decode_base_layer = = 1 ) )

- the return value of the function is_constrained_inter_layer_prediction( ) is set equal to the value of the variable result.

- modification to sub clause S.7.3.7 Slice data in scalable extension syntax

| slice_data_in_scalable_extension( ) { | C | Descriptor |
| --- | --- | --- |
| while( !byte_aligned( ) ) |  |  |
| cabac_alignment_one_bit | 2 | f(1) |
| CurrMbAddr = first_mb_in_slice * ( 1 + MbaffFrameFlag ) |  |  |
| Constrained_inter_layer_prediction = is_constrained_inter_layer_prediction( ) |  |  |
| do { |  |  |
| if( MbaffFrameFlag && ( CurrMbAddr % 2 = = 0 ) ) |  |  |
| mb_field_decoding_flag | 2 | ae(v) |
| macroblock_layer_in_scalable_extension( ) | 2 ¦ 3 ¦ 4 |  |
| if( MbaffFrameFlag && ( CurrMbAddr % 2 = = 0 ) ) |  |  |
| moreDataFlag = 1 |  |  |
| else { |  |  |
| end_of_slice_flag | 2 | ae(v) |
| moreDataFlag = ! end_of_slice_flag |  |  |
| } |  |  |
| CurrMbAddr = NextMbAddress( CurrMbAddr ) |  |  |
| } while( moreDataFlag ) |  |  |
| } |  |  |

- modification to sub clause S.7.3.8 Macroblock layer in scalable extension syntax

| macroblock_layer_in_scalable_extension( ) { | C | Descriptor |
| --- | --- | --- |
| if( in_crop_window( CurrMbAddr ) ) |  |  |
| if( base_id_plus1 != 0 && adaptive_prediction_flag ) { |  |  |
| base_mode_flag | 2 | ae(v) |
| if( ! base_mode_flag && SpatialScalabilityType > 0 && ! intra_base_mb( CurrMbAddr ) ) |  |  |
| base_mode_refinement_flag | 2 | ae(v) |
| } |  |  |
| } |  |  |
| if( ! base_mode_flag && ! base_mode_refinement_flag ) { |  |  |
| mb_type | 2 | ae(v) |
| if( mb_type = = I_NxN && base_id_plus1 != 0 && Constrained_inter_layer_prediction = = 0 ) |  |  |
| intra_base_flag | 2 | ae(v) |
| } |  |  |
| ==== the remaining parts are identical to JSVM 2 ==== |  |  |

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name |  |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: