| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 16th Meeting: Poznań, PL, 24-29 July, 2005 | Document: JVT-P087 Filename: JVT-P087.doc NOTE – That first character after the dash up there in the document number is an "O", not a zero. |
| --- | --- |

| Title: | PFGS with bi-linear interpolation filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woo-Jin Han Bae-Keun Lee Yeongtong-Gu Suwon-City Gyeonggi-Do South Korea Digital Media R&D Center Samsung Electronics Co., Ltd. | Email: | wjhan.han@samsung.com baekeun.lee@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

Compared to the current implementation of FGS technique in JSVM, progressive fine grain scalability (PFGS) uses the FGS layer as the reference frames to improve the coding efficiency. However, the computational complexity is the major burden of PFGS since motion compensation is needed for each FGS layer. In this contribution, we slightly modify the scheme for estimating the FGS residual in the PFGS scheme for the robustness of the interpolation filter effects. In addition, we apply the less complex bi-linear interpolation filter in the FGS layer to reduce the overall complexity of PFGS-based schemes. From the experimental results, it can be shown that the use of bi-linear interpolation filter does not degrade the PSNR values in the FGS layer, and the modified residual estimation process provides up to 7% bit-saving compared to the direct PFGS residual computation process.

# Motivation

JSVM2 [1] includes Fine Grain Scalability (FGS) feature for encoding the residue between the spatial transform coefficients of the original picture in a SNR-scalable way, which is obtained after the MCTF of the corresponding spatial layer. Each quality enhancement layer contains a refinement signal that corresponds to a bisection of the quantization step size. FGS performance is not optimum in JSVM2, since only using base-layer’s temporal information. But JVT-O054 and some researchers already have proposed the new scheme to enable the prediction from the neighboring FGS layers. In addition, leaky prediction for restricting the drift effect due to the truncation of FGS layers was proposed. For the simplicity's sake, Let this kinds of algorithm category be the progressive fine grain scalability (PFGS). It is well-known that PFGS outperforms other FGS techniques since it utilizes the temporal correlation between FGS layers. But the one of the main problem of PFGS is its heavy complexity requirements, since additional motion compensation is needed to utilize the updated temporal information of reference frames for each layer. In this contribution, we would like to propose the use of the bilinear interpolation filter for the motion compensation in the FGS layer and a different equation to cancel the interpolation effects for reducing the computational burden in PFGS-based schemes.

# Modified residual estimation process of PFGS

## Residual estimation of PFGS

In the current implemenation of JSVM2, the residual of the FGS layer RF is defined by the difference between the residual signal R and the quantized version from the lower layer RB as

RF=R−RB. 						(1)

Equation (1) can be rewritten by using the original and prediction signals as

RF=O−PB−RB 					  (2)

where O is the original signal and PB is the prediction signal at the lower-layer. It should be noted that the prediction signal at the base-layer, PB, is used instead of the current layer prediction signal. Progressive fine grain scalability (PFGS) technique enables us to use the prediction signal at the FGS layer. If we assume that the prediction signal of the current layer, PF, is used, the residual of the FGS layer RF can be written as:

RF=O−PF−RB. 					 (3)

Equation (3) is very similar to the residual prediction in the multi-layer approach. It indicates that the most simplified form of the PFGS can be represented by the multi-layer residual prediction technique. Since Equation (2) only utilizes the base-layer texture information whereas Equation (3) utilizes the base-layer texture as well as the temporal correlation between temporal neighbours, PFGS naturally provides the better performance compared to the normal FGS schemes. However, the prediction from the FGS layer may produce the drift error when the neighbouring FGS layer is truncated. This type of drift error can be reduced by the leaky prediction method proposed in JVT-O054. In this proposal, we do not use the leaky prediction for focusing the interpolation filter performance.

## Application of bi-linear interpolation to PFGS process

In Equation (3), the base-layer residual RB can be divided into two terms: OB and PB, which are the reconstructed lower-layer signal and the reconstructed low-layer prediction signal, respectively. By reorganization of Equation (3) can be rewritten yields

RF=(O−OB)−(PF−PB) 					(4)

which means that the PFGS residual can be derived from the difference between the intra-base residual and the prediction differences. The first parenthesis term in Equation (4) can be computed without any motion compensation process. The second parenthesis term in Equation (4), which means the difference between two prediction signals, is more robust compared to Equation (3) due to that the interpolation error can be cancelled together, since both terms have the similar pattern of the interpolation errors. We propose Equation (4) to compute the PFGS residual and apply the bi-linear interpolation filter to reduce the computational complexity. That is, the bi-linear interpolation is applied for computing PF and PB in Equation (4).

# Experimental results

Due to the late release of JSVM2, JSVM1 software was used as the baseline. We use one base-layer of Qp 40 and three FGS layers for QCIF 15Hz, CIF 30Hz, and 4CIF 60Hz. There are four systems to be compared as the followings: We will present QCIF and CIF cases for CIF sequences and CIF and 4CIF cases for 4CIF sequences.

- Anchor: JSVM1 software

- PFGS: JSVM1 software with modified FGS scheme using Equation (3)

- PFGS-B: JSVM1 software with modified FGS scheme using Equation (3) and bi-linear filter

- FPFGS-B: JSVM1 software with modified FGS scheme using Equation (4) and bi-linear filter

Figure 1 PSNR curves of BUS sequence.

As shown in Figure 1, PFGS-based system clearly outperforms the current FGS implementation, however, if leaky scheme is used, the performance gap may be reduced. Therefore, the direct comparision compared to the anchor is not much meaningful. It can be proved if we can merge our scheme to Nokia's leaky predicton technique. The use of bi-linear filter slightly improves the performance while the modified residual computation can reduce the bit-rates up to 7% with virtually same PSNR compared to the PFGS-B.

Figure 2 PSNR curves of Foreman sequence.

Figure 3 PSNR curves of Football sequence.

In Football sequence, the PSNR gain is not large compared to other sequences. It is mainly due to the fact that the temporal correlation of this sequence is relatively weak, thus the schemes using the temporal correlation such as PFGS do not have much advantages.

Figure 4 PSNR curves of Mobile sequence.

In Mobile sequence, the original PFGS scheme using original interpolation filter shows better performance due to the high-texture nature of the sequence. However, the modified residual estimation process can reduce the performance gap from the use of bi-linear interpolation filter.

Figure 5 PSNR curve of City sequence.

It is interesting that the bi-linear interpolation filter improves the performance in 4CIF resolution of City sequence. Furthermore, the modified residual estimation process is better than the original PFGS with bi-linear interpolation filter.

Figure 6 PSNR curve of Crew sequence.

Figure 7 PSNR curve of Harbour sequence.

Figure 8 PSNR curve of Soccer sequence.

# Conclusion

From these results, it can be concluded that the use of bi-linear filter does not degrade the performance, and it even improves the PSNR value by up to 0.5 dB with the much low complexity. Furthermore, the modified residual estimation process can reduce the bit-rates up to 8% with the similar PSNR values compared to the unmodified one. Thus the combination of the modified residual estimation process and the bi-linear interpolation filter can be an effective tool for the SNR scalability of JSVM.

References

- Julien Reichel, Heiko Schwarz, and Mathias Wien, "Joint Scalable Video Model JSVM-2," 15th JVT meeting, JVT-O202, Busan, Korea.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Samsung Electronics |  |  |
| Mailing address | 416, Maetan 3-dong, Paldal-gu, Suwon-si, Gyeonggi-do, 442-742 |  |  |
| Country | Korea |  |  |
| Contact person | Woo-Jin Han |  |  |
| Telephone | +82-31-200-9319 |  |  |
| Fax | +82-31-200-3427 |  |  |
| Email | wjhan.han@samsung.com |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | PFGS with bi-linear interpolation filter |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: