| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 16th Meeting: Poznań, PL, 24-29 July, 2005 | Document: JVT-P089 Filename: input.doc |
| --- | --- |

| Title: | CE 3. Improve coding efficiency of entropy coding - Improved coding scheme of residual prediction flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bae-Keun Lee Yeongtong-Gu Suwon-City Gyeonggi-Do, Rep of Korea Digital Media R&D Center Samsung Electronics Co., Ltd. | Email: | baekeun.lee @samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Introduction

To improve the coding efficiency of SVC (Scalable Video Coding), we propose symbol prediction technique to reduce the bit-rates of residual prediction flag in JSVM 2.0 [1] . We suggested that change residual prediction flag coding process to improve coding efficiency [3] [4]

# Improved coding scheme of residual prediction flag

According to the current JSVM2 implementation, residual_pred_flag is set to one if the base-layer residual is used for predicting the current inter residual. However, if the base-layer residual in the macroblock has no non-zero pixel, the coding of residual_pred_flag is not required. Thus we encode reversed residual prediction flag instead of the residual_pred_flag itself to increase number of '0'. It provides some performance gain when the VLC is used instead of CABAC. Figure 1 shows this case.

This process can be summarized as:

- Skip coding of residual_pred_flag if base-layer residual has no non-zero pixels.

- Code reversed residual prediction flag (= 1-resiudal_pred_flag )

It should be noted that this kind of prediction technique is especially useful when VLC-based coder is used since the new technique generates more zeros than the original one. Fig. 1 shows the proposed residual_pred_flag coding process.

Furthermore, CABAC model is used same as BLSKIP flag. It gives more bit-rate saving than that of residual prediction flag.

Figure 1 Coding process of residual_pred_flag

# Decoding Process and Syntax and semantics modifications

## Decoding process

Decoding process can be summarized as:

- If base-layer’s energy is zero, residual prediction flag is inferred as `0`

- If bas-layer’s energy is non-zero, parsing the reversed_residual_pred_flag and set residual_pred_flag to (1- reversed_residual_pred_flag)

## Syntax and semantics modification

Syntax changes:

Change residual_pred_flag as reversed_residual_pred_flag

Semantics:

Reversed_residual_pred_flag represents a reversed symbol with residual_pred_flag. If base-layer’s energy(excluding FGS layer’s energy) is zero, residual_pred_flag can be inferred `0`.

# Experimental Results

We used 4 CIF and 4 4CIF sequences and we simulated two test conditions, Munich and Spatial scalability. The configuration is based on Munich configuration except the inter-layer prediction option is set to 2 (adaptive prediction) for all layers since the proposed method only works for these cases. The proposed method was integrated into JSVM2.0 software and the result was compare to those of JSVM2.

## Munich Test Condition

Since our methods do not change the PSNR values, exact same Qp values used for generating the anchor results are re-applied to generate the PSNR curves of the proposed method, thus all PSNR values are the same and only bit-rate reduction can be shown. As you can see, about 0.4-3.3 % bit-rate reductions in each layer can be observed for almost all test points.

| Anchor | Bit-rate Reduction | Net gain | Reduction ratio in each layer |
| --- | --- | --- | --- |
| 516.15 | N/A |  |  |
| 386.82 |  |  |  |
| 193.73 |  |  |  |
| 95.28 |  |  |  |
| ResPred |  |  |  |
| 509.63 | -6.52 | -3.09 | -2.45% |
| 383.40 | -3.43 | -1.57 | -0.82% |
| 191.88 | -1.86 | -1.38 | -1.42% |
| 94.80 | -0.48 | -0.47 | -1.51% |

Table 1 Bit-rate saving analysis of Bus sequence

| Anchor | Bit-rate Reduction | Net gain | Reduction ratio in each layer |
| --- | --- | --- | --- |
| 1027.01 | N/A |  |  |
| 508.83 |  |  |  |
| 384.51 |  |  |  |
| 193.79 |  |  |  |
| ResPred |  |  |  |
| 1016.74 | -10.27 | -3.85 | -0.75% |
| 502.41 | -6.42 | -3.57 | -2.96% |
| 381.66 | -2.85 | -2.61 | -1.39% |
| 193.55 | -0.24 | -0.24 | -0.37% |

Table 2 Bit-rate saving analysis of Football sequence

| Anchor | Bit-rate Reduction | Net gain | Reduction ratio in each layer |
| --- | --- | --- | --- |
| 258.15 | N/A |  |  |
| 193.84 |  |  |  |
| 96.10 |  |  |  |
| 47.56 |  |  |  |
| ResPred |  |  |  |
| 253.86 | -4.30 | -2.04 | -3.27% |
| 191.58 | -2.26 | -1.02 | -1.05% |
| 94.86 | -1.24 | -0.92 | -1.93% |
| 47.23 | -0.32 | -0.32 | -2.08% |

Table 3 Bit-rate saving analysis of Foreman sequence

| Anchor | Bit-rate Reduction | Net gain | Reduction ratio in each layer |
| --- | --- | --- | --- |
| 385.69 | N/A |  |  |
| 256.37 |  |  |  |
| 128.76 |  |  |  |
| 64.10 |  |  |  |
| ResPred |  |  |  |
| 381.88 | -3.81 | -1.71 | -1.34% |
| 254.28 | -2.10 | -0.96 | -0.76% |
| 127.63 | -1.13 | -0.80 | -1.26% |
| 63.78 | -0.33 | -0.33 | -2.08% |

Table 4 Bit-rate saving analysis of Mobile sequence

| Anchor | Bit-rate Reduction | Net gain | Reduction ratio in each layer |
| --- | --- | --- | --- |
| 2029.10 | N/A |  |  |
| 1018.64 |  |  |  |
| 516.51 |  |  |  |
| 258.23 |  |  |  |
| 129.02 |  |  |  |
| ResPred |  |  |  |
| 2015.18 | -13.92 | -6.86 | -0.68% |
| 1011.57 | -7.06 | -3.88 | -0.78% |
| 513.33 | -3.19 | -1.47 | -0.57% |
| 256.51 | -1.71 | -1.11 | -0.86% |
| 128.42 | -0.61 | -0.61 | -0.94% |

Table 5 Bit-rate saving analysis of City sequence

| Anchor | Bit-rate Reduction | Net gain | Reduction ratio in each layer |
| --- | --- | --- | --- |
| 3069.06 | N/A |  |  |
| 1545.32 |  |  |  |
| 767.72 |  |  |  |
| 386.57 |  |  |  |
| 193.17 |  |  |  |
| ResPred |  |  |  |
| 3031.11 | -37.95 | -18.38 | -1.22% |
| 1525.75 | -19.57 | -13.39 | -1.75% |
| 761.54 | -6.18 | -3.53 | -0.93% |
| 383.92 | -2.65 | -2.30 | -1.21% |
| 192.82 | -0.35 | -0.35 | -0.36% |

Table 6 Bit-rate saving analysis of Harbour sequence

| Anchor | Bit-rate Reduction | Net gain | Reduction ratio in each layer |
| --- | --- | --- | --- |
| 3057.06 | N/A |  |  |
| 1543.28 |  |  |  |
| 764.89 |  |  |  |
| 386.81 |  |  |  |
| 191.52 |  |  |  |
| ResPred |  |  |  |
| 3009.68 | -47.38 | -22.70 | -1.52% |
| 1518.59 | -24.69 | -16.59 | -2.18% |
| 756.79 | -8.10 | -3.90 | -1.04% |
| 382.61 | -4.20 | -3.33 | -1.74% |
| 190.66 | -0.87 | -0.87 | -0.92% |

Table 7 Bit-rate saving analysis of Crew sequence

| Anchor | Bit-rate Reduction | Net gain | Reduction ratio in each layer |
| --- | --- | --- | --- |
| 3089.84 | N/A |  |  |
| 1528.98 |  |  |  |
| 767.08 |  |  |  |
| 381.02 |  |  |  |
| 191.23 |  |  |  |
| ResPred |  |  |  |
| 3047.93 | -41.91 | -18.90 | -1.23% |
| 1505.97 | -23.01 | -14.43 | -1.93% |
| 758.49 | -8.58 | -4.28 | -1.12% |
| 376.72 | -4.31 | -3.25 | -1.74% |
| 190.17 | -1.06 | -1.06 | -1.12% |

Table 8 Bit-saving analysis of Soccer sequence

As shown in the previous eight tables, the bit-rate reduction ratio in each layer is up to 3.3% without any changes of PSNR values. Although the bits allocated to these symbols are not large, clearly better coding efficiency can be achieved by the proposed methods. Furthermore, if VLC is used instead of CABAC, the prediction approach of the proposed method can be more beneficial since more zero symbols will be used.

## Spatial Scalability Test condition

We simulated in spatial scalability test condition with R-D cost estimation. As you can see, about 0-0.1dB PSNR gain can be observed for almost all test points. Some sequence (see Foreman, City, Harbour) clearly improved by proposed prediction scheme of residual prediction flag. The following figures show simulated results.

Figure 2 PSNR curves of Bus CIF@30Hz sequence.

Figure 3 PSNR curves of Football CIF@30Hz sequence.

Figure 4 PSNR curves of Foreman CIF@30Hz sequence.

Figure 5 PSNR curves of Mobile CIF@30Hz sequence.

Figure 6 PSNR curves of CITY CIF@30Hz sequence.

Figure 7 PSNR curves of CITY 4CIF@30Hz sequence.

Figure 8 PSNR curves of Harbour CIF@30Hz sequence.

Figure 9 PSNR curves of Harbour 4CIF@30Hz sequence.

Figure 10 PSNR curves of Soccer CIF@30Hz sequence.

Figure 11 PSNR curves of Soccer 4CIF@30Hz sequence.

Figure 12 PSNR curves of Crew CIF@30Hz sequence.

Figure 13 PSNR curves of Crew 4CIF@30Hz sequence.

# Conclusion

We propose symbol prediction techniques to reduce the bit-rates of residual prediction flag. The bit-rates can be clearly reduced about 1-7% of total bit-rates in Munich test condition. In spatial scalability test condition, we can achieve PSNR gain up to about 0.1dB. So we conclude that proposed method is efficient in residual prediction flag.

In previous meeting, someone pointed out that proposed method has problem, due to mismatch between encoder and decoder, when if an enhancement layer is coded on top of an FGS Layer. In that case, we only use base-layer’s energy with excluding base-layer’s FGS picture energy.

# Reference

[1] JVT-O202, “JSVM 2.0 document,” Busan, April., 2005.

[2] JVT-O201, “Scalable Video Coding – Working Draft 2,” Busan, April., 2005.

[3] JVT-O303, “CE 3. Improve coding efficiency of entropy coding,” Busan, April., 2005.

[4] JVT-O063, “Symbol prediction techniques for SVC,” Busan, April., 2005.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Samsung Electronics |  |  |
| Mailing address | 416, Maetan 3-dong, Paldal-gu, Suwon-si, Gyeonggi-do, 442-742 DM R&D Center, Samsung Electronics. |  |  |
| Country | Korea |  |  |
| Contact person | Bae-Keun Lee |  |  |
| Telephone | +82-31-200-3409 |  |  |
| Fax | +82-31-200-3427 |  |  |
| Email | baekeun.lee@samsung.com |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Improved coding scheme of residual prediction flag |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: