| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 16th Meeting: Poznań, PL, 24-29 July, 2005 | Document: JVT-P090 Filename: input.doc |
| --- | --- |

| Title: | Adaptive FGS process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bae-Keun Lee Woo-Jin Han Yeongtong-Gu Suwon-City Gyeonggi-Do, Rep of Korea Digital Media R&D Center Samsung Electronics Co., Ltd. | Email: | baekeun.lee@samsung.com wjhan.han@samsung.co.kr |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Introduction

JSVM2.0 [1] includes Fine Grain Scalability (FGS) for encoding the residue between the spatial transform coefficients of the original picture obtained after the MCTF of the corresponding spatial layer and their reconstructed base layer representation (or the subordinate enhancement layer representation). Each quality enhancement layer contains a refinement signal that corresponds to a bisection of the quantization step size. If base-layer macroblock represents an intra macroblock, the current macroblock is set to Intra-BL- intra macroblock with prediction from base layer in inter-layer coding process. But FGS coding process is very similar with residual prediction coding process. However, it is efficient to apply Intra-BL coding process in intra macroblock in FGS layer. Our main proposal is to modify FGS residual with the residual between original and reconstructed base layer picture in low-pass frames.

# Adaptive FGS coding process

## Motivation

In current JSVM2.0, FGS coding process is conceptually same as residual prediction coding process except refinement with transform coefficients.

Therefore FGS residual is roughly expressed as following:

(OF−PB)−RB,                                    (1)

where OFis the original picture and PB is prediction signal obtained from base layer and RB is the residual signal, respectively. In JSVM2, there are two inter-layer coding methods, one is residual prediction, which is determined by R-D cost function, and the other is Intra-BL coding, which uses base-layer macroblock (when mb_mode is intra). FGS coding process is similar to residual prediction coding process. But it seems reasonable to use intra-BL coding process when base layer macroblock represents intra macroblock, since residual between original and reconstructed base-layer is smoother compared to the residue in JSVM2.

The following equation represents Intra-BL coding

OF−OB=OF−D(PB+RB),                            (2)

where D denotes the deblocking filter specified in AVC. Furthermore, lowpass frame’s residual have many block artifacts due to directional intra prediction and temporally longer reference frame. In that case , prediction from base-layer is a reasonable choice instead of residual prediction coding.

## Proposed Method

If we use smoothed residual for FGS layer coding in low-pass frames, we can achieve bit-rate saving at a comparable quality. Therefore we propose an adaptive FGS process as explained below.

- Encode OF−OB if current frame is low-pass picture, OB is the reconstructed base layer picture.

- Encode same residue as specified in JSVM2, otherwise.

Intra-BL coding process in a low-pass frame is more effective than residual prediction coding process because base-layer macroblock type is intra. If we use same deblocking strength as JSVM2 after FGS layer encoding, it might cause over-smoothing effect, since base layer reconstructed picture is already deblocked. Therefore we deblock strength to 1 for all macroblocks after FGS layer encoding. i.e. JSVM specifies a certain deblocking strength in Intra-BL, but we set it to 1 to prevent over-smoothing effect.  Fig 1 shows the proposed FGS encoding process.

Figure 1 Proposed FGS coding process

# Experimental Results

We used four CIF and two 4CIF sequences and we simulated Intra picture only configuration (GOP size=1), since our methods has no change in H frame. The proposed method was integrated into JSVM1.0 software [2] and the results were compared to those of JSVM1.0 software. In our simulations the FGS layer encoding option was set to be in the range from 0.1 to 2.0. We achieved a bit-rate saving of about 1.3-7% in FGS layer 1 and about 2-7% in FGS layer 2 respectively.

Truncation performance is comparable in FGS layer 1 and better in FGS layer 2. But some performance drop was observed in some cases – most of them occurred when the FGS option was under 0.3. However, the overall performance improved compared to the current JSVM1.0. The following figures show the experimental results. As you can see, we can achieve more bit-rate savings as the number of FGS layer is increased.

Figure 2 Foreman CIF 15Hz

Figure 3 Bus CIF 15Hz

Figure 4 Football CIF 15 Hz

Figure 5 Mobile CIF 15 Hz

Figure 6 Soccer 4CIF 30 Hz

Figure 7 Harbour 4CIF 30 Hz

# Conclusion

We propose an adaptive FGS process that encodes the residue between original picture and reconstructed base-layer in low-pass frames. We can achieve a bit-rate saving of about 1.3%-7% in FGS layer 1 and about 2-7% in FGS layer 2 compared to the JSVM 1.0 using Intra frame only configuration.

The proposed adaptive FGS coding process outperformed JSVM1, as FGS layer setting is greater than 0.3. But the PSNR values were slightly decreased for all test sequences by the proposed method, when the FGS option is set under 0.3. However an overall performance gain was achieved. We will investigate mode adaptive or MB adaptive technique and incorporate an adaptive method between residual prediction and intra-BL coding process with in an R-D cost estimation process.

# Reference

[1] JVT-O202, “JSVM 2.0 document,” Busan, April., 2005.

[2] JVT-N024, “JSVM 1.0 software,” Hong Kong, Jan., 2005.

[3] JVT-O201, “Scalable Video Coding – Working Draft 2,” Busan, April., 2005.

(Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Samsung Electronics |  |  |
| Mailing address | 416, Maetan 3-dong, Paldal-gu, Suwon-si, Gyeonggi-do, 442-742 DM R&D Center, Samsung Electronics. |  |  |
| Country | Korea |  |  |
| Contact person | Bae-Keun Lee |  |  |
| Telephone | +82-31-200-3409 |  |  |
| Fax | +82-31-200-3427 |  |  |
| Email | baekeun.lee@samsung.com |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Adaptive FGS process |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: