| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 16th Meeting: Poznań, PL, 24-29 July, 2005 | Document: JVT-P095 Filename: input.doc |
| --- | --- |

| Title: | Locally adaptive quantization: concept and first results |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Thomas Rusert, Mathias Wien RWTH Aachen University Melatener Str. 23 52074 Aachen, Germany | Tel: Email: | +49/241/8027679 rusert@ient.rwth-aachen.de |
| Source: | RWTH Aachen University |  |  |

_____________________________

# Abstract

We present a concept for bit rate allocation with locally adaptive quantization for the JSVM. The error-propagation characteristics within the MCTF scheme are considered on a macroblock basis by modified computation of the distortion measure used for rate-distortion optimization. In an analytic approach, a performance gain of up to 0.3 dB is observed for four decomposition levels. In a model-based approach, a performance gain of up to 0.2 dB is observed for one decomposition level.

# 1 Motivation

Bit rate allocation is important for optimization of video coding systems. Roughly speaking, the problem is how to allocate the available bit rate among different macroblocks in order to maximize the quality of the a given video sequence after decoding. As a solution to this problem, Lagrangian rate-distortion optimization techniques are commonly used, aiming at minimization of the cost function

JMB=DMB+λRMB	(1)

for each macroblock MB, where DMB is the distortion associated with the macroblock, RMB is the corresponding amount of bits, and λ is the Lagrange factor, which can be fixed for all macroblocks if they affect the reconstruction approximately independently.

One main challenge in rate-distortion optimization is to consider the dependencies between different macroblocks. Exemplarily, in closed-loop/predictive coding, the rate allocation for a specific macroblock will not only affect the distortion of the corresponding reconstructed macroblock, but it will also affect the prediction quality and thus the rate-distortion performance of subsequently coded macroblocks.

In open-loop systems such as the JSVM, the dependencies between macroblocks are different, since encoder and decoder do not use identical references. Specifically, for inter blocks in the MCTF-structure of the JSVM, the encoder uses unquantized references, which will be quantized when referred to at the decoder. Due to the iterative temporal decomposition, quantization of a specific macroblock thus leads to controlled propagation of the quantization error during reconstruction. Error propagation is restricted by the temporal filter lengths and heavily depends on the decomposition structure and the temporal subband to be quantized. Typically, quantization errors applied to subband frames at coarser temporal decomposition levels will affect a larger number of samples after reconstruction. This demonstratively justifies that coarser temporal subbands should be quantized more accurately. In the JSVM encoder, this is considered by choosing appropriate λ factors on a frame basis, while DMB is measured as distortion between the unquantized macroblock and the quantized macroblock.

Depending on the motion information, the error propagation characteristics can also vary within a given subband frame. Especially for sequences with inhomogeneous motion, certain macroblocks in a subband frame may correspond to longer motion trajectories, while other macroblocks in the same subband frame may be associated with short trajectories. Equal quantization of the respective macroblocks will then cause different amounts of quantization noise in the reconstructed sequence. Our preliminary investigations indicate that considering the locally varying error propagation characteristics during rate-distortion optimization can improve the overall coding performance of the JSVM.

# 2 Concept

To optimize the reconstruction quality at the decoder, we modify the cost function (1) at the encoder to

JMB=DMBrec+λRMB,	(2)

now considering the distortion DMBrec after temporal reconstruction. At the same time, we fix the value λ for all temporal subbands and macroblocks. Then rate-distortion optimization is performed in the same manner as in JSVM1.

We derive DMBrec using two different approaches,

- computation by explicit reconstruction (analytic approach),

- estimation as a function of DMB and local motion parameters (model-based approach).

While the first approach is computationally very complex, it provides an upper bound for the achievable gains brought by locally adaptive quantization. The second approach can be used to approximate these gains with low complexity. It can be employed for encoder optimization, generating a rate-distortion optimized deltaQp value for each macroblock. Alternatively, if the motion parameters used for derivation of DMBrec are also available at the decoder, then the quantization can be adapted without transmission of side information.

# 3 First Results

## 3.1 Simulation Settings

In our experiments we use JSVM1 in single layer mode (single spatial layer, fixed QP, QPmode = QPresidual, no FGS layers). We also extend the function MbEncoder::encodeIntra to support selection of deltaQp values unequal to zero, which is originally not supported in the JSVM1 encoder. Then deltaQp values unequal to zero can be assigned to any macroblock in both H and L frames. The adaptive update step is enabled in all experiments.

## 3.2 Analytic Approach

We derive DMBrec by explicit reconstruction. In our analytic experiments we use a GOP size of 16 and a maximum absolute deltaQp of 3. We compare three different settings.

| DMB, intraDeltaQp=off | Unmodified JSVM1 |
| --- | --- |
| DMB, intraDeltaQp=on | JSVM1 with deltaQp support for all macroblocks |
| DMBrec, intraDeltaQp=on | DMBrec derived by explicit reconstruction |

Results are exemplarily depicted in Figures 1, 2. First we observe that introduction of deltaQp support for intra blocks does not significantly improve the performance in the standard JSVM1. Second, while use of the analytically derived values DMBrec leads to slight gains for sequences with homogeneous motion (e.g. Figure 2), we observe PSNR gains of up to 0.3 dB for sequences with inhomogeneous motion (e.g. Figure 1).

## 3.3 Model-based Approach

We now estimate DMBrec as a function of DMB and local motion parameters, employing a simple error propagation model. We use a simple decomposition structure with a GOP size of 2 and a maximum absolute deltaQp of 6. We compare the following settings. For all settings, deltaQp is supported for all macroblocks.

| DMB | Unmodified JSVM1 |
| --- | --- |
| DMBrec model | DMBrec estimated using motion parameters |
| DMBrec model, deltaQpBypass=on | bit rate for transmission of deltaQp values not considered |

Results are exemplarily depicted in Figures 3, 4. We observe for all tested sequences that for the simple decomposition structure, use of the simple DMBrec model does not lead to performance gains if the deltaQp values need to be transmitted. Since the motion parameters used for estimation of DMBrec are also available at the decoder, the deltaQp values can hypothetically also be derived at the decoder. We consider this by disregard the transmission of the deltaQp values, observing performance gains of up to 0.2 dB for sequences with inhomogeneous motion.

# 4 Conclusion

We have presented a concept and first results of a bit rate allocation scheme with locally adaptive quantization for the JSVM. The results obtained from the analytic approach show performance gains of up to 0.3 dB. Using the model-based approach and a simple temporal decomposition structure, we obtain performance gains of up to 0.2 dB. Higher performance gains may be expected for higher spatial resolutions and more complex decomposition structures.

Figure 1: Example for locally adaptive quantization using analytic approach.

Figure 2: Example for locally adaptive quantization using analytic approach.

Figure 3: Example for locally adaptive quantization using model-based approach.

Figure 4: Example for locally adaptive quantization using model-based approach.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | RWTH Aachen University |  |  |
| Mailing address | Melatener Str. 23 52074 Aachen |  |  |
| Country | Germany |  |  |
| Contact person | Thomas Rusert |  |  |
| Telephone | +49/241/8027679 |  |  |
| Fax | +49/241/8022196 |  |  |
| Email | rusert@ient.rwth-aachen.de |  |  |
| Place and date of submission | Aachen, 2005-07-20 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Locally adaptive quantization: |  |
| Contribution number |  | JVT-P095 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| x | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| x | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: