| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 16th Meeting: Poznań, PL, 24-29 July, 2005 | Document: JVT-P112 Filename: input.doc |
| --- | --- |

| Title: | Revised syntax for FGS pass fractionning |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s: | Stephane Pateux France Telecom Division R&D 4, rue du clos Courtel. BP 59 35 512 Cesson Sevigne. FRANCE Ye-Kui Wang Nokia Research Center Visiokatu 1 33720 Tampere, Finland Peter Amon Siemens AG, Corporate Technology CT IC 2, 81730 Munich, Germany | Tel: Email: | +33 2 99 12 41 77 stephane.pateux@francetelecom.com +358 50 486 7004 ye-kui.wang@nokia.com +49 (89) 636 54642 p.amon@siemens.com |
| Source: | France Telecom Division R&D, Nokia Corporation, Siemens AG |  |  |

_____________________________

# Abstract

This document presents a revised syntax for FGS pass fractioning to JVT-P031 as agreed during Poznan meeting.

# Introduction

This document presents a revised syntax for FGS pass fractioning to JVT-P031 as agreed during Poznan meeting.

A fragmented progressive refinement NAL unit is signaled thanks to a is_fragmented flag. For such fragmented progressive refinement a slice_id field is used in order to identify unambiguously the slice a fragment refers to. Further fragment_order field and last_fragment flag are introduced for ordering fragments.

# Proposed syntax modifications

S.7.2 Specification of syntax functions, categories, and descriptors

next_nalu_not_belongs_to_current_PRF ( )  is specified as follows (PRF stands for "Progressive Refinement Fragment") :

this function scans the following NAL unit and check whether it belongs to the progressive refinement of the current slice or not (return 1 if it does not belongs to same PRF otherwise 0). This can be implemented by parsing the syntax elements of next NAL unit (quality_level and  slice_id if available).

Remark: the introduction of this function is a trick we propose at the syntax level in order to specify when "progressive_refinement_slice_data_in_scalable_extension" should be called. This kind of trick may also be a solution for defining when to launch inverse MCTF for reconstruction (this point is not still clear in the syntax as you have pointed out). This concept is quite similar to what is done currently in JSVM where such decoding is launched on the next NAL processing. In this proposed case, we perform the decoding during the current NAL processing thanks to a simple parse (only NAL header parsed) of the following NAL. This does not imply additional delay and lead to a simpler syntax.

Remark For this new syntax function it is necessary to have NAL units associated to fragments of a progressive refinement slice that are adjacent and in an increasing fragment_order.

swap_buffers ( cBuffer0, cBuffer1 )  is specified as follows.

-	It swaps the two buffers. At the end of this process, cBuffer0 contains the datas previously contained in cBuffer1 , and cBuffer1contains the datas previously contained in cBuffer0.

## S.7.3 Syntax in tabular form

#### S.7.3.2.12	Slice layer in scalable extension RBSP syntax

| slice_layer_in_scalable_extension_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| slice_header_in_scalable_extension( ) | 2 |  |
| if ( slice_type != PR ) |  |  |
| slice_data_in_scalable_extension( ) | 2 ¦ 3 ¦ 4 |  |
| else { |  |  |
| if ( fragmented_flag = = 1) { |  |  |
| if ( fragment_order = = 0 ) |  |  |
| NumBytesInPRF = 0 |  |  |
| while( !byte_aligned( ) ) |  |  |
| cabac_alignment_one_bit | 2 | f(1) |
| while( more_rbsp_data( ) ) |  |  |
| rbtmp_byte[ NumBytesInPRF++ ] | 2 ¦ 3 ¦ 4 | b(8) |
| if ( (last_fragment_flag = = 1 ) ¦ ¦ next_nalu_not_belongs_to_current_PRF ) |  |  |
| swap_buffers( rbtmp_byte, rbsp_byte ) |  |  |
| } |  |  |
| if ( ( fragmented_flag = = 0 ) ¦ ¦ last_fragment_flag = = 1 ) ¦ ¦ next_nalu_not_belongs_to_current_PRF ) ) |  |  |
| progressive_refinement_slice_data_in_scalable_extension( ) |  |  |
| } |  |  |
| rbsp_slice_trailing_bits( ) | 2 |  |
| } |  |  |

### S.7.3.6	Slice header in scalable extension syntax

| slice_header_in_scalable_extension( ) { | C | Descriptor |
| --- | --- | --- |
| slice_type | 2 | ue(v) |
| if ( slice_type = = PR ) { |  |  |
| fragmented_flag | 2 | b(1) |
| if ( fragmented_flag = = 1 ) { |  |  |
| fragment_order | 2 | ue(v) |
| if ( fragment_order != 0) |  |  |
| last_fragment_flag | 2 | b(1) |
| } |  |  |
| if ( ( fragmented_flag = = 0 ) ¦ ¦ ( fragment_order = 0 ) ) |  |  |
| first_mb_in_slice | 2 | ue(v) |
| if ( fragmented_flag = = 1 ) |  |  |
| slice_id | 2 | ue(v) |
| frame_num | 2 | u(v) |
| if ( ( fragmented_flag = = 0 ) ¦ ¦ ( fragment_order = 0 ) ) { |  |  |
| [remaing informations from slice_header_in_scalable_extension] |  |  |
| } |  |  |
| } |  |  |

## S.7.4 Semantics

#### S.7.2.4.8	Slice layer without partitioning RBSP semantics

The specification of this subclause in AVC shall apply.

cabac_alignment_one_bit has the same semantics as cabac_alignment_one_bit in subclause Error: Reference source not found.

rbtmp_byte[ i ]  is the i-th byte of a progressive_refinement_slice_data_in_scalable_extension( ) payload, it does not include the slice headers.  The rbtmp_byte[ ] is used to append sub-parts of the slice data of a progressive refinement slice before parsing its syntax elements.

### S.7.4.6	Slice header in scalable extension semantics

fragmented_flag  equal to 1 specifies that the current NAL unit is fractioned.  fragmented_flag equal to 0 specifies that the current NAL unit is not fractioned. If not present it should be inferred to the 0 value.

When fragmented_flag is equal to 1, the NAL unit cannot be parsed independently; in this case the RBSP bytes of the slice data of all NAL units with identical values of first_mb_in_slice, slice_type, pic_parameter_set_id, num_mbs_in_slice_minus1, luma_chroma_sep_flag, and fragmented_flag shall be stored in a temporary RBSP byte buffer in increasing order of fragment order, and the parsing process is started when a NAL unit with last_fragment_flag equal to 1 is received or when the next NAL unit is not a NAL unit with identical values of first_mb_in_slice, slice_type, pic_parameter_set_id, num_mbs_in_slice_minus1, luma_chroma_sep_flag, and fragmented_flag.

fragment_order  specifies the order in which the NAL units with fragmented_flag equal to 1 shall be ordered before the parsing process is started. If not present it should be inferred to the 0 value.

A NAL unit with fragmentOrder = fragment_order greater than 0 shall immediately follow a NAL unit with identical values of first_mb_in_slice, slice_id, slice_type, pic_parameter_set_id, num_mbs_in_slice_minus1, luma_chroma_sep_flag, and fragmented_flag, and a value of fragment_order equal to fragmentOrder – 1.

last_fragment_flag  equal to 1 specifies that the current NAL unit is the last fragment of a progressive refinement slice and that the parsing process can be started.  last_fragment_flag equal to 0 specifies that zero or more NAL units containing fraction of the current progressive refinement slice may follow.

If not present last_fragment_flag is inferred to be 0 for a fragmented progressive refinement slice with fragment_order equal to 0 and inferred to be 1 otherwise.

slice_id specify an unambiguous identifier for a slice. slice_id is only relevant inside a layer (specified by dependency_id) of an access unit. If not present it should be infered to be equal to first_mb_in_slice.

base_id_plus1  minus 1 specifies the value of dependency_id, quality_level and fragment_order for base pictures that are used for inter-layer prediction of coding mode, motion, samples values, and/or residual values of the current slice. base_id_plus1 equal to 0 specifies that no inter-layer prediction (of coding mode, motion, sample value, and/or residual prediction) is used for the current slice. base_id_plus1 greater than 0 specifies that an inter-layer prediction (of coding mode, motion, sample value, and/or residual prediction) may be used for the current slice when signalled in the macroblock layer.

If base_id_plus1 is greater than 0, the variables DependencyIdBase, QualityLevelBase and FragmentOrderBase are derived as follows.:

DependencyIdBase = ( base_id_plus1 – 1 ) >> 4
	QualityLevelBase   = (( base_id_plus1 – 1 )>>2) & 3.

FragmentOrderBase   = ( base_id_plus1 – 1 ) & 3.

Otherwise (base_id_plus1 is equal to 0), DependencyIdBase and QualityLevelBase are set equal to –1.

[ remaining unchanged]

Remark

In this syntax proposal, following conditions regarding stream ordering are assumed to be true:

-	All NAL units of the same slice are consecutive for a given dependency_id and must be ordered by non decreasing QualityLevel (This assumption is related to NAL ordering considering the adopted AU semantic).

-	All FGS fractions of a progressive refinement slice must be ordered by increasing value of priority_id

This ordering is necessary in order to use the next_nalu_belongs_to_new_PRS syntax function introduced in order to control the decoding process.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name |  |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: