| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 17th Meeting: Nice, FR, 14-21 October, 2005 | Document: JVT-Pxxx Filename: input.doc |
| --- | --- |

| Title: | Meeting Report, Draft 0 |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary J. Sullivan Thomas Wiegand Jens-Rainer Ohm | Tel: Email: Tel: Email: Tel: Email: | +1 (425) 703-5308 garysull@microsoft.com +49 30 31002 617 wiegand@hhi.de |
| Source: | JVT |  |  |

_____________________________

# Abstract

The Joint Video Team (JVT) of ITU-T Q.6/16 and ISO/IEC JTC 1/SC 29/WG 11 held its 17th meeting during 14-21 October 2005 in Nice, France. The JVT meeting was held under the chairmanship of Dr. Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany), and under the associate chairmanship of Dr. Thomas Wiegand (Fraunhofer HHI/Germany) and Dr. Ajay Luthra (Motorola/USA). The JVT meetings opened at approximately 9:30 on Friday October 14 2005 and closed at ??:?? on Friday 21 October 2005.  Approximately ??? people attended the JVT meetings (as recorded on a sign-in sheet passed at the meeting) and approximately ??? input documents were discussed.  The meetings took place in a co-located fashion with a meeting of ISO/IEC JTC 1/SC 29/WG 11 (MPEG).  The subject matter of these activities consisted of work on video coding.

# Contents

1.	Abstract	1

2.	Contents	2

3.	Documents of the JVT meeting	2

3.1.	Input documents	2

3.2.	Major output documents	4

3.3.	JVT internal output documents	4

3.4.	SVC core experiment output documents	5

4.	JVT Administrative topics	5

4.1.	Administrative documents	5

4.2.	IPR policy reminder	5

4.3.	Late documents	5

5.	JVT Scalable video coding work	6

5.1.	CE1 & related docs: Update step	6

5.2.	CE 2 & related docs: Inter-layer motion prediction	6

5.3.	CE 3 & related docs: Inter-layer texture prediction	8

5.4.	CE 4 & related docs: ROI	11

5.5.	CE 5 & related docs: Quantization	12

5.6.	CE 6 & related docs: Extended spatial scalability	12

5.7.	CE 7 & related docs: Low-delay FGS	12

5.8.	CE 8 & related docs: CAVLC	12

5.9.	Improved FGS coding	13

5.10.	Improved entropy coding	13

5.11.	High-Level Syntax	13

5.12.	Error Resiliency	14

5.13.	Profiles	15

6.	SVC non-normative modifications	15

7.	JVT Non-SVC normative modifications	16

7.1.	Improved 4:4:4 coding	16

7.2.	Logo insertion	17

8.	JVT Non-SVC non-normative aspects	17

8.1.	Encoder optimization techniques	17

8.2.	Improved Error resilience	17

8.3.	Reference Software & Test Material	18

9.	Withdrawn JVT documents	18

10.	Updating of JVT group documents	18

11.	List of Adoptions	18

11.1.	Normative SVC adoptions	18

11.2.	Non-Normative SVC adoptions	18

11.3.	Normative non SVC-related adoptions	18

11.4.	Non-Normative non SVC-related adoptions	18

12.	List of AHGs established	18

13.	Resolutions of the meeting	18

14.	SW integration plan	18

15.	Attendance	18

# Documents of the JVT meeting

## Input documents

JVT-Q000 List of documents of Nice meeting

JVT-Q001 [G.J. Sullivan, J.-R. Ohm, A. Luthra, T. Wiegand] AHG Report: Proj mgmt and errata

JVT-Q002 [T. Wiegand, K. Suehring, A. Tourapis, K.P. Lim] AHG Report: JM text and ref soft

JVT-Q003 [T. Suzuki, L. Winger] AHG Report: Bitstreams & conformance

JVT-Q004 [Withdrawn] Withdrawn

JVT-Q005 [G. Cook] AHG Report: JSVM s/W and new func. integ.

JVT-Q006 [J. Reichel, H. Schwarz, M. Wien] AHG Report: JSVM & WD text

JVT-Q007 [G. Sullivan] AHG Report: Spatial scalability resampling

JVT-Q008 [Y.-K. Wang] AHG Report: Err resil test cond's & apps

JVT-Q009 [M. Wien, H. Schwarz] AHG Report: Coding eff & JSVM perf test cond

JVT-Q010 [T. Suzuki] AHG Report: Study of 4:4:4 functionality

JVT-Q011 [L. Xiong] AHG Report: Enh. slice complexity reduction

JVT-Q012 [G.J. Sullivan, J.-R. Ohm, A. Luthra, T. Wiegand] Meeting report of Poznan meeting

JVT-Q013 [E. Francois, J. Vieron, V. Bottreau] Additional results on SVC ESS evaluation

JVT-Q014 [J. Vieron, E. Francois, V. Bottreau] Additional test conds for SVC EsS JSVM Perf Eval

JVT-Q015 [E. Francois, J. Vieron, V. Bottreau] General. SVC ESS for inter-layer ratios >2

JVT-Q016 [Withdrawn] Withdrawn

JVT-Q017 [Z. Lu, Z. Li, S. Rahardja, X. Lin, K.P. Lim, W. Lin, J. Zheng, W. Yao] On high-level syntax for SVC ROI

JVT-Q018 [Z. Lu, Z. Li, S. Rahardja, X. Lin, K.P. Lim, W. Lin, J. Zheng, W. Yao] Progress report of SVC CE4: ROI

JVT-Q019 [P. Bordes, E. Francois] SEI for logo insertion

JVT-Q020 [D. Ichimura, Y. Honda, M.H. Lee] Slice group map for mult. interactive ROI scal.

JVT-Q021 [D. Ichimura, Y. Honda, M.H. Lee] Stream struct and CABAC for mult. inter. ROI scal.

JVT-Q022 [J. Reichel, X. Ziliani, P. Amon, X. Boltz, U. Benzler] Surveillance profile for SVC

JVT-Q023 [T. Suzuki] Some COR issues

JVT-Q024 [T. Suzuki] Verif. of new 4:4:4 tools

JVT-Q025 [E. Setton, B. Girod] R-D analysis and streaming of SP and SI frames

JVT-Q026 [M. Visharam] Study of effect of MCTF update step

JVT-Q027 [J. Xin, A. Vetro, H. Sun] Verif. of 4:4:4 in JVT-P017

JVT-Q028 [P. Yin, J. Boyce, P. Pandit] High-level SVC Syntax

JVT-Q029 [P. Yin, J. Boyce, P. Pandit] FMO and ROI scalability

JVT-Q030 [H. Schwarz, D. Marpe, T. Wiegand] Closed loop coding with quality layers

JVT-Q031 [M. Winken, H. Schwarz, D. Marpe, T. Wiegand] Adapt. motion refinement for FGS slices

JVT-Q032 [S. Sekiguchi, Y. Isu, Y. Yamada, K. Asai, T. Murakami] Improved entropy coding for intra pred. modes

JVT-Q033 [Y. Isu, S. Sekiguchi, Y. Yamada, K. Asai, T. Murakami] Verif. on existing Adv. 4:4:4 proposals

JVT-Q034 [I.H. Shin and H.W. Park] CE3: Adapt. upsampling

JVT-Q035 [Y.L. Lee] Complexity of prop. lossless intra for 4:4:4

JVT-Q036 [D. Hench, P. Topiwala] Joint source-channel encoding

JVT-Q037 [P. Topiwala] Prop. 14-b sample support in Adv. 4:4:4 Pr

JVT-Q038 [S. Sun] New results on direct interp. for upsamp. in ESS

JVT-Q039 [Y. Bao, M. Karczewicz] CE7: FGS for low delay

JVT-Q040 [J. Ridge, M. Karczewicz] CE8: VLCs for SVC

JVT-Q041 [Withdrawn] Withdrawn

JVT-Q042 [A. Tourapis] Updates to reference software manual

JVT-Q043 [K.H. Lee] CE2: Revision of interlayer mot pred & TDM

JVT-Q044 [H. Choi] Quantization for improved FGS coding

JVT-Q045 [K.H. Lee] CE3: Verif. of NEC JVT-Q048

JVT-Q046 [Y. Chen, J. Boyce, K. Xie] SVC frame loss concealment

JVT-Q047 [T. Kimoto] CE2: Verif.

JVT-Q048 [T. Kimoto, Y. Miyamoto] CE3: Unrestricted temporal decomposition in SVC

JVT-Q049 [L. Xiong] Enh. layer intra pred using lower layer

JVT-Q050 [L. Xiong] Enh. layer intra pred modes

JVT-Q051 [L. Xiong] Spatial upsampling filter

JVT-Q052 [L. Xiong] Adaptive intra slice coding

JVT-Q053 [J. Jia, H.K. Kim, H.C. Choi, J.G. Kim] CE3: Polyphase downsample bef DCT for SVC FGS

JVT-Q054 [J. Jia, H.K. Kim, H.C. Choi, J.G. Kim] Implementation of redundant pictures in JSVM

JVT-Q055 [J. Jia, H.K. Kim, H.C. Choi, J.G. Kim] Polyphase downsample-based redundant pics

JVT-Q056 [J. Jia, H.K. Kim, H.C. Choi, J.G. Kim] CE5: Verif of Tool5 results

JVT-Q057 [B.K. Lee] VLC for 8x8 transform FGS

JVT-Q058 [W.-S. Kim, D. Cho, D. Birinov, H.M. Kim] Verif and analysis of JVT-P017 on 4:4:4 profile

JVT-Q059 [W.-S. Kim, D. Cho, D. Birinov, H.M. Kim] Advanced residual color transform

JVT-Q060 [I.H. Shin, H.W. Park] CE3: Verif of Sejong/JVT-Q053

JVT-Q061 [I.H. Shin, H.W. Park] CE3: Verif of Samsung/JVT-Q062

JVT-Q062 [S.-Y. Kim] CE3: Smoothed reference prediction

JVT-Q063 [W.-J. Han] CE5: Distribution-based dequant for SVC

JVT-Q064 [Y.-K. Wang, M.M. Hannuksela, S. Wenger] SVC DPB management

JVT-Q065 [Y.-K. Wang, M.M. Hannuksela] Enhancement-layer IDR picture ("EIDR")

JVT-Q066 [Q. Shen, H. Li, Y.-K. Wang] Showcase of non-required picture SEI message

JVT-Q067 [H. Liu, H. Li, Y.-K. Wang] Showcase of scalability information SEI message

JVT-Q068 [Y. Guo, C. Zhu, H. Li, Y.-K. Wang] Error resilient implementations for JSVM

JVT-Q069 [Y. Guo, H. Li, Y.-K. Wang] SVC/AVC loss simulator donation

JVT-Q070 [Y.-K. Wang, S. Wenger] SVC RTP payload format internet draft

JVT-Q071 [J.-H. Park, S.-W. Park, B.-M. Jeon] Context modeling of residual prediction flag

JVT-Q072 [J.-H. Park, S.-W. Park, B.-M. Jeon] Inter-layer pred for temporally enh pictures

JVT-Q073 [J.-H. Park, S.-W. Park, B.-M. Jeon] Modified temporal direct mode for SVC

JVT-Q074 [Withdrawn] Withdrawn

JVT-Q075 [T.C. Thang, T.M. Bae, J.W. Kang, Y.M. Ro, J.-G. Kim] Improved scalability info SEI message

JVT-Q076 [T.C. Thang, T.M. Bae, J.W. Kang, Y.M. Ro, J.-G. Kim] Boundary handling for ROI scalability

JVT-Q077 [T.C. Thang, T.M. Bae, J.W. Kang, Y.M. Ro, J.-G. Kim] Showcase of ROI extraction using scal info SEI

JVT-Q078 [C.-X. Zhang, L. Yu, L. Xiong] Simplification of upsampling for IntraBL mode

JVT-Q079 [A. Tourapis, P. Topivala] Sub-pel ME for enh. predictive zonal search

JVT-Q080 [Z. Heng] R-D function of H.264/AVC transf. coeffs.

JVT-Q081 [I. Amonou, N. Cammas, S. Kervadec, S. Pateux] Layered quality opt. of JSVM-3 & closed-loop

JVT-Q082 [D. Marpe, V. George, H. Kirchhoffer, T. Wiegand] 4:4:4 intra using adapt. color transform

JVT-Q083 [A. Segall ] Up-/Down-sampling for Spatial SVC

JVT-Q084 [Y. Libo, C. Ying, Z. Jiefu, Z. Feng] Low complexity intra pred for enh layer

JVT-Q085 [J.-Z. Xu] CE7: Core experiment 7 proposal

JVT-Q086 [T. Wedi] Intra-only 4:4:4 profile

JVT-Q087 [F. Pauchet] Prop SVC bit error resilience test conds

JVT-Q088 [X. Xu, Y. He] Dynamic search range for ME in JM

JVT-Q089 [X. Xu, Y. He] Comments on ME in current JM software

JVT-Q090 [P. Topiwala, H. Yu] New Viper HD test sequences

JVT-Q091 [S. Jeong, M. Park, G. Park, K Kim, J. Hong] Numbering of Key pictures

## Major output documents

(Dates listed are planned dates of availability.  Note that document JVT-P001 should also be considered output documents.)

##### JVT-Q200 Meeting report of the 17th JVT meeting [05/??/??]

##### JVT-Q201/N???? Working Draft 4 of Scalable Video Coding [05/??/??]

##### JVT-Q202/N???? Joint Scalable Video Model (JSVM) 4 [05/??/??]

##### JVT-Q203/N???? JSVM 4 Software [05/??/??]

## JVT internal output documents

##### JVT-Q205 Testing conditions for coding efficiency proposals [05/??/??]

##### JVT-Q206 Test conditions for error resiliency proposals [05/??/??]

## SVC core experiment output documents

# JVT Administrative topics

## Administrative documents

##### JVT-Q001 [G.J. Sullivan, J.-R. Ohm, A. Luthra, T. Wiegand] AHG Report: Proj mgmt and errata

##### JVT-Q002 [T. Wiegand, K. Suehring, A. Tourapis, K.P. Lim] AHG Report: JM text and ref soft

##### JVT-Q003 [T. Suzuki, L. Winger] AHG Report: Bitstreams & conformance

##### JVT-Q005 [G. Cook] AHG Report: JSVM s/W and new func. integ.

AHG report has been noted. SW integration not finished. 4 weeks estimated to be left for the work. Short test set for SW integration. Revisit.

##### JVT-Q006 [J. Reichel, H. Schwarz, M. Wien] AHG Report: JSVM & WD text

AHG report has been noted. JSVM 3 and WD 3 text versions have been apporved.

##### JVT-Q007 [G. Sullivan] AHG Report: Spatial scalability resampling

##### JVT-Q008 [Y.-K. Wang] AHG Report: Err resil test cond's & apps

##### JVT-Q009 [M. Wien, H. Schwarz] AHG Report: Coding eff & JSVM perf test cond

AHG report has been noted. Work will continue including scope of providing anchors for CE experiments. Post bit streams on ftp site in compressed form.

##### JVT-Q010 [T. Suzuki] AHG Report: Study of 4:4:4 functionality

##### JVT-Q011 [L. Xiong] AHG Report: Enh. slice complexity reduction

##### JVT-Q012 [G.J. Sullivan, J.-R. Ohm, A. Luthra, T. Wiegand] Meeting report of Poznan meeting

## IPR policy reminder

Participants were reminded of the IPR policies established by the parent organizations of the JVT and were referred to the parent body web sites for further information.

Participants were particularly reminded of the need to supply a completed JVT IPR status reporting form in all technical proposals for normative standardization.

## Late documents

No objections / objections were voiced to the consideration of the late documents.  Documents listed in this report in italics and with a "-L" suffix to their document numbers were classified as late. Those documents marked with [Not available at first meeting day] will only be considered as information documents at the end of the meeting if time permits.

JVT-Q001 [G.J. Sullivan, J.-R. Ohm, A. Luthra, T. Wiegand] AHG Report: Proj mgmt and errata

JVT-Q002 [T. Wiegand, K. Suehring, A. Tourapis, K.P. Lim] AHG Report: JM text and ref soft

JVT-Q003 [T. Suzuki, L. Winger] AHG Report: Bitstreams & conformance

JVT-Q006 [J. Reichel, H. Schwarz, M. Wien] AHG Report: JSVM & WD text

JVT-Q007 [G. Sullivan] AHG Report: Spatial scalability resampling

JVT-Q008 [Y.-K. Wang] AHG Report: Err resil test cond's & apps

JVT-Q009 [M. Wien, H. Schwarz] AHG Report: Coding eff & JSVM perf test cond

JVT-Q011 [L. Xiong] AHG Report: Enh. slice complexity reduction

JVT-Q012 [G.J. Sullivan, J.-R. Ohm, A. Luthra, T. Wiegand] Meeting report of Poznan meeting

JVT-Q018 [Z. Lu, Z. Li, S. Rahardja, X. Lin, K.P. Lim, W. Lin, J. Zheng, W. Yao] Progress report of SVC CE4: ROI

JVT-Q039 [Y. Bao, M. Karczewicz] CE7: FGS for low delay

JVT-Q042 [A. Tourapis] Updates to reference software manual

JVT-Q044 [H. Choi] Quantization for improved FGS coding [Not available at first meeting day]

JVT-Q058 [W.-S. Kim, D. Cho, D. Birinov, H.M. Kim] Verif and analysis of JVT-P017 on 4:4:4 profile

JVT-Q059 [W.-S. Kim, D. Cho, D. Birinov, H.M. Kim] Advanced residual color transform

JVT-Q061 [I.H. Shin, H.W. Park] CE3: Verif of Samsung/JVT-Q062  [Not available at first meeting day]

JVT-Q068 [Y. Guo, C. Zhu, H. Li, Y.-K. Wang] Error resilient implementations for JSVM  [Not available at first meeting day]

JVT-Q079 [A. Tourapis, P. Topivala] Sub-pel ME for enh. predictive zonal search

JVT-Q082 [D. Marpe, V. George, H. Kirchhoffer, T. Wiegand] 4:4:4 intra using adapt. color transform  [Not available at first meeting day]

JVT-Q085 [J.-Z. Xu] CE7: Core experiment 7 proposal  [Not available at first meeting day]

JVT-Q088 [X. Xu, Y. He] Dynamic search range for ME in JM

JVT-Q089 [X. Xu, Y. He] Comments on ME in current JM software

# JVT SVC normative modifications

## CE1 & related docs: Update step

##### JVT-Q026 [M. Visharam] Study of effect of MCTF update step

In this contribution a summary of a study about the role and effects of the update stage within the MCTF process in the JSVM encoding/decoding framework is provided. The studies indicates that disabling the update step at the decoder does not degrade the visual quality and PSNR performance of the decoded video, when compared to the case of “update on”. Also a few tests to study the effect of update step at very high qualities are performed as well. At very high qualities (QP ~ 0) a difference in PSNR between update On vs. Off at the decoder is noticed. No difference in terms of subjective visual quality are observed.

Update step

Pros:

Sometimes provides coding efficiency gains

Reduces fluctuations when omitting update step in open-loop coding (not the case in closed-loop coding)

Pre-processor provides these gains for PSNR from 30-45 dB

Cons:

Bit-depth issues affects memory bandwidth

Reference picture buffering

Delay issues

Complexity increase

Interlaced

Remove the update step from the WD starting with WD4.

Make update step a non-normative tool in JSVM 4 not necessarily being compatible with all features of the standard (e.g. interlaced).

## CE 2 & related docs: Inter-layer motion prediction

##### JVT-Q043 [K.H. Lee] CE2: Revision of interlayer mot pred & TDM

Revision of inter layer motion prediction and AVC TDM (Temporal Direct Mode) relative to JVT-O058 and JVT-P075. This contribution is based on closed loop analysis encoding and has two kind of new technical proposals. One is revision of inter layer motion prediction and the other is revision of AVC TDM.

JVT-Q043's temporal direct mode modification aspect

See notes below.  Does not appear especially promising considering the small gain in compression performance versus the necessary re-design of existing tools and added complexity.

JVT-Q043's inter-layer motion prediction aspect

See notes below.  Adds another motion prediction mode.  No indication of significant gain.  Does not appear especially promising.

##### JVT-Q047 [T. Kimoto] CE2: Verif.

Verification of JVT-Q043. As a verification task, implementation inspection and coding performance check were carried out.

No sufficient support for adoption.

##### JVT-Q072 [J.-H. Park, S.-W. Park, B.-M. Jeon] Inter-layer pred for temporally enh pictures

This document presents the experimental results of CE2 which aims to improve coding efficiency for temporally enhanced pictures. In this document, the performance of the proposed method is compared with that of temporal/spatial direct mode which is selected by RD-optimization at slice layer. Experimental result shows a gain up to 0.2dB (in case of CIF sequence) against the RD-optimized temporal/spatial direct mode.

Something for temporal enhancement pictures (when no corresponding base layer picture).

Compares to "JSVM 3.0" – but JSVM 3.0 doesn't exist yet – perhaps this point of comparison is the current CVS code – ver 2.2.

JVT-Q043 tested a different proposed method for a similar issue.  JVT-Q043 compared against JSVM 2.1.  Up to 3.7% bit rate savings was shown in JVT-Q043 (including both revision of inter-layer motion prediction and revision of temporal direct mode – the latter having max 1.5% gain).  Most tested sequences showed some improvement.  Average not reported (but appears very small).

JVT-Q047 is a verification of JVT-Q043.

Some gain (up to 0.2 dB on CIF, up to 0.3 dB on 4CIF) shown in some cases.

##### JVT-Q073 [J.-H. Park, S.-W. Park, B.-M. Jeon] Modified temporal direct mode for SVC

##### Temporal direct mode is allowed to be used at every high pass picture in SVC structure. Problem that mvCol points to the picture located at temporally long distance from the current picture. Proposal to extend temporal direct mode with slight modification to solve this issue.

Differs from what is tested/proposed in JVT-Q043, JVT-Q047, and JVT-Q072.

Modifies process to reference a closer picture in time rather than the picture referenced by the co-located MB.

Basically adds a third direct mode that can be chosen in addition to the temporal and spatial direct modes.

Tested in JSVM 2.2.

Compared to temporal direct always.  Remark: What if spatial direct would be better?

No R-D improvement shown.  The proponent indicates that further study may show benefits. Further contributions encouraged provided that these benefits can be proven (no CE).

## CE 3 & related docs: Inter-layer texture prediction

##### JVT-Q062 [S.-Y. Kim] CE3: Smoothed reference prediction

Single-loop decoding is a very useful technique to reduce the decoding complexity in the multi-layer structure, however, the restrictions of the intra-base prediction sometimes degrades the performance especially for the fast-motion sequences. The earlier proposed smoothed reference technique that based on the residual prediction process to simulate the intra-base prediction by adding the smoothing function to the prediction signal with consideration of the residual prediction process. In this proposal, a new prediction mode is presented, which utilizes the actual downsampling function to generate the smoothed prediction signal. It downsamples the prediction signal composed of the inter-mode prediction of the current layer and the intra-mode prediction of the base-layer to simulate the decoded base-layer signal. The proposed method is reported to reduce blocking artifacts from residual predicted macroblocks. PSNR improvements in fast-motion sequences such as Football and Soccer sequences are reported within the context of the low-complexity decoding mode.

Method does smoothing of the prediction signal prior to motion compensating it requiring processing of the reference picture.

##### JVT-Q061 [I.H. Shin, H.W. Park] CE3: Verif of Samsung/JVT-Q062

Results have been verified.

##### JVT-Q034 [I.H. Shin and H.W. Park] CE3: Adapt. upsampling

This document presents the technical description and experimental results for the adaptive up-sampling considered in CE3 with details in the JVT-P045. In the current JSVM, the I_BL macroblock mode uses an up-sampling process for providing the prediction signal of the current spatial layer. Current up-sampling filter for JSVM uses six-tap interpolation, which has good performance. An adaptive filtering method for the luminance component in the type-1 DCT up-sampling is proposed, which applies different weighting parameters to DCT coefficients. Since the decoder requires the weighting parameters of up-sampling matrix, the weighting parameters of horizontal and vertical up-sampling are transmitted.

##### Very small bit-rate savings for Inter frame coding cases.  Larger gains for Intra frame coding cases.

##### JVT-Q049 [L. Xiong] Enh. layer intra pred using lower layer

The enhancement layer intra directional prediction is claimed to be improved by using lower layer information when some samples are not available.

No gain. Contribution noted.

##### JVT-Q050 [L. Xiong] Enh. layer intra pred modes

JVT-P041 proposes that only 3 intra directional prediction modes which are DC, Horizontal and Vertical are used in the enhancement layer. This proposal gives the further experimental results for GOP=1 and GOP=16. (no results)

Proposal to use only 3 spatial prediction modes instead of 9 in I_BL mode: PSNR loss of 0.1 dB at very high rates (PSNR >50 dB), typically no differences observed below 45 dB. Could be useful to reduce encoder complexity, not much impact on decoder.

##### JVT-Q051 [L. Xiong] Spatial upsampling filter

Since the human visual system is less sensitive to chrominance than to luminance, a different upsampling filter can be used on luminance and chrominance for the I_BL mode. The proposal uses a simple upsample filter [-1 5 5 -1]/8 to replace the original 6 taps filter on chrominance. And for residual prediction, the proposal uses the filter [-1 5 5 -1]/8 as the residual upsampling filter on both luminance and chrominance, or uses the filter [-1 5 5 -1]/8 as the residual upsampling filter on luminance and the filter [1 1]/2 on chrominance. Very small differences in the results have been reported. No separate testing on Intra vs. Inter frame coding.

For case of I_BL upsampling JVT-Q078 gives more generic results. For case of residual prediction upsampling, usage of 4-tap filter does not give any significant advantage. Contribution noted.

##### JVT-Q052 [L. Xiong] Adaptive intra slice coding

Every slice of intra frame is encoded twice. First, the slice is encoded as usual. Second, the original slice is rotated 180 degree then the rotated result is encoded.

Results only given for Football and Bus. Irrelevant improvement in PSNR (<<0.1 dB at high rates).

##### JVT-Q053 [J. Jia, H.K. Kim, H.C. Choi, J.G. Kim] CE3: Polyphase downsample bef DCT for SVC FGS

In this proposal, a method for improving SVC coding efficiency on the basis of polyphase downsampling (PD) is described. The presented method proposes to implement PD on FGS residual signal before integer transformation in SVC enhancement layer coding process. Experiments based on JSVM3.0 were performed using the JVT common testing conditions. Experimental results are reported to show that the presented method gives 0 – 0.1dB PSNR improvement on luma component and outperforms 0.35 – 0.96 dB on chroma components compared to the current JSVM.

##### JVT-Q056 [J. Jia, H.K. Kim, H.C. Choi, J.G. Kim] CE3: Verif of Tool5 results

This document reports the cross-check results for the proposal for CE3 Tool5 as described in JVT-Q034. The verification has been performed by decoding the coded bitstreams which are provided by KAIST. Both of the PSNR against the original sequence and the bit-rate according to the file size have been verified. It is shown that the RD results obtained by decoding the provided bitstreams match the experimental results presented in JVT-Q034 quiet well. Verification results are shown in verification_CE3_T5.xls.

##### JVT-Q060 [I.H. Shin, H.W. Park] CE3: Verif of Sejong/JVT-Q053

##### JVT-Q078 [C.-X. Zhang, L. Yu, L. Xiong] Simplification of upsampling for IntraBL mode

In the current JSVM, a 6-tap interpolation filter (1, -5, 20, 20, -5, 1)/32 is used in the up-sampling process of IntraBL mode coding both for luma and chroma components and the complexity of this interpolation is even higher than the half-pel interpolation of reference frames in AVC. Considering that the spatial correlation may be different from the temporal correlation, experiments are carried out and it is reported that the complexity can be reduced without obvious penalty by using more simple interpolation filters.

IPR statement missing.

Intra_BL upsample uses [1, -5, 20, 20, -5, 1] filter for upsampling both luma and chroma

Reports that bilinear is not good enough for luma

But advocates simplifying luma to [-1, 5, 5, -1] and simplifying chroma to [-1, 5, 5, -1] or bilinear [1, 1].

Remark: Should analyze chroma impact.

Remark: Test conditions say "FRExt mode: No".  Why?  Off in common conditions.  All-intra coding, since this is testing Intra_BL upsampling impact.

Remark: We must have better generation of test sequences before drawing any firm conclusions on such issues.  Sequences that were downsampled using the JSVM method do not use any appropriate alignment of chroma to luma, and sequences that were downsampled using the MPEG-4 VM method (or perhaps it was the MPEG-2 TM method) do not have appropriate alignment with the upsampling of chroma in our current JSVM decoder design (except perhaps in "ESS" mode).

Remark: Based on results in AHG report, perhaps OK to interpret results according to this experiment for the sequences that were generated using the JSVM downsampling approach.  Look at whether the chroma resampling impact is different on those sequences in the tests versus the other sequences.

Remark: Look at ESS case and its interaction with "non-ESS" case.  That needs to be fixed.

Verification?  Not yet.

Designate Shijun Sun to coordinate side activity to resolve these issues.

Revisit.

##### JVT-Q083 [A. Segall ] Up-/Down-sampling for Spatial SVC

This contribution investigates the upsampling and down-sampling operation utilized for spatial scalability.  The problem of upsampling/down-sampling is posed as an optimization problem, and two “optimal” designs for the upsampling filter are derived.  The investigation begins by considering the down-sampling process currently utilized for SVC testing.  It then identifies application specific weaknesses of this down-sampler and explores an alternative down-sampling approach.  Results motivate a modification to the current upsampler.  Specifically, the upsampler should filter the pixels co-located in the high- and low-resolution frames.  This extends the current method that interpolates missing pixels.

Considers upsampler as and estimation problem dependent on downsampler.  Analyzes using Wiener filter design concept, uses RLS optimization for separable design optimization.  Tests with different fidelities.  Concludes that current 6-tap filter is good (relative to other hypothetical 6-tap filter) for upsampling video that was downsampled using the current JSVM downsampling filter.

The current upsampler is an "interpolator" – every other sample horizontally and vertically is simply copied rather than filtered.  (One out of 4 in the 2-d grid are copied.)

Reports that the current upsampler is not optimal in that sense.  RLS optimal upsampler design seems to depend on the fidelity of the video.  RLS upsampler differs from one-tap design now used in JSVM.

Investigates current (13-tap) downsampler.  Reports that current downsampler introduces some ringing and is rather long.

Suggests considering alternatively a shorter downsampling filter, perhaps with less ringing.  e.g., a Gaussian filter design may be an interesting choice to consider.

Actually the best downsampling method to use may be an application-dependent issue.

If desire to use and "interpolating" upsampler, the current upsampler design is reportedly fine (ignoring luma/chroma alignment issues).

Suggests creating a parameterized upsampler design.

Remark: How much does the parameterization help?  Not really quantified, but proponent says significant.

Max average gain of 1.8% by parameterized non-interpolating upsampler relative to current interpolating upsampler when using current downsampler.

However, with a Gaussian downsampler, up to 10% average improvement, overall average across different fidelities roughly 6% average improvement.

Perceptual?

Core experiment: Consider down/up-sampling pairs and alignment issues, incl. ESS, perceptual issues, etc.

##### JVT-Q084 [Y. Libo, C. Ying, Z. Jiefu, Z. Feng] Low complexity intra pred for enh layer

This document presents a low complexity intra prediction method for spatial enhancement layer. In current JSVM, intra prediction is implemented by using three intra modes Intra4x4, Intra8x8, Intra16x16, together with I_BL in enhancement layer. Based on the claim that Intra8x8 and Intra16x16 have almost no impact on the coding efficiency in enhancement layer, it is proposed to remove these two modes to reduce the complexity of intra coding. At the same time, the corresponding syntax bits can be saved. Simulation results show that compared with the current JSVM, this low complexity approach keeps the same performance.

##### JVT-Q048 [T. Kimoto, Y. Miyamoto] CE3: Unrestricted temporal decomposition in SVC

This contribution presents the proposal for SVC CE3 on “Inter-layer texture prediction”. In Unrestricted Temporal Decomposition (UTD), MC structure is decoupled between a base and enhancement layers. The pictures of a GOP in a base layer are encoded first, preceded by the corresponding pictures in enhancement layers. UTD achieves low computational complexity and small memory at a base layer while keeping comparable coding performance at higher layers. In the experimental results, when the base layer has IPPP structure, UTD is reported to outperform a short GOP in all layers by up to 3.0 dB at higher layers.

## CE 4 & related docs: ROI

##### JVT-Q017 [Z. Lu, Z. Li, S. Rahardja, X. Lin, K.P. Lim, W. Lin, J. Zheng, W. Yao] On high-level syntax for SVC ROI

This contribution consists of two parts. One is on the geometrical relationship between two successive spatial resolutions, which is based on JVT-O008. The other is on the prediction between syntaxes in picture parameter set and slice header that are related to ROI in JVT-O056.

##### JVT-Q018 [Z. Lu, Z. Li, S. Rahardja, X. Lin, K.P. Lim, W. Lin, J. Zheng, W. Yao] Progress report of SVC CE4: ROI

##### JVT-Q020 [D. Ichimura, Y. Honda, M.H. Lee] Slice group map for mult. interactive ROI scal.

Interactive region of interest (IROI) scalability is distinguished from predefined ROI fixed at encoder side, where current FMO (e.g. FMO map type 2) is known to be capable of achieving. The applications of IROI are discussed. The solution to support interactive ROI scalability by coding rectangular grid of MBs as independent slices is proposed. Coding of grid slices can be specified by a new FMO map type 7 for grid slice groups to reduce overhead bits. From the experimental results, the coding efficiency penalty to support IROI scalability is 15% bitrate increase for the overall bitstream but IROI coding is reported to show a 3dB gain in PSNR for the user-selected ROI region. Subjective quality of ROI region is also reported to be enhanced with more details. Moreover, for IROI decoding an additional benefit of lower decoder complexity according to experimental results is reported. Lastly, a bug fix in JSVM software is also reported in this contribution.

##### JVT-Q021 [D. Ichimura, Y. Honda, M.H. Lee] Stream struct and CABAC for mult. inter. ROI scal.

In this contribution, the proposal to improve the coding efficiency of Interactive Region of Interest (IROI) scalability is presented. In IROI coding, a picture is divided into many rectangular grid slices. One problem with dividing into many slices is the huge overhead of coding many NAL unit headers and slice headers using JSVM stream structure because each quality layer is coded in a separate slice. A new stream structure is proposed to reduce the header overhead, where several quality layers are coded in one slice. Another problem with dividing into many slices is the reduced coding efficiency of CABAC due to more initialization and less updating of CABAC contexts. Inter quality layer context continuation is proposed to improve the coding efficiency of CABAC. In inter quality layer context continuation, the context states are used continuously through all quality layers in a slice without initialization. From the experimental results, an analysis of the overheads comparison between JSVM stream structure and proposed IROI stream structure with 3 quality layers shows an overhead bitrate reduction by 75%. A 5% overall bitrate reduction and a PSNR gain of 0.3dB is reported for the ROI region when using inter quality layer context continuation.

##### JVT-Q076 [T.C. Thang, T.M. Bae, J.W. Kang, Y.M. Ro, J.-G. Kim] Boundary handling for ROI scalability

At the last meeting, an FMO-based method to enable encoding and extraction of ROIs from SVC bitstream was presented. It was claimed that ROI boundaries should be treated as picture boundaries. In this contribution, the importance of ROI boundary handling in SVC is studied. When ROI boundaries are not handled, two different kinds of errors are shown related to upsampling and fractional sample prediction. Also, some related works in MPEG-4 and H.263 is reviewed.

##### JVT-Q077 [T.C. Thang, T.M. Bae, J.W. Kang, Y.M. Ro, J.-G. Kim] Showcase of ROI extraction using scal info SEI

At the last meeting, a method to enable ROI extraction from SVC bitstream was proposed in JVT-P042. To help the extractor in extracting ROI bitstreams, the information of ROIs inside a SVC bitstream was included in scalability information SEI message. In this contribution, a showcase of ROI extraction is presented using the scalability information SEI message. Here, the focus is on the application where ROI is extracted according to ROI id. In addition, one correction to the current syntax of scalability information SEI message is proposed, so as to support overlapped regions between ROIs.

## CE 5 & related docs: Quantization

##### JVT-Q063 [W.-J. Han] CE5: Distribution-based dequant for SVC

## CE 6 & related docs: Extended spatial scalability

##### JVT-Q013 [E. Francois, J. Vieron, V. Bottreau] Additional results on SVC ESS evaluation

JVT-P018 provided evaluation results of ESS, in comparison with single layer and simulcast-like configurations. This document provides additional results, using different tests conditions as JVT-P018. In previous tests, comparisons were achieved for equivalent bit-rates. These new tests have been performed by targeting a same quality (PSNR) for the different compared solutions, and by comparing the required bit-rates.

##### JVT-Q014 [J. Vieron, E. Francois, V. Bottreau] Additional test conds for SVC EsS JSVM Perf Eval

The aim of this contribution is to set-up test conditions for JSVM performance evaluation for the Extended Spatial Scalability (ESS) tools. It is recommended that the proposed test sets are included in the testing conditions superset already provided by the AhG on Testing Conditions for Coding Efficiency and JSVM Performance Evaluation in document [JVT-P205].

##### JVT-Q015 [E. Francois, J. Vieron, V. Bottreau] General. SVC ESS for inter-layer ratios >2

This document addresses two issues. The first issue is a bug fixing in ESS that has been revealed in specific cropping configurations. The second issue aims at generalizing ESS for inter-layer spatial ratios greater than 2. In JSVM3, [JVT-P202], Extended Spatial Scalability (ESS) is limited to inter-layer spatial ratios from 1 to 2. A straightforward generalization of ESS, enabling to use any ratio greater than 1, is proposed. Corresponding JSVM modifications are described.

##### JVT-Q038 [S. Sun] New results on direct interp. for upsamp. in ESS

In Poznan meeting, the direct interpolation method (JVT-P012) for upsampling in extended spatial scalability was adopted into the JSVM software as an option to be further tested.  This contribution shows new test results of the technique based on the latest JSVM software.  In the resampling experiments, this upsampling option is reported to be up to 4.11 dB (and on average 1.96 dB) better than the default option based on the quarter-pixel interpolation.  In the ESS experiments (with IntraPeriod equal to 32), the direct interpolation option is reported to show a performance gain of 0.21 dB on average comparing to the default option.  It is therefore proposed to formally adopt this method into the SVC Extension.

## CE 7 & related docs: Low-delay FGS

##### JVT-Q039 [Y. Bao, M. Karczewicz] CE7: FGS for low delay

##### JVT-Q085 [J.-Z. Xu] CE7: Core experiment 7 proposal

## CE 8 & related docs: CAVLC

##### JVT-Q040 [J. Ridge, M. Karczewicz] CE8: VLCs for SVC

At the previous JVT meeting, software enabling entropy coding using VLCs in progressive refinement (FGS) layers was presented. A core experiment was created to enable further consideration of that feature prior to acceptance into the JSVM. This contribution reviews the VLC for FGS feature, notes some minor improvements made during the course of the CE, and presents results indicating that the overhead associated with VLC for FGS is in line with the single-layer case.

##### JVT-Q041 [J. Ridge] Coding efficiency improvements for VLCs

##### JVT-Q057 [B.K. Lee] VLC for 8x8 transform FGS

JVT-P056 proposed CAVLC for FGS. In JVT-P056 flag coding for significant pass in 8x8 transform is done. This is reported to be improved for 8x8 transform. The VLC technique of 4x4 transform is extended for significant pass in 8x8 transform. The modification is small. But our experimental results shows bit-rate saving up to 12%.

## Improved FGS coding

##### JVT-Q031 [M. Winken, H. Schwarz, D. Marpe, T. Wiegand] Adapt. motion refinement for FGS slices

In this contribution, the adaptive coding of motion refinements as part of progressive refinement slices is investigated.  The impact of using a constant set of motion data for all progressive refinement slices of a pictures is analyzed.  Based on this analysis a first approach for adaptively refining motion data in progressive refinement slices has been investigated.  First experimental results show that coding efficiency gains of more than 1 dB can be obtained, when motion data refinements are coded in progressive refinement slices in addition to the refinements of texture information.  It is proposed to set up a new Core Experiment on that issue with the goal to significantly improve the coding efficiency of fine-granular SNR scalable coding, especially for large rate intervals.

##### Improved CABAC coding

##### JVT-Q071 [J.-H. Park, S.-W. Park, B.-M. Jeon] Context modeling of residual prediction flag

This document presents the information about context modeling of residual prediction flag. In case that CBP value of base MB is zero and at the same time residual prediction flag of base MB is equal to 1, the current model is reported to not fully utilize base layer residual information. Since originally the base MB has access to the residual signal, although CBP value of base MB is zero, ctxIdxInc should have non-zero value in that case. A simple modified condition for context modeling of residual prediction flag is introduced.

## High-Level Syntax

##### JVT-Q028 [P. Yin, J. Boyce, P. Pandit] High-level SVC Syntax

The concept of FGS fragment NAL units was adopted in Poznan. So the concatenated [quality_level, fragment_order] is used to support MGS/FGS. But in the current design, the quality level is indicated in NAL unit header or SPS, but fragment_order is indicated in slice header. For consistency, it is proposed to add fragment_order information in NAL unit header or SPS, without changing the existing number of bytes in the NAL unit header (e.g. either 1 or 2).  The 6-bits allocated but unused for simple_priority_id in the two-byte mode, are used for fragment order and a short_priority_id field which can be used as determined by the application.

##### JVT-Q029 [P. Yin, J. Boyce, P. Pandit] FMO and ROI scalability

FMO has been discussed to implement the feature of ROI scalability. The base specification does not allow any missing slice groups, which means even for slice groups which contain non-ROIs still need to be coded and send them into the network. In this contribution, it is proposed to relax this requirement and allow missing slice groups in enhancement layers, and define a normative behavior for intentionally missing slices. The base layer remains compatible.

##### JVT-Q064 [Y.-K. Wang, M.M. Hannuksela, S. Wenger] SVC DPB management

This contribution proposes some changes to the DPB management syntax and process for handling of decoded pictures used for inter-layer prediction referencing in SVC. The goal of the proposal is to significantly save the required DPB buffer size, which is demonstrated by the example in the end of the contribution.

##### JVT-Q065 [Y.-K. Wang, M.M. Hannuksela] Enhancement-layer IDR picture ("EIDR")

This contribution proposes the coding of enhancement-layer IDR (EIDR) picture to enable efficient low-to-high layer switching, which is important in scalable stream adaptation.

##### JVT-Q066 [Q. Shen, H. Li, Y.-K. Wang] Showcase of non-required picture SEI message

This contribution describes the showcase of non-required picture SEI message, which was adopted in the Poznan meeting, to demonstrate the usefulness of the SEI message. The demonstration tool set is attached such that anyone interested can perform the demo independently. The source code for creation of the demonstration tool set is also attached.

##### JVT-Q067 [H. Liu, H. Li, Y.-K. Wang] Showcase of scalability information SEI message

This contribution describes the showcase of the scalability information SEI message, which was adopted in the Poznan meeting, to demonstrate the usefulness of the SEI message. The showcase of the part for support of sub-picture coding based ROI scalability is described in JVT-Q077. The demonstration tool set is attached such that anyone interested can perform the demo independently. The source code for creation of the demonstration tool set is also attached.

##### JVT-Q070 [Y.-K. Wang, S. Wenger] SVC RTP payload format internet draft

This is an informative contribution to announce that the work towards an RTP payload format for SVC coded video has started.  The first Internet Draft is available at http://www.ietf.org/internet-drafts/draft-wenger-avt-rtp-svc-00.txt.  As for all Internet Drafts, comments are encouraged in the appropriate forum, which is the IETF AVT WG's mailing list.  Please see http://www.ietf.org/html.charters/avt-charter.html.

##### JVT-Q075 [T.C. Thang, T.M. Bae, J.W. Kang, Y.M. Ro, J.-G. Kim] Improved scalability info SEI message

In last Poznan meeting, a new syntax structure of scalability information SEI message was proposed. This syntax describes important information of all the possible layers of a SVC bitstream, namely base layer, temporal/spatial/SNR enhancement layers.

From the implementation of the ROI extraction show case, it was found that, when there exist ROIs in a SVC bitstream, the current syntax of scalability information SEI message is still inconvenient to describe the layers of different ROIs. For this purpose, some improvements to the current syntax are proposed.

##### JVT-Q091 [S. Jeong, M. Park, G. Park, K Kim, J. Hong] Numbering of Key pictures

In the current JSVM the interval of key pictures, i.e. GOP size, can be fixed or can be variable. When temporal scalability is used, this interval is variable. JSVM uses the numbering scheme for all pictures by its displaying order. Therefore, it is difficult to detect the drop of key pictures. When the proposed numbering scheme for key pictures is used, it is easy to detect the drop or lost error of key pictures. Furthermore it is possible to conceal the error by the dropped key picture at the decoder.

## Error Resiliency

##### JVT-Q054 [J. Jia, H.K. Kim, H.C. Choi, J.G. Kim] Implementation of redundant pictures in JSVM

In order to enhance the robustness to packet loss, redundant picture is one of the error resilient tools, which allows an encoder to send redundant coded pictures ( or part of the macroblocks in the primary coded picture). As current JSVM software doesn’t support the redundant pictures, in this contribution, the redundant pictures have been implemented into the current JSVM software. Also, a simple description of the usage of this tool is given here.

##### JVT-Q055 [J. Jia, H.K. Kim, H.C. Choi, J.G. Kim] Polyphase downsample-based redundant pics

In order to enhance error resilience of SVC, Polyphase Downsampling (PD) based redundant picture coding in JSVM is described in this proposal. The presented method implements PD to residual signals before DCT in the redundant picture coding process, which enables the redundant coded picture to be sent flexibly with somewhat degradation on the picture quality corresponding to the quantity of DCT coefficients that have been sent. When part of the DCT coefficients in one redundant picture have been sent, reconstruction utilizing the spatial redundancy inherent in natural pictures between neighboring pixels can be performed to improve the reconstruction quality.

##### JVT-Q068 [Y. Guo, C. Zhu, H. Li, Y.-K. Wang] Error resilient implementations for JSVM

##### JVT-Q069 [Y. Guo, H. Li, Y.-K. Wang] SVC/AVC loss simulator donation

This is an informative contribution to announce that a loss simulator is donated to JVT for use in SVC/AVC error resilience simulations. The simulator is intended for free and public use. Both the source code and the executable file are contained in the attachment of this document.

##### JVT-Q087 [F. Pauchet] Prop SVC bit error resilience test conds

This document proposes common conditions for SVC error resilience testing taking into account bit errors.

## Profiles

##### JVT-Q022 [J. Reichel, X. Ziliani, P. Amon, X. Boltz, U. Benzler] Surveillance profile for SVC

A proposal is made to define a Surveillance Profile for scalable video coding in JVT with the aim of creating a subset of tools for applications requiring low-delay, low-complexity encoder and decoder solutions and defining those in one specific profile.

# JVT SVC non-normative modifications

##### JVT-Q081 [I. Amonou, N. Cammas, S. Kervadec, S. Pateux] Layered quality opt. of JSVM-3 & closed-loop

This document presents an improvement of the Joint Scalable Video Software 3 (JSVM 3.0). It extends the notion of priority levels introduced in JVT-PO202 to the context of closed-loop coding. In closed-loop coding, the impact of the distortion between frames is no more uncorrelated as it was the case in open-loop coding. This document introduces a novel approach for computing quality layers. The priority levels are then introduced in the bitstream using the SVC extended NAL unit header (priority_id). Results show improvement of up to 0.66 dB compared to JSVM3.0. It is proposed that the improved algorithm for the determination of quality layers is adopted as non-normative tool for the Joint Scalable Video Model (JSVM) 4.

##### JVT-Q030 [H. Schwarz, D. Marpe, T. Wiegand] Closed loop coding with quality layers

In this contribution, the usage of quality layers in connection with closed-loop coding is investigated.  Furthermore, an improved algorithm for the determination of quality layers has been developed.  The developed algorithm has been tested for closed-loop coding with hierarchical B pictures, where coding efficiency gains are reported to be up to 0.28 dB compared to the algorithm proposed in JVT-P029.  In comparison to closed-loop coding without the usage of quality layers, coding efficiency gains of up to 0.63 dB have been measured.  It is proposed that the improved algorithm for the determination of quality layers is adopted as non-normative tool for the Joint Scalable Video Model (JSVM) 4.

Conclusion from quality layers: Clear advantage of QL shown for closed-loop coding by both proposals. Both props may target different rate points, but Q081 seems to be the better starting point. Proponents will make proposal whether elements of Q030 can be added or be made switchable as an  option. Except for City, closed loop performs clearly better than open-loop MCTF.

##### JVT-Q046 [Y. Chen, J. Boyce, K. Xie] SVC frame loss concealment

The current JSVM3.0 software has no error concealment support. The software crashes with any loss. Four error concealment algorithms are proposed to be added to the JSVM to handle frame loss – frame copy and temporal direct mode motion generation, BlSkip mode and reconstruction base layer upsampling. The proposed error concealment algorithms were tested using the packet loss model of VCEG Q15-I-16r1, and the testing conditions are referred to JVT-P026. The described scenario focuses on the two spatial layer cases.

##### JVT Non-SVC normative modifications

## Improved 4:4:4 coding

##### JVT-Q027 [J. Xin, A. Vetro, H. Sun] Verif. of 4:4:4 in JVT-P017

The “Advanced 4:4:4 Profile” as proposed in JVT-O013 and JVT-P017 has been implemented by the authors of the above two contributions. In this contribution, the performance of the encoding schemes proposed in JVT-P017 is verified.

##### JVT-Q032 [S. Sekiguchi, Y. Isu, Y. Yamada, K. Asai, T. Murakami] Improved entropy coding for intra pred. modes

Introduction of improved entropy coding of intra prediction modes is proposed in this contribution. In some proposals, independent intra prediction for each color component is adopted, which can improve prediction efficiency but leads increase of overhead bits to code additional intra prediction modes. The efficiency of entropy coding for intra prediction modes is investigated for the case that independent intra prediction per each color component is employed. Two types of entropy coding modifications are presented to achieve further coding gain. First one is the modification to the process of predictor determination for coding intra NxN pred modes. The other is a direct CABAC application to the intra NxN pred mode syntax. A set of preliminary experimental results is provided.

##### JVT-Q033 [Y. Isu, S. Sekiguchi, Y. Yamada, K. Asai, T. Murakami] Verif. on existing Adv. 4:4:4 proposals

This document provides verification results on the existing advanced 4:4:4 coding proposals that can be categorized as short-term approach (i.e., with minimum modifications to the current AVC coding tools). Both intra-only coding case and inter coding case will be presented. The main purpose of this verification is to evaluate the efficiency of independent spatio-temporal prediction for each color component, which is a new concept for the video coding and can be implemented with minimum modifications to the current AVC coding framework. The common set of HDTV materials discussed in 4:4:4 AHG [1] have been used as test sequences. The existing AVC High 4:4:4 profile (“ref”) is used as reference. For intra-only coding, independent 4:0:0 coding for each color component (“400x3”) and the proposed implementation of “new3” method proposed in JVT-P017 [2] (“new3_melco”) are evaluated. As for the inter coding case, a simple comparison between ref and 400x3 for IPPP… picture type configuration is conducted.

##### JVT-Q035 [Y.L. Lee] Complexity of prop. lossless intra for 4:4:4

The new 4:4:4 profile will use a multi-directional spatial prediction method to reduce the spatial redundancy by using block boundary pixels. A new lossless Intra residual DPCM coding method was proposed at the JVT meeting for the Advanced 4:4:4 Profile. At the previous meeting, the proposed lossless Intra coding method based on residual DPCM showed the bit rate reduction approximately 12.53 % in comparison to the lossless Intra coding method included in FRExt. The purpose of this document is to analyze the complexity of the residual DPCM when the proposed one is used. In the experiments, the decoding complexity is reduced when the proposed method is applied to the Advanced 4:4:4 Profile.

##### JVT-Q037 [P. Topiwala] Prop. 14-b sample support in Adv. 4:4:4 Pr

For support of 14-bit input video sample data in the upcoming Advanced 4:4:4 Profile. Applications using existing 14-bit video sensors ranging from military surveillance to emerging commercial television and film industry require 14-bit support.

##### JVT-Q058 [W.-S. Kim, D. Cho, D. Birinov, H.M. Kim] Verif and analysis of JVT-P017 on 4:4:4 profile

##### JVT-Q059 [W.-S. Kim, D. Cho, D. Birinov, H.M. Kim] Advanced residual color transform

##### JVT-Q082 [D. Marpe, V. George, H. Kirchhoffer, T. Wiegand] 4:4:4 intra using adapt. color transform

##### JVT-Q086 [T. Wedi] Intra-only 4:4:4 profile

For professional applications, Intra-only coding is a common coding method. Furthermore, several experiments and demonstrations have shown that intra coding has an excellent performance, even compared to state of the art still image coding schemes.

##### JVT-Q090 [P. Topiwala, H. Yu] New Viper HD test sequences

Three new test sequences are introduced in the existing VIPER 10-bit HD data.  These replace two previously selected sequences that have been found to be inappropriate for 444 coding.

## Logo insertion

##### JVT-Q019 [P. Bordes, E. Francois] SEI for logo insertion

The problem of the logo insertion in video is addressed. It is proposed to transmit the encoded logo in a standardized SEI message. This will greatly facilitate the logo insertion, removing and replacing in a video byte-stream without modifying the quality of the video (VCL).

# JVT Non-SVC non-normative aspects

## Encoder optimization techniques

##### JVT-Q079 [ABC] ABC

##### JVT-Q080 [Z. Heng] R-D function of H.264/AVC transf. coeffs.

In recent literatures on rate control, a quadratic model is widely used. This model was first introduced in assuming the source to be continuous. It is claimed that the transform coefficients are actually discrete random variables, thus the conclusions of continuous source may not hold. This article derives the R-D function transform coefficients by first obtaining the R-D function of simple discrete sources with unique alphabet step, and then composing these simple cases to a product source. Results reveal that the continuous and discrete source has different R-D function form and the latter can approximate to a logarithmic form as that of the former one.

## Improved Error resilience

##### JVT-Q036 [D. Hench, P. Topiwala] Joint source-channel encoding

A “Joint Source-Channel friendly” real-time implementation of normative AVC is discussed.  The motivation is both to support basic research and to be an early adopter by performing a technology demonstration.

## Reference Software

##### JVT-Q025 [E. Setton, B. Girod] R-D analysis and streaming of SP and SI frames

##### JVT-Q088 [X. Xu, Y. He] Dynamic search range for ME in JM

##### JVT-Q089 [X. Xu, Y. He] Comments on ME in current JM software

# Withdrawn JVT documents

JVT-Q004, JVT-Q016, JVT-Q041, JVT-Q074

# Updating of JVT group documents

##### JVT-Q023 [T. Suzuki] Some COR issues

##### JVT-Q042 [A. Tourapis] Updates to reference software manual

# List of Adoptions

Person listed in bracket is responsible for provisioning of text and software integration.

## Normative SVC adoptions

## Non-Normative SVC adoptions

## Normative non SVC-related adoptions

## Non-Normative non SVC-related adoptions

# List of AHGs established

Copy mandate for all AHGs from previous description.

# Resolutions of the meeting

# SW integration plan

# Attendance