| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 17th Meeting: Nice, France, 14-22 October, 2005 | Document: JVT-Q022 Filename: JVT-Q022.doc |
| --- | --- |

| Title: | Surveillance Profile for Scalable JVT Coding Standard |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marco Boltz, Ulrich Benzler (Bosch) Francesco Ziliani, Julien Reichel (Visiowave GE Security), Peter Amon (Siemens); MenHuang Lee, ShengMei Shen(Panasonic) | Tel: Email: | Ulrich.benzler.-at-de.bosch.com |
| Source: | Bosch, Visiowave GE Security, Siemens, Panasonic |  |  |

# 1. Introduction

A proposal is made to define a Surveillance Profile for scalable video coding in JVT with the aim of creating a subset of tools for applications requiring  low-delay, low-complexity encoder and decoder solutions and defining those in one specific profile.

2. Definition of video coding tools for the Surveillance profile

The video coding tools to be included/excluded in this profile are suggested in the following table:

Table 1. Surveillance Profile Tools

| Video Tools | Included in Surveillance Profile | Restrictions in Surveillance Profile |
| --- | --- | --- |
| IDR-slice | X | Definitely needed for easy random access. See comments below this table. |
| I-slice | X |  |
| P-slice | X |  |
| B-slice | X | Still under discussion. Maybe not used as reference? Maybe restrict the number of compensations in consecutive MBs similarly to MaxMvsPer2Mb to limit complexity |
| SP-slice | No |  |
| - KeyPicture | No | Not needed, because FGS is not used. This would introduce additional memory constraints. |
| - Motion vector block sizes | X | All, 16x16 to 4x4. Some restriction on number of 4x4 vectors maybe placed (TBD). |
| - Multiple Reference frames | X | See comments below this table |
| - 16 bit 4x4 transform and quantization | X |  |
| - 8x8 transform and quantization | X | See comments below this table |
| 1/4 pel motion vector resolution | X |  |
| Interlace Coding Tools – Adaptive field/frame at sequence level | X | For levels supporting interlace video |
| Intra Predictions Modes | X | Maybe not all possible modes to be supported |
| VLC | X |  |
| CABAC | X |  |
| Startcodes (in Network Adaptation Enabling Layer) | X |  |
| - Loop filter | X | Allow option in the Sequence/Picture layer to flag that Loop filter is switched off. |
| - Data Partitioning | No |  |
| - Motion vector restriction | X | See restrictions in Table 2 |
| - Chroma resolution | 4:2:0 |  |
| - Number of bits per Luma and Chroma component | 8 |  |
| - Quantization | X | Fixed and same for Luma and Chroma? (should use same settings as Baseline) |
| - MCTF update | No | Not obligatory in the decoder |
| - Spatial Scalability | X | Only dyadic factors, but maybe different for horizontal/vertical (e.g. 2CIF – CIF scalability) |
| - Fine Grain Scalability | No |  |
| - Coarse grain SNR scalability | X | Some restriction on the number of layers TBD |
| - Multi-loop decoding | X | Maybe make this level dependent. Still under discussion, may not be needed if coding efficiency can be sufficiently achieved without it. |
| - Slice Groups (FMO) | X | Interactive Region of Interest on slice level (MB aligned). This will enable Interactive ROI support for bitstream parsers and decoding. How many groups should be supported? Level dependent? (e.g. AVC, Base line profile supports up to 8). |
| - Weighted prediction | No |  |
| - Temporal direct mode | No | Only spatial direct mode supported |

IDR slices:

IDR slices must be supported, however, it is not necessary to constraint their frequency. Imposing a minimum IDR frequency may prevent the effective use of GDR (for example in optimizing random access) which is a useful feature in video surveillance.

Multiple References:

Maximum 5 for both P and (if supported) B pictures (this implies 1 future reference frame in B and maximum of 4 past frames) for all resolutions. In SVC the maximum number of temporal level is equal to log2(num_ref_frame-1) (see JVT-P014). So in order to support a minimum of 3 temporal levels, support for 4 references frames is a minimum for all resolutions.

8x8 transform:

8x8 transform is supported only by FREx. The additional complexity linked to the use of 8x8 transform might not be necessary (at least for resolution CIF and under).

CGS:

A maximum of 3 CGS layer should be authorized per spatial resolution. Note that this would already multiply the memory requirement by 3 if multiple decoding loops are allowed.

In fact what needs to be constrained is the maximum number of MBs to be decoded, from the current layer and from all layers it depends on.

3. Definition of Levels for Surveillance Profile

Within the Surveillance Profile, levels are defined as in Table 2.

Table 2. Surveillance Profile Levels (from AVC Baseline Profile)

| Level number | Max macroblock processing rate MaxMBPS (MB/s) For the layer (n) | Max frame size MaxFS (MBs) | Max decoded picture buffer size MaxDPB (1024 bytes for 4:2:0) For the layer (n) | Max video bit rate MaxBR (1000 bits/s, 1200 bits/s, cpbBrVclFactor bits/s, or cpbBrNalFactor bits/s) For the layer (n) | Max CPB size MaxCPB (1000 bits, 1200 bits, cpbBrVclFactor bits, or cpbBrNalFactor bits) | Vertical MV component range MaxVmvR (luma frame samples) | Min compression ratio MinCR | Max number of motion vectors per two consecutive MBs MaxMvsPer2Mb |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 485 | 99 | 148.5 | 64 | 175 | [-64,+63.75] | 2 | - |
| 1b | 1 485 | 99 | 148.5 | 128 | 350 | [-64,+63.75] | 2 | - |
| 1.1 | 3 000 | 396 | 337.5 | 192 | 500 | [-128,+127.75] | 2 | - |
| 1.2 | 6 000 | 396 | 891.0 | 384 | 1 000 | [-128,+127.75] | 2 | - |
| 1.3 | 11 880 | 396 | 891.0 | 768 | 2 000 | [-128,+127.75] | 2 | - |
| 2 | 11 880 | 396 | 891.0 | 2 000 | 2 000 | [-128,+127.75] | 2 | - |
| 2.1 | 19 800 | 792 | 1 782.0 | 4 000 | 4 000 | [-256,+255.75] | 2 | - |
| 2.2 | 20 250 | 1 620 | 3 037.5 | 4 000 | 4 000 | [-256,+255.75] | 2 | - |
| 3 | 40 500 | 1 620 | 3 037.5 | 10 000 | 10 000 | [-256,+255.75] | 2 | 32 |
| 3.1 | 108 000 | 3 600 | 6 750.0 | 14 000 | 14 000 | [-512,+511.75] | 4 | 16 |
| 3.2 | 216 000 | 5 120 | 7 680.0 | 20 000 | 20 000 | [-512,+511.75] | 4 | 16 |
| 4 | 245 760 | 8 192 | 12 288.0 | 20 000 | 25 000 | [-512,+511.75] | 4 | 16 |
| 4.1 | 245 760 | 8 192 | 12 288.0 | 50 000 | 62 500 | [-512,+511.75] | 2 | 16 |
| 4.2/Lo | 491 520 | 8 192 | 12 288.0 | 50 000 | 62 500 | [-512,+511.75] | 2 | 16 |
| 4.2/Hi | 522 240 | 8 704 | 13 056.0 | 50 000 | 62 500 | [-512,+511.75] | 2 | 16 |
| 5 | 589 824 | 22 080 | 41 400.0 | 135 000 | 135 000 | [-512,+511.75] | 2 | 16 |
| 5.1 | 983 040 | 36 864 | 69 120.0 | 240 000 | 240 000 | [-512,+511.75] | 2 | 16 |

| Level number | Max macroblock processing rate MaxMBPS (MB/s) For all decoded layers (0..n) | Max decoded picture buffer size MaxDPB (1024 bytes for 4:2:0) For all decoded layers (0..n) | Max video bit rate MaxBR (1000 bits/s, 1200 bits/s, cpbBrVclFactor bits/s, or cpbBrNalFactor bits/s) For all decoded layers (0..n) |
| --- | --- | --- | --- |
| 1 | 4 456 | TBD (not needed for single loop) | 128 |
| 1b | 4 456 | TBD (not needed for single loop) | 256 |
| 1.1 | 9 000 | TBD (not needed for single loop) | 384 |
| 1.2 | 18 000 | TBD (not needed for single loop) | 768 |
| 1.3 | 35 640 | TBD (not needed for single loop) | 1 536 |
| 2 | 35 640 | TBD (not needed for single loop) | 4 000 |
| 2.1 | 59 400 | TBD (not needed for single loop) | 8 000 |
| 2.2 | 60 750 | TBD (not needed for single loop) | 8 000 |
| 3 | 121 500 | TBD (not needed for single loop) | 20 000 |
| 3.1 | 324 000 | TBD (not needed for single loop) | 28 000 |
| 3.2 | 648 000 | TBD (not needed for single loop) | 40 000 |
| 4 | 737 280 | TBD (not needed for single loop) | 40 000 |
| 4.1 | 737 280 | TBD (not needed for single loop) | 100 000 |
| 4.2/Lo | 1 474 560 | TBD (not needed for single loop) | 100 000 |
| 4.2/Hi | 1 566 720 | TBD (not needed for single loop) | 100 000 |
| 5 | 1 769 472 | TBD (not needed for single loop) | 270 000 |
| 5.1 | 2 949 120 | TBD (not needed for single loop) | 580 000 |

Comments:

-  Limit the maximum aspect ratio (Horizontal size / Vertical size) to 4:1.

-  It may also be acceptable to have no maximum motion vector range (constrained by the picture size). TBD.

-  HRD buffer (what used to be called VBV in MPEG-2) size is the maximum size that an encoder can assume and is the minimum size that decoder must have.

In comparison with AVC, in SVC it is not sufficient to declare the current level and profile: it is necessary to define how the levels and profiles of the different layers interact. We propose the following rules extensions:

- The levels of the different layer shall be strictly non-decreasing, i.e. if layer 3 is of level 2, then layer 0 to 2 are not allowed to be of level higher than 2

- A layer of a given profile shall only include layer of the same profile, or of a profile (or part of profile) which can be decoded by the profile of the current layer, e.g. if layer 3 is coded using profile X which doesn’t support 8x8 transform, then layer 0 to 2 can not be coded with profile Y which allows such transform, unless a flag is used that specify that such transform are not used (see constraint_set0_flag)

- An additional set of constraints should specify the maximum total number of MB needed to decode a layer, including the number of MB of previous layers (to be added to the MaxMBPS)

- There are 3 types of bitrate constraints in SVC: the bitrate of the current layer, the total bitrate of the current layer and all sub layers, and the bitrate of the current layer and the bitrate of all layers that are needed for the decoding of the current layer.

- If multiple loop decoding is allowed, then the size of the DPB should also be specified for the current layer AND for all sub layers.

4. Discussion

In this profile the use of B-frames is (possibly) allowed. An encoder used for the video surveillance applications can choose to not use the B-frames to have low delay. In the past this type of approach may have put unnecessary cost burden on the decoders as the B-frames required more memory and the cost of memory was high in comparison to the rest of the system’s cost. However, in scalable JVT multiple reference frames are already used for prediction in P-frames. Proposal here keeps the total number of frames, used as reference, the same for both P and B frames.  Incremental cost to support B-frames now is related to mainly the logic or the software code and the reference memory access to implement B-frame decoding. Considering the benefit of achieving wider interoperability with other applications, this is considered to be a good compromise. Additionally, further restrictions on the number of motion compensation operations (in consecutive MBs) may be added in some levels.

The maximum number of reference frame should be based on the profile (as for AVC).

There is no restriction on the Slice Type. For error resilience purposes encoder can choose to limit the slice types such that they are independently coded.

A higher Level decoder in Surveillance Profile must be able to decode the bit stream compliant with a lower level in the profile.

It may also be acceptable to have unconstrained maximum motion vector range (constrained by the picture size).

References:

[1] Dave Lindbergh and Ajay Luthra, “Profile and levels framework”, JVT-B108, Feb. 2002.

[2] Ajay Luthra, “Baseline Profile for JVT Coding Standard”, JVT-C113, March 2002.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Robert Bosch GmbH |  |  |
| Mailing address | CR/AEM PO Box 77 77 77 31132 Hildesheim |  |  |
| Country | Germany |  |  |
| Contact person | Ulrich Benzler |  |  |
| Telephone | +49 (5121) 49-3087 |  |  |
| Fax | +49 (5121) 49-173087 |  |  |
| Email | Ulrich.benzler.-at-.de.bosch.com |  |  |
| Place and date of submission | Nice, France, October 14th 2005 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Surveillance Profile for Scalable JVT Coding Standard |  |
| Contribution number |  | JVT-Q022 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| x | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of this contribution or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: