| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 17th Meeting: Nice, FR, 14-21 October, 2005 | Document: JVT-Q023 Filename: input.doc |
| --- | --- |

| Title: | Some COR issues |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki Sony Corp. 6-7-35, Kitashinagawa, Shinagawa-ku, Tokyo, 141-0001, JAPAN | Tel: Email: | +81 3 5795 8650 teruhiko@av.crl.sony.co.jp |
| Source: | Sony Corp. |  |  |

_____________________________

# Introduction

We found some ambiguous descriptions and problems in the latest specifications, the document to consent in ITU in August.

# Annex A 3.3 (a)

Encoder can not generate the bitstreams compliant with Annex A 3.3.a, when encoder try to encode pictures, whose MB processing rate is much smaller than that of Max value defined by Table A-1. The following is an example.

The definition of Annex A 3.3.a is as follows.

"

a)	In bitstreams conforming to the Main, High, High 10, High 4:2:2, or High 4:4:4 profiles, the removal time of access unit 0 shall satisfy the constraint that the number of slices in picture 0 is less than or equal to ( PicSizeInMbs + MaxMBPS * ( tr( 0 )   tr,n( 0 ) ) ) ÷ SliceRate, where SliceRate is the value specified in Table A 4 that applies to picture 0.

"

This is the restriction on the number of slices in picture 0 and it is less than or equal to

( PicSizeInMbs + MaxMBPS * ( tr( 0 ) - tr,n( 0 ) ) ) ÷ SliceRate

But when low_delay_hrd_flag is equal to 0, tr(n) and tr,n(n) is constrained by C.1.2 (C-10) as followins,

tr( n ) = tr,n( n )

In this case, ( tr( 0 ) - tr,n( 0 ) ) will be zero and the formula in A.3.3.a will be

( PicSizeInMbs ) ÷ SliceRate

When encoder want to encode 48x48 pixel size picture @ 29.97 fps (3x3 MB picture) at level 3.0, the formula in Annex 3.3.a will be

( PicSizeInMbs ) ÷ SliceRate = ( 9 ) ÷ 22 = 0.409

In this case, the maximum number of slice is less than 1 and the bitstream can not comply this restriction, even if picture is encoded with only 1 slice. However this problem only occurred in picture 0.

Annex A.3.3.a is for the picture 0 and Annex A 3.3.b is the rest of pictures. Annex A 3.3.b defines the constraint on the number of slices in picture n as follows.

the number of slices in picture n is less than or equal to

MaxMBPS * ( tr( n ) - tr( n - 1 ) ) ÷ SliceRate

In this case PicSizeInMbs is not included in the formula and the value of this formula for the above example will be

40500 * ( 1 / 29.97 ) / 22 = 61.42

In this case 1 slice = 1 picture is allowed. If the above example is true, the first frame can not be compliant, but the rest of the frames can be compliant.

# cpb_removal_delay

In D.2.2, there is the following sentence at the semantics of cpb_removal_delay.

"The value of cpb_removal_delay for the first picture in the bitstream shall be equal to 0."

This constraint is too restrictive and AVC can not support very simple editing due to this constraint. The example is to split one bitstream into two bitstream at IDR.

The value of cpb_removal_delay of the first IDR in the first bitstream is equal to 0. However, the value

of cpb_removal_delay of the first picture of the second bitstream may not be 0. Therefore we need to change the value of cpb_removal_delay and simple editor need to decode variable length code to identify cpb_removal_delay in the bitstream.

Therefore I propose to remove this constraint or change to support simple editing.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Sony Corp. |  |  |
| Mailing address | 6-7-35 Kitashinagawa, Shinagawa-ku, Tokyo |  |  |
| Country | Japan |  |  |
| Contact person | Teruhiko Suzuki |  |  |
| Telephone | +81-3-5795-8650 |  |  |
| Fax | +81-3-5795-7523 |  |  |
| Email | teruhiko@av.crl.sony.co.jp |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Some COR issues |  |
| Contribution number |  | JVT-Q023 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| X | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: