| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 17th Meeting: Nice, France, 14-21 October, 2005 | Document: JVT-Q028 Filename: JVT-Q028.doc |
| --- | --- |

| Title: | Some Comments on High-level Syntax for SVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peng Yin, Jill Boyce, Purvin Pandit 2 Independence Way Princeton, NJ 08540, USA | Tel: Email: | 1-609.987.7355 peng.yin@thomson.net jill.boyce@thomson.net purvin.pandit@thomson.net |
| Source: | Thomson Inc. |  |  |

_____________________________

## Abstract

The concept of FGS fragment NAL units was adopted in Poznan meeting. So the concatenated [quality_level, fragment_order] is used to support MGS/FGS. But in the current design, the quality level is indicated in NAL unit header or SPS, but fragment_order is indicated in slice header. For a more consistent design, we propose to add fragment_order information in NAL unit header or SPS, without changing the existing number of bytes in the NAL unit header (e.g. either 1 or 2).  The 6-bits allocated but unused for simple_priority_id in the two-byte mode, are used for fragment order and a short_priority_id field which can be used as determined by the application.

### 1. Introduction

At the Poznan July 2005 meeting, the concept of FGS fragment NAL units was adopted as described in [2].. So fragment_order information together with quality_level information (concatenated as [quality_level, fragment_order]) is used to support medium and fine grain SNR scalability, as shown in Figure 1 [4]. In the current design, quality_level is indicated in NAL unit header if nal unit extension_flag equals to 1 or in SPS if nal_unit_extension_flag equals to 0, but fragment_order is indicated in slice header. This makes processing fragment_order challenging for a router or gateway.  To be consistent and allow parsing FGS fragment information at NAL unit header or SPS, we propose to add fragment_order information in NAL unit header or SPS, without changing the lengths of the headers in the existing design. The 6-bits allocated but unused for simple_priority_id in the two-byte mode, are used for fragment order and a short_priority_id field which can be used as determined by the application.

SNR

Temporal

Spatial

15Hz

30Hz

low

high

CIF

QCIF

Figure 1 NAL units for combined scalability

### 2. Proposal

In [1], the NAL unit header has an option to support one- or two-byte solutions for parsing as shown in Table 1 and Table 2, respectively. The one-byte solution can be used to: a) support fixed path bitstream extraction by dropping packets that are smaller than or equal to a given target value; b) support adaptation path but at the cost of parsing SPS to establish 1D-3D relationship.  Routers that support a simpler one-dimensional decision can use the one-byte NAL unit header solution.  The two-byte solution is to use explicit 3D scalability information to determine the adaptation path but at the cost of one byte overhead per NAL unit. Routers that can support a more sophisticated three-dimensional decision can use the two-byte NAL unit header solution. For the two-byte solution, simple_priority_id is not used by the decoding process specified in JSVM3 [1].

Table 1  JSVM3 one-byte solution for NAL unit header

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| simple_priority_id |  |  |  |  |  | discardable_flag | 0 |

Table 2 JSVM3 two-byte solution for NAL unit header

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| simple_priority_id |  |  |  |  |  | discardable_flag | 1 | temporal_level |  |  | dependency_id |  |  | quality_level |  |

To add fragment_order information to support one-byte solution b), we can add fragment_order in SPS, marked red in  .

fragment_order_list[ priority_id ] specifies the inferring process for the syntax elements fragment_order.

The two-byte solution is aimed at 3-D routers, which can make 3-dimensional packet dropping decisions based on spatial, temporal, and quality dimensions.  However, when a bitstream is generated, it is not necessarily known in advance whether the bitstream will be processed using 1-D routers or 3-D routers.

In the current JSVM3 design, for the two-byte solution the 6 bits for simple_priority_id are not used by decoding process. To add fragment_order information in the two-byte solution, we can use two of the low order bits in the space allocated for the simple_priority_id, as described in .  The remaining four bits are used as a short_priority_id and maybe used as determined by the application to indicate 1-D priority.

Our proposal for fragment order information differs from JVT-P100 which used all 6 bits of simple_priority_id for fragment information [2]. In our case, we only use low order two bits, which is enough in current JSVM design, as specified in slice header. As in [2], the quality_level and the frame_order values are concatenated together for the 3rd dimension which indicates SNR scalability for use by the 3-D router. Using only two bits for the fragment order has the advantage of leaving 4-bits for use as determined by the application, by defining a 4-bit short_priority_id field, which the encoder would be free to use to provide a coarse indication of 1-D priority.

short_priority_id When extension_flag is equal to 1, short_priority_id is not used by the decoding process specified in this Recommendation | International Standard. The syntax element short_priority_id may be used as determined by the application.

Table 3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| profile_idc | 0 | u(8) |
| constraint_set0_flag | 0 | u(1) |
| constraint_set1_flag | 0 | u(1) |
| constraint_set2_flag | 0 | u(1) |
| constraint_set3_flag | 0 | u(1) |
| reserved_zero_4bits /* equal to 0 */ | 0 | u(4) |
| level_idc | 0 | u(8) |
| seq_parameter_set_id | 0 | ue(v) |
| if( profile_idc = = 83 ) { |  |  |
| nal_unit_extension_flag | 0 | u(1) |
| if( nal_unit_extension_flag = = 0 ) { |  |  |
| number_of_simple_priority_id_values_minus1 | 0 | ue(v) |
| for( i = 0; i <= number_of_simple_priority_id_values_minus1; i++ ) { |  |  |
| priority_id | 0 | u(6) |
| temporal_level_list[ priority_id ] | 0 | u(3) |
| dependency_id_list[ priority_id ] | 0 | u(3) |
| quality_level_list[ priority_id ] | 0 | u(2) |
| fragment_order_list[ priority_id ] | 0 | u(2) |
| } |  |  |
| } |  |  |
| } |  |  |
| ……………….. |  |  |
| } |  |  |

Table 4 NAL unit syntax

| nal_unit( NumBytesInNALunit ) { | C | Descriptor |
| --- | --- | --- |
| forbidden_zero_bit | All | f(1) |
| nal_ref_idc | All | u(2) |
| nal_unit_type | All | u(5) |
| nalUnitHeaderBytes = 1 |  |  |
| if( nal_unit_type = = 20 ¦ ¦ nal_unit_type = = 21 ) { |  |  |
| extension_flag | All | u(1) |
| discardable_flag | All | u(1) |
| if (extension_flag == 0 ) |  |  |
| simple_priority_id | All | u(6) |
| else { |  |  |
| short_priority_id | All | u(4) |
| fragment_order | All | u(2) |
| temporal_level | All | u(3) |
| dependency_id | All | u(3) |
| quality_level | All | u(2) |
| nalUnitHeaderBytes++ |  |  |
| } |  |  |
| nalUnitHeaderBytes++ |  |  |
| } |  |  |
| … |  |  |
| } |  |  |

Since fragment_order information is specified in NAL unit header or SPS, we can remove it from slice header, as shown in Table 5.

Table 5 Slice header in scalable extension syntax

| slice_header_in_scalable_extension( ) { | C | Descriptor |
| --- | --- | --- |
| first_mb_in_slice | 2 | ue(v) |
| slice_type | 2 | ue(v) |
| if( slice_type = = PR ) { |  |  |
| fragmented_flag | 2 | u(1) |
| if ( fragmented_flag = = 1 ) { |  |  |
| fragment_order | 2 | ue(v) |
| if ( fragment_order != 0) |  |  |
| last_fragment_flag | 2 | u(1) |
| } |  |  |
| if ( fragment_order = = 0 ) { |  |  |
| num_mbs_in_slice_minus1 | 2 | ue(v) |
| luma_chroma_sep_flag | 2 | u(1) |
| } |  |  |
| } |  |  |
| … |  |  |
| } |  |  |

For the same reason, in Scalability Information SEI message, we should add fragment_order as marked in red in .

Table 6 Scalable information SEI message syntax

| scalability_info( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| num_layers_minus1 | 5 | ue(v) |
| for ( i = 0; i <= num_layers_minus1; i++ ) { |  |  |
| layer_id[ i ] | 5 | u(8) |
| fgs_layer_flag[ i ] | 5 | u(1) |
| sub_pic_layer_flag[ i ] | 5 | u(1) |
| sub_region_layer_flag[ i ] | 5 | u(1) |
| profile_level_info_present_flag[ i ] | 5 | u(1) |
| decoding_dependency_info_present_flag[ i ] | 5 | u(1) |
| bitrate_info_present_flag[ i ] | 5 | u(1) |
| frm_rate_info_present_flag[ i ] | 5 | u(1) |
| frm_size_info_present_flag[ i ] | 5 | u(1) |
| layer_dependency_info_present_flag[ i ] | 5 | u(1) |
| init_parameter_sets_info_present_flag[ i ] | 5 | u(1) |
| if (profile_level_info_present_flag[ i ]) { |  |  |
| layer_profile_idc[ i ] | 5 | u(8) |
| layer_constraint_set0_flag[ i ] | 5 | u(1) |
| layer_constraint_set1_flag[ i ] | 5 | u(1) |
| layer_constraint_set2_flag[ i ] | 5 | u(1) |
| layer_constraint_set3_flag[ i ] | 5 | u(1) |
| reserved_zero_4bits /* equal to 0 */ | 5 | u(4) |
| layer_level_idc[ i ] | 5 | u(8) |
| } |  |  |
| if (decoding_dependency_info_present_flag[ i ]) { |  |  |
| temporal_level[ i ] | 5 | u(3) |
| dependency_id[ i ] | 5 | u(3) |
| quality_level[ i ] | 5 | u(2) |
| fragment_order[ i ] | 5 | u(2) |
| } |  |  |
| …. |  |  |
| } |  |  |
| } |  |  |

fragment_order[ i ] is equal to fragment_order of the NAL units in the scalable layer with layer identifier equal to i.

### 3. Conclusion

In this contribution, we propose to move fragment_order information into NAL unit header to keep the information related to quality scalability [quality_level, fragment_order] consistent. For one-byte solution, to establish 1D-3D relationship, we add fragment_order in SPS. For two-byte solution, we use the low order 2 bits of simple_priority_id for fragment_order and leave 4 bits for application. We also propose to add fragment_order in scalable information SEI message.

### 4. References

- J. Reichel, M. Wien and H. Schwarz, “Joint Scalable Video Model JSVM-3 Annex S”, JVT-P202, Poznan, Poland, July 2005

- S. Pateux, Y. Wang and P. Amon, "Revised Syntax for FGS Pass Fractioning", JVT-P112, Poznan, Poland, July 2005

- P. Amon, A. hutter and T. Rathgen, "High-Level Syntax for SVC", JVT-P100, Poznan, Poland, July 2005

- I. Amonou, N. Cammas, S. Kervadec. S. Pateux, "On high level syntax for SVC", JVT-P032, Poznan, Poland, July 2005

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Thomson Inc. |  |  |
| Mailing address | 2 Independence Way Princeton, NJ 08540 |  |  |
| Country | U.S.A |  |  |
| Contact person | Peng Yin |  |  |
| Telephone | +1 609 987 7355 |  |  |
| Fax | +1 609 987 7299 |  |  |
| Email | peng.yin@thomson.net |  |  |
| Place and date of submission | Poznan, Poland July 25, 2005 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT-P |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| x | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| x | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks:

In JSVM-3, fragment_order is coded in slice header using ue(v). But in the definition of base_id_plus_1 and non-required picture SEI message, fragment_order is described as u(2), which means only 2 bits are used for fragment_order.