| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 17th Meeting: Nice, FR, 14-21 October, 2005 | Document: JVT-Q049 Filename: input.doc |
| --- | --- |

| Title: | Enh. layer intra pred using lower layer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lianhuan Xiong，Qingpeng Xie Multimedia Product Line Huawei Technologies Co., Ltd. Shenzhen 518129 P.R.China Zhang Wang, Huazhong university of science and technology Wuhan P.R. China | Tel: Email: | 0086-0755-8965-0174 lh_xiong@huawei.com |
| Source: | Huawei Tech. Co., Ltd. |  |  |

_____________________________

# Abstract

The enhancement layer intra directional prediction is improved by using lower layer information when some samples are not available.

# 1.  Introduction

H.264 INTRA prediction models are applied in the JSVM1 enhancement layer INTRA prediction and the I_BL model is added. For I_BL, the corresponding encoded macroblock information in layer k can be used to process the INTRA prediction of current macroblock in the layer k+1. By using I_BL, on some occasions, better efficiency of prediction and encoding can be achieved.

Figure 1  H.264 luma 1616 INTRA prediction

Figure 2   H.264 luma 44 INTRA prediction

Figure 3   H.264 chroma 88 INTRA prediction

Figure 4  I_BL INTRA prediction

In H.264, the block to be coded in intra mode is predicted by using the up and left pixels around the current block, when coding the enhance layer, the corresponding block in base layer is available, it can be used for intra prediction. But in JSVM3, the enhance layer intra prediction mostly using the intra prediction modes in H.264 except I_BL mode, the base layer information is not used enough. Specially, in H.264 when the up or left pixels around the current block are not available, the corresponding prediction direction will be unavailable, for example in DC prediction, if the up and left pixels are all unavailable, 128 will be used as the prediction value, it is not a good choice in enhance layer intra prediction.

In this proposal, the intra prediction of enhance layer is extended, in enhance layer intra prediction, when the up or left pixels around the current block are not available, the up or left prediction pixels substituted by the corresponding base layer pixels. Except that, the intra prediction mode is similar with the H.264, so it can improve the intra coding efficiency with importing a little computing complexity.

# 2.  Enh. layer intra pred using lower layer

Figure 2,3,4 show that, the intra prediction of JSVM 3.0 is using the up and left pixels of the current block, when the prediction pixels is not available, the direction of that prediction is unavailable. For enhance layer intra prediction, the base layer information can be used for prediction like I_BL mode.

The extended intra prediction method of enhance layer is proposed in this proposal. As the figure 2,3,4 show, when the prediction pixels are available, the intra prediction method is the same with the H.264, if the prediction pixels are unavailable, the prediction pixels substituted by the corresponding base layer pixels. As the figure 5 showed, the pixels in the corresponding base layer block around the block edge are available for intra prediction, for example: if the up pixels are unavailable, the vertical prediction will be done by using the average value of up and bottom pixels in the corresponding base layer block, the others prediction modes for the 16*16 and 4*4 intra block will be done like that.

n n

Layer  k

Layer  k-1

n n

Figure 5 enhance layer intra prediction n=4 or 8

By using the base layer information to extend the intra prediction of the enhance layer, it can improve the intra coding efficiency with importing a little computing complexity.

# 3.  Experimental result

The experimental result shows that the proposed method can improve the enhance layer coding efficiency. The number of intra coding blocks increased. The test results and more detailed information is shown in“JVT-Q049.xls”document.

# 4.  Conclusions:

An improving enhancement INTRA prediction method is given in this proposal. By using the information of the corresponding block in the lower layer, the enhancement encoding efficiency can be improved. The added computational complexity is very small. We suggest that JVT accept it in the SVC.

References:

【1】JVT-P201，Scalable Video Coding - Working Draft 3。

【2】JVT-P202，Joint Scalable Video Model JSVM 3。

【3】ITU-T Recommendation H.264：Advanced video coding for generic audiovisual services, May, 2003.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Huawei Tech. Co.,Ltd. |  |  |
| Mailing address | Lianhuan Xiong Multimedia Product Line Huawei Technologies Co., Ltd. Shenzhen 518129 P.R.China |  |  |
| Country | China |  |  |
| Contact person | Lianhuan Xiong |  |  |
| Telephone | 0086-0755-89650174 |  |  |
| Fax | 0086-0755-89650161 |  |  |
| Email | lh_xiong@huawei.com |  |  |
| Place and date of submission | Shenzhen, 2005-10-11 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Improving enh. layer intra directional prediction using lower layer infor. |  |
| Contribution number |  | JVT-Q050.doc |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| √ | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: