| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 17th Meeting: Nice, FR, 14-21 October, 2005 | Document: JVT-Q054 Filename: input.doc |
| --- | --- |

| Title: | Implementation of redundant pictures in JSVM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jie Jia, Hae-Kwang Kim Multimedia Framework Lab Sejong University Seoul, 143-747, KOREA Hae-Chul Choi, Jae-Gon Kim Broadcasting Media Research Group ETRI Daejeon, 305-350, KOREA | Tel: Email: Tel: Email: | +82 02 3408 3757 joriejiejia@gmail.com hkkim@sejong.ac.kr +82-42-860-1808 choihc@etri.re.kr jgkim@etri.re.kr |
| Source: | Sejong Univ. and ETRI |  |  |

_____________________________

# Abstract

In order to enhance the robustness to packet loss, redundant picture is one of the error resilient tools in MPEG4 H.264/AVC, which allows an encoder to send redundant coded pictures ( or part of the macroblocks in the primary coded picture). As current JSVM software doesn’t support the redundant pictures, in this contribution, the redundant pictures have been implemented into the current JSVM software. Also, a simple description of the usage of this tool is given here.

# Introduction

Redundant picture is a coded representation of a primary picture or a part of a primary picture. The content of the redundant picture will not be decoded except the corresponding primary picture was lost due to packet loss. The redundant coded picture can be coded as one exactly same picture as the primary picture, however, it can also be coded with somewhat quality degradation, in that way, fewer bits will be used to represent the redundant pictures.

In current JSVM, the redundant pictures haven’t been supported, so in this contribution, the redundant pictures were implemented to the current JSVM.

# Redundant pictures in SVC

## Syntax and Semantics in SVC

In SVC, pictures are coded in a layered way, so, the redundant pictures should also be coded in a layered way. When redundant pictures are concerned for each layer, the coding process will be same as what has been described in AVC [1]. [2] gives the syntax and semantics relating to the redundant pictures in SVC. Tab. 1 illustrates the syntax related to redundant pictures. Further description can be referred to [2].

| slice_header_in_scalable_extension( ) { |  |  |
| --- | --- | --- |
| …… |  |  |
| if( redundant_pic_cnt_present_flag ) |  |  |
| redundant_pic_cnt | 2 | ue(v) |
| …… |  |  |
| } |  |  |

Tab. 1 Slice header in scalable extension syntax

## Usage of redundant pictures in JSVM

The usage of redundant pictures can be enabled in configure file. Fig. 1 illustrates the option of using redundant pictures in the current JSVM. When UseRedundantPic is set to 0, JSVM will work in normal way, while when UseRedundantPic is set to 1, redundant pictures will be applied to the current JSVM.

Fig. 1 Redundant Picture can be easily enabled in configure file.

# Current restrictions of redundant pictures in JSVM

As JSVM is different from JM when layered coding is considered, the current implementation of redundant pictures in JSVM will code redundant pictures for all the primary pictures. However, further flexibility can be easily added to SVC with simple modifications to the current semantics. For instance, redundant pictures can be coded for every key picture in each GOP, also redundant pictures can be coded only for the primary pictures which belong to the base layer. This needs to be further implemented in the current JSVM based on the work which has been done in this contribution.

# Reference

- Text of ISO/IEC FDIS 14496-10: Advanced Video Coding. N5555, Mar. 2003.

- ITU-T and ISO/IEC JTC1, “Scalable Video Coding - Working Draft 3”, JVT-P201, Jul 2005

- ITU-T and ISO/IEC JTC1, “JSVM 3 Software”, JVT-P203, Jul 2005.

# (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name |  |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: