| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 17th Meeting: Nice, FR, 14-21 October, 2005 | Document: JVT-Q055 Filename: input.doc |
| --- | --- |

| Title: | Polyphase downsampling based redundant pictures in JSVM for error resilience |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Jia, Hae-Kwang Kim Multimedia Framework Lab Sejong University Seoul, 143-747, KOREA Hae-Chul Choi, Jae-Gon Kim Broadcasting Media Research Group ETRI Daejeon, 305-350, KOREA | Tel: Email: Tel: Email: | +82 02 3408 3757 joriejiejia@gmail.com hkkim@sejong.ac.kr +82-42-860-1808 choihc@etri.re.kr jgkim@etri.re.kr |
| Source: | Sejong Univ. and ETRI |  |  |

_____________________________

# Abstract

In order to enhance error resilience of SVC, Polyphase Downsampling (PD) based redundant picture coding in JSVM is described in this proposal. The presented method implements PD to residual signals before DCT in the redundant picture coding process, which enables the redundant coded picture to be sent flexibly with somewhat degradation on the picture quality corresponding to the quantity of DCT coefficients that have been sent. When part of the DCT coefficients in one redundant picture have been sent, reconstruction utilizing the spatial redundancy inherent in natural pictures between neighboring pixels can be performed to improve the reconstruction quality.

# Introduction

Redundant pictures effectively enhance the robustness of AVC [1] to packet-loss. For the similar reason, in this proposal, redundant picture is applied to SVC [2][3]. The difference is that the redundant picture was coded with Polyphase downsampling (PD) method which has been described in detail in [4].

# Problem to solve

When redundant picture is concerned, the redundant picture can be coded as a simple repetition of a primary picture. However, coding redundant picture in this way will decrease the coding efficiency a lot. Considering both error resilience and coding efficiency, how to encode the redundant picture in a more effective way is a problem need to be solved. For this reason, the PD based redundant picture coding method is proposed in this document.

# Polyphase Downsampling based redundant picture in SVC

## PD based redundant picture coding

The PD based video coding method as described in [5] is adopted in this proposal for redundant picture coding. The proposed method suggest to apply polyphase downsampling to each 8x8 inter residual block before DCT, where 4x4 transformation is adopted. After PD, N/4 of the residual signals can be flexible coded and transmitted according to the available bandwidth, where N can be any integer value between 1 and 4. In this way, the video encoder will generate different bit streams corresponding to different bit rates, respectively.

Besides working independently, the PD based video coding method can also be implemented in a cooperative way, where redundant pictures are encoded not only with the PD based method but also with the other redundant picture coding methods as described in AVC standard to provide further bit-rate flexibility. For instance, redundant pictures can be encoded with both the PD based method and the higher QP based method. In this case, after PD and DCT process, the transformed coefficients are encoded with higher QP values.

## PD based redundant picture reconstruction

In error free environment, video data are reconstructed as described in [2][3]. When the decoded primary picture can not be correctly decoded due to the errors or losses in transmission of the sequence while a coded redundant picture can be correctly decoded, the decoder replace the samples of the decoded primary picture with the corresponding samples of the decoded redundant picture.

In PD based redundant picture coding method, when error occurred, redundant pictures are first decoded from the encoded bit stream, after that, such pixels that are not sent in the encoder are reconstructed with the prediction values and their neighboring pixel values which are decoded from the bit stream.

Fig. 1 illustrated the method for reconstructing such pixels that are not sent in the encoder.

Fig. 1 Pixel reconstruction in PD based redundant slice coding method

In the above figure, the gray color pixels in the current frame represent the reconstructed pixels which are decoded from the bit stream, while the white color pixels represent pixels which are reconstructed by predicting from both the previous frame and their spatial neighboring pixels, as illustrated by the yellow color pixel. When the reconstruction process of the white color pixel is concerned, the prediction value from previous frame is first used as the preliminary reconstruction value, after that, the average value from its neighboring pixels is compared with the preliminary reconstruction value. If the difference between the two prediction values is less than the threshold, the preliminary value is used as the reconstruction value, or else, the average value is used as the reconstruction value.

## Modification in syntax and semantics

### Syntax change:

New syntax (redundant_pic_type) is added into picture parameter set RBSP syntax (S.7.3.2.2) just after redundant_pic_cnt_present_flag as follows: (the new syntax is marked yellow)

| constrained_intra_pred_flag | 1 | u(1) |
| --- | --- | --- |
| redundant_pic_cnt_present_flag | 1 | u(1) |
| if (redundant_pic_cnt_present_flag) { |  |  |
| redundant_pic_type | 1 | ue(v) |
| } |  |  |

Tab. 1 Syntax modification of PPS

### Semantics:

redundant_pic_type specifies how the redundant picture is coded. The value of redundant_pic_type shall be in the range of integer from 0 to 1, inclusive. (Enlarging of the range is possible when new redundant picture coding method is adopted.)

redundant_pic_type equal to 0 specifies the redundant picture is coded as a simple repetition of a primary picture.

redundant_pic_type equal to 1 specifies the redundant picture is coded with PD method proposed in this document.

## Modification in configure file

A new entry is added to the end of encoder configure file for indicating the redundant picture coding type, which is illustrated as .

Fig. 2 Indication of redundant_pic_type in JSVM encoder configure file

# Conclusion

The PD based redundant picture coding for SVC error resilience is described in this document. Further experimental results and performance comparison will be hopefully obtained till next meeting.

# Reference

- Text of ISO/IEC FDIS 14496-10: Advanced Video Coding. N5555, Mar. 2003.

- ITU-T and ISO/IEC JTC1, “Scalable Video Coding - Working Draft 3”, JVT-P201, Jul 2005

- ITU-T and ISO/IEC JTC1, “JSVM 3 Software”, JVT-P203, Jul 2005.

- J. Jia, H.K. Kim, H.C. Choi, J.G. Kim, “Polyphase down sampling based redundant slice coding for SVC error resilience,” JVT-P050, July. 2005.

- J. Jia, and H.K. Kim, “Error resilience for SVC base and enhanced layers,” in JVT-O048, Apr. 2005

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | ETRI |  |  |
| Mailing address | Broadcasting Media Research Group Digital Broadcasting Media Research Division ETRI(Electronics and Telecommunications Research Institute) 161 Gajeong-ding, Yuseong-gu, Daejeon, 305-350, Republic of KOREA |  |  |
| Country | Republic of KOREA |  |  |
| Contact person | Haechul Choi |  |  |
| Telephone | +82-42-860-1808 |  |  |
| Fax | +82-42-860-5479 |  |  |
| Email | choihc@etri.re.kr |  |  |
| Place and date of submission | Nice, FR, 14-21 October, 2005 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Polyphase downsampling based redundant pictures in JSVM for error resilience |  |
| Contribution number |  | JVT-Q055 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| X | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: