| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 17th Meeting: Nice, FR, 14-21 October, 2005 | Document: JVT-Q058 Filename: input.doc |
| --- | --- |

| Title: | Verification And Analysis of Experiments in JVT-P017 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim, Dae-Sung Cho, Dmitry Birinov, and Hyun Mun Kim Multimedia Lab. Samsung AIT P.O.Box 111, Suwon 440-600 Korea | Tel: Email: | 82-31-280-9204 hyunmun27.kim@samsung.com |
| Source: | Samsung AIT |  |  |

_____________________________

# Abstract

A new tool for Advanced 4:4:4 profile has been proposed in JVT-P017 [1]. This document is to verify and discuss the experimental results of JVT-P017 and discuss some issues related to the comparisons. In this comparison we can see that JVT High 4:4:4 (including RCT) is comparable to the method of JVT-P017 in terms of coding efficiency.

# Introduction

In JVT-P017, a luma-like 4x4 intra prediction method is proposed to be applied to chroma components for coding efficiency improvement. It also states that its method prevails over the RCT (residual color transform) in the current JVT FRExt standard. While verifying its experimental results, we found several issues to address. First, we find out some problems in the JVT reference software (JM9.6), which affect the performance of RCT specifically. Hence the comparison in JVT-P017 is revisited in the next section. For the evaluation of the Thomson’s “NEW1” method we used the executable file provided by Thomson.

# Test Conditions

## Reference codec enhancements

The reference software JM9.6 used in the evalution in JVT-P017 has some bugs and improper parameters for encoding have been applied to evaluate RCT. Hence the JM9.6 reference software has been modified to revisit RCT as follows:

- Bug fix of adaptive rounding control for RCT

- Bug fix of skip mode when residue_colour_transform==1

- Lambda value fixing for RCT

Fig.1. RCT improvements over JM9.6 RCT

The above updates will be found in the attached software JVT-Q058_reference_sw.zip. The improvements from the above modification can be seen in Fig.1. The test condition is same as in section 2.3 except the rounding control setting and external lambda value setting. In Fig. 1 the SAIT_RCT_Old means the RCT method of JM9.6 and SAIT_RCT means the RCT method of the modified JM9.6 (JVT-Q058_reference_sw.zip).

We compare the two methods around 45-dB reconstructed quality as shown in Table 1. We can see up to 1 dB (average 0.5 dB) improvements on the update RCT method compared to JM9.6 RCT. The gain goes higher as the image quality increases.

Table 1. PSNR Comparisons in 40Mbps or 100Mbps around 45-dB

|  | Mbps | PSNR Y |  |  |
| --- | --- | --- | --- | --- |
|  |  | RCT_OLD | RCT |  PSNR |
| Analog TV | 100 | 44.1 | 44.7 | 0.6 |
| Bicycle | 40 | 46.4 | 46.5 | 0.1 |
| Card Toss | 100 | 45.5 | 46.0 | 0.5 |
| Dinner | 100 | 45.2 | 45.7 | 0.5 |
| Restaurant | 40 | 48.4 | 48.6 | 0.2 |
| Tomatoes | 100 | 47.0 | 48.0 | 1.0 |
| Avg |  | 46.1 | 46.6 | 0.5 |

## Adaptive rounding control

Fig.2. Results of adaptive rounding control

Adaptive rounding control based on JVT-N011 is currently implemented to enhance the coding efficiency in JM9.6. Now the rounding control was also used in the comparison in JVT-P017 as follows:

- AdaptiveRounding = 1

- AdaptRndPeriod = 1

- AdaptRndChroma = 1

But we saw some strange results when adaptive rounding for chroma component is enabled (AdaptRndChroma==1). The simple results are shown in Fig. 2. Each method has the following meaning:

- RCT_no_rnd: RCT with AdaptiveRouding == 0

- RCT_rnd_luma: RCT with AdaptiveRoudning == 1 & AdaptRndChroma == 0

- RCT_rnd: RCT with AdaptiveRouding ==1 & AdapRndChroma == 1

- YCgCo_no_rnd: YCgCo with AdaptiveRouding == 0

- YCgCo_rnd_luma: YCgCo with AdaptiveRoudning == 1 & AdaptRndChroma == 0

- YCgCo_rnd: YCgCo with AdaptiveRouding ==1 & AdapRndChroma == 1

As shown in Fig.2 the rounding control for each luma and chroma component doesn’t show major improvements compared to rounding control of luma component only.

We don’t know whether this is a bug in JM9.6 or it originally shows the results in the case of 4:4:4 image format. Hence in the final experiments we turn off the rounding control operation, (i.e., AdaptiveRounding == 0) in each method.

## Lamda value fixing for RCT

In RD-optimized mode selection scheme, the rate and the distortion are measured separately for luma and chroma. In intra coding case, the coding mode for each luma and chroma is selected independently to each other. However, in case of RCT, the RD-optimized mode selection scheme is implemented to consider the luma and the chroma together. Therefore, a proper lambda value should be applied to choose best mode both for luma and chroma. We determined this value by experiments, and applied this value by means of explicit lambda value indication in the reference SW.

RCT Off:

LMluma=Dluma+λluma⋅RlumaLMchroma=Dchroma+λchroma⋅Rchroma

RCT On:

LMluma,Mchroma=Dluma+Dchroma+λRCT⋅(Rluma+Rchroma)

## Experimental settings

The test conditions are as follows:

- Test material : 
Film scanned sequences (1920x1088 @ 24 Hz, progressive, 10 bit per pixel),
Viper sequences (1920x1088 @ 24 Hz, progressive, 10 bit per pixel),

- QP : (6,) 12, 18, (24)

- Entropy coding: CABAC

- RD-optimized mode selection: On

- Number of slice groups: 1

- Rounding control : Off

- IBBPBBP… (no reference B picture)

- Search range : 64

- Fast ME : On

Each method is described as follows:

- Thomson : Best one between Thomson RGB and Thomson YCgCo [1]

- JVT : Best one among RCT, YCgCo, and RGB [2]

## Other comments

- PSNR

Average PSNR of Y, Cg, and Co would not be a good measurement for video coding efficiency. In most cases, while Y has much information which requires large bit consumption, Cg and Co can easily get higher PSNR values using fewer amounts of bits than Y. It is also well known that each component has different impact on human visual system where Y has greater impact than others. Therefore, Y PSNR measurement would be the proper approach to objectively measure the coding efficiency. It would be better measuring Y PSNR with rectifying chroma PSNRs of difference methods to have similar values between methods, however this is very much time consuming, which will require several number of iteration after manually observing chroma PSNR disparities.

- CBP

Since previously released Thomson codec had some bugs, we develop independent implementation for this verification test. Only recently, Thomson has released their new codec after fixing bugs. Though the coding efficiency performances of two implementations were similar, there were some differences between them.

The first bullet is the discrepancy in CBP bits between SAIT and Thomson implementation as shown in Table 2. It is obvious that Thomson codec spends much bits for CBP.

Table 2. CBP bits comparison between independent implmentation with different QP setteings.

| Seq. | CBP bits (QP = 12) |  | % |
| --- | --- | --- | --- |
|  | Thomson | SAIT |  |
| AnalogTV | 3800 | 1970 | 93 |
| Bicycle | 5776 | 1817 | 218 |
| CardToss | 3800 | 1851 | 105 |

| Seq. | CBP bits (QP = 12) |  | % |
| --- | --- | --- | --- |
|  | Thomson | SAIT |  |
| AnalogTV | 49365 | 13639 | 262 |
| Bicycle | 57248 | 10978 | 421 |
| CardToss | 54872 | 11904 | 361 |

- Bit-depth expansion for Y component

The second bullet is degradation of coding efficiency in Thomson’s implementation when input data has different bit-depth among color components, for example, 10 bit Y and 11 bit Cg and Co. Therefore, the input data should have same bit-depth for all color components even in YCgCo sequence. We observed that 1 bit expansion in Y component in YCgCo sequence results in small coding efficiency improvement, about 0.1 to 0.2 dB.

# Verification Test Results

With above test conditions, we performed the experiment to verify the coding efficiency of Thomson’s proposal, JVT-P017. We compare the Y PSNR between Thomson and JVT FRExt. For Thomson’s method, we choose best one between RGB and YCgCo, and for JVT FRExt, we choose best one among RCT, RGB, and YCgCo. Table 3 show the Y PSNR differences to verify the coding efficiency of Thomson’s proposed method.

In this table, we select PSNR at two specific bitrate, 40 and 100 Mbps, which had been used as a common test condition during FRExt standardization. And we choose one of them which is closer to 45 dB as Thomson required this level of image quality at the previous JVT meeting in Poznan.

Table 3. Y PSNR differences

| Seq. | Mbps | PSNR Y |  |  |
| --- | --- | --- | --- | --- |
|  |  | Thomson | JVT |  PSNR |
| Analog TV | 100 | 44.4 | 44.2 | -0.2 |
| Bicycle | 40 | 46.25 | 46.1 | -0.1 |
| Card Toss | 100 | 46.2 | 46 | -0.2 |
| Dinner | 100 | 45.8 | 45.6 | -0.2 |
| Restaurant | 40 | 48.45 | 48.25 | -0.2 |
| Tomatoes | 40 | 43 | 43 | 0.0 |
| Avg |  | 45.7 | 45.5 | -0.2 |

In JVT-P017, there is concern about RCT performance. So we compared the performance between inside color transform (RCT) and outside color transform (YCgCo) with applying same intra luma prediction method for all color components to both methods for fair comparison. Table 4 shows the Y PSNR differences between Thomson YCgCo and RCT.

Table 4. Y PSNR differences between RCT and YCgCo.

| Seq. | Mbps | PSNR Y |  |  |
| --- | --- | --- | --- | --- |
|  |  | RCT | YCgCo |  PSNR |
| AnalogTV | 100 | 44.45 | 43.9 | -0.55 |
| Bicycle | 40 | 45.2 | 45 | -0.2 |
| CardToss | 100 | 45.3 | 44.8 | -0.5 |
| Dinner | 100 | 45.2 | 44.75 | -0.45 |
| Restaurant | 40 | 47.3 | 47.1 | -0.2 |
| Tomatoes | 100 | 47.7 | 47.4 | -0.3 |
| Avg |  | 45.9 | 45.5 | -0.4 |

Attached Excel file contains whole results of bitrates, average RGB, and Y PSNR with R-D curve plot. As shown in the experimental results, the in-loop color transform, RCT show better performance than out-loop color transform with improvements as Thomson proposed. Consequently, the overall gain of Thomson’s proposal over the current standard is negligible.

# Conclusion

Thomson has proposed intra prediction scheme and MC interpolation scheme, and there seems to be other changes that have not been fully described in their proposal JVT-P017. From the verification results, we can conclude that JVT High 4:4:4 Profile (including RCT) is comparable to the method of JVT-P017 in terms of coding efficiency. Therefore, we should be cautious to amend the current design.

# References

- H. Yu, and L. Liu, “Advanced 4:4:4 Profile for MPEG4-Part10/H.264,” ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6, Document JVT-P017, July 2005.

- “Fidelity range extensions amendment to ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC,” ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6, Document JVT-L047 (2004).

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Samsung Elctronics. Co. Ltd. |  |  |
| Mailing address | POBox 111 Suwon Samsung AIT Computing Lab. |  |  |
| Country | Korea |  |  |
| Contact person | Hyun Mun Kim |  |  |
| Telephone | 82-31-280-9204 |  |  |
| Fax | 82-31-280-9207 |  |  |
| Email | hyunmun27.kim@samsung.com |  |  |
| Place and date of submission | Poznań, PL, 24-29 July, 2005 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Further experimental results on 4:4:4 coding |  |
| Contribution number |  | JVT-Q |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| X | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: