| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 17th Meeting: Nice, FR, 14-21 October, 2005 | Document: JVT-Q059 Filename: input.doc |
| --- | --- |

| Title: | Advanced Residual Color Transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim, Dae-Sung Cho, Dmitry Birinov, and Hyun Mun Kim Multimedia Lab. Samsung AIT P.O.Box 111, Suwon 440-600 Korea | Tel: Email: | 82-31-280-9204 hyunmun27.kim@samsung.com |
| Source: | Samsung AIT |  |  |

_____________________________

# Abstract

In this document, we propose improved coding methods such as inter-plane prediction and single mode intra prediction to increase the coding efficiency of the current FRExt 4:4:4 video coding technology, especially for high quality image coding using RCT. Our proposed method shows better coding efficiency than the existing FRExt 4:4:4 video coding and the Thomson’s method in JVT-O013 and JVT-P017.

# Introduction

H.264/MPEG-4 AVC Fidelity Range Extensions (FRExt) [1] includes 4:4:4 profile, which provides effective coding tools to deal with not only YCbCr 4:4:4 image but also various color spaces such as RGB and XYZ, supports higher bit-depth up to 12 bit per pixel, and has capability of lossless coding.

During the course of the FRExt standardization the new RGB coding technology had been developed of which the name is residual color transform (RCT) [2-4] as a result of responding to the call for proposal [5] issued to satisfy increasing demand from industry for high quality video coding. The RCT showed effectiveness for RGB coding for various high quality test materials, and adopted to the FRExt. Though RGB video coding is promising area for high quality video coding, it has not been matured enough since it has just take its step a while ago.

In last 16th JVT meeting in Poznan additional research showed room to improve current technology for 4:4:4 video coding and new standardization work was recommended [6]. Since the research area for high quality video coding has just opened recently, we think the research on 4:4:4 coding is desirable and additional standardization work can be encouraged.

In this document, we propose some improvements on the current FRExt technology to increase the coding efficiency of 4:4:4 video coding, especially for RGB coding using RCT. This is the re-collection of our previous proposals for advanced 4:4:4 video coding in addition to the current standard. The methods were not adopted since any major technical changes only for 4:4:4 were not agreed and the consistent structure with 4:2:0 and 4:2:2 formats were preferred at that time. But now it is good time to consider them again since the situation has been changed that there is brisk interest in advanced 4:4:4 profiles.

# Review of proposed methods

## Inter-plane prediction (IPP)

In JVT-H018, JVT-I023, and JVT-IJ017, we proposed IPP (inter plane prediction) to exploit the redundancy between color components after intra/inter predictions [8][10][11]. The equations for the forward and backward IPP operations are:

Encoding:                Decoding:

where X denotes the intra/inter predicted residual value, X' denotes the reconstructed residual value, and 2X and 2X' denote the inter-plane predicted and reconstructed signals, respectively.

It is noted that the residual data of R and B components are predicted not from the original data of residual G component but from the reconstructed residual G data. Compared to the residual color transform (RCT) in FREXT the IPP gives higher coding efficiency specially in Film-grain sequences in relatively higher bit-rates. So it is useful to apply the IPP and the RCT methods adaptively based on the bit-rates or the characteristic of the coded sequences.

## Single mode intra prediction

In JVT-H018 and JVT-J018, new intra chroma prediction was proposed, in which the chroma components use the same prediction mode as the luminance component [8][12]. In FREXT there are 9 modes of 44 intra prediction and 4 modes of 1616 intra prediction for luma component, and 4 modes of 8x8 prediction for chroma components. Since the prediction modes of luma are different from those of chroma, the correlation becomes weak. This would not be optimal for RCT and IPP or for similar in-loop transforms. Hence, it is better to use the same intra prediction mode for all color components to increase the coding efficiency. As proposed in [8][12], this can be achieved by applying the luma intra prediction scheme such as 4x4, 8x8, and 16x16 to the chroma prediction scheme.

## Inter prediction for 4:4:4 chroma format

In JVT-I012 and JVT-J018, we conducted an extensive simulation to compare the performances between 6-tab filter and bilinear transform for various 4:4:4 chroma format contents both in YUV and RGB spaces [9][12]. Table 1 shows the summary of MC interpolation results when we apply the filtering to high resolution images and low resolution images in both spaces [9][12].

## Table1: Selection of best performance for MC interpolation comparison

|  | High Resolution (HD) | Low Resolution (SD) |
| --- | --- | --- |
| RGB space | Bilinear Transform (BT) | 6-tab filter |
| YUV space | BT | Y:6-tab UV:BT |

It shows that bilinear transform gives better performance for more correlated images such as high resolution images or UV components. For low resolution images, 6-tab shows better results for RGB images while 6-tab for Y and bilinear transform for UV shows better performance. In RGB space each component shows the similar characteristics among components. But in YUV space Y component and UV components show the different characteristics. Based on the observations we need to change the MC interpolation scheme for advanced 4:4:4 format. But this change complicates the existing coding structure while much gain from such a change cannot be expected. So we propose to use bilinear interpolation for 4:4:4 format Chroma components as in FREXT. But this should be further investigated for the final decision.

## CBP for 4:4:4 image format

In JVT-I012 and JVT-J018, we proposed how to code the coded_block_pattern (CBP) efficiently for 4:4:4 chroma format [9][12]. Depending on partitions, the bit saving gain differs. For example we can uses CBP method as in FREXT. In the case we cannot take advantage of the CBP since the entire two 16x16 chroma blocks are treated as one block pattern. Instead we can assign coded_block_pattern bit independently for each 8x8 block from YUV components as in Fig 1. In this case we have 212 different cases to code the coded_block_pattern. This may give some problem because it needs a lot overhead bits. On the other hand, we can combine the coded_block_pattern bit for the collocated 8x8 blocks in UV components as shown in Fig 2. In this case there are 28 different cases. Either of Fig. 1 and Fig. 2 are good CBP methods for 4:4:4 chroma format. We need to further investigate which is better for advanced 4:4:4 profile.

Fig.1. 12 independent Coded Block Pattern coding

Fig. 2. 8 independent Coded Block Pattern coding (4 for Y and 4 for UV components)

## Experimental results

The aforementioned intra prediction scheme is implemented in the reference SW, JM96, and the coding efficiency is measured by means of RD-curve and Y-PSNR difference. Test condition is same with those described in section 4 except that the GOP structure is I frame only.

Fig. 3 is the RD-curves which show the improvements using 4x4 chroma intra prediction method described in section 2.2. Table 2 and Table 3 show the coding efficiency gain of single intra prediction method in case of RCT and IPP, respectively.

Table 2. Coding efficiency gain of single mode intra prediction (RCT)

|  | Mbps | PSNR Y |  |  |
| --- | --- | --- | --- | --- |
|  |  | Separate Mode | Single Mode |  PSNR |
| Analog TV | 100 | 45.55 | 45.9 | 0.35 |
| Bicycle | 40 | 48.25 | 49 | 0.75 |
| Card Toss | 100 | 45.9 | 46.55 | 0.65 |
| Dinner | 100 | 46.1 | 46.65 | 0.55 |
| Restaurant | 40 | 50.35 | 50.9 | 0.55 |
| Tomatoes | 100 | 47.4 | 47.85 | 0.45 |
| Avg |  | 47.3 | 47.8 | 0.6 |

Table 3. Coding efficiency gain of single mode intra prediction (IPP)

|  | Mbps | PSNR Y |  |  |
| --- | --- | --- | --- | --- |
|  |  | Separate Mode | Single Mode |  PSNR |
| Analog TV | 100 | 44 | 44.45 | 0.5 |
| Bicycle | 40 | 47.8 | 48.5 | 0.7 |
| Card Toss | 100 | 44.55 | 45.25 | 0.7 |
| Dinner | 100 | 44.7 | 45.2 | 0.5 |
| Restaurant | 40 | 49.7 | 50.2 | 0.5 |
| Tomatoes | 100 | 47.2 | 47.7 | 0.5 |
| Avg |  | 46.3 | 46.9 | 0.6 |

According to Table 2 and Table 3, it is highly desirable to apply same intra prediction scheme for all color components.

Fig. 3. R-D curve showing coding efficiency gain of single intra prediction in case of IPP and RCT

# Adaptive Residual Color Transform

So far we observed that the coding efficiency of inside/outside color transforms is very dependent to the image sequence characteristics. For example, IPP shows good performance where color component independent noise such as film grain is dominant since it reduces inter-color correlation without intermingle each color component unlike RCT. However, if there are strong correlations among color components, RCT shows better results. Therefore, it is preferable to apply RCT adaptively according to the characteristic of given image sequence.

To support this adaptive feature, we propose to change RCT_flag to RCT_type_idc in SPS so that it can indicate proper RCT type according to given image characteristics.

| RCT_type_idc | Meaning |
| --- | --- |
| 0 | No RCT/IPP |
| 1 | RCT |
| 2 | IPP |

# Experimental results

In this section we verify the coding efficiency of proposed methods.

The test conditions are as follows:

- Test material : 
Film scanned sequences (1920x1088 @ 24 Hz, progressive, 10 bit per pixel),
Viper sequences (1920x1088 @ 24 Hz, progressive, 10 bit per pixel)

- QP : (6,) 12, 18, (24)

- Entropy coding: CABAC

- RD-optimized mode selection: On

- Number of slice groups: 1

- Rounding control : Off

- IBBPBBP… (no reference B picture)

- Search range : 64

- Fast ME : On

Each method is described as follows:

- Proposed : Adaptive RCT (RCT and IPP)

- Thomson : Best one between Thomson RGB and Thomson YCgCo [6][7] (Using software provided by Thomson)

- JVT : Best one among RCT, YCgCo, and RGB [1]

Table 4 shows the PSNR Y differences.

Table 4. PSNR Y difference in the experimental results

| Seq. | Mbps | PSNR Y |  |  |
| --- | --- | --- | --- | --- |
|  |  | Thomson | Proposed |  PSNR |
| Analog TV | 100 | 44.3 | 45.5 | 1.2 |
| Bicycle | 40 | 46.2 | 46.7 | 0.5 |
| Card Toss | 100 | 46.3 | 47.0 | 0.8 |
| Dinner | 100 | 45.8 | 46.7 | 0.9 |
| Restaurant | 40 | 48.5 | 48.9 | 0.4 |
| Tomatoes | 100 | 47.6 | 47.6 | 0.0 |
| Avg |  | 46.2 | 47.1 | 0.6 |

Attached Excel file contains whole results of bitrates, RGB average PSNR, and Y PSNR with R-D curve plot.

# Conclusion

In this document we proposed methods to improve the coding efficiency of RGB coding using adaptive RCT in addition to the previous proposals. The experimental results showed that our proposed method gives better coding efficiency than the existing FRExt 4:4:4 video coding and the Thomson’s method in JVT-O013 and JVT-P017. Thus, we recommend our method to be adopted for advanced 4:4:4 coding.

# References

- “Fidelity range extensions amendment to ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC,” ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6, Document JVT-L047 (2004).

- W.-S. Kim and H. M. Kim, “Residue transform,” ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6, Document JVT-J038, Dec. 2003.

- W.-S. Kim, D. Birinov, and H. M. Kim, “Adaptive residue transform and sampling,” ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6, Document JVT-K018, Mar. 2004.

- W.-S. Kim, D. Birinov, and H. M. Kim, “Residue color transform,” ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6, Document JVT-L025, Jul. 2004.

- WG11 Requirements Group, “Call for proposals for extended sample bit depth and chroma format support in the advanced video coding standard (ITU-T H.264 & ISO/IEC 14496-10)”, ISO/IEC JTC1/SC29/WG11, Document N5523 (2003).

- H. Yu, “Performance Improved 4:4:4 Coding for MPEG4-Part10/H.264,” ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6, Document JVT-O013, Apr. 2005.

- H. Yu, and L. Liu, “Advanced 4:4:4 Profile for MPEG4-Part10/H.264,” ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6, Document JVT-P017, July 2005.

- W.-S. Kim, D. Cho and H. M. Kim, “Color Format Extension,” ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6 Document JVT-H018, May 2003.

- W.-S. Kim, D. Cho and H. M. Kim, “Proposal for the unsolved issues in professional extensions,” ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6, Document JVT-I012, Jul. 2003.

- W.-S. Kim, D. Cho and H. M. Kim, “Inter-plane Prediction for RGB Coding,” ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6, Document JVT-I023, Sep. 2003.

- W.-S. Kim, D. Cho and H. M. Kim, “Inter-plane Prediction for RGB Coding II,” ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6, Document JVT-J017, Dec. 2003.

- W.-S. Kim, D. Cho and H. M. Kim, “Proposal for the unsolved issues in Professional Extensions II,” ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6, Document JVT-J018, Dec. 2003.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Samsung Elctronics. Co. Ltd. |  |  |
| Mailing address | POBox 111 Suwon Samsung AIT Computing Lab. |  |  |
| Country | Korea |  |  |
| Contact person | Hyun Mun Kim |  |  |
| Telephone | 82-31-280-9204 |  |  |
| Fax | 82-31-280-9207 |  |  |
| Email | hyunmun27.kim@samsung.com |  |  |
| Place and date of submission | Poznań, PL, 24-29 July, 2005 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Advanced Residual Color Transform |  |
| Contribution number |  | JVT-Q |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: