| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 17th Meeting: Nice, France, 17-21 Oct 2005 | Document: JVT-Q075 Filename: input.doc |
| --- | --- |

| Title: | Improvements of Scalability Information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Truong Cong Thang1, Tae Meon Bae1, Yong Man Ro1, Jung Won Kang2, Jae-Gon Kim2, | Tel: Email: | {tcthang, yro}@icu.ac.kr jungwon@etri.re.kr |
| Source: | 1ICU, 2ETRI |  |  |

_____________________________

1. Introduction

In last Poznan meeting, a new syntax structure of scalability information SEI message was proposed. This syntax describe important information of all the possible layers of a SVC bitstream, namely base layer, temporal/spatial/SNR enhancement layers.

From our experience in implementing the ROI extraction show case, we found that, when there exist ROIs in a SVC bitstream, the current syntax of scalability information SEI message is still inconvenient to describe the layers of different ROIs. For this purpose, we propose some improvements to the current syntax.

2. Proposed changes

2.1 Reorganization of the syntax

Currently, there is one loop in the syntax to describe layer-by-layer. When there are ROIs and background in the bitstream, the scalability information of each ROI (or background) is repeated in a single loop. With the current syntax, layer descriptions of an ROI can be mixed with descriptions of another ROI as long as the rule of layer id numbering is not violated. This issue is illustrated in Fig. 1. In this example, there is one ROI and background, each has 6 scalable layers.

ROI_1

layer_id=0, QCIF Base layer info.

layer_id=1, QCIF FGS1 layer info.

layer_id=2, QCIF Base layer info.

layer_id=4, QCIF FGS1 layer info.

layer_id=8, CIF spatial enh. layer info.

layer_id=3, QCIF FGS2 layer info.

for ROI_1

layer_id=5, QCIF FGS2 layer info.

layer_id=6, CIF spatial enh. layer info.

layer_id=7, CIF FGS1 layer info.

for background

for background

Background

layer_id=9, CIF FGS1 layer info.

layer_id=10, CIF FGS2 layer info.

layer_id=11, CIF FGS2 layer info.

Fig. 1: Example of scalable layer information for SVC bitstream with ROI

The difficulty with this syntax structure is that, to find information of all scalable layers of an ROI, we always need to go through the whole SEI message. To avoid this complexity, we propose to divide the whole message into "sections", and each section is identified by one section_id. A section can be used to contain all layers of an ROI, or all layers which does not belong to any ROI. This reorganization is illustrated in Fig. 2. When there is no ROI in the bitstream, the number of sections is 1 which is the same as the current syntax structure.

section_id = 0

layer_id=0, QCIF Base layer info.

layer_id=1, QCIF FGS1 layer info.

layer_id=2, QCIF FGS2 layer info.

layer_id=3, CIF spatial enh. layer info.

layer_id=4, CIF FGS1 layer info.

layer_id=5, CIF FGS2 layer info.

for ROI_1

section_id = 1

layer_id=6, QCIF Base layer info.

layer_id=7, QCIF FGS1 layer info.

layer_id=8, QCIF FGS2 layer info.

layer_id=9, CIF spatial enh. layer info.

layer_id=10, CIF FGS1 layer info.

layer_id=11, CIF FGS2 layer info.

for background

Fig. 2: Reorganizing scalable layer information into sections

Because for ROI scalability, one section now describes one ROI, so the information of ROI id can be put outside the loop. That is, ROI id information will not have to be repeated in every layer anymore. To assign ROI id to each section, we propose to remove the current sub_pic_layer_flag and have a separate syntax for ROI-to-section mapping. This syntax will be enabled/disabled by a roi_flag. It should be noted that there may be overlapped regions between ROIs (as shown in Fig. 3). So the mapping must support that a section id corresponding an overlapped region is associated to more than one ROI id's. In this syntax, the element num_component_sections_minus1 plus 1 indicates the number of sections (and component_section_id indicates id of each section) that constitute an ROI. The detailed reorganization of the syntax is shown in section 3 (in red).

ROI 1

(ROI_2-OR)

Overlapped region

(OR)

(ROI_1 - OR)

ROI 2

Fig. 3: Illustration of overlapped region between two ROIs

2.2 Removal of sub_pic_scalable_layer_info SEI message

The use of sub_pic_scalable_layer_info SEI message is to connect each scalable layer with one motion-constrained slice group set message, which is used to describe the id's of slice groups belonging to one ROI. We find that searching and parsing these two specific SEI messages (among many other messages) for each scalable layer is rather time-consuming. So we propose to put the id of the related slice group right in the loop for each layer. The presence of slice group id is now enabled/disabled by the roi_flag.

3. Improved syntax

Semantics of the new elements:

- roi_flag: indicates if ROI information exists

- num_rois_minus1: plus 1 indicates the number of ROIs

- roi_id: indicates the identification of an ROI

- num_component_sections_minus1: plus 1 indicates the number of sections that constitute an ROI

- component_section_id: indicates the id of a section which belongs to an ROI

- num_section_minus1: plus 1 indicates the number of sections

- section_id: indicates the id of a section

- slice_group_id: indicates the slice group id which corresponds to the related section

| scalability_info( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| roi_flag | 5 | u(1) |
| if (roi_flag) { |  |  |
| num_rois_minus1 | 5 | u(3) |
| for (i = 0; i <= num_rois_minus1; i++) { |  |  |
| roi_id[ i ] | 5 | u(8) |
| num_component_sections_minus1[ i ] | 5 | u(3) |
| for (j = 0; j <= num_component_sections_minus1; j++) { |  |  |
| component_section_id[ i ][ j ] | 5 | u(8) |
| } |  |  |
| } |  |  |
| } |  |  |
| num_sections_minus1 | 5 | u(3) |
| for ( k = 0; k <= num_sections_minus1; k++) { |  |  |
| if(roi_flag) |  |  |
| section_id[ k ] | 5 | u(8) |
| num_layers_minus1[ k ] | 5 | ue(v) |
| for ( i = 0; i <= num_layers_minus1[ k ]; i++ ) { |  |  |
| layer_id[ k ][ i ] | 5 | u(8) |
| fgs_layer_flag[ k ][ i ] | 5 | u(1) |
| sub_region_layer_flag[ k ][ i ] | 5 | u(1) |
| profile_level_info_present_flag[ k ][ i ] | 5 | u(1) |
| decoding_dependency_info_present_flag[ k ][ i ] | 5 | u(1) |
| bitrate_info_present_flag[ k ][ i ] | 5 | u(1) |
| frm_rate_info_present_flag[ k ][ i ] | 5 | u(1) |
| frm_size_info_present_flag[ k ][ i ] | 5 | u(1) |
| layer_dependency_info_present_flag[ k ][ i ] | 5 | u(1) |
| init_parameter_sets_info_present_flag[ k ][ i ] | 5 | u(1) |
| if (profile_level_info_present_flag[ k ][ i ]) { |  |  |
| layer_profile_idc[ k ][ i ] | 5 | u(8) |
| layer_constraint_set0_flag[ k ][ i ] | 5 | u(1) |
| layer_constraint_set1_flag[ k ][ i ] | 5 | u(1) |
| layer_constraint_set2_flag[ k ][ i ] | 5 | u(1) |
| layer_constraint_set3_flag[ k ][ i ] | 5 | u(1) |
| reserved_zero_4bits /* equal to 0 */ | 5 | u(4) |
| layer_level_idc[ k ][ i ] | 5 | u(8) |
| } |  |  |
| if (decoding_dependency_info_present_flag[ k ][ i ]) { |  |  |
| temporal_level[ k ][ i ] | 5 | u(3) |
| dependency_id[ k ][ i ] | 5 | u(3) |
| quality_level[ k ][ i ] | 5 | u(2) |
| } |  |  |
| if (bitrate_info_present_flag[ k ][ i ]) { |  |  |
| avg_bitrate[ k ][ i ] | 5 | u(16) |
| max_bitrate[ k ][ i ] | 5 | u(16) |
| } |  |  |
| if (frm_rate_info_present_flag[ k ][ i ]) { |  |  |
| constant_frm_rate_idc[ k ][ i ] | 5 | u(2) |
| avg_frm_rate[ k ][ i ] | 5 | u(16) |
| } |  |  |
| if (frm_size_info_present_flag[ k ][ i ]) { |  |  |
| frm_width_in_mbs_minus1[ k ][ i ] | 5 | ue(v) |
| frm_height_in_mbs_minus1[ k ][ i ] | 5 | ue(v) |
| } |  |  |
| if (sub_region_layer_flag[ k ][ i ]) { |  |  |
| base_region_layer_id[ k ][ i ] | 5 | u(8) |
| dynamic_rect_flag[ k ][ i ] | 5 | u(1) |
| if (dynamic_rect_flag[ k ][ i ]) { |  |  |
| horizontal_offset[ k ][ i ] | 5 | u(16) |
| verticial_offset[ k ][ i ] | 5 | u(16) |
| region_width[ k ][ i ] | 5 | u(16) |
| region_height[ k ][ i ] | 5 | u(16) |
| } |  |  |
| } |  |  |
| if (roi_flag) { |  |  |
| slice_group_id[ k ][ i ] | 5 | u(3) |
| } |  |  |
| If (layer_depdendency_info_present_flag[ k ][ i ]) { |  |  |
| num_directly_dependent_layers[ k ][ i ] | 5 | ue(v) |
| for ( j = 0; j < num_directly_dependent_layers[ k ][ i ]; j++ ) |  |  |
| directly_dependent_layer_id_delta[ k ][ i ][ j ] | 5 | ue(v) |
| } |  |  |
| If (init_parameter_sets_info_present_flag[ k ][ i ]) { |  |  |
| num_ init_seq_parameter_set_minus1[ k ][ i ] | 5 | ue(v) |
| for ( j = 0; j <= num_seq_parameter_set_minus1[ k ][ i ]; j++ ) |  |  |
| init_seq_parameter_set_id_delta[ k ][ i ][ j ] | 5 | ue(v) |
| num_ init_pic_parameter_set_minus1[ k ][ i ] | 5 | ue(v) |
| for ( j = 0; j <= num_pic_parameter_set_minus1[ k ][ i ]; j++ ) |  |  |
| init_pic_parameter_set_id_delta[ k ][ i ][ j ] | 5 | ue(v) |
| } |  |  |
| } |  |  |
| } |  |  |

(Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Information and Communications University |  |  |
| Mailing address | 119, Munjiro, Yusong-gu, Daejeon, Korea |  |  |
| Country | Korea |  |  |
| Contact person | Yong Man Ro |  |  |
| Telephone | 82-42-866-6129 |  |  |
| Fax | 82-42-866-6245 |  |  |
| Email | yro@icu.ac.kr |  |  |
| Place and date of submission | Nice, France, 14-21 Oct. 2005 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | ITU-T Rec. H.264 ¦ ISO/IEC 14496-10 |  |
| Title |  | Improvements of Scalability Information SEI message |  |
| Contribution number |  | JVT-Q075 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| V | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| V | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: