| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 17th Meeting: Nice, France, 17-21 Oct 2005 | Document: JVT-Q077 Filename: input.doc |
| --- | --- |

| Title: | Show case of ROI extraction using scalability information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Truong Cong Thang1, Duckeon Kim1, Tae Meon Bae1, Jung Won Kang2, Yong Man Ro1, Jae-Gon Kim2 | Tel: Email: | {tcthang, yro}@icu.ac.kr jungwon@etri.re.kr |
| Source: | 1ICU, 2ETRI |  |  |

_____________________________

1. Introduction

In last Poznan meeting, a method to enable ROI extraction from SVC bitstream was proposed [1]. To help the extractor in extracting ROI bitstreams, the information of ROIs inside a SVC bitstream was included in scalability information SEI message [2]. In this contribution, we present a show case of ROI extraction using scalability information SEI message. Here, we focus on the application where ROI is extracted according to ROI id. In addition, we propose one correction to the current syntax of scalability information SEI message, so as to support overlapped regions between ROIs.

2. System overview

An overview of the system for ROI-based video communication is shown in Fig. 1. At the encoder, raw video data is encoded into SVC NAL units. Also, the information about ROIs in bitstream (and other scalability information) is encoded into one NAL unit. In this system, this SEI NAL unit is put before video data NAL unit. At the extractor, the SEI message is parsed to find the necessary information of ROIs. The extractor also takes as input the id's of ROIs which should be extracted. These ROI ids can be provided by end user or decided based on the limited resources of connection link or end-device. The extractor then examine each video NAL unit to determine if that NAL unit is to be discarded or not. Finally, the decoder decodes the stream of extracted (or remained) NAL units.

raw video data

Encoding of video data

Removing unneeded NAL units

Decoding/ parsing of SEI message

requested ROI ids

Encoder

Extractor

original bitstream

extracted  ROI-bitstream

Decoding extracted ROIs

Decoder

Encoding of SEI message

Fig. 1: Overview of a video communication with ROI extraction

3. ROI extraction procedure

As mentioned above, from the original bitstream, the extractor can take any ROI at any available spatial resolution. The requested ROIs together with their requested spatial resolution can be decided by the user or by the constraints of end-devices and networks. Here, we suppose that the request information is already given. In ROI extraction, operation consists of 2 steps. First step is parsing SEI message to get the information for ROI extraction. The last step is to extract the bitstream from original SVC bitstream with the information.

3.1 Parsing SEI

To extract ROI, it needs to get information about ROI from input bitstream. The information is described in the SEI in NAL bitstream. It may get the information of ROI description by parsing the SEI data. Spatial scalable Info. SEI, Sub-picture scalable layer info. SEI, motion-constrained slice group set SEI are parsed to get ROI information. The extracted information related with ROI are layer_id of ROI containing scalable layer, number of related ROIs with the scalable layer, and the scalable information of the layer such as temporal level, dependency id, and quality level. By checking sub_pic_layer_flag, the scalable layer that contains ROI is identified. And then, for the layers that contain ROIs, num_related_rois that represents number of ROIs related with the scalable layer is read. And ROI_ids for each related ROI are read. Sub-picture scalable layer info. SEI and the motion-constrained slice group set SEI are always paired. And from these SEIs, the mapping information between slice group id and layer_id is extracted, thereby each slice group is mapped into ROI_id.

3.2 Sequence Extraction Operation

ROI Extraction is the operation that selects NALs related with ROI_id which is inputted by input parameter. By parsing NAL header, the scalable information as temporal level, dependency id, and quality level of the NAL is informed, and the NALs related with the inputted extraction point are chosen. And, from the mapping information between ROI_id and slice group id, the slice groups related with the ROI_id are identified. We assume that slice group id of each slice data are in increasing order. And among the NALs chosen by scalability extraction point, the ones that contain slice group id related with ROI are finally selected. The extracted bitstream is composed of the selected NALs.

4. Extraction example

In this section, we show an example of ROI extraction. In this example, there are two ROIs and one overlapped regions between ROIs as illustrated in Fig. 2. This bitstream is encoded with two spatial layers (QCIF and CIF).

Fig. 2: Illustration of two ROIs in the Bus sequence

The instruction format to run the extractor program is as follows:

>> [exractor.exe] [input sequence] [output sequence] [-r R] [-e WxH@F:B] r: ROI Extraction Mode; R : ROI id; W : Frame width; H : Frame height; F : Frame rate; B : Target bitrate,

Figures 3, 4, and 5 show the screen captures of extraction processes and the displays of extracted ROIs.

Fig. 3: Display of extraction process of extractor

Fig. 4: Display of extracted ROI 1

Fig. 5: Display of extracted ROI 2

5. Correction of current syntax

In the current syntax of scalability information SEI message, each scalable layer is associated with only one ROI id. However, as shown in JVT-042, there are some overlapped regions between ROIs, and a scalable layer of an overlapped region needs to be associated with all the ROIs which contain the overlapped region. Actually this is a missed point in integrating two proposal JVT-P042 and JVT-P061. So, we propose to make a correction to the current syntax as follows (highlighted in red).

Semantics of added element

num_related_rois: indicates the numbers of ROIs to which the scalable layer belongs.

| scalability_info( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| num_layers_minus1 | 5 | ue(v) |
| for ( i = 0; i <= num_layers_minus1; i++ ) { |  |  |
| layer_id[ i ] | 5 | u(8) |
| fgs_layer_flag[ i ] | 5 | u(1) |
| sub_pic_layer_flag[ i ] | 5 | u(1) |
| sub_region_layer_flag[ i ] | 5 | u(1) |
| profile_level_info_present_flag[ i ] | 5 | u(1) |
| decoding_dependency_info_present_flag[ i ] | 5 | u(1) |
| bitrate_info_present_flag[ i ] | 5 | u(1) |
| frm_rate_info_present_flag[ i ] | 5 | u(1) |
| frm_size_info_present_flag[ i ] | 5 | u(1) |
| layer_dependency_info_present_flag[ i ] | 5 | u(1) |
| init_parameter_sets_info_present_flag[ i ] | 5 | u(1) |
| if (profile_level_info_present_flag[ i ]) { |  |  |
| layer_profile_idc[ i ] | 5 | u(8) |
| layer_constraint_set0_flag[ i ] | 5 | u(1) |
| layer_constraint_set1_flag[ i ] | 5 | u(1) |
| layer_constraint_set2_flag[ i ] | 5 | u(1) |
| layer_constraint_set3_flag[ i ] | 5 | u(1) |
| reserved_zero_4bits /* equal to 0 */ | 5 | u(4) |
| layer_level_idc[ i ] | 5 | u(8) |
| } |  |  |
| if (decoding_dependency_info_present_flag[ i ]) { |  |  |
| temporal_level[ i ] | 5 | u(3) |
| dependency_id[ i ] | 5 | u(3) |
| quality_level[ i ] | 5 | u(2) |
| } |  |  |
| if (bitrate_info_present_flag[ i ]) { |  |  |
| avg_bitrate[ i ] | 5 | u(16) |
| max_bitrate[ i ] | 5 | u(16) |
| } |  |  |
| if (frm_rate_info_present_flag[ i ]) { |  |  |
| constant_frm_rate_idc[ i ] | 5 | u(2) |
| avg_frm_rate[ i ] | 5 | u(16) |
| } |  |  |
| if (frm_size_info_present_flag[ i ]) { |  |  |
| frm_width_in_mbs_minus1[ i ] | 5 | ue(v) |
| frm_height_in_mbs_minus1[ i ] | 5 | ue(v) |
| } |  |  |
| if (sub_region_layer_flag[ i ]) { |  |  |
| base_region_layer_id[ i ] | 5 | u(8) |
| dynamic_rect_flag[ i ] | 5 | u(1) |
| if (dynamic_rect_flag[ i ]) { |  |  |
| horizontal_offset[ i ] | 5 | u(16) |
| verticial_offset[ i ] | 5 | u(16) |
| region_width[ i ] | 5 | u(16) |
| region_height[ i ] | 5 | u(16) |
| } |  |  |
| } |  |  |
| if (sub_pic_layer_flag[ i ]) { |  |  |
| num_related_rois[ i ] | 5 | u(3) |
| for ( j = 0; j < num_related_rois[ i ]; j++) |  |  |
| roi_id [ i ][ j ] | 5 | u(3) |
| } |  |  |
| If (layer_depdendency_info_present_flag[ i ]) { |  |  |
| num_directly_dependent_layers[ i ] | 5 | ue(v) |
| for ( j = 0; j < num_directly_dependent_layers[ i ]; j++ ) |  |  |
| directly_dependent_layer_id_delta[ i ][ j ] | 5 | ue(v) |
| } |  |  |
| If (init_parameter_sets_info_present_flag[ i ]) { |  |  |
| num_ init_seq_parameter_set_minus1[ i ] | 5 | ue(v) |
| for ( j = 0; j <= num_seq_parameter_set_minus1[ i ]; j++ ) |  |  |
| init_seq_parameter_set_id_delta[ i ][ j ] | 5 | ue(v) |
| num_ init_pic_parameter_set_minus1[ i ] | 5 | ue(v) |
| for ( j = 0; j <= num_pic_parameter_set_minus1[ i ]; j++ ) |  |  |
| init_pic_parameter_set_id_delta[ i ][ j ] | 5 | ue(v) |
| } |  |  |
| } |  |  |

References

- Truong Cong Thang, et al., "SVC CE8: Spatial scalability of multiple ROIs", JVT-P042, Poznan, July 2005.

- Ye-Kui Wang and Truong Cong Thang, "Revised signaling of scalability information", JVT-P113, Poznan, July 2005.

(Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Information and Communications University |  |  |
| Mailing address | 119, Munjiro, Yusong-gu, Daejeon, Korea |  |  |
| Country | Korea |  |  |
| Contact person | Yong Man Ro |  |  |
| Telephone | 82-42-866-6129 |  |  |
| Fax | 82-42-866-6245 |  |  |
| Email | yro@icu.ac.kr |  |  |
| Place and date of submission | Nice, France, 14-21 Oct. 2005 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | ITU-T Rec. H.264 ¦ ISO/IEC 14496-10 |  |
| Title |  | Show case of ROI extraction using scalability information SEI message |  |
| Contribution number |  | JVT-Q077 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| V | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| V | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: