| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 17th Meeting: Nice, FR, 14-21 October, 2005 | Document: JVT-Q086r1 Filename: input.doc |
| --- | --- |

| Title: | Intra-only 4:4:4 Profile for H.264/AVC FRExt |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Steffen Wittmann, Thomas Wedi Panasonic R&D Center Germany Monzastr. 4c 63225 Langen, Germany Satoshi Kondo Matsushita Electric Industrial Co., Ltd. 1006, Kadoma, Kadoma City Osaka, 571-8501, Japan Hiroshi Saito Professional AV Development Center Matsushita Electric Industrial Co., Ltd 2-15 Matsuba-cho, Kadoma-shi Osaka, 571-8503, Japan | Tel: Email: Tel: Email: Tel: Email: Tel: Email: | +49 6103 766 1306 Steffen.Wittmann -at- eu.panasonic.com +49 6103 766 1194 Thomas.Wedi -at- eu.panasonic.com +81-6-6900-9689 Kondo.Satoshi -at- jp.panasonic.com +81-6-6905-4097 Saito.Hiroshi002 -at- jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

For professional applications, Intra-only coding is a common coding method. Furthermore, several experiments and demonstrations have shown that intra coding of H.264/AVC has an excellent performance, even compared to state of the art still image coding schemes.

The current H.264/AVC coding scheme uses less modes for the chroma color-components of a MB. This is a drawback for RGB or YUV-4:4:4 video material. Furthermore, in the current scheme, the color components cannot be coded independently and the possibility of parallel processing is limited. This limitation is yet another drawback, because parallelization is essential, especially for future professional cameras that record video content at incessant higher resolutions.

In this contribution an Intra-only coding scheme is proposed that allows independently encoding and decoding of each color component using the luma prediction modes. Therefore, separate mode information is associated to each color component on MB basis. In order to restrict the memory consumption, it is further proposed to keep the interleaving of color components on MB-level. Thus, the possibility of parallel processing for H.264/AVC Intra only applications is significantly enhanced. Additionally, experimental results show a gain of more than 1dB in coding efficiency for higher bitrates as they are valid for professional applications.

In order to take the advantage of different coding schemes (e.g. 1 common mode for all components or 3 individual modes), it is proposed to signal the used coding scheme in the bitstream. Thus, encoders can select the most appropriate coding scheme.

Since Intra-only is an important coding method for professional applications and future products, we further propose to create an Intra-only 4:4:4 profile.

Along with this proposal we provide a software implementation (JVT-Q086-software.zip) and the necessary syntax modifications (chapter 5). It turned out, that these syntax modifications are only minor.

# Current Intra Coding

The following Figure 1 shows a block diagram of an exemplary intra-coding scheme.

Figure 1: Block diagram of current YUV based coding scheme that is used in H.264/AVC

The current H.264/AVC standard uses one individual mode for the Y-component and one common mode for the U- and V-components. Due to the usage of the same mode for U- and V-components, the possibility for parallel processing is limited.

In Figure 2 the mode selection that is based on the G-component is used to code the R-and B-component. Due to the usage of the same mode for all three color-components, the possibility for parallel processing is limited.

Figure 2: Block diagram of a modified coding scheme that is based on same modes for all three components

Note: In the example of Figure 2, only the G-component is used to select the prediction mode. If more than one component is used to select the prediction mode, it is even more difficult to use a parallel processing structure.

# Proposal

In order to enable completely parallel processing of each color-component (e.g. 3 single-chip encoding in parallel) we propose:

- Parallel and independently coding of each color component using the luma modes. This is proposed in order to be able to have an efficient parallel processing. Each color component has an own header including the mode information.

- Interleaving the encoded data on MB level. This is proposed, in order to reduce the buffer size in the coding process.

- Creation of a new Intra-only 4:4:4 profile with a flag that signals the applied coding scheme, e.g. each color component uses independent prediction modes or all color components use the same prediction mode.

The following Figure 3 shows an exemplary block diagram of an encoder for the proposed modifications.

Figure 3: Block diagram for the proposed Intra-only coding structure.

As it can be seen in Figure 3, it is proposed to use separate mode information for each color component. Furthermore it is proposed to apply the luma mode selection to each color component. Since each component-coder does not have to consider the mode that is used in another component-coder, parallel processing is much easier. In order to reduce buffer sizes, it is proposed to interleave the coded information on MB basis.

In order to take the advantage of different coding schemes (e.g. Figure 2 or Figure 3), it is proposed to signal the used coding scheme in the bitstream for the new Intra-only 4:4:4 profile. Thus, encoders can select the most appropriate coding method. We also proposed to consider this signaling for inter case.

Since Intra-only is an important coding mode for professional applications and future products, we further propose to create an Intra-only 4:4:4 profile.

# Experimental results

## Test sequences

The modified jm10.1 codec is used for the simulations and assigns independent mode information to each color component. The correspondent source code is provided together with this proposal. Sequences of the “Dolby 4-4-4 Film Content Kit Two” serve as input sequences with a size of 1920x1080, a clip-length of 50 frames and 10-bit per color component. The YUV data was converted from the RGB data beforehand.

## Settings

Intra-only coding along with the Hi-4:4:4 profile is performed. The settings for the quantization parameters are QP=6, 12, 18, 24, 30. Other settings are in compliance with the encoder.cfg file that is provided together with the jm10.1 codec.

## Results

The following RD-plots show the results of the simulation. For each sequence exist two different plots: One is for RGB coding including a curve for residue color transform, the other one is for YUV input signals. The curve with the label ‘JM10.1’ shows the results generated with the state-of-the-art Hi-4:4:4 Profile, whereas ‘Jm10.1 ModeExt’ stands for the results generated by the proposal (each color component has its own spatial prediction mode). Each PSNR value in the plot represents the arithmetic mean of the PSNR values for each color component.

In the RGB case:	PSNR = (PSNR-R + PSNR-G + PSNR-B) / 3

In the YUV case: 	PSNR = (PSNR-Y + PSNR-U + PSNR-V) / 3

The proposal outperforms the JM10.1 codec for RGB or YUV-4:4:4 sequences. In the area for professional applications (PSNR > 40dB) gains around 1dB can be observed.

# Syntax and semantic modifications

As stated above, each color component is encoded independently and with an own header in order to allow parallel processing. Hence the syntax changes are quite small. The modification consists in adding a new profile_idc and the mode_extension_flag, in a loop over the macroblock layer and in the storage of the corresponding prediction modes.

In this example, it is switched between the state-of-the-art coding and the method that allows 3 independent spatial prediction modes. Using the proposed mode extension flag, this scheme can be extended easily in order to enable different methods like applying one common prediction mode for all color components.

7.3.2.1 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| profile_idc | 0 | u(8) |
| constraint_set0_flag | 0 | u(1) |
| constraint_set1_flag | 0 | u(1) |
| constraint_set2_flag | 0 | u(1) |
| constraint_set3_flag | 0 | u(1) |
| reserved_zero_4bits /* equal to 0 */ | 0 | u(4) |
| level_idc | 0 | u(8) |
| seq_parameter_set_id | 0 | ue(v) |
| if( profile_idc = = 100 ¦ ¦ profile_idc = = 110 ¦ ¦ profile_idc = = 122 ¦ ¦ profile_idc = = 144 ¦ ¦ profile_idc = = 145 ) { |  |  |
| chroma_format_idc | 0 | ue(v) |
| if( chroma_format_idc = = 3 ) |  |  |
| residual_colour_transform_flag | 0 | u(1) |
| bit_depth_luma_minus8 | 0 | ue(v) |
| bit_depth_chroma_minus8 | 0 | ue(v) |
| if( profile_idc = = 145 ) |  |  |
| mode_extension_flag | 0 | u(1) |
| qpprime_y_zero_transform_bypass_flag | 0 | u(1) |
| seq_scaling_matrix_present_flag | 0 | u(1) |
| if( seq_scaling_matrix_present_flag ) |  |  |
| for( i = 0; i < 8; i++ ) { |  |  |
| seq_scaling_list_present_flag[ i ] | 0 | u(1) |
| if( seq_scaling_list_present_flag[ i ] ) |  |  |
| if( i < 6 ) |  |  |
| scaling_list( ScalingList4x4[ i ], 16, UseDefaultScalingMatrix4x4Flag[ i ]) | 0 |  |
| else |  |  |
| scaling_list( ScalingList8x8[ i – 6 ], 64, UseDefaultScalingMatrix8x8Flag[ i – 6 ] ) | 0 |  |
| } |  |  |
| } |  |  |
| ...... |  |  |

7.3.4  Slice data syntax

| slice_data( ) { | C | Descriptor |
| --- | --- | --- |
| if( entropy_coding_mode_flag ) |  |  |
| while( !byte_aligned( ) ) |  |  |
| cabac_alignment_one_bit | 2 | f(1) |
| CurrMbAddr = first_mb_in_slice * ( 1 + MbaffFrameFlag ) |  |  |
| moreDataFlag = 1 |  |  |
| prevMbSkipped = 0 |  |  |
| do { |  |  |
| if( slice_type != I && slice_type != SI ) |  |  |
| if( !entropy_coding_mode_flag ) { |  |  |
| mb_skip_run | 2 | ue(v) |
| prevMbSkipped = ( mb_skip_run > 0 ) |  |  |
| for( i=0; i<mb_skip_run; i++ ) |  |  |
| CurrMbAddr = NextMbAddress( CurrMbAddr ) |  |  |
| moreDataFlag = more_rbsp_data( ) |  |  |
| } else { |  |  |
| mb_skip_flag | 2 | ae(v) |
| moreDataFlag = !mb_skip_flag |  |  |
| } |  |  |
| if( moreDataFlag ) { |  |  |
| if( MbaffFrameFlag && ( CurrMbAddr % 2 = = 0 ¦ ¦ ( CurrMbAddr % 2 = = 1 && prevMbSkipped ) ) ) |  |  |
| mb_field_decoding_flag | 2 | u(1) ¦ ae(v) |
| if( mode_extension_flag ) |  |  |
| max_color_comp = 3 /* loop 3-times for 4:4:4 intra */ |  |  |
| else |  |  |
| max_color_comp = 1 /* loop only 1-times for !4:4:4 intra */ |  |  |
| for( color_ comp = 0; color_ comp < max_color_comp; color_ comp ++ ) |  |  |
| macroblock_layer( color_ comp ) | 2 ¦ 3 ¦ 4 |  |
| } |  |  |
| if( !entropy_coding_mode_flag ) |  |  |
| moreDataFlag = more_rbsp_data( ) |  |  |
| else { |  |  |
| if( slice_type != I && slice_type != SI ) |  |  |
| prevMbSkipped = mb_skip_flag |  |  |
| if( MbaffFrameFlag && CurrMbAddr % 2 = = 0 ) |  |  |
| moreDataFlag = 1 |  |  |
| else { |  |  |
| end_of_slice_flag | 2 | ae(v) |
| moreDataFlag = !end_of_slice_flag |  |  |
| } |  |  |
| } |  |  |
| CurrMbAddr = NextMbAddress( CurrMbAddr ) |  |  |
| } while( moreDataFlag ) } |  |  |
| } |  |  |

7.3.5   Macroblock layer syntax

| macroblock_layer( color_ comp ) { | C | Descriptor |
| --- | --- | --- |
| mb_type | 2 | ue(v) ¦ ae(v) |
| if( mb_type = = I_PCM ) { |  |  |
| while( !byte_aligned( ) ) |  |  |
| pcm_alignment_zero_bit | 2 | f(1) |
| for( i = 0; i < 256; i++ ) |  |  |
| pcm_sample_luma[ i ] | 2 | u(v) |
| for( i = 0; i < 2 * MbWidthC * MbHeightC; i++ ) |  |  |
| pcm_sample_chroma[ i ] | 2 | u(v) |
| } else { |  |  |
| noSubMbPartSizeLessThan8x8Flag = 1 |  |  |
| if( mb_type != I_NxN && MbPartPredMode( mb_type, 0 ) != Intra_16x16 && NumMbPart( mb_type ) = = 4 ) { |  |  |
| sub_mb_pred( mb_type ) | 2 |  |
| for( mbPartIdx = 0; mbPartIdx < 4; mbPartIdx++ ) |  |  |
| if( sub_mb_type[ mbPartIdx ] != B_Direct_8x8 ) { |  |  |
| if( NumSubMbPart( sub_mb_type[ mbPartIdx ] ) > 1 ) |  |  |
| noSubMbPartSizeLessThan8x8Flag = 0 |  |  |
| } else if( !direct_8x8_inference_flag ) |  |  |
| noSubMbPartSizeLessThan8x8Flag = 0 |  |  |
| } else { |  |  |
| if( transform_8x8_mode_flag && mb_type = = I_NxN ) |  |  |
| transform_size_8x8_flag | 2 | u(1) ¦ ae(v) |
| mb_pred( mb_type, color_comp ) | 2 |  |
| } |  |  |
| if( MbPartPredMode( mb_type, 0 ) != Intra_16x16 ) { |  |  |
| coded_block_pattern | 2 | me(v) ¦ ae(v) |
| if( CodedBlockPatternLuma > 0 && transform_8x8_mode_flag && mb_type != I_NxN && noSubMbPartSizeLessThan8x8Flag && ( mb_type != B_Direct_16x16 ¦ ¦ direct_8x8_inference_flag ) ) |  |  |
| transform_size_8x8_flag | 2 | u(1) ¦ ae(v) |
| } |  |  |
| if( CodedBlockPatternLuma > 0 ¦ ¦ CodedBlockPatternChroma > 0 ¦ ¦ MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) { |  |  |
| mb_qp_delta | 2 | se(v) ¦ ae(v) |
| residual( ) | 3 ¦ 4 |  |
| } |  |  |
| } |  |  |
| } |  |  |

7.3.5.1  Macroblock prediction syntax

| mb_pred( mb_type, color_comp ) { | C | Descriptor |
| --- | --- | --- |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_4x4 ¦ ¦ MbPartPredMode( mb_type, 0 ) = = Intra_8x8 ¦ ¦ MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) { |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_4x4 ) |  |  |
| for( luma4x4BlkIdx=0; luma4x4BlkIdx<16; luma4x4BlkIdx++ ) { |  |  |
| prev_intra4x4_pred_mode_flag[color_comp][ luma4x4BlkIdx ] | 2 | u(1) ¦ ae(v) |
| if( !prev_intra4x4_pred_mode_flag[color_comp][ luma4x4BlkIdx ] ) |  |  |
| rem_intra4x4_pred_mode[color_comp][ luma4x4BlkIdx ] | 2 | u(3) ¦ ae(v) |
| } |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_8x8 ) |  |  |
| for( luma8x8BlkIdx=0; luma8x8BlkIdx<4; luma8x8BlkIdx++ ) { |  |  |
| prev_intra8x8_pred_mode_flag[color_comp][ luma8x8BlkIdx ] | 2 | u(1) ¦ ae(v) |
| if( !prev_intra8x8_pred_mode_flag[color_comp][ luma8x8BlkIdx ] ) |  |  |
| rem_intra8x8_pred_mode[color_comp][ luma8x8BlkIdx ] | 2 | u(3) ¦ ae(v) |
| } |  |  |
| if( chroma_format_idc != 0 ) |  |  |
| intra_chroma_pred_mode | 2 | ue(v) ¦ ae(v) |
| } else if( MbPartPredMode( mb_type, 0 ) != Direct ) { |  |  |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( ( num_ref_idx_l0_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag ) && MbPartPredMode( mb_type, mbPartIdx ) != Pred_L1 ) |  |  |
| ref_idx_l0[ mbPartIdx ] | 2 | te(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( ( num_ref_idx_l1_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag ) && MbPartPredMode( mb_type, mbPartIdx ) != Pred_L0 ) |  |  |
| ref_idx_l1[ mbPartIdx ] | 2 | te(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( MbPartPredMode ( mb_type, mbPartIdx ) != Pred_L1 ) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_l0[ mbPartIdx ][ 0 ][ compIdx ] | 2 | se(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( MbPartPredMode( mb_type, mbPartIdx ) != Pred_L0 ) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_l1[ mbPartIdx ][ 0 ][ compIdx ] | 2 | se(v) ¦ ae(v) |
| } |  |  |
| } |  |  |

7.4.5.1 Macroblock prediction semantics

prev_intra4x4_pred_mode_flag[color_comp][ luma4x4BlkIdx ] and rem_intra4x4_pred_mode[color_comp][ luma4x4BlkIdx ] specify the Intra_4x4 prediction of the 4x4 luma block of the appropriate color component (color_comp = 0..2) with index luma4x4BlkIdx = 0..15.

prev_intra8x8_pred_mode_flag[color_comp][ luma8x8BlkIdx ] and rem_intra8x8_pred_mode[color_comp][ luma8x8BlkIdx ] specify the Intra_8x8 prediction of the 8x8 luma block of the appropriate color component (color_comp = 0..2) with index luma8x8BlkIdx = 0..3.

A.2.8 Intra-only High 4:4:4 profile

Bitstreams conforming to the Intra-only High 4:4:4 profile shall obey the following constraints:

–	Only I slice types may be present.

–	NAL unit streams shall not contain nal_unit_type values in the range of 2 to 4, inclusive.

–	Arbitrary slice order is not allowed.

–	Picture parameter sets shall have num_slice_groups_minus1 equal to 0 only.

–	Picture parameter sets shall have redundant_pic_cnt_present_flag equal to 0 only.

–	Sequence parameter sets shall have bit_depth_luma_minus8 in the range of 0 to 4 inclusive.

–	Sequence parameter sets shall have bit_depth_chroma_minus8 in the range of 0 to 4 inclusive.

–	The level constraints specified for the High 4:4:4 profile in subclause Error: Reference source not found shall be fulfilled.

- Conformance of a bitstream to the High 4:4:4 intra-only profile is specified by profile_idc being equal to 145.

# Software

The software used for the simulation is provided along with this proposal. In order to enable this proposal, please set the “Period of I-frames” to 1 and set the “Profile IDC” to 144. Using the “ModeExtensionFlag” it is possible to switch between the mode selection of the current AVC Hi-4:4:4 profile and the extended mode selection as proposed herein on sequence level (RCT is not possible with this feature!). Extended mode selection assigns different spatial prediction modes to each color component.

(Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name |  | Panasonic R&D Center Germany |  |
| Mailing address |  | ## Monzastr. 4c, 63225 Langen |  |
| Country |  | Germany |  |
| Contact person |  | Steffen Wittmann |  |
| Telephone |  | +49 6103 766 1306 |  |
| Fax |  | +49 6103 766 166 |  |
| Email |  | Steffen.Wittmann-at-eu.panasonic.com |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) | JVT |  |  |
| Title | Intra-only 4:4:4 profile for H.264/AVC FRExt |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| X | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: