| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 17th Meeting: Nice, FR, 14-21 October, 2005 | Document: JVT-Q087_r1 Filename: input.doc |
| --- | --- |

| Title: | Conditions for SVC bit error resilience testing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Florelle Pauchet, Christine Guillemot IRISA-INRIA Campus de Beaulieu 35042 Rennes cedex, France Michel Kerdranvat Thomson R&D 1, avenue de Belle Fontaine CS 17616 35576 Cesson-Sévigné Cedex France Stéphane Pateux, Pierre Siohan France Télécom 4 Rue du Clos Courtel 35510 Cesson-Sévigné, France Catherine Lamy-Bergot, Didier Nicholson THALES Communications 160 boulevard de Valmy 92704 Colombes Cedex, France | Tel: Email: | +33 (0) 2 99 84 71 71 fpauchet@irisa.fr +33 (0)2 99 27 33 95 Michel.Kerdranvat@thomson.net +33 (0) 2 99 12 44 77 stephane.pateux@francetelecom.com +33 1 46 13 25 79 catherine.lamy@fr.thalesgroup.com didier.nicholson@fr.thalesgroup.com |
| Source: | IRISA-INRIA, THOMSON, FRANCE TELECOM, THALES Communications |  |  |

_____________________________

### Abstract

This document proposes common conditions for SVC error resilience testing taking into account bit errors.

# Introduction

So far error resilience has been designed mainly for packet-loss environments. This results from two main considerations: firstly that wired channels mostly led to packets losses due to buffering problems and secondly that the OSI traditional approach led to the transmission to upper layers of only valid packets. However, emergence of wireless error-prone links network opens a new era, as it was found then that entire packets could be discarded due to only a few bit errors in the payload.. As a consequence, new standards include the possibility to pass erroneous SDUs to the upper layers. The motivation is to save bandwidth by avoiding discarding entire packets if they are only damaged by some bit errors. This implies that application layers should handle corrupted data, and consequently requires new modes of resilience adapted to the presence of bit errors. In that context we propose error resilience tests taking into account bit errors.

# Need for bit error resilience

Variable Length Codes achieve good compression efficiency but suffer from a high sensitivity to noise: a single error in the bitstream can lead to a complete desynchronisation of the decoder. As a result, all the bitstream from the part where the error occurs until the next position where the decoder is able to resynchronize is lost.

Compression systems have been designed with the assumption that the quality of service was guaranteed by the lower layers of the communication systems or storage media. For instance, for transmission applications, lower layers of the OSI model were supposed to have error detection mechanisms. Those error detection mechanisms lead to discard each corrupted packets. As a result, a few hundred bytes will be discarded even if there is a single bit error in the whole payload. Packets drop impacts on the bandwidth, which is critical for wireless and mobile applications.

These observations have led some normalization groups to work on new modes that will not discard systematically damaged packets but will forward them to the upper layers (hence to the decoder). This relies obviously on the assumption that the decoder is able to cope with errors.

In particular, 3GPP [5] and Bluetooth norms include transparent mode for the Medium Access Control (MAC) layer that allows to pass erroneous SDUs to the application layer. This evolution of the lower layer specifications of communication systems can be found also for the transport layer. The Internet Engineering Task Force (IETF) has elevated UDP lite (lightweight User Datagram Protocol) [2] to the RFC status. UDP lite allows corrupted packet from the MAC layer to reach applications level. This is done by having the Cyclic Redundancy Check (CRC) error checking covering the header only. The checksum minimally covers the UDP lite header and can extend to any offset within the UDP lite packet. Similar evolutions have been made for the specifications of DCCP (Datagram Congestion Control Protocol): appendix B deals with Partial Check summing Design Motivation. The current development of network header compression such as in RFC 3095 [6] accepted by IETF allows for the transmission of these bit erroneous SDUs even with more traditional transport layer such as UDP, for the error checks can then be re-derived at the receiver side and hence be reported valid even though the SDU was corrupted.

Moreover, it should be stated that even if many transmissions now rely on an IP networking layer, some still do not face the OSI protocol stack and hence do not necessarily have error detection mechanisms between the physical level and the source decoding. This is for instance the case of a broadcasting application such as Digital Radio Mondiale [7] where the data link can be used to transmit video information (use of JPEG 2000 has already been foreseen for instance).

Last, bit errors are not only a problem occurring in wireless transmissions systems: storage of digital media such as DVD or CD raises also the error resiliency problem. Degradation may render the disc unreadable. The life expectancy of DVD or CD record is linked to the error resilience tool used. Record systems usually use error correction codes; they consist in adding some redundancy to the signal which impacts on the storage capacity. In compact disc system error correction is provided by Cross-Interleaved Reed Solomon Codes (CIRC) that can reach 25% of redundancy. The upper limit of error rate indicated in DVD specifications is 1e-2 before correction so as to obtain a rate of 1e-20 after correction. New trend is to accept a higher error rate in order to increase the storage capacity. However it leads actually to decrease the signal quality. Digital media could highly benefit from new robust decoding solutions that could reduce the need of error correction codes and so, increase the storage capacity while limiting the effect of errors on quality.

# Tests conditions

## Bitstream

The tests are based on the assumption that erroneous SDUs are not discarded but forwarded to the application layer.

We propose to introduce errors into the Base Layer. In practice, it has been observed that the presence of errors in header information (NAL, slide headers) having a very different impact on the decoding process when compared with presence of errors in the video information (VCL part except slide headers). As a consequence, two modes should be considered:

- NAL and slide headers assumed heavily protected and thus error free, with errors located only in the video data (testing case)

- errors occurring on the whole bitstream (realistic case)

It could also be possible to consider that the sequence parameters and picture parameters are error free.

The bitstream can in practice be generated using different modes: in particular, one of the testing case could be the Data Partitioning mode considered by the H.264/AVC standard in its extended profile.

The Data Partitioning consists in separating elements of a slice in three classes depending on their sensitivity to bit errors, thus enabling the possibility of performing unequal error protection. Each class is encapsulated into one NAL unit. The current data partition mode supports three partitions: The first one, referred as DP A contains header and motion vector information which have a high sensitivity to bit errors while the two others are less sensitive and contain residual data, transform coefficients of intra blocks for DP B, and coefficients of inter blocks for DP C.

Data Partitioning enables unequal error protection based on the priority of the packet. The loss of DP A with header data makes it impossible to decode the slice and thus makes DP B and DP C useless whereas efficient error resilience tool could allow coping with the loss of DP B and DPC.

Since data partitioning is not available in the JSVM scalable software, we propose first to assess error resilience mechanisms for Intra coded pictures, i.e. picture without motion information.

Depending on the benefits obtained on the Intra pictures, the next step will be to corrupt Inter coded pictures, and if possible introduce data partitioning to allow separation of the elements in classes based on their sensitivity to errors, opening the path to an unequal error protection functionality.

## Errors

Proposals should precisely describe the following points:

- the overhead in the bitstream (if any) introduced by the error resiliency method

- encoding configuration and options

- the impact at coder/decoder part and the compatibility with the standard

- syntax elements impacted by the error resilience method

- complexity of the proposed method

- channel/network simulation environment. The simulator provides a corrupted bitstream.

Experiments are performed for two kinds of error conditions: random bit errors and bit error patterns.

For random bit errors, BER parameter is to be specified.

Simulating and testing transmission over wireless radio channels is not a completely new problem, and as such SVC can take advantage of already existing patterns. In practice, ITU-T Q15 group, focussed on video codec development, has been requesting representative simulations or patterns for such channels in order to develop and test H.264/AVC solutions, which are aiming at being error robust. In the framework of ITU-T Q11 group, 3GPP bit-error patterns have been generated, either through direct capture of real signals or by emulation of mobile radio channels. Those patterns, provided by Nokia and Ericsson, were captured above the physical layer and below the RLC/RLP layer, such that in practice they act as the physical layer simulation (turbo codes are used in all cases). From those two sets of patterns, a selection of six patterns was done by ITU-T Q15 group to be used for H.264/AVC standard as common test conditions [1], which are the first six ones shown in Table 1.

Similarly, 802.11b error traces were captured at Michigan State University, with an 802.11b Access Point (AP) operating in Distributed Coordination Function mode and three wireless stations communicating in the infrastructure network configuration (one as server, the remaining two as multicast clients) [4]. The capture aimed at reflecting a realistic business/classroom/home-network wireless setup, in the sense where the client and servers were positioned in separate rooms, without line of sight (LoS) link. A selection of two patterns corresponding to 2Mbps data rate is proposed, as shown in Table 2.

We propose to use the following error configurations:

- Random bit errors BER 10e-4

- Random bit errors BER 10e-3

- 3GPP bit error patterns [see Table 1] detailed in [1]

- 802.11b bit error patterns [see Table 2] detailed in [4]

| Name | Bit rate | BER |
| --- | --- | --- |
| 18681.3 | 64 kbps | 9.3e-3 |
| 18681.4 | 64 kbps | 2.9e-3 |
| Wcdma_64kb_3kph_5e-04.bin | 64 kbps | 5.1e-4 |
| Wcdma_64kb_50kph_2e-04.bin | 64 kbps | 1.7e-4 |
| Wcdma_128kb_3kph_5e-04.bin | 128 kbps | 5.0e-4 |
| Wcdma_128kb_50kph_2e-04.bin | 128 kbps | 2.0e-4 |
| Wcdma_384kb_3kph_5e-04.bin | 384 kpbs | 5.1e-4 |
| Wcdma_384kb_50kph_1e-04.bin | 384 kpbs | 1.4e-4 |

Table 1: 3GPP bit error patterns

| Name | Bit rate |
| --- | --- |
| bit_trace2mbps_1.bin | 2 mbps |
| bit_trace2mbps_2.bin | 2 mbps |

Table 2: 802.11b error pattern

## Test sequences

The numbers of trials should be sufficient to avoid influence of the position of errors. The number of runs over the whole sequence should be chosen so as to obtain at least 4000 coded pictures:

- for random errors, using different seeds for the random number generators;

- for bit error patterns, using different starting positions of the pattern.

The table below defines the sequences and birates to be used in simulations:

| Sequence | Size | Frame rate | Bit rate |
| --- | --- | --- | --- |
| Foreman | QCIF | 7.5 fps | 64 kbps |
| Mobile | QCIF | 7.5 fps | 64 kbps |
| Soccer | QCIF | 15 fps | 128 kbps |
| Bus | QCIF | 15 fps | 128 kbps |
| Foreman | CIF | 30 fps | 384 kbps |
| Mobile | CIF | 15 fps | 384 kbps |
| City | 4CIF | 60 fps | 2048 kbps |

Table 3: Sequences

## Evaluation criteria

The PSNR value is computed between each frame of the coded sequence and the corresponding reconstructed frame. The PSNR is averaged over the sequence. Then, average, maximal and minimal PSNR over the trials are required.

For subjective quality measurement, some tests sequences should be provided.

# References

[1] ITU-T VCEG-N80 “Common test conditions for RTP/IP over 3GPP/3GPP2”

[2] RFC 3828 “The UDP-Lite Protocol”, L-A. Larzon, M. Degermark, S. Pink, L-E. Jonsson, G. Fairhurst, 2004

[3] JVT-P206 “Test conditions for error resiliency proposals”

[4] S. Khayam, S. Karande, H. Radha, and D. Loguinov, "Performance Analysis and Modeling of Errors and Losses over 802.11b LANs for High-Bitrate Real-time Multimedia," Signal Processing: Image Communication, vol. 18, no.7, pp. 575-595, August 2003.

[5] 3GPP TS 23.107, "QoS concept and Architecture", V6.2.0, 12/2004

[6] RFC 3095 “RObust Header Compression (ROHC): Framework and four profiles: RTP,UDP,ESP and uncompressed,” C. Bormann et Al., 2001.

[7] ETSI TS 101 980 V1.11, 'Digitale Radio Mondiale; System Specification', September 2001.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | INRIA |  |  |
| Mailing address | Campus Universitaire de Beaulieu 35 042 Rennes Cedex |  |  |
| Country | FRANCE |  |  |
| Contact person | Christine Guillemot |  |  |
| Telephone | 33 2 99 84 74 29 |  |  |
| Fax | 33 2 99 84 71 71 |  |  |
| Email | cguillem@irisa.fr |  |  |
| Place and date of submission | 17th Meeting: Nice, France, October 2005 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT/SVC |  |
| Title |  | New conditions for SVC bit error resilience testing |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: