| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 17th Meeting: Nice, FR, 14-21 October, 2005 | Document: JVT-Q304 Filename: input.doc |
| --- | --- |

| Title: | CE4: ROI Scalability |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JVT |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | ZhongKang Lu Institute for Infocomm Research 21 Heng Mui Keng Terrace, Singapore 119613 | Tel: Email: | +65 6874 6720 zklu@i2r.a-star.edu.sg |
| Source: | CE Coordinator |  |  |

_____________________________

# Participants

Coordinator: ZhongKang Lu (I2R)

| Participants | Verification Partner | Contact | Email |
| --- | --- | --- | --- |
| I2R |  | ZhongKang Lu | zklu@i2r.a-star.edu.sg |
| Panasonic |  | MenHuang Lee | MenHuang.Lee@sg.panasonic.com |
| ICU |  | Truong Cong Thang | tcthang@icu.ac.kr |
| ETRI |  | Jung Won Kang | jungwon@etri.re.kr |
| Nokia |  | Y.-K. Wang | Ye-kui.Wang@nokia.com |

# Functionality addressed

This CE addresses the following aspects of Region-of-Interest (ROI) scalability:

- coding efficiency improvement

- method to signal the processes of ROI handling

Tools relating to two different kinds of ROI scalability will be investigated under a common set of test conditions:

- Pre-defined ROI tool: The ROI is pre-defined during encoding.

- Dynamic ROI tool: The ROI may change, appear and disappear during coding.

- Interactive ROI tool: The ROI is not defined and not fixed during encoding. The user selects ROI interactively at decoder-side after bitstream is encoded.

# Tool Descriptions

Tool 1: New FMO Type for Interactive ROI Scalability [JVT-Q020]

This is a tool to improve the coding efficiency of Interactive Region of Interest (IROI) scalability. IROI scalability can be supported by coding rectangular grid of MBs as independent slices. This can be achieved using existing FMO map type 2: foreground + leftover slice groups or FMO map type 6: explicit assignment. A new FMO map type 7: grid slice group is proposed to save overhead bits for coding rectangular grid slices.

Tool 2: Stream Structure and CABAC Improvement for Interactive ROI Scalability [JVT-Q021]

This is a tool to improve the coding efficiency of Interactive Region of Interest (IROI) scalability. In IROI coding, a picture is divided into many rectangular grid slices. One problem with dividing into many slices is the huge overhead of coding many NAL unit headers and slice headers using JSVM stream structure because each quality layer is coded in a separate slice. A new stream structure is proposed to reduce the header overhead, where several quality layers are coded in one ROI slice. For N quality layers, the header overhead can be reduced by a factor of approximately N times. Another problem with dividing into many slices is the reduced coding efficiency of CABAC due to more initialization and less updating of CABAC contexts. Inter quality layer context continuation is proposed to improve the coding efficiency of CABAC. In inter quality layer context continuation, the context states are used continuously through all quality layers in a slice without initialization.

Tool 3: Improve high-level syntax in dynamic ROI conditions [JVT-O018]

This tool is to reduce the overhead on high-level syntax in dynamic ROI conditions. Current implementation of dynamic frame partition by FMO is to insert a picture parameter set before every frame that its partition is different to previous frame. In an extreme case, to insert a PPS before every frame will cost about 4-6k bits/sec. The cost is unacceptable in low and very-low bit rate conditions.

# Testing conditions

Software

The latest JSVM4 software output from this 17th JVT meeting will be used by all the participants for this CE. This software shall have implementation of FMO tool. The software will be sent to the participants by the coordinator to synchronize the anchor software, once the software is released by the JSVM software AHG. Every modified software source will be shared across all participants and verificators of this CE.

Coding Conditions

Tool1 and Tool2:

| Sequence | Format |  | Bit rates (kbit/sec) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Crew, Soccer | Layer 0 CIF 30Hz | I P P P Intra Period = 30 | 768 |  |  |  |  |
|  | 4CIF 30Hz whole frame & ROI | I I I I | 1536 | 1792 | 2048 | 2560 | 3072 |

Tool 3:

Frame size:	QCIF and CIF

Test sequences:

‘Bus’, ‘Crew’, ‘Football’, ‘Foreman’, ‘Harbour’ and ‘Soccer’

Frame rate:

- base layer:			7.5Hz

- 1st upper layer:		15Hz

- 2nd upper layer:		30Hz

ROI conditions:

- Single ROI

- Multiple ROIs

ROI Description

The percentage of ROI slices on whole frame size is:

  - Single ROI condition:		15% ~ 30%

  - Two ROIs condition:			20% ~ 40%

  - Three ROIs condition:		25% ~ 50%

For tools relating to pre-defined ROI, ROI description is pre-defined for each test sequence.

For tools relating to interactive ROI, a set of multiple possible ROI descriptions is defined for each test sequence. The actual ROI region to be compared will be selected randomly after the bitstreams are encoded and submitted. This is distinguished from pre-defined ROI.

The ROI descriptions shall be generated using eye tracker. It can include single or multiple ROIs. The number of ROI can change from picture to picture.

The ROI description shall follow a fixed format (see below) and the parsing of the ROI description will be implemented into the JSVM4 software used for this CE.

N1 = (x1,y1,w1,h1), (x2,y2,w2,h2), etc…

N2 = (x1,y1,w1,h1), (x2,y2,w2,h2), etc…

etc……

where each line specifies the ROI (or multiple ROIs) of each frame (e.g. N1, N2)

xi specifies the horizontal coordinate of top-left corner of the ith ROI from the top-left corner of the picture in pixel unit

yi specifies the vertical coordinate of top-left corner of the ith ROI from the top-left corner of the picture in pixel unit

wi specifies the width of the ith ROI in pixel unit

hi specifies the height of the ith ROI in pixel unit

Anchors

The following anchors should be generated and compared against:

| Anchor # | Description |
| --- | --- |
| Anchor1 | JSVM4 without ROI coding whole picture coded as 1 slice group |
| Anchor2 | non-scalable JM10 without ROI coding whole picture coded as 1 slice group |
| Anchor3 | JSVM4 with pre-defined ROI coding FMO type 2 to describe the pre-defined ROIs as foreground slice groups |
| Anchor4 | Current FMO methods related to ROI |

Anchor1, Anchor3, Anchor4 and proposed tools will have the same AVC base layer

Random ROI Test

For tools relating to interactive ROI, the proponents shall submit the encoded bitstreams according to the timeline. After bitstream submission, 2 sets of ROI descriptions will be selected randomly to simulate interactive user-selection of ROI. The submitted bitstreams will be extracted, decoded and evaluated according to these 2 ROI descriptions. No re-encoding is allowed.

Evaluation criteria

For tools proposing coding efficiency improvement, the following evaluation criteria are used.

Objective test results

PSNR and/or the percentage bitrate saving can be used to evaluate the improvements.

PSNR (Luma) results generated for the whole frame and ROI region will be compared for the tool against the anchor.

The percentage bitrate saving at the same PSNR can also be compared, where it is computed by:

percentage bitrate saving = 100% * (anchor_rate – tool_rate)/anchor_rate

A tool to compute PSNR of ROI region (“ROI_PSNR” tool) will be provided to all proponents.

Subjective visual tests

Subjective visual test will be used to confirm the consistency of the objective test results.

## Time lines

| Time | Action to take | Action by |
| --- | --- | --- |
| Nice+2W | ROI descriptions will be ready | CE coordinator |
| JSVM4 | Release of JSVM software used for this CE | JSVM S/W AHG |
| Bangkok-4W | Final update of technical description of tool being proposed | Proponents |
| Bangkok-3W | Submission of software and bitstream Random selection of ROI description | Proponents CE coordinator |
| Bangkok-2W | Submission of extracted bitstreams, YUV files & PSNR results Submission of verification results | Proponents Verificators |
| Bangkok-1W | Submission of technical contribution Submission of verification report | Proponents Verificators |