ITU - Telecommunication Standardization Sector	Temporary Document XX(PLEN)

STUDY GROUP 16

Geneva, 7-18 February 2000

Question(s):	11

SOURCE:	Rapporteur Q11 (T. Geary)

TITLE:	Draft new Implementor's Guide for the ITU-T H.320 Recommendation series (H.221, 	H.242)

__________________

# Introduction

This document is a new draft Implementor’s Guide for H.320 series recommendations. Changes are made against the 05/99 revision of H.221 and H.242. The changes will allow the use of G.722.1 in H.320 systems as they appeared and were agreed in Q11-K-06 (Red Bank 10/99).

Changes appear in the following sections:

- Technical and editorial corrections to H.221

- Table A1/H.221

- Section A.1/H.221

- Section A.5/H.221

- Bit position for G.722.1

- Clarifications to include revised T.35 country codes

- Technical and editorial corrections to H.242

- Table 5/H.242

- Appendix 6

- Clarifications to include revised T.35 country codes

# Implementor's Guide for the ITU-T H.320 Recommendation series - Narrow-band visual telephone systems and terminal equipment

# Abstract

This document is a compilation of reported defects identified with the 1997-2000 editions of the ITU-T H.320-series Recommendations. It is intended to be read in conjunction with the Recommendations to serve as an additional authoritative source of information for implementors. The changes, clarifications and corrections defined herein are expected to be included in future versions of affected H.320-series Recommendations.

# Contact information

| ITU-T Study Group 16/Question 11 Rapporteur | Tom Geary Conexant Systems Inc. 4311 Jamboree Road Newport Beach, CA 92660-3095 USA | Tel: +1 949 483 4092 Fax: +1 949 483 6511 Email: tom.geary@connexant.com |
| --- | --- | --- |
| Implementor's Guide Editor | Patrick Luthi PictureTel Corp. 100 Minuteman Road Andover, MA 01810 USA | Tel: +1 978 292 5000 Fax: +1 978 292 3328 Email: luthip@pictel.com |
| ITU-T Recommendation H.320 Editor |  |  |

# Document history

| Revision | Date | Description |
| --- | --- | --- |
| 1 | 7-18 February 2000 | Initial version - Reviewed at the Q.11/16 Rapporteur meeting. |

# 1.  Introduction

This document is a compilation of reported defects identified with the 1997-2000 editions of the ITU-T H.320-series Recommendations. It is intended to be read in conjunction with the Recommendations to serve as an additional authoritative source of information for implementors. The changes, clarifications and corrections defined herein are expected to be included in future versions of affected H.320-series Recommendations.

The first version of the guide was produced following the October 1999 ITU-T Study Group 16/Question 11 Rapporteur meeting. Wide distribution of this document is expected and encouraged.

# 2.  Scope

This guide resolves defects in the following categories:

•	editorial errors;

•	technical errors such as omissions or inconsistencies;

•	ambiguities.

In addition the Guide may include explanatory text found necessary as a result of interpretation difficulties apparent from the defect reports.

This Guide will not address proposed additions, deletions or modifications to the Recommendations that are not strictly related to implementation difficulties in the above categories. Proposals for new features should be made in the normal way through contributions to the ITU-T.

# 3.  Policies for updating this document

This document is managed by the ITU-T Study Group 16 Question 11 Rapporteur's Group. It can be revised at any recognized Q.11/16 Rapporteur's Group meeting provided the proposed revisions are unanimously accepted by the members of the group. A revision history cataloguing the evolution of this document is included.

# 4.  Defect resolution procedure

Upon discovering technical defects with any components of the H.320 Recommendations series, please provide a written description directly to the editors of the affected Recommendations with a copy to the Q.11/16 Rapporteur. The template for a defect report is enclosed. Contact information for these parties is included in this document. Return contact information should also be supplied so a dialogue can be established to resolve the matter and an appropriate reply to the defect report can be conveyed. This defect resolution process is open to anyone interested in H.320-series Recommendations. Formal membership in the ITU is not required to participate in this process.

# 5.  References

This document refers to the following H.320-series Recommendations:

–	ITU-T Recommendation H.221 (1999), Frame Structure for a 64 to 1920 kbit/s channel in audiovisual teleservices.

–	ITU-T Recommendation H.242 (1999), System for establishing communication between audiovisual terminals using digital channel up to 2 Mbit/s.

- ITU-T Recommendation H.320 (1999), Narrow-band visual telephone systems and terminal equipment.

- ITU-T Recommendation T.35 (2000), Procedure for the allocation of ITU-T defined codes for non-standard facilities

# 6.  Nomenclature

In addition to traditional revision marks, the following marks and symbols are used to indicate to the reader how changes to the text of a Recommendation should be applied:

| Symbol | Description |
| --- | --- |
| [Begin Correction] | Identifies the start of revision marked text based on extractions from the published Recommendations affected by the correction being described. |
| [End Correction] | Identifies the end of revision marked text based on extractions from the published Recommendations affected by the correction being described. |
| ... | Indicates that the portion of the Recommendation between the text appearing before and after this symbol has remained unaffected by the correction being described and has been omitted for brevity. |
| --- SPECIAL INSTRUCTIONS --- {instructions} | Indicates a set of special editing instructions to be followed. |

# 7.  Technical and editorial corrections

## 7.1  Technical and editorial corrections to ITU-T Recommendation H.221

To support G.722.1 in H.320 systems, new BAS capability and command codes need to be added in Recommendation H.221.

In order to provide H.320/H.32x gateways the ability to constrain the bit rate allowed for use by the H.320 endpoint (for the purposes of enforcing bit rate symmetry, for example), capability codes for each bit rate are necessary. In the case of G.722.1, this means separate capability codes for 24 kbit/s and 32 kbit/s.

### 7.1.1  Table A.1/H.221 - BAS numerical values

The following new BAS code values are being added (to be updated in Table 1/H.221):

[Begin Correction]

G.722.1-32	(000)[27]

G.722.1-24	(000)[28]

G.722.1-32 (cap)	(110)[5]

G.722.1-24 (cap)	(110)[6]

[End Correction]

### 7.1.2 Section A.1/H.221 - Audio command values (000)

Section A.1/H.221 is being updated as follows:

[Begin Correction]

…

G.722.1-32	G.722.1 7 kHz audio at 32 kbit/s, in bits 1-4.

G.722.1-24	G.722.1 7 kHz audio at 24 kbit/s, in bits 1-3.

…

[End Correction]

### 7.1.3  Section A.5/H.221 - Audio capabilities (100)

Section A.5/H.221 is being updated as follows:

…

G.722-64	Capable of decoding audio to Recommendation G.722 (mode 1) and to Recommendation G.711.

G.722-48		Capable of decoding audio to Recommendation G.722 (modes 1, 2, 3) and to 			Recommendation G.711.

[Begin Correction]

G.722.1-32 (cap)	Capable of decoding audio to Recommendation G.722.1 at 32 kbit/s and to Recommendation G.711.

G.722.1-24 (cap)	Capable of decoding audio to Recommendation G.722.1 at 24 kbit/s and to Recommendation G.711.

…

[End Correction]

### 7.1.4  Bit Position for G.722.1 audio

This new section is being added at the end of section 4.2 (encoded audio streams) as follows:

4.2  Encoded audio streams

…

[Begin Correction]

###### G.722.1 audio

G.722.1 provides two bit rates, 24 kbit/s or 32 kbit/s, and uses a frame size of 20ms.  This results in either 480 bits (60 octets) or 640 bits (80 octets) in any one frame respectively. The bitrate may be changed at any 20 ms audio frame boundary. Alignment of H.221 audio mode changes with a submultiframe boundary is required by section 3.2/H.221.

Figure 1 & 2 illustrate the bit allocation of the two G.722.1 frames for a bitrate of 32 kbit/s and 24 kbit/s respectively.

| Figure 1 - Bit positions for G.722.1 audio at 32 kbit/s H.221 Frame | Sub-ChannelBit # | Sub-Channel |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
|  | 1 | 1 | 2 | 3 | 4 |  |  |  | FAS |
|  | 2 | 5 | 6 | 7 | 8 |  |  |  | FAS |
|  | 3 | 9 | 10 | 11 | 12 |  |  |  | FAS |
|  | 4 | 13 | 14 | 15 | 16 |  |  |  | FAS |
| First | 5 | ... | ... | ... | ... |  |  |  | FAS |
| H.221 | 6 |  |  |  |  |  |  |  | FAS |
| Frame | 7 |  |  |  |  |  |  |  | FAS |
|  | 8 |  |  |  |  |  |  |  | FAS |
|  | 9 |  |  |  |  |  |  |  |  |
|  | ... |  |  |  |  |  |  |  |  |
|  | 80 | 317 | 318 | 319 | 320 |  |  |  |  |
|  | 81 | 321 | 322 | 323 | 324 |  |  |  | FAS |
|  | 82 | ... | ... | ... | ... |  |  |  | FAS |
|  | 83 |  |  |  |  |  |  |  | FAS |
|  | 84 |  |  |  |  |  |  |  | FAS |
| Second | 85 |  |  |  |  |  |  |  | FAS |
| H.221 | 86 |  |  |  |  |  |  |  | FAS |
| Frame | 87 |  |  |  |  |  |  |  | FAS |
|  | 88 |  |  |  |  |  |  |  | FAS |
|  | 89 |  |  |  |  |  |  |  |  |
|  | ... |  |  |  |  |  |  |  |  |
|  | 160 | 637 | 638 | 639 | 640 |  |  |  |  |

| Figure 2 - Bit positions for G.722.1 audio at 24 kbit/s H.221 Frame | Bit # | Sub-Channel |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
|  | 1 | 1 | 2 | 3 |  |  |  |  | FAS |
|  | 2 | 4 | 5 | 6 |  |  |  |  | FAS |
|  | 3 | 7 | 8 | 9 |  |  |  |  | FAS |
|  | 4 | 10 | 11 | 12 |  |  |  |  | FAS |
| First | 5 | ... | ... | ... |  |  |  |  | FAS |
| H.221 | 6 |  |  |  |  |  |  |  | FAS |
| Frame | 7 |  |  |  |  |  |  |  | FAS |
|  | 8 |  |  |  |  |  |  |  | FAS |
|  | 9 |  |  |  |  |  |  |  |  |
|  | ... |  |  |  |  |  |  |  |  |
|  | 80 | 218 | 219 | 220 |  |  |  |  |  |
|  | 81 | 221 | 222 | 223 |  |  |  |  | FAS |
|  | 82 | 224 | 225 | 226 |  |  |  |  | FAS |
|  | 83 | ... | ... | ... |  |  |  |  | FAS |
|  | 84 |  |  |  |  |  |  |  | FAS |
| Second | 85 |  |  |  |  |  |  |  | FAS |
| H.221 | 86 |  |  |  |  |  |  |  | FAS |
| Frame | 87 |  |  |  |  |  |  |  | FAS |
|  | 88 |  |  |  |  |  |  |  | FAS |
|  | 89 |  |  |  |  |  |  |  |  |
|  | ... |  |  |  |  |  |  |  |  |
|  | 160 | 478 | 479 | 480 |  |  |  |  |  |

[End Correction]

### 7.1.5 Clarifications to include revised T.35 country codes

The revised Recommendation T.35 (2000) extends the number of possible country codes to allow more than 254 countries. This is being achieved by introducing a second byte for new countries being reached by an escape code in the first byte of “1111 1111” (0xFF). Countries defined in the first byte are listed in Annex A/T.35 and countries defined in the second byte are listed in Annex B/T.35.

T.35 country codes are used by NS-cap and NS-comm as described in section A.9 - Escape table values (111). In footnote 4, the following text should be added:

[Begin Correction]

4 	Country code consists of two bytes, the first being according to Recommendation T.35 Annex A. The second byte is assigned nationally, unless the first byte is 1111 1111, in which case the second byte shall contain the country code according to T.35 Annex B.The terminal manufacturer code consist of two bytes assigned nationally.

[End Correction]

## 7.2  Technical and editorial corrections to ITU-T Recommendation H.242

### 7.2.1  Table 5/H.242 - BAS capabilities that can be included in a valid capability set

Table 5/H.242 is being updated as follows:

[Begin Correction]

| Audio | Absent or One or more valuesb) from A-law, m-law, G.722-48, G.722-64, G.728, G.723.1, G.729, G.722.1-24, G.722.1-32 |
| --- | --- |

…

[End Correction]

b) See Appendix VII for interpretation of received audio capabilities.

### 7.2.2  Appendix 6 - Hierarchical capability BAS codes

Appendix 6 is being updated as follows:

[Begin Correction]

The following capability codes are hierarchically structured:

G.711 (A or  or both) < G.722-64 < G.722-48

G.711 (A or  or both) < G.728

G.711 (A or  or both) < G.723.1

G.711 (A or  or both) < G.729

G.711 (A or  or both) < G.722.1-24

G.711 (A or  or both) < G.722.1-32

…

[End Correction]

Note that, while they have not been included previously, G.723.1 and G.729 have been now added to the list for completeness.

### 7.2.3 Clarifications to include revised T.35 country codes

In Appendix III, two sections are affected by the extension of T.35 country codes (see details in section 7.1.5).

### 4.2.1 Section III.2 - Subsequent capability exchange, including MBE capability message

Section III.2 is updated as follows:

[Begin Correction]

…

{M}	Information will be M-bytes

{byte 1}	Country code according to Recommendation T.35 Annex A

{byte 2}	Country code assigned nationally, unless the first byte is 1111 1111, in which case this field shall contain the country code according to T.35 Annex B

{bytes 3, 4}	Manufacturer code (Company XYZ)

{bytes 5-M}	Type identity

…

[End Correction]

### 4.2.2 Section III.3 Mode switch to non-standard mode using MBE command

Section III.3 is updated as follows:

…

{N}		Information will be N-bytes

[Begin Correction]

{byte 1}		Country code according to Recommendation T.35 Annex A

{byte 2}	Country code assigned nationally, unless the first byte is 1111 1111, in which case this field shall contain the country code according to T.35 Annex B

[End Correction]

{bytes 3, 4}	Manufacturer code (Company XYZ)

{bytes 5-N}	Type identity

…

H.320 Recommendation Series Defect Report Form

| DATE: |  |
| --- | --- |
| CONTACT INFORMATION NAME: COMPANY: ADDRESS: TEL: FAX: EMAIL: |  |
| AFFECTED RECOMMENDATIONS: |  |
| DESCRIPTION OF PROBLEM: |  |
| SUGGESTIONS FOR RESOLUTION: |  |

NOTE - Attach additional pages if more space is required than is provided above.

Contact:	Patrick Luthi (Editor)	Phone: +1 978-292-4354

Fax: +1 978-292-3328

E-mail: luthip@pictel.com