ITU Telecommunication Standardization Sector		Document Q11-M-06

Study Group 16

Q.11-15/16 Rapporteur Meeting

Portland, Oregon, 21-25 August 2000		Date: August 7, 2000

SOURCE*: 	Editor (P. Luthi)

TITLE:	Proposed changes to H.221 to fix an editorial bug

###### PURPOSE:	Proposal

______________________________________________________________________________

# Introduction

An editorial anomaly related to the use of Restrict (a command) and Restrict_required (a capability) was discovered in table A.1 of H.221. It seems that this anomaly was introduced at the time when the Restrict_P, Restrict_L and NoRestrict caps were added to the Recommendations.  The "restrict" cap was renamed to Restrict_Required, but the change was incorrectly made in Table A.1/H.221, and it appears that the change was never made in section A.7/H.221.

Section 13.4.1 (Capability values to be transmitted) and 13.4.3 (Restricted mode transmitted) of H.242 correctly describe the use of a [restrict_required] capability and a [restrict] command as following:

…

An endpoint which can only send and receive restricted signals shall include {Restrict_Required} in its capset;

…

An endpoint already transmitting a restricted mode shall include the command [restrict] in all repetitive transmissions of valid commands in force. An endpoint transmitting unrestricted after previously transmitting a restricted mode shall include the command [derestrict] in all repetitive transmissions of valid commands in force.

…

# 2.  Proposed Solution

In table A.1 of H.221, Restricted_required [27] is incorrectly listed in the video, encryption, loop and other command (010) column, and restrict [22] is incorrectly listed in the audio and transfer rate capabilities column (100).

The solution is to simply swap the command and with the cap to become:

(010)[27]		Restrict

(100)[22]		Restrict_required

The text in section A.3 correctly describes the [restrict] command, but the text in section A.7 needs to be changed from [restrict] to [restrict_required]. The proposed corrected text of section A.7 follows:

[Begin Correction]

## A.7	Transfer-rate capabilities (100)

…

Restrict_required	Can work only at p  56 kbit/s, rate-adapted to p  64 kbit/s by moving the SC to bit position 7 and setting bit 8 to "one" in every channel or time-slot; a constant "one", however, may be set in bit 8 if it is known by out-of-band signalling prior to the connection that the restriction exists; this code has the effect of forcing the remote terminal to work in the p  56 kbit/s mode (see Annex B).

…

[End Correction]

--END--

* Contact:	Patrick Luthi	Phone: +1 978-292-4354

PictureTel Corp., USA	E-mail: luthip@pictel.com