UIT - Secteur de la normalisation des télécommunications

ITU - Telecommunication Standardization Sector

UIT - Sector de Normalización de las Telecomunicaciones

Study Period 1997-2000

Commission d'étudesStudy Group  16	Contribution tardiveDelayed Contribution  D.xxx

Geneva,  November 13-17, 2000

Texte disponible seulement enText available only in  E

Question(s):	Q.11, 14

SOURCE:	NTT DoCoMo, Inc.

TITLE:	Support of HTTP in H.324

___________________

This contribution proposes support of HTTP - Hyper Text Transfer Protocol in H.324 as an optional data application protocol. This enables the H.324 terminals to be used as terminals for non-conversational services with a user interface through web-like menus.

New H.245 codepoints required to manage the logical channel for HTTP messages are also proposed in this contribution.

ANNEX X

HTTP Data Application Protocol Definition

A==

(This annex forms an integral part of this Recommendation)

Table X.1 defines the capability identifier for HTTP - Hypertext Transfer Protocol [HTTP]. Table X.2 to X.x define the associated capability parameters. This is an application-level protocol for distributed, collaborative, hypermedia information systems. The technical specifications of HTTP are provided in IETF RFC 2068. This standard is published by IETF.

TABLE X.1/H.245

Capability Identifier for HTTP Capability

| Capability name: | HTTP |
| --- | --- |
| Capability class: | Data application |
| Capability identifier type: | Standard |
| Capability identifier value: | itu-t (0) recommendation (0) h (8) 245 generic-capabilities (1) data (2) HTTP (0) |
| maxBitRate: | This field shall be included. |
| nonCollapsingRaw: | This field shall not be included. |
| transport: | This field shall be included. |

TABLE X.2/H.245

Mode for HTTP Capability

| Parameter name: | mode |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. mode indicates the operating mode of the terminal: - Server - Client - Server & Client (this mode shall not be set in Logical Channel Signaling) |
| Parameter identifier value: | 0 |
| Parameter Status: | Mandatory |
| Parameter type: | UnsignedMin. |
| Supersedes: | - |

TABLE X.3/H.245

Underlying protocol for HTTP Capability

| Parameter name: | underlyingProtocol |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. underlyingProtocol indicates the protocol under HTTP: 0: None 1: TCP/IP/PPP 2 ~ : Reserved for future extension |
| Parameter identifier value: | 1 |
| Parameter Status: | Mandatory |
| Parameter type: | UnsignedMin |
| Supersedes: | - |

Normative Reference

[HTTP]  FIELDING (R. et al.): Hypertext Transfer Protocol - HTTP/1.1, RFC 2068, Internet Engineering Task Force, January 1997.

___________

Contact:

Toshiro Kawahara,			NTT DoCoMo, Inc.		kawahara@spg.yrp.nttdocomo.co.jp

Takashi Suzuki,				NTT DoCoMo, Inc.		suzuki@spg.yrp.nttdocomo.co.jp