UIT - Secteur de la normalisation des télécommunications

ITU - Telecommunication Standardization Sector

UIT - Sector de Normalización de las Telecomunicaciones

Study Period 1997-2000

16	  D.xxx

Geneva, 13-18, November 2000

E

Question(s):	11/16

SOURCE*:	NTT DoCoMo, Inc.

TITLE:	Editorial revision to H.324 Annex H white document (COM 16-119-E)

___________________

This contribution proposes an editorial revision to H.324 Annex H white contribution (COM 16-119-E). This revision is to clarify the usage of "Stuffing Frame" in section H.5.3.

# Proposed revision

H.5.3 Stuffing Frames

In the event that the mobile multilink layer has no information to send on a channel, such as when a channel is no longer part of the multilink session but has not been disconnected yet, a flag, shown in Figure3, followed by 5 zero octets shall be sent as the stuffing sequence.

For connections that are part of a multilink session it is the responsibility of the H.223 multiplex to supply the multilink with the correct amount of data, taking into account the overhead for the multilink framing.

H.324 Annex H defines two kinds of synchronization flags, flag in Figure 3/Annex H and its one's complement, to distinguish two communication modes, full header mode and compressed header mode respectively. In the stuffing frames, however, only flag as shown in Figure 3 is used, so that the terminals can treat them as a special full header mode frames.

Although the original sentence implied the above usage by stating "followed by 5 zero octets", there exists some ambiguity. Therefore, the above revision is proposed for clarification.

____________________