ITU - Telecommunication Standardization Sector       Document Q11A10/AVD2054

STUDY GROUP 16

Q.D, F, G, 1-5/16 Rapporteur Meeting

- Launceston, 5 – 9 March 2001

SOURCE*:	Editor

TITLE:	Draft new Annex I to ITU-T Recommendation H.324

PURPOSE:	Proposal

__________________

- Abstract: This contribution contains the updated draft of new Annex I to H.324: Usage of HTTP Generic Capability in H.324 terminals. For clarification, several editorial revisions are proposed to the draft which was agreed in the last SG16 meeting.

The Capability identifier defined in the Table I.1/H.324 needs to be added to Table VIII.1/H.245 in the appendix VIII of H.245.

ANNEX I

Usage of HTTP Generic Capability in H.324 terminals

(This annex forms an integral part of this Recommendation)

# General

This annex defines the usage of HTTP - Hypertext Transfer Protocol [1] Capability in H.324 terminals. HTTP is an application-level protocol for distributed, collaborative, hypermedia information systems, and its technical specifications are provided in IETF RFC 2068. The capability presented in this annex is used for applications that want to make use of HTTP capabilities in H.324 terminals.

The purpose of using a HTTP channel associated with a H.324 call is to permit the HTTP client (e.g. Web browser) to remotely operate a far-end H.324 endpoint (in which the HTTP server is implemented).  This is especially useful in cases where the far-end H.324 endpoint is an automatic device.

For example, by selecting items on a Web page, the human user could cause the far-end system to switch input video or audio sources, or control far-end audio pickup.  In another example, the human user could choose via a Web page to view one of a set of stored audiovisual streams, which might contain entertainment or educational material.

Figure I. 1 illustrates such ae examply. In this example, an H.324 Annex I terminal (on the left) receives audiovisual contens from a Content server where H.324 Annex I is implemented. The logical channel for HTTP transactions, which is used tolexchangeselect the audiovisual contentrinformatioeto be send, is opened using H.245 logical channel signaling. Separate logical channels for delivery of audio and video data may be opened using H.245 logical channel signaling, if necessary.

Figure I. 1 An application using H.324 Annex I

# Logical channel for HTTP

The terminals which intend to use the HTTP capability shall open bi-directional logical channels for HTTP messages encapsulated by the underlying protocol specified in Table I.3/H.324.

The error protection for these logical channels may be arbitrarily negotiated, requested and chosen by use of the "transport" field in the Generic Capability.

# HTTP Generic Capability

Table I.1 defines the capability identifier for HTTP Generic Capability. Table I.2 and I.3 define the associated capability parameters.

TABLE I.1/H.324

Capability Identifier for HTTP Capability

| Capability name: | HTTP |
| --- | --- |
| Capability class: | Data application |
| Capability identifier type: | Standard |
| Capability identifier value: | itu-t (0) recommendation (0) h (8) 324 generic-capabilities (1) 0 |
| maxBitRate: | This field shall be included. |
| nonCollapsingRaw: | This field shall not be included. |
| transport: | This field shall be included. |

TABLE I.2/H.324

Mode for HTTP Capability

| Parameter name: | mode |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. mode indicates the operating mode of the terminal: - Server - Client - Server & Client (this mode may be used in Capability exchange, but shall not be set in Logical Channel Signaling) |
| Parameter identifier value: | 0 |
| Parameter Status: | Mandatory |
| Parameter type: | UnsignedMin. |
| Supersedes: | - |

TABLE I.3/H.324

Underlying protocol for HTTP Capability

| Parameter name: | underlyingProtocol |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. underlyingProtocol indicates the protocol under HTTP: 0: None 1: TCP/IP/PPP 2 ~ : Reserved for future extension |
| Parameter identifier value: | 1 |
| Parameter Status: | Mandatory |
| Parameter type: | UnsignedMin |
| Supersedes: | - |

# References

- FIELDING (R. et al.): Hypertext Transfer Protocol - HTTP/1.1, RFC 2068, Internet Engineering Task Force, January 1997.

*Contact:

Takashi Suzuki		Tel:+81 468 40 3520 	E-Mail: suzuki@spg.yrp.nttdocomo.co.jp