ITU - Telecommunication Standardization Sector	Temporary Document XXX

STUDY GROUP 16

Porto Seguro, 28 May - 8 June 2001

Question(s):	1

SOURCE*:	Rapporteur for Q.1 (P. Luthi)

TITLE:	Revised Implementor's Guide for the ITU-T T.120 Recommendation series

__________________

# Introduction

This document is a text revision of the Implementor’s Guide for T.120 series recommendations. The changes from the previous published version appear in the following sections:

- 7.5.1  Clarifications to Section B.9

- 7.6.1  Addition of Unicode Support – ASN1 syntax correction (additions of a comma after “…”)

# Implementor's Guide for the ITU-T T.120 Recommendation series - Data Protocols for Multimedia Conferencing

# Abstract

This document is a compilation of reported defects identified with the 1993-1999 editions of the ITU-T T.120-series Recommendations. It is intended to be read in conjunction with the Recommendations to serve as an additional authoritative source of information for implementors. The changes, clarifications and corrections defined herein are expected to be included in future versions of affected T.120-series Recommendations.

# Contact information

| ITU-T Study Group 16/Question 1 Rapporteur | Patrick Luthi PictureTel Corp. 100 Minuteman Road Andover, MA 01810 USA | Tel: +1 978 292 5000 Fax: +1 978 292 3309 Email: luthip@pictel.com |
| --- | --- | --- |

# Document History

| Version | Date | Description |
| --- | --- | --- |
| 1 | 15 February 1996 | Initial version - completed at the ITU-T Study Group 8 meeting, Geneva, 6-15 February 1996 |
| 2 | 26 April 1996 | Updated at the Q.10/8 Rapporteur's meeting, Boston, 21-26 April 1996 Sections added: 7.5.4 and 7.5.5 |
| 3 | 9 August 1996 | Updated at the Q.10/8 Rapporteur's meeting, Santa Rosa, 5-9 August 1996 Sections added: 7.5.6 |
| 4 | 4 October 1996 | Updated at the Q.10/8 Rapporteur's meeting, Ismaning, 30 September - 4 October 1996 |
| 5 | 17 January 1997 | Updated at the Q.10/8 Rapporteur's meeting, Newport Beach, 13-17 January 1997 |
| 6 | 29 January 1998 | Updated at the ITU-T SG 16 meeting, Geneva, 26 January - 6 February 1998 Sections added: 6.7.1.1 |
| 7 | 20 May 1999 | Updated at the ITU-T SG 16 meeting, Santiago, 17-28 May 1999 Added sections 6.8.6.1 and 6.8.9.1 |
| 8 | 8 June 2001 | Eighth version – Completed at the ITU-T Study Group 16 Rapporteurs meeting. |

# 1.  Introduction

This document is a compilation of reported defects identified with the 1993-1999 editions of the ITU-T T.120-series Recommendations. It is intended to be read in conjunction with the Recommendations to serve as an additional authoritative source of information for implementors. The changes, clarifications and corrections defined herein are expected to be included in future versions of affected T.120-series Recommendations.

The first version of the guide was produced following the February 1996 ITU-T Study Group 8 meeting. Wide distribution of this document is expected and encouraged.

# 2.  Scope

This Guide resolves defects in the following categories:

•	Editorial errors

•	Technical errors such as omissions or inconsistencies

•	Ambiguities.

In addition, the Guide may include explanatory text found necessary because of interpretation difficulties apparent from the defect reports.

This Guide will not address proposed additions, deletions, or modifications to the Recommendations that are not strictly related to implementation difficulties in the above categories. Proposals for new features should be made in the normal way through contributions to the ITU-T.

# 3.  Policies for updating this document

This document is managed by the ITU-T Study Group 16 Question 1 Rapporteur's Group. It can be revised at any recognized Q.1/16 Rapporteur's Group meeting provided the proposed revisions are unanimously accepted by the members of the group. A revision history cataloguing the evolution of this document is included.

# 4.  Defect Resolution Procedure

Upon discovering technical defects with any components of the T.120 Recommendations series, please provide a written description directly to the Q.1/16 Rapporteur.

# 5.  References

This document refers to the following T.120-series Recommendations:

•	ITU-T Recommendation T.120 (07/96) - Data Protocols for Multimedia Conferencing.

•	ITU-T Recommendation T.120, Annex C (02/98) - Lightweight Profiles for the T.120 Architecture.

•	ITU-T Recommendation T.121 (07/96) - Generic Application Template.

•	ITU-T Recommendation T.122 (02/98) - Multipoint Communication Service - Service Definition.

•	ITU-T Recommendation T.123 (05/99) - Network Specific Data Protocol Stacks for Multimedia Conferencing.

•	ITU-T Recommendation T.124 (02/98) - Generic Conference Control.

•	ITU-T Recommendation T.125 (02/98) - Multipoint Communication Service - Protocol Specification.

•	ITU-T Recommendation T.126 (07/97) - Multipoint Still Image and Annotation Protocol.

•	ITU-T Recommendation T.127 (08/95) - Multipoint Binary File Transfer Protocol.

•	ITU-T Recommendation T.128 (02/98) - Multipoint Application Sharing.

•	ITU-T Recommendation T.134 (02/98) - Text Chat Application Entity.

•	ITU-T Recommendation T.135 (02/98) - User-To-Reservation System Transactions within T.120 Conferences.

•	ITU-T Recommendation T.136 (05/99) - Remote Device Control Application Protocol.

•	ITU-T Recommendation H.282 (05/99)- A Far End Camera Control Protocol for Video Conferencing Using H.224.

# 6.  Nomenclature

In addition to traditional revision marks, the following marks and symbols are used to indicate to the reader how changes to the text of a Recommendation should be applied:

| Symbol | Description |
| --- | --- |
| [Begin Correction] | Identifies the start of revision marked text based on extractions from the published Recommendations affected by the correction being described. |
| [End Correction] | Identifies the end of revision marked text based on extractions from the published Recommendations affected by the correction being described. |
| ... | Indicates that the portion of the Recommendation between the text appearing before and after this symbol has remained unaffected by the correction being described and has been omitted for brevity. |
| --- SPECIAL INSTRUCTIONS --- {instructions} | Indicates a set of special editing instructions to be followed. |

# 7.  Technical and Editorial Corrections

## 7.1  Technical and Editorial Corrections to ITU-T Recommendation T.120

### 7.1.2  Addition of new MCS static channel assignments

Description:	T.120 Annex A1.1 contains a table of MCS Static Channel ID numbers. Two new Application Protocol Entities have been defined, each with a new Channel ID. The table is updated to add Channel IDs 11 and 12.

[Begin Correction]

## A1.1	Static Channels

| Symbolic Name | MCS Channel ID | Description | Recommendation |
| --- | --- | --- | --- |
| GCC-CHANNEL-0 | 1 | GCC Broadcast Channel | T.124 |
| GCC-CHANNEL-1 | 2 | Convenor Channel | T.124 |
| SI-CHANNEL-0 | 8 | MSIA Communications Channel | T.126 |
| MBFT-CHANNEL-0 | 9 | Control Channel | T.127 |
| MBFT-CHANNEL-1 | 10 | Data Channel | T.127 |
| AS-CHANNEL-0 | 11 | Application Sharing Channel | T.128 |
| CHAT-CHANNEL-0 | 12 | Chat Data Channel | T.134 |

[End Correction]

## 7.2  Technical and Editorial Corrections to ITU-T Recommendation T.120, Annex C

-None-

## 7.3  Technical and Editorial Corrections to ITU-T Recommendation T.121

-None-

## 7.4  Technical and Editorial Corrections to ITU-T Recommendation T.122

-None-

## 7.5  Technical and Editorial Corrections to ITU-T Recommendation T.123

### 7.5.1  Clarifications to Section B.9 – Connection Negotiation Protocol (CNP)

Text to clarify the usage of TPDU identifiers is being added to the overview of Section B.9 as follows:

[Begin Correction]

B.9.1	Overview

The PDUs defined in this protocol are based on their X.224/Class 0 counterparts. Both X.224 and CNP provide Connect Request, Connect Confirm, Disconnect Request, Data, and Error TPDUs. In addition, CNP provides a NonStandardPDU for protocol extensibility.

The CNP TPDUs defined in this Annex shall not be used outside the context of T.120 communications.

[End Correction]

## 7.6  Technical and Editorial Corrections to ITU-T Recommendation T.124

### 7.6.1  Addition of Unicode Support for Conference Names and Passwords

Description:	In order to better support these users of Unicode languages the definitions of ConferenceName and Password shall be modified with additional optional fields past the extension markers. These fields will allow the full support of Unicode strings for the identification of conference names and for conference passwords. The following are the proposed modifications to the ASN.1.

[Begin Correction]

ConferenceName ::= SEQUENCE

{

numeric		SimpleNumericString,

text			SimpleTextString OPTIONAL,

...,

unicode_text		TextString OPTIONAL

}

ConferenceNameSelector ::= CHOICE

{

numeric		SimpleNumericString,

text			SimpleTextString,

...,

unicode_text		TextString

}

Password ::= SEQUENCE

{

numeric		SimpleNumericString,

text			SimpleTextString OPTIONAL,

...,

unicode_text		TextString OPTIONAL

}

PasswordSelector ::= CHOICE

{

numeric		SimpleNumericString,

text			SimpleTextString,

...,

unicode_text		TextString

}

[End Correction]

## 7.7  Technical and Editorial Corrections to ITU-T Recommendation T.125

-None-

## 7.8  Technical and Editorial Corrections to ITU-T Recommendation T.126

-None-

## 7.9  Technical and Editorial Corrections to ITU-T Recommendation T.127

### 7.9.1  ASN.1 Errata

Description:	The following additions to the ASN.1 for the File-OfferPDU were omitted from the published text, as documented in the editor's errata sheet issued at the Paris Rapporteur meeting of 10/95.

The ASN.1 text shown below shall be added.

[Begin Correction]

File-OfferPDU ::= SEQUENCE

{

file-header			FileHeader,

data-channel-id		ChannelID,

file-handle			Handle,

roster-instance		INTEGER (0..65535) OPTIONAL,

file-transmit-token		TokenID OPTIONAL,

file-request-token		TokenID OPTIONAL,

file-handle			Handle OPTIONAL,

mbftID			UserID OPTIONAL

compression-specifier	CompressionSpecifier OPTIONAL,

compressed-filesize		INTEGER OPTIONAL,

ack-flag			BOOLEAN,			-- True if acknowledgements

-- required

...,

}

[End Correction]

## 7.10  Technical and Editorial Corrections to ITU-T Recommendation T.128

-None-

## 7.11  Technical and Editorial Corrections to ITU-T Recommendation T.134

-None-

## 7.12  Technical and Editorial Corrections to ITU-T Recommendation T.135

-None-

## 7.13  Technical and Editorial Corrections to ITU-T Recommendation T.136

-None-

## 7.14  Technical and Editorial Corrections to ITU-T Recommendation H.282

-None-

T.120 RECOMMENDATION SERIES DEFECT REPORT FORM

| DATE: |  |
| --- | --- |
| CONTACT INFORMATION NAME: COMPANY: ADDRESS: TEL: FAX: EMAIL: |  |
| AFFECTED RECOMMENDATIONS: |  |
| DESCRIPTION OF PROBLEM: |  |
| SUGGESTIONS FOR RESOLUTION: |  |

NOTE - Attach additional pages if more space is required than is provided above.