| Question(s): | 1, 2, 3, 6 |  | Meeting, date: |  | 20-30 May 2003 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Study Group: | 16 | Working Party: | 3 | Intended type of document (R-C-D-TD): |  | TD |
| Source: | Chair, WP2/16 (S. OKUBO) |  |  |  |  |  |
| Title: | Draft new H.241 “Extended Video Procedures and Control Signals for H.300 Series Terminals”, for Consent |  |  |  |  |  |
| Contact: | Dave Lindbergh Polycom Inc. 100 Minuteman Road Andover MA 01810 USA |  |  |  | Tel: +1 978 292 5366 Email: lindbergh@92F1.com |  |

AAP Summary: This Recommendation defines the procedures for use of advanced video codecs, including Rec. H.264, with H.300 series terminals, including H.310, H.320, H.321, H.322, H.323, and H.324.  Such procedures include control, indication, capability exchange, and transport mechanisms.

Additionally, this Recommendation defines generic extended video control, indication, and capability signalling, applicable for use with all video codecs used in H.300 series multimedia terminals.

There are advanced plans to deploy H.264 video coding in the marketplace; this Rec. provides the necessary signalling to use H.264 in H.310, H.320, and H.323 systems.

Traditional Summary: This Recommendation defines the procedures for use of advanced video codecs, including Rec. H.264, with H.300 series terminals, including H.310, H.320, H.321, H.322, H.323, and H.324.  Such procedures include control, indication, capability exchange, and transport mechanisms.

Additionally, this Recommendation defines generic extended video control, indication, and capability signalling, applicable for use with all video codecs used in H.300 series multimedia terminals.

1	Scope	4

2	References	4

3	Definitions	5

4	Abbreviations	5

5	Conventions	5

5.1	System terminology	5

5.2	Message names	5

5.3	Requirement terminology	6

6	Commands and Indications	6

6.1	C&I applicable to all video codecs	6

6.2	C&I for use with Rec. H.264	6

6.2.1	videoFreezePicture command in H.264	7

6.2.2	videoFastUpdatePicture command in H.264	7

6.2.3	Recovery point SEI message	8

7	Transport of coded video in H.300 series systems	8

7.1	Transport of Rec. H.264 video streams	8

7.1.1	Parameter set transmission	8

7.1.2	Use of H.264 in BAS based systems	9

7.1.3	Transport of H.264 streams in H.310 systems	9

7.1.4	Transport of H.264 streams in H.323 systems	9

7.1.5	Transport of H.264 streams in H.324 systems	9

8	Capability exchange signalling	9

8.1	General	9

8.2	Signalling of H.245 generic parameters in BAS based systems	11

8.3	H.264 capabilities	14

8.3.1	General	14

8.3.2	H.264 Generic Capability for H.245	14

8.3.3	H.264 capabilities for BAS based systems	20

Annex A – H.264 transport for H.323	22

A.1	Introduction	22

A.2	RTP payload format for H.264 video	23

A.2.1	RTP header usage	23

A.2.2	Simple packet	24

A.3	Packetization rules	24

A.4	De-Packetization Process (non-normative)	25

Appendix I – ASN.1 OIDs defined in this Recommendation	25

Proposed Draft H.241

Extended Video Procedures and Control Signals for H.300 Series Terminals

Summary

This Recommendation defines the use of advanced video codecs, including Rec. H.264, in Rec. H.310, H.320, H.321, H.322, H.323, and H.324 terminals.  It also defines generic extended signalling for use with all video codecs in H.300 series terminals.

Keywords

H.310, H.320, H.321, H.322, H.323, H.324, H.264, video coding, video codec, video, videoconferencing, videotelephony, signalling, signaling, commands, capability exchange

# 1	Scope

This Recommendation defines the procedures for use of advanced video codecs, including Rec. H.264, with H.300 series terminals, including H.310, H.320, H.321, H.322, H.323, and H.324.  Such procedures include control, indication, capability exchange, and transport mechanisms.

Additionally, this Recommendation defines generic extended video control, indication, and capability signalling, applicable for use with all video codecs used in H.300 series multimedia terminals.

# 2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

–	IETF RFC 1889, RTP: A Transport Protocol for Real-Time Applications

–	ITU-T Recommendation H.221 (1999), Frame structure for a 64 or 1920 kbit/s channel in audiovisual teleservices.

–	ITU-T Recommendation H.239 (2003), Role Management and Additional Media Channels for H.300-series Terminals

–	ITU-T Recommendation H.230 (1999), Frame-synchronous control and indication signals for audiovisual systems.

–	ITU-T Recommendation H.242 (1999), System for establishing communication between two audiovisual terminals using digital channels up to 2 Mbit/s.

–	ITU-T Recommendation H.245 (2001), Control protocol for multimedia communication.

–	ITU-T Recommendation H.261 (1993), Video codec for audiovisual services at p  64 kbit/s.

–	ITU-T Recommendation H.263 (1998), Video coding for low bit rate communication.

–	ITU-T Recommendation H.264 (2003), Advanced Video Coding for Generic Audiovisual Services.

–	ITU-T Recommendation H.310 (1998), Broadband audiovisual communication systems and terminals.

–	ITU-T Recommendation H.320 (1999), Narrow-band visual telephone systems and terminal equipment.

–	ITU-T Recommendation H.323 (2002), Packet based multimedia communications systems.

–	ITU-T Recommendation H.324 (1998), Terminal for low bit-rate multimedia communications.

# 3	Definitions

terminal: A terminal is any endpoint and may be a user's terminal or some other communication system such as an MCU or an information server.

# 4	Abbreviations

ASN.1		Abstract Syntax Notation One (see H.245)

AL-SDU	Adaptation Layer Service Data Unit (see H.324)

BAS 		Bit-rate Allocation Signal (see H.221)

C&I		Control & Indication

IDR		Instantaneous Decoding Refresh (see H.264)

MBE		Multiple Byte Extension (see H.221)

OID		Object Identifier (see H.245)

# 5	Conventions

## 5.1	System terminology

In order to simplify references, this Recommendation refers to two classes of signalling systems for H.300 series terminals.

“BAS based systems” refers to those systems that use signalling in the H.221 BAS channel; these include H.320, H.321, and H.322 systems.

“H.245 based systems” refers to those systems that use signalling according to Rec. H.245; these include H.310, H.323, and H.324 systems.

## 5.2	Message names

In this Recommendation signalling messages which are common to both H.245 and BAS signalling systems are referred to by their names as given in Annex A/H.245, except in cases where their use in the unique BAS signalling environment are described.  Message names are presented in bold font to distinguish them from the other text of this Recommendation.

Table 1 provides a reference for corresponding H.245 and H.242/H.230 messages mentioned in this Recommendation.

Table 1/H.241 – Corresponding H.245 and BAS video signals

| H.245 name | H.230 mnemonic |
| --- | --- |
| h263Options.customPictureClockFrequency | ØCPCF |
| h263Options.customPictureFormat | ØCSFMT |
| h263Options.customPictureFormat | ØCPAR |
| h263VideoCapability.enhancementLayerInfo | ØSCLPREF |
| lostPartialPicture | lostPartialPicture |
| lostPicture | lostPicture |
| recoveryReferencePicture | recoveryReferencePicture |
| videoBadMBs | VBMBC |
| videoFastUpdateGOB | videoFastUpdateGOB |
| videoFastUpdateMB | videoFastUpdateMB |
| videoFastUpdatePicture | VCU |
| videoFreezePicture | VCF |
| videoNotDecodedMBs | videoNotDecodedMBs |
| videoSendSyncEveryGOB | ØGHOP |
| videoSendSyncEveryGOBCancel | Øcancel-GHOP |

## 5.3	Requirement terminology

In this Recommendation the following conventions are used:

"Shall" indicates a mandatory requirement.

"Should" indicates a suggested but optional course of action.

"May" indicates an optional course of action rather than a recommendation that something take place.

# 6	Commands and Indications

## 6.1	C&I applicable to all video codecs

For further study.

## 6.2	C&I for use with Rec. H.264

The following C&I signals shall not be used with regard to any channel operating according to Rec. H.264:

BAS signals ØCPCF, ØCSFMT, ØCPAR, ØSCLPREF

lostPartialPicture

lostPicture

recoveryReferencePicture

videoBadMBs

videoFastUpdateGOB

videoFastUpdateMB

videoNotDecodedMBs

videoSendSyncEveryGOB

videoSendSyncEveryGOBCancel

NOTE – The above signals are either specific to Rec. H.263 or have parameters which do not correspond to Rec. H.264 structures or value ranges.  Replacement signals which could be used either with Rec. H.264 or in a generic form for any video codec, are for further study.

All other C&I not mentioned in this section shall be used as specified elsewhere.

NOTE - For example, the use of videoIndicateReadyToActivate and corresponding BAS signal VIR is not affected by this Recommendation.

### 6.2.1	videoFreezePicture command in H.264

When a video decoder according to H.264 receives a videoFreezePicture command it shall freeze its displayed picture until one of the following events:

- A recovery point signalled in a recovery point SEI message (D.2.7/H.264) is reached.

- Reception of an IDR picture.

- A timeout period of at least 6 seconds has elapsed since the videoFreezePicture command was received.

### 6.2.2	videoFastUpdatePicture command in H.264

When a video encoder according to H.264 receives a videoFastUpdatePicture command, the encoder shall enter the fast update mode by using one of the procedures specified in section 6.2.2.1 or 6.2.2.2 below.  The procedure in 6.2.2.1 is the preferred response in a lossless transmission environment.  Both procedures satisfy the requirement to enter the fast update mode for H.264 video encoding.

NOTE – The procedures re-initialize a H.264 decoder completely such that valid video frames will be decoded.  Such re-initialization is effective regardless of whether or not the decoder was previously decoding any video stream from any endpoint.

The procedure should be accomplished as quickly as possible, but the re-initialization video stream shall be completely transmitted within 3 seconds of receiving the videoFastUpdatePicture command.

NOTE – The 3 second requirement is needed to avoid timeout of the 6 second timer associated with the videoFreezePicture command, taking into account network and system latencies and possible cascaded MCUs.  The videoFreezePicture command is used by MCUs as part of the video switching procedure (see 6.1.1/H.243).

#### 6.2.2.1	IDR procedure to respond to videoFastUpdatePicture

This section gives one possible way to respond to videoFastUpdatePicture.

The encoder shall, in the order presented here:

- Immediately prepare to send an IDR picture (see section 3/H.264).

- Send a H.264 sequence parameter set corresponding to the IDR picture to be sent.  The encoder may optionally also send other parameter sets.

- Send a H.264 picture parameter set corresponding to the IDR picture to be sent.  The encoder may optionally also send other parameter sets.

- Send the IDR picture.

- From this point forward in time, send or re-send any other sequence or picture parameter sets, not sent in this procedure, prior to their reference by any H.264 slice, regardless of whether such parameter sets were previously sent prior to receiving the videoFastUpdatePicture command.  Such parameter sets may be sent all at once (within the limits of Rec. H.264), one at a time as needed, or in any combination of these methods. Parameter sets may be re-sent at any time for redundancy.

#### 6.2.2.2	Gradual recovery procedure to respond to videoFastUpdatePicture

This section gives one possible way to respond to videoFastUpdatePicture.

The encoder shall, in the order presented here:

- Send a recovery point SEI message (D.2.7/H.264).

- Repeat any sequence and picture parameter sets that were sent before the recovery point SEI message, prior to their reference in a H.264 slice.

The encoder shall ensure that the decoder has access to all reference pictures for inter prediction of pictures at or after the recovery point in output order.  For example, the encoder may mark all reference pictures as “unused for reference” by issuing a memory_management_control_operation equal to 5 (see subclause 8.2.5/H.264).

The value of the recovery_frame_cnt syntax element in the recovery point SEI message shall be such that the time between the reception of the videoFastUpdatePicture command and completing the transmission of the access unit including the recovery point as specified in subclause D.2.7/H.264 is less than or equal to 3 seconds.

Re-sending of parameter sets may be done all at once (within the limits of Rec. H.264), one at a time as needed, or in any combination of these methods. Parameter sets may be re-sent at any time for redundancy.

### 6.2.3	Recovery point SEI message

H.264 video decoders in H.300 series terminals shall support reception of the recovery point SEI message (see D.2.7/H.264) and identify the signalled recovery point.

Upon reception of a recovery point SEI message, the decoder shall continue to decode until the recovery point regardless of apparent errors in the stream such as reference to absent pictures, and should not send a videoFastUpdatePicture command in response to such apparent error.

If a videoFreezePicture is in force the decoder shall not display the decoded pictures, and shall continue to display the previously frozen picture.  If the broken_link_flag in the recovery point SEI message is set, the decoder may choose not to display decoded pictures until the recovery point is reached.

If the decoder detects bitstream corruption between the SEI message and the recovery point in decoding order, a videoFastUpdatePicture command should be sent.

# 7	Transport of coded video in H.300 series systems

## 7.1	Transport of Rec. H.264 video streams

## 7.1.1	Parameter set transmission

H.264 parameter set information shall be transmitted in-band to the H.264 video stream (see “NOTE” in 7.4.1.2.1/H.264).

Terminals sending H.264 video shall transmit each sequence or picture parameter set at a time prior to its reference by any H.264 slice.  These parameter sets may be re-sent at any time for redundancy.

NOTE – There is no requirement that parameter sets must be transmitted each time they are about to be referenced by a H.264 slice.  The transmission may take place at any time prior to the reference.  Ordinarily, many H.264 slices will refer to the same parameter set, with the parameter set being sent only once.

### 7.1.2	Use of H.264 in BAS based systems

When carried in a BAS signalling based system, the H.264 video shall make use of the byte stream format given in Annex B/H.264.

The resulting byte stream shall be transmitted using the framing and forward error correction method given in section 5.4/H.261. This procedure is the same as that used for Recommendations H.261 and H.263.

Terminals encoding H.264 video may insert fill bits using the fill indicator (Fi) as described in 5.4.3/H.261.

NOTE – Insertion of such fill may be useful, for example, to reduce the effective coded video data rate within the video channel to avoid exceeding a H.264 decoder’s maximum video bitrate (MaxBR) as given in Annex A/H.264.

### 7.1.3	Transport of H.264 streams in H.310 systems

In H.310 systems, the H.264 video shall make use of the byte stream format given in Annex B/H.264.  H.264 shall be used without BCH error correction and without error correction framing.

### 7.1.4	Transport of H.264 streams in H.323 systems

In H.323 systems, H.264 shall be used without BCH error correction and without error correction framing.  Such systems shall not make use of the byte stream format given in Annex B/H.264.

All H.323 systems that support H.264 shall support carriage of the H.264 video stream according to Annex A, and shall signal this in their capability set by including  MediaPacketizationCapability.rtpPayload.Type.payloadDescriptor.oid, with the OID having the value {itu-t(0) recommendation(0) h(8) 241(241) specificVideoCodecCapabilities(0) h264(0) iPpacketization(0) h241AnnexA(0)}.

NOTE - If in the future the IETF defines an RFC for the packetization of Rec. H.264 on RTP/UDP/IP, ITU-T will consider specifying the optional use of such RFCs.

### 7.1.5	Transport of H.264 streams in H.324 systems

In H.324 systems, H.264 shall be used without BCH error correction and without error correction framing, and shall make use of the byte stream format given in Annex B/H.264.

H.264 encoders shall align the Annex B/H.264 start code prefix for the first NAL unit of each access unit with the start of an AL-SDU.

# 8	Capability exchange signalling

## 8.1	General

Terminals which display received video shall be capable of displaying any picture format and frame rate for which they signal the capability.  The format used to display such received video streams is not required to match the exact format transmitted.

NOTE – For example, a video conferencing system which decodes Rec. H.264 at a given Profile and Level must display any picture format and frame rate allowed by that Profile and Level.

If during an ongoing connection a terminal which is transmitting video receives a changed capability set, the terminal shall adapt its video coding method to conform with all the limitations signalled in the received capability set.

## Signalling of H.245 generic parameters in BAS based systems

This Recommendation signals a subset of H.245 GenericParameter structures in BAS channel MBE messages.  These messages shall be carried in H.245-based systems using the procedures of Annex A/H.239.  These procedures avoid emulation of the MBE BAS code.

## H.264 capabilities

### 8.3.1	General

H.300 series terminals may optionally support video according to Recommendation H.264.

The H.264 capability set is structured as a list of one or more H.264 capabilities, each of which includes:

- Profile (mandatory)

- Level (mandatory)

- Zero or more optional parameters

These capabilities indicate the ability to decode using one or more H.264 Profiles.  The exact syntax and semantics are given in the sections below.  In the case of H.245 based systems, each capability is contained in a GenericCapability structure.  For BAS based systems, all capabilities are carried in a single MBE message.

The bitrate made available for a video stream by an H.300 series system may be less than the maximum video bit rate which decoders are required to support by Annex A/H.264.  Terminals are not required to decode video streams which they do not receive.

#### 8.3.1.1	Optional parameters

For each H.264 capability, optional parameters may be signalled.  These parameters permit a terminal to signal that, in addition to meeting the support requirements for the signalled Profile and Level, the terminal has additional capabilities.  Such additional capabilities in decoders may permit encoders to send a video stream which takes advantage of these capabilities.

The optional parameters are:

- CustomMaxMBPS – If present, indicates the decoder has a higher processing rate capability.

- CustomMaxFS – If present, indicates the decoder can decode larger picture (frame) sizes.

- CustomMaxDPB – If present, indicates the decoder has additional decoded picture buffer memory.

- CustomMaxBR&CPB – If present, indicates the decoder can decode a higher video bit rate and has a correspondingly larger coded picture buffer.

If these parameters are present, the signalled values replace the MaxMBPS, MaxFS, MaxDPB, MaxBR, and MaxCPB values, respectively, in Table A-1/H.264 for the given Profile and Level, and indicate that in addition to fully conforming with the Profile and Level requirements, these additional capabilities are available at the decoder.

These optional parameters permit, for example, support of 1024x768x3 Hz while using Level 2 (CIF/30 Hz), a common mode for videoconferencing systems.

All H.300 series systems which support H.264 shall support Baseline Profile, Level 1, in addition to any other Profiles, Levels or optional parameters.

### H.264 Generic Capability for H.245

This section defines the Generic Capabilities for H.264 in the H.245 signalling system.

If a terminal has the capability to decode according to more than one H.264 Profile with different Levels capabilities (for example, Baseline Profile at Level 3 and Extended Profile at Level 2) or with different optional parameters for each Profile, this may be signalled by a separate Generic Capability for each supported Profile.

NOTE – Parameter identifier value 0 is not defined, and should not be defined in the future.  This value is reserved so that it can be used in the equivalent BAS signalling based system MBE message as a demarcation between separate capabilities within the single MBE message, as defined in Annex A/H.239.

#### 8.3.2.1	H.264 Capability Identifier

| Table 2/H.241  H.264 Capability Identifier |  |
| --- | --- |
| Capability name | ITU-T Recommendation H.241 H.264 Video Capabilities |
| Capability identifier type | standard |
| Capability identifier value | {itu-t(0) recommendation(0) h(8) 241(241) specificVideoCodecCapabilities(0) h264(0) generic-capabilities(1)} |
| maxBitRate | This field shall be included, in units of 100 bits/s |
| collapsing | This field shall contain the H.264 Capability Parameters as given below. |
| nonCollapsing | This field shall not be included. |
| nonCollapsingRaw | This field shall not be included. |
| transport | This field shall not be included. |

#### 8.3.2.2	H.264 Profile parameter

Table 3/H.241  H.264 Capability Parameter – Profile

| Parameter name | Profile |
| --- | --- |
| Parameter description | This parameter is a Boolean array. If bit 2 (value 64) is 1, this indicates the Baseline Profile. If bit 3 (value 32) is 1, this indicates the Main Profile. If bit 4 (value 16) is 1, this indicates the Extended Profile. All other bits are reserved, shall be set to 0, and shall be ignored by receivers. In a decoder capability, for each bit set to 1, this means that the terminal is capable of decoding the indicated Profile(s) using the Level and other optional parameters in this Generic Capability. In an OpenLogicalChannel message, for each bit set to 1, this means that the logical channel contents obey all constraints of the indicated Profile(s). NOTE – If in the future more H.264 Profiles are defined than the number of reserved bits can accommodate, additional Profiles could be signalled by allocating another parameter for additional Profiles. |
| Parameter identifier value | 41 |
| Parameter status | Mandatory. This parameter shall appear exactly once in each Generic Capability. |
| Parameter type | booleanArray |
| Supersedes | This field shall not be included. |

#### 8.3.2.3	H.264 Level parameter

The Level parameter signals the H.264 Level.

Table 4/H.241  H.264 Capability Parameter – Level

| Parameter name | Level |
| --- | --- |
| Parameter description | Signals a value according to Table 5, indicating the H.264 Level. All other values are reserved and shall not be transmitted. Terminals that receive this signal with a Level parameter value less than the lowest Level parameter value shown in Table 5 shall ignore this capability parameter. NOTE – Such values are reserved for future use. For all other received Level parameter values, the terminal shall interpret the signalled H.264 Level number as the H.264 Level number corresponding to the highest Level parameter value in Table 5 which is less than or equal to the received Level parameter value. |
| Parameter identifier value | 42 |
| Parameter status | Mandatory. This parameter shall appear exactly once in each Generic Capability. |
| Parameter type | unsignedMin |
| Supersedes | This field shall not be included. |

Table 5/H.241  Level parameter values

| Level parameter value | H.264 Level number |
| --- | --- |
| 15 | 1 |
| 22 | 1.1 |
| 29 | 1.2 |
| 36 | 1.3 |
| 43 | 2 |
| 50 | 2.1 |
| 57 | 2.2 |
| 64 | 3 |
| 71 | 3.1 |
| 78 | 3.2 |
| 85 | 4 |
| 92 | 4.1 |
| 99 | 4.2 |
| 106 | 5 |
| 113 | 5.1 |

NOTE – Table 5 and this parameter description are constructed such that new H.264 Levels defined in the future, which are strictly between or above existing Levels, may be inserted into Table 5 in the future.  If new Levels are defined which do not meet these constraints, they may be inserted below the lowest existing Level.  In that case new rules for interpreting such Level parameter values will be needed.

#### 8.3.2.4	H.264 CustomMaxMBPS processing rate parameter

The optional CustomMaxMBPS parameter permits a decoder to signal that it is capable of decoding video at a higher rate than required by the signalled Level.  Encoders may use this knowledge to, for example, send pictures of a given size at a higher frame rate..

Table 6/H.241  H.264 Capability Parameter – CustomMaxMBPS

| Parameter name | CustomMaxMBPS |
| --- | --- |
| Parameter description | CustomMaxMBPS is the maximum macroblock processing rate, in units of 500 macroblocks per second. This optional parameter, when present, shall be considered to replace the MaxMBPS value in Table A-1/H.264 for the signalled Level. The value of (CustomMaxMBPS * 500) shall not be less than the value MaxMBPS for the Level given in Table A-1/H.264. |
| Parameter identifier value | 3 |
| Parameter status | Optional. This parameter shall appear at most once in each Generic Capability. |
| Parameter type | unsignedMin |
| Supersedes | This field shall not be included. |

#### 8.3.2.5	H.264 CustomMaxFS frame size parameter

The optional CustomMaxFS parameter permits a decoder to signal that it is capable of decoding larger picture sizes than required by the signalled Level.  Encoders may use this knowledge to, for example, send larger pictures at a proportionally lower frame rate.

Table 7/H.241  H.264 Capability Parameter – CustomMaxFS

| Parameter name | CustomMaxFS |
| --- | --- |
| Parameter description | CustomMaxFS is the maximum frame size, in units of 256 luma macroblocks. This optional parameter, when present, shall be considered to replace the MaxFS value in Table A-1/H.264 for the signalled Level. The value of (CustomMaxFS * 256) shall not be less than the value MaxFS for the Level given in Table A-1/H.264. |
| Parameter identifier value | 4 |
| Parameter status | Optional. This parameter shall appear at most once in each Generic Capability. |
| Parameter type | unsignedMin |
| Supersedes | This field shall not be included. |

#### 8.3.2.6	H.264 CustomMaxDPB memory parameter

The optional CustomMaxDPB parameter permits a decoder to signal that it has more than the minimum amount of decoded picture buffer memory required by the signalled Level.  Encoders may use this knowledge to construct coded video streams with improved compression.

A system which signals CustomMaxDPB shall be capable of storing the following number of decoded frames in its decoded picture buffer:

Min(32768 * CustomMaxDPB / ( PicWidthInMbs * FrameHeightInMbs * 256 * ChromaFormatFactor ), 16)

PicWidthInMbs, FrameHeightInMbs, and ChromaFormatFactor are defined in Rec. H.264.

Table 8/H.241  H.264 Capability Parameter – CustomMaxDPB

| Parameter name | CustomMaxDPB |
| --- | --- |
| Parameter description | CustomMaxDPB is the maximum decoded picture buffer size, in units of 32,768 bytes. This optional parameter, when present, shall be considered to replace the MaxDPB value in Table A-1/H.264 for the signalled Level. The value of (CustomMaxDBP * 32768) shall not be less than the value (MaxDBP * 1024) for the Level given in Table A-1/H.264. |
| Parameter identifier value | 5 |
| Parameter status | Optional. This parameter shall appear at most once in each Generic Capability. |
| Parameter type | unsignedMin |
| Supersedes | This field shall not be included. |

#### 8.3.2.7	H.264 CustomMaxBR&CBP bit rate and coded picture buffer size parameter

The optional CustomMaxBR&CBP parameter permits a decoder to signal that it is capable of decoding video streams of higher bitrate, and that it has a correspondingly larger coded picture buffer, than required by the signalled Level.  Encoders may use this knowledge to, for example, send higher bitrate video to achieve improved video quality.

Table 9/H.241  H.264 Capability Parameter – CustomMaxBRandCPB

| Parameter name | CustomMaxBR&CPB |
| --- | --- |
| Parameter description | CustomMaxBR&CPB is the maximum video bit rate. The maximum coded picture buffer (CBP) size is derived from the maximum video bit rate. The units for maximum video bit rate are 25,000 bits/second for the VCL HRD parameters (see A.3.1 item i/H.264) and 30,000 bits/second for the NAL HRD parameters (see A.3.1 item j/H.264). The CPB size shall be derived as equal to the MaxCBP for the signalled Level (see Table A-1/H.264), multiplied by the ratio of the signalled maximum bit rate to the MaxBR for the signalled level. For example, if a terminal signals Level 1.2 with CustomMaxBRandCPB equal to 62, this indicates a maximum video bitrate of 1.550 Mbits/sec for VCL HRD parameters, a maximum video bitrate of 1.860 Mbits/sec for NAL HRD parameters, and a CPB size of 4,036,458 bits ((62*25,000)/384,000) * 1000 * 1000). This optional parameter, when present, shall be considered to replace the MaxBR and MaxCBP values in Table A-1/H.264 for the signalled Level. The value of (CustomMaxBR&CBP * 25,000) shall not be less than the value MaxFS computed using units of 1000 bits/sec for the Level given in Table A-1/H.264. |
| Parameter identifier value | 6 |
| Parameter status | Optional. This parameter shall appear at most once in each Generic Capability. |
| Parameter type | unsignedMin |
| Supersedes | This field shall not be included. |

### 8.3.3	H.264 capabilities for BAS based systems

#### 8.3.3.1	H.320 video algorithm hierarchy

The H.320 enhanced video algorithm hierarchy in Annex A/H.320 is not extended for H.264.  No relative level in the hierarchy is specified for H.264 with regard to other video codecs.

#### 8.3.3.2	H.264 capabilities MBE message format

For H.264 operation, the capabilities exchange is handled by an MBE message (see 2.2.3/H.230). This MBE message uses the type identification byte <H.264> (see Table 2/H.230). A terminal shall signal the H.264 capability by including within its capability set the message:

{ Start-MBE / N / <H.264>  / B1 /  . . .  / BN-1 }

The H.264 capability MBE bytes B1 through BN-1 may contain one or more encoding or decoding capabilities for H.264.

Each capability, which corresponds to a single H.245 GenericCapability message, consists of the mandatory Profile and Level parameters, and an optional set of zero or more parameterIdentifier/parameterValue pairs from the set of H.264 generic capability parameters defined in section 8.3.2.  These pairs are carried in the format given in section 8.2 above.

Encoder capabilities are for further study.

The first two bytes of each decoder capability within the MBE shall contain the H.264 Profile parameter, followed by the H.264 Level parameter, as defined in Tables 3 and 4.  No parameter identifier is included in the MBE, as these mandatory parameters are identified by their position in the decoder capability string.

Following the Profile and Level parameters, zero or more parameterIdentifier/parameterValue pairs containing the optional CustomMaxMBPS, CustomMaxFS, CustomMaxDPB, and CustomMaxBR&CBP parameters may be included, according to the syntax and semantics given for these parameters.  The set of parameterIdentifier/parameterValue pairs may appear in any order within the capability.

If the H.264 capability MBE contains more than one capability, the second and succeeding capabilities within the MBE message shall be demarcated by a single byte of value zero  immediately before the start of each succeeding capability.

NOTE – This zero byte appears in the position where a Parameter ID would otherwise appear.  Since the H.264 generic capabilities do not define a parameter with a parameterIdentifier value of zero, no confusion results.

Receivers shall ignore the value of any parameterValue following an undefined parameterIdentifier.

Table 10 below gives an example of an MBE with a single decoder capability indicating Baseline Profile, Level 3.1, with a CustomMaxMBPS parameter of 246,000 macroblocks/second:

Table 10 – Baseline Profile example MBE

| MBE | Value | Description |
| --- | --- | --- |
| Byte 1 | Start-MBE | Start of MBE. From H.230 |
| Byte 2 | 6 | Number of bytes to follow |
| Byte 3 | <H.264> | Indicates H.264 MBE. From H.230 |
| Byte 4 | 64 | Profile parameter - indicates Baseline Profile |
| Byte 5 | 63 | Level parameter - indicates Level 3.1 |
| Byte 6 | 3 | Parameter ID – CustomMaxMBPS |
| Byte 7 | 172 | Lowest 6 bits of 492 (equals 246,000 / 500), ORed with 128 |
| Byte 8 | 7 | Highest 7 bits of 492 |

Table 11 below gives an example H.264 capability MBE for a system that supports two capabilities:

  - Baseline Profile, Level 2.2, and,

  - Main Profile, Level 2, with CustomMaxFS supporting 800x600 SVGA format and CustomMaxMBPS supporting this format at a rate of 10 frames per second.

Table 11 – Two Profile example MBE

| MBE | Value | Description |
| --- | --- | --- |
| Byte 1 | Start-MBE | Start of MBE. From H.230 |
| Byte 2 | 10 | Number of bytes to follow |
| Byte 3 | <H.264> | Indicates H.264 MBE. From H.230 |
| Byte 4 | 32 | Profile parameter - indicates Main Profile. |
| Byte 5 | 35 | Level parameter - indicates Level 2 |
| Byte 6 | 4 | Parameter ID – CustomMaxFS |
| Byte 7 | 8 | Indicates 2048 macroblock frame size (1900 needed for 800x600) |
| Byte 8 | 3 | Parameter ID – CustomMaxMBPS |
| Byte 9 | 38 | Indicates 19,000 macroblocks/sec processing rate. |
| Byte 10 | 0 | Demarcates start of new capability |
| Byte 11 | 64 | Profile parameter - indicates Baseline Profile. |
| Byte 12 | 49 | Level parameter - indicates Level 2.2 |

# Annex A – H.264 transport for H.323

This annex is a normative part of this Recommendation.

## A.1	Introduction

All details required to implement the H.264 RTP payload format for H.264 are contained in this annex and its references.

Readers should note that this annex is not the complete and primary specification of the RTP payload specification for H.264; please refer to the appropriate IETF RFC for this informative reference.  This annex is intended only for use with H.241.

Readers should also note that the terminology used in this annex differs somewhat from that used in the body of H.241 and other ITU-T Recommendations according to Table A.1:

Table A.1/H.241

| ITU Recommendation Term (H.241 and others in the H.323 framework) | Section A.2/Annex A/H.241 (RTP payload spec for H.264) Term |
| --- | --- |
| Annex A | specification or document |
| may | MAY |
| shall | MUST |
| shall not | MUST NOT |
| should | SHOULD |
| should not | SHOULD NOT |

BCH forward error correction and the byte stream format of Annex B/H.264 shall not be used for H.323 transport.

## A.2	RTP payload format for H.264 video

### A.2.1	RTP header usage

The format of the RTP header is specified in RFC 1889 and is reprinted in Figure A.1 for convenience.  This payload format uses the fields of the header in a manner consistent with that specification.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|V=2|P|X|  CC   |M|     PT      |       sequence number         |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                           timestamp                           |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|           synchronization source (SSRC) identifier            |

+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+

|            contributing source (CSRC) identifiers             |

|                             ....                              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure A.1/H.241 - RTP header according to RFC 1889

The RTP header information shall be set as follows:

Version (V): 2 bits

Set to 2 according to RFC 1889.

Padding (P): 1 bit

Used according to RFC 1889.

Extension (X): 1 bit

Specified in the RTP profile in use.

CSRC count (CC): 4 bits

Used according to RFC 1889.

Marker bit (M): 1 bit

Set for the very last packet of the Access Unit indicated by the RTP timestamp, in line with the normal use of the M bit and to allow efficient playout buffer handling.  Decoders MAY use this bit as an early indication of the last packet of a coded picture, but MUST not rely on this property because the last packet of the picture may get lost, and because future backward compatible extensions of this payload spec will allow for packet aggregation that does not necessarily preserve the M bit values for all NALUs.

Payload type (PT): 7 bits

The assignment of an RTP payload type for this new packet format is outside the scope of this document, and will not be specified here.  It is expected that the RTP profile under which this payload format is being used will assign a payload type for this encoding or specify that the payload type is to be bound dynamically.

Sequence number (SN): 16 bit

Increased by one for each sent packet.  Set to a random value during startup as per RFC1889.

Timestamp: 32 bits

The RTP timestamp shall be set to the sampling timestamp of the content.  If the NALU has no timing properties of its own (for example, parameter set and SEI NAL units), the RTP timestamp shall be set to the RTP timestamp of the primary coded picture that is associated with the same access unit as the NALU according to section 7.4.1.2.3/H.264.

Synchronization source (SSRC) identifier: 32 bits

Used according to RFC 1889.

Contributing source (CSRC) identifiers: 0 to 15 items, 32 bits each

Used according to RFC 1889.

### A.2.2	Simple packet

The RTP payload of a Simple Packet according to this specification shall consist of one NALU as depicted in Figure A.2.  The type of the NALU MUST be one of those specified in Rec. H.264.  A NALU stream composed by de-encapsulating Simple Packets in RTP sequence number order MUST conform to the NAL unit decoding order according to section 7.4.1.2/H.264.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                                                               |

|                              NALU                             |

|                                                               |

|                               +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               :...OPTIONAL RTP padding        |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure A.2/H.241 - RTP payload format for Simple Packet

## A.3	Packetization rules

- VCL NALUs specified according to section 7.4.1/H.264 (i.e., NALUs containing a coded slice or a coded slice data partition) belonging to the same picture (and hence share the same RTP timestamp value) MAY be sent in any order permitted by the applicable profile defined in Rec. H.264, although, for delay critical systems, they SHOULD be sent in their original coding order to minimize the delay.  Note that the coding order is not necessarily the scan order, but the order the NAL packets become available to the RTP stack.

- Packets containing SEI NALUs MAY be sent anytime allowed by Rec. H.264.

- Parameter set NALUs MUST NOT be sent in an RTP session whose Parameter Sets were already changed by control protocol messages during the lifetime of the RTP session.  If parameter set NALUs are allowed by this condition, they MAY be sent at any time.

- All NALU types MAY be mixed freely, provided that above rules are obeyed.  In particular, it is allowed to mix coded slices and coded slice data partitions when allowed by the applicable profile defined in Rec. H.264.

## A.4	De-Packetization Process (non-normative)

The de-packetization process is implementation dependent.  Hence, the following description is an example of a suitable implementation.  Other schemes MAY be used as well.  Optimizations relative to the described algorithms are likely possible.

Since the packetization rules above already enforce a standard-compliant NALU stream when processing received RTP packets in their original order, the most straightforward de-packetization scheme is to re-order the RTP packets according to their sequence numbers and forward the RTP payload to the decoder.

The following additional de-packetization rules MAY be used to implement a more optimized operational JVT de-packetizer:

- Intelligent RTP receivers (e.g. in gateways) MAY identify lost NALUs of type "coded slice data partition A" (DPA). If a lost DPA is found, the associated coded slice data partition B (DPB) and coded slice data partition C (DPC) NALUs are meaningless to the decoder and MAY be discarded.  Gateways, for example, MAY decide not to forward the DPB and DPC NALUs in this case, to ease the network load.

- Receivers MAY discard all packets that have a value of nal_ref_idc equal to 0.  However, it is preferable to process those packets if possible, because the user experience may suffer if the packets are discarded.

# Appendix I – ASN.1 OIDs defined in this Recommendation

| OID | Section Reference |
| --- | --- |
| {itu-t(0) recommendation(0) h(8) 241(241) specificVideoCodecCapabilities(0) h264(0) iPpacketization(0) h241AnnexA(0)} | 7.1.4 |
| {itu-t(0) recommendation(0) h(8) 241(241) specificVideoCodecCapabilities(0) h264(0) generic-capabilities(1)} | 8.3.2.1 |

[end]