ITU Telecommunication Standardization Sector	Document Q1-C04r1

Study Group 16

Q.F,G,K,1-5/16 Rapporteur Meeting

San Jose, 30 August – 3 September 2004

Question(s):	1/16

Source*:	RADVISION

Title:		Audio and Video muting in H.324

Purpose:	Proposal

___________________

# Introduction

# The purpose of this recommendation is to clarify the proper procedure of muting audio and video channels in the context of 3G-324M and H.324. The information covered in this recommendation is targeted to the H.324 3G-324M implementer guide.

# According to H.245 recommendation, the proper behavior for muting audio or video stream is to use the Miscellaneous Indication messages logicalChannelActive and logicalChannelInactive. However, many 3G–324M and H.324 implementations do not support this procedure. Therefore, we recommend that H.324 and 3G-324M receivers   should be able to handle A/V muting also in absence of the associated Miscellaneous Indication messages

# This document is a result of a sizable set of interoperability tests in the context of IMTC 3G-324M Activity Group.

# Proposal

The following section should be added to the H.324 recommendation:

#### 6.5.2.1	Channel muting

Logical channels may be temporarily inactive. Such temporary inactivation (muting) should be indicated to the far-end terminal using the H.245 Miscellaneous Indication logicalChannelInactive.

The H.245 Miscellaneous Indication logicalChannelActive should be used to indicate when a normal signal has resumed on the logical channel. These indications are intended to be used to inform the human user that the far-end has muted or unmuted the channel.

Before sending the logicalChannelInactive message the transmitter should ensure that no data is sent on the logical channel.

However, regardless of whether or not logicalChannelInactive or logicalChannelActive messages are received, receivers shall decode the contents of the logical channel normally.

___________________