ITU Telecommunication Standardization Sector	Document Q1-C06r1 (AVD-2539)

Study Group 16

Q.F,G,K,1-5/16 Rapporteur Meeting

San Jose, 30 August – 3 September 2004

Question(s):	1/16, 3/16

Source*:	RADVISION

Title:		Call reset procedure in H.324

Purpose:	Proposal

___________________

# Introduction

This document proposes the use of an “H.324 Call Reset procedure” in order to reset an established call without disconnecting the physical connection. In other words, moving directly from Phase G (sub clause 7.7 in H.324 recommendation) to Phase D (sub clause 7.4 in H.324 recommendation) without losing the existing physical link.

The purpose of this proposal is mainly to clarify the procedure and extend it to a negotiation of capabilities using the H.245 capability exchange procedure. The H.324 Call Reset is actually defined in Phase G and Phase F in H.324 recommendation but very vaguely and implicitly. We prefer describing this procedure explicitly under these phases and also add a new Generic

Control Capability that indicates a support for this procedure.

H.245 already has a call reset indication in the endSessionCommand.

From tests made with today’s equipment, it seems that sending such information on the endSesisonCommand causes today’s equipment to either drop the call or hold (i.e – stop working properly).

Our goal is to allow a terminal to indicate support for such a reset procedure and to clarify the procedure itself in the standard.

# Proposal

## Terminology

Reset is the procedure of moving directly from Phase G (sub clause 7.7 in H.324 recommendation) to Phase D (sub clause 7.4 in H.324 recommendation) without losing the existing physical link.

## Call Reset Procedure Description

Phase F and Phase G in H.324 specification (sub clauses 7.6 and 7.7 respectively) describe the procedure of H.324 session ending. They specify an ASN.1 choice from within the H.245 EndSessionCommand message that is used to distinguish between disconnecting a session along with the physical connection and disconnecting a session without the physical connection. The choice member called “disconnect” is used to disconnect a session including the physical connection. We think that the “v34H324” choice member of gstnOptions or isdnOptions from within EndSessionCommand choice, should be used to tell the receiver of the message that the disconnection of the session should not include the physical connection.

.

The receiver of the message that agrees to the offer should send a EndSessionCommand message of it’s own with the “v34H324” choice. If the receiver rejects the offer, EndSessionCommand message should be sent with the “disconnect” choice.

The procedure described here shall occur only if both terminals have indicated the support in the ability to reset a call. This support is indicated as a GenericContorolCapability in the capabilityTable of TerminalCapabilitySet message. This capability should be defined in a new annex to H.324 or H.245 recommendations or under Phase G in the H.324 recommendation it self.

## Table 1 – Call Reset Capability Parameters

All other parameters of the GenericCapability shall not be used.

## H.324 recommendation changes

### 7.7.1	Reset of H.324 session

In Phase G, if both the terminal and the far end have the SessionResetCapability defined in Annex J in Capability.genericControlCapability, and the mode indicated in the  EndSessionCommand message is gstnOptions.v34H324, the terminal shall reset the H.324 session by moving immediately to Phase D without changing communication mode and without disconnecting the physical connection.

Annex J

ASN.1 OIDs defined in this Recommendation

Table J.1/H.324 – Summary of OIDs defined in H.324

| OID | Clause reference |
| --- | --- |
| { itu-t(0) recommendation(0) h(8) 324 generic-capabilities(1) SessionResetCapability(1) } | 7.7.1 |

Table J.2/H.324 – SessionResetCapability Capability Identifier

| Capability name | SessionResetCapability |
| --- | --- |
| Capability identifier type | Standard |
| Capability identifier value | { itu-t(0) recommendation(0) h(8) 324 generic-capabilities(1) SessionResetCapability(1) } |
| maxBitRate | This parameter is not used. |
| Collapsing | This field shall not be used and shall be ignored by receivers. |
| nonCollapsing | This field shall not be used and shall be ignored by receivers. |
| nonCollapsingRaw | This field shall not be used and shall be ignored by receivers. |
| Transport | This field shall not be used and shall be ignored by receivers. |

# Conclusion

This proposal provides a simple way to reset an H.324 / 3G – 324M call and gives devices the ability to connect two terminals without losing the already established physical connection.

___________________