ITU Telecommunication Standardization Sector	Document Q1-D06

Study Group 16

Q.1/16 Rapporteur Meeting	5 May 2005

Biel-Bienne, Switzerland 19-20 May 2005

| Source*: | Polycom |
| --- | --- |
| Title: | Proposed H.320 BAS codes for support of G.722.1 Annex C |
| Purpose: | Proposal |

_______________________________

# 1.  Introduction

Revised ITU-T Rec. G.722.1, including new Annex C, a 14 kHz audio bandwidth extension of the G.722.1 audio codec, was approved to go to AAP Last Call by WP3/16 in April 2005, and in the absence of Last Call comments will be formally approved on 14 May 2005.

Revised Annex A/G.722.1 contains the necessary signalling to support Annex C/G.722.1 in H.245-based systems.

This contribution proposes the BAS code signalling and H.221 multiplex allocation to support Annex C/G.722.1 in H.320 systems.  Recs. H.221 and H.242 are affected.

We propose these additions should be incorporated in revised Recommendations for Consent at the July/August 2005 meeting of SG16.

Underlined red text sections mark proposed additions.

# 2.  Change to 4.2.5/H.221

This change describes the bit allocation for Annex C/G.722.1 at the standardized rates of 24, 32, and 48 kbit/s.

[Begin Change]

…

ITU-T Rec. G.722.1 provides two bit rates, 24 kbit/s or 32 kbit/s, and uses a frame size of 20 ms.  This results in either 480 bits (60 octets) or 640 bits (80 octets) in any one frame respectively. The bit rate may be changed at any 20 ms audio frame boundary. Alignment of H.221 audio mode changes with a submultiframe boundary is required by 3.2/H.221. Figures 5h and 5i illustrate the bit allocation of the two G.722.1 frames for a bit rate of 32 kbit/s and 24 kbit/s respectively.

Bit allocation for Annex C/G.722.1 at 24 and 32 kbit/s shall be identical to bit allocation for G.722.1 at these rates.  Bit allocation for Annex C/G.722.1 at 48 kbit/s shall be identical to bit allocation for G.722 at 48 kbit/s.

…

[End Change]

# 3.  Changes to Annex A/H.221

These changes supply capabilities and commands for Annex C/G.722.1.

[Begin Change]

Annex A

Definitions and tables of BAS values

The definitions of BAS values are given in this annex, and the corresponding numerical values are listed in Tables A.1 and A.2. In these tables, the column header gives the attribute designation as bits (b0, b1, b2); the left-hand column gives the decimal value of bits [b3, b4, b5, b6, b7]; for example, "Dig-loop" has the value (010) [10100]. All unassigned values are reserved, as are values marked (R).

| Table A.1/H.221 – BAS numerical values |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (000) | (001) | (010) | (011) | (100) | (101) | (110) | (111) |
| [0] | neutrala) | 64k | Video-off | LSD-off | neutral | var-LSD | Restrict_L | class (R) |
| [1] | capex | 2  64k | H.261-on | LSD_300 | A-law | LSD_300 | Restrict_P | class (R) |
| [2] | (R) | 3  64k | H.263-on | LSD_1200 | µ-law | LSD_1200 | NoRestrict | class (R) |
| [3] | (R) | 4  64k | video-MPEG-1-on | LSD_4800 | G.722-64 | LSD_4800 | G.723.1b) | class (R) |
| [4] | A-law, 0U | 5  64k | H.264-on | LSD_6400 | G.722-48 | LSD_6400 | G.729 | class (R) |
| [5] | -law, 0U | 6  64k | MLP-8k | LSD_8000 | G.728 | LSD_8000 | G.722.1-32 (cap) | class (R) |
| [6] | G.722, m1a) | 384k | encryp-on | LSD_9600 | (R) | LSD_9600 | G.722.1-24 (cap) | class (R) |
| [7] | Au-off, Ua) | 2  384k | encryp-off | LSD_14.4k | SM-comp | LSD_14.4k | G.722.1 Annex C-48 (cap) | class (R) |
| [8] | (R) | 3  384k | H.262S-on | LSD_16k | 128k | LSD_16k | G.722.1 Annex C-32 (cap) | family (R) |
| [9] | (R) | 4  384k | H.262M-on | LSD_24k | 192k | LSD_24k | G.722.1 Annex C-24 (cap) | family (R) |
| [10] | G.723.1 | 5  384k | DOP | LSD_32k | 256k | LSD_32k | (R) | family (R) |
| [11] | G.729 | 1536k | DCP | LSD_40k | 320k | LSD_40k | (R) | family (R) |
| [12] | (R) G-4k | 1920k | DOIP | LSD_48k | 512k | LSD_48k | (R) | family (R) |
| [13] | G.722.1 Annex C-48 | 128k | DCIP | LSD_56k | 768k | LSD_56k | (R) | family (R) |
| [14] | G.722.1 Annex C-32 | 192k | PRAO | LSD_62.4k | Null | LSD_62.4k | (R) | family (R) |
| [15] | G.722.1 Annex C-24 | 256k | PRAC | LSD_64k | 1152k | LSD_64k | (R) | Table_A.6 |
| [16] | (R) | 320k | freeze-pic | MLP-off | 1B | MLP-4k | (R) | Table_A.2 |
| [17] | (R) | loss i.c. | Fast-update | MLP-4k | 2B | MLP-6.4k | (R) | H.230 |
| [18] | A-law, 0Fa) | (R) | Au-loop | MLP-6.4k | 3B | var-MLP | (R) | Table_A.4 |
| [19] | -law, 0Fa) | (R) | Vid-loop | var-MLP | 4B | MLP_Set 1 | (R) | SBE numbers |
| [20] | A-law, F6a) | (R) | Dig-loop | MLP-14.4k | 5B | H.261-QCIF | (R) | SBE characters |
| [21] | -law, F6a) | (R) | Loop-off | MLP-22.4k | 6B | H.261-CIF | (R) | SBE (R) |
| [22] | (R) | (R) | (R) | MLP-30.4k | Restrict_required | 1/29.97 | (R) | SBE (R) |
| [23] | (R) | 512k | SM-comp | MLP-38.4k | 6B-H0-comp | 2/29.97 | (R) | SBE (R) |
| [24] | G.722, m2a) | 768k | not-SM-comp | MLP-46.4k | H0 | 3/29.97 | (R) | cap-mark |
| [25] | G.722, m3a) | (R) | 6B-H0-comp | MLP-16k | 2H0 | 4/29.97 | (R) | start-MBE |
| [26] | Au-40k (R) | 1152k | not-6B-H0-comp | MLP-24k | 3H0 | H.263(2000) | (R) | (R) |
| [27] | G.722.1-32 | (R) | Restrict | MLP-32k | 4H0 | video-MPEG-1 | (R) | (R) |
| [28] | G.722.1-24 | (R) | derestrict | MLP-40k | 5H0 | MLP_Set2 | (R) | (R) |
| [29] | G.728a) | 1472k | (R) | MLP-62.4k | 1472k | esc-CF (R) | (R) | (R) |
| [30] | (R) | (R) | (R) | MLP-64k | H11 | encryp. | (R) | ns-cap |
| [31] | Au-off, Fa) | (R) | (R) | var-LSD | H12 | MBE-cap | (R) | ns-comm |
| a) Use of these codes in the 56 kbit/s environments is defined in Annex B. b) Use of H.223 AL2 CRC is required as specified in 4.2. |  |  |  |  |  |  |  |  |

…

## A.1	Audio command values (000)

For audio bit position illustrations, see clause 4. Abbreviations "G.711", "G.722" and so on refer to Recommendations.

Neutral	Neutralized I-channel, containing only FAS and BAS; all other bits are to be ignored at the receiver.

Capex	Transmitted by a Channel Aggregation Unit (see ITU-T Rec. H.244).

Au-off, U	Switches off G.711/722/728 audio (but not Au-ISO as in Table A.2) and switches off the frame structure in the I-channel; all the I-channel is available for use under commands other than (000)[n]Error: Reference source not found, .

Au-off, F	Switches off G.711/722/728 audio (but not Au-ISO as in Table A.2); FAS and BAS in use (mode 9); 62.4 kbit/s in the I-channel available for use under commands other than (000)[n].

A-law, 0U	G.711 audio at 64 kbit/s, A-law, no framing (Mode 0U)Error: Reference source not found.

A-law, 0F	G.711 audio at 56 kbit/s, A-law, truncated to 7 bits in bits 1-7, with FAS and BAS in bit 8; bit 8 is set to zero at the PCM audio decoder (Mode 0F).

-law, 0U	G.711 audio at 64 kbit/s, -law, no framing (Mode 0U)3.

-law, 0F	G.711 audio at 56 kbit/s, -law, truncated to 7 bits in bits 1-7, with FAS and BAS in bit 8; bit 8 is set to zero at the PCM audio decoder (Mode 0F).

A-law, F6	Audio according to ITU-T Rec. G.711 at 48 kbit/s, A-law truncated to 6 bits, with FAS and BAS in bit 8 (use only according to 13.4/H.242).

-law, F6	Audio according to ITU-T Rec. G.711 at 48 kbit/s, -law truncated to 6 bits, with FAS and BAS in bit 8 (use only according to 13.4/H.242).

G.722, m1	G.722 7 kHz audio at 64 kbit/s, no framing (mode 1)Error: Reference source not found.

G.722, m2	G.722 7 kHz audio at 56 kbit/s, in bits 1-7 (mode 2).

G.722, m3	G.722 7 kHz audio at 48 kbit/s, in bits 1-6 (mode 3).

Au-40k	Reserved for audio at less than 48 kbit/s (for example, 40 kbit/s in bits 1-5).

G.722.1-32	G.722.1 7 kHz audio at 32 kbit/s, in bits 1-4.

G.722.1-24	G.722.1 7 kHz audio at 24 kbit/s, in bits 1-3.

G.722.1 Annex C-48	G.722.1 Annex C 14 kHz audio at 48 kbit/s, in bits 1-6.

G.722.1 Annex C-32	G.722.1 Annex C 14 kHz audio at 32 kbit/s, in bits 1-4.

G.722.1 Annex C-24	G.722.1 Annex C 14 kHz audio at 24 kbit/s, in bits 1-3.

G.728	Audio at 16 kbit/s to ITU-T Rec. G.728 in bits 1 and 2 according to clause 4 (mode 7).

G.729	Audio at 8 kbit/s to ITU-T Rec. G.729 according to clause 4 (mode 8a).

G.723.1	Audio at <7 kbit/s to ITU-T Rec. G.723.1 according to clause 4 (mode 8b).

Au-4k	Reserved for audio at less than 5 kbit/s in bit 1.

…

## A.5	Audio capabilities (100)

Neutral	Neutral capability: no change in the current capabilities of the terminal.

A-law	Capable of decoding audio to ITU-T Rec. G.711, A-law.

-law	Capable of decoding audio to ITU-T Rec. G.711, -law.

G.722-64	Capable of decoding audio to ITU-T Rec. G.722 (mode 1) and to ITU-T Rec. G.711.

G.722-48	Capable of decoding audio to ITU-T Rec. G.722 (modes 1, 2, 3) and to ITU-T Rec. G.711.

G.722.1-32 (cap)	Capable of decoding audio to ITU-T Rec. G.722.1 at 32 kbit/s and to ITU-T Rec. G.711.

G.722.1-24 (cap)	Capable of decoding audio to ITU-T Rec. G.722.1 at 24 kbit/s and to ITU-T Rec. G.711.

G.722.1 Annex C-48 (cap)	Capable of decoding audio to ITU-T Rec. G.722.1 Annex C at 48 kbit/s and to ITU-T Rec. G.711.

G.722.1 Annex C-32 (cap)	Capable of decoding audio to ITU-T Rec. G.722.1 Annex C at 32 kbit/s and to ITU-T Rec. G.711.

G.722.1 Annex C-24 (cap)	Capable of decoding audio to ITU-T Rec. G.722.1 Annex C at 24 kbit/s and to ITU-T Rec. G.711.

G.728	Capable of decoding audio, both to ITU-T Recs G.728 and G.711.

G.723.1	Capable of decoding audio, both to ITU-T Recs G.723.1 and G.711.

G.729	Capable of decoding audio, both to ITU-T Recs G.729 (including Annex A) and ITU-T Rec. G.711.

Null	Capability having no significance other than as a filler.

NOTE – This value may occur any number of times within a capability set transmitted towards a Single-Channel Equipment – see ITU-T Rec. H.244 (Channel Aggregation).

[End Change]

# 4.  Changes to Table 52/H.242

.

[Begin Change]

Table 52/H.242 – BAS capabilities that can be included in a valid capability set (Note 1)

| Audio | Absent or One or more values (Note 2) from A-law, -law, G.722-48, G.722-64, G.722.1-24, G.722.1-32, G.722.1 Annex C-48, G.722.1 Annex C-32, G.722.1 Annex C-24, G.723.1, G.728, G.729 |
| --- | --- |

[End Change]

# 5.  Changes to Appendix VI/H.242

.

[Begin Change]

Appendix VI

Hierarchical capability BAS codes

The following capability codes are hierarchically structured:

G.711 (A or  or both) < G.722-64 < G.722-48

G.711 (A or  or both) < G.722.1-24

G.711 (A or  or both) < G.722.1-32

G.711 (A or  or both) < G.722.1 Annex C-48

G.711 (A or  or both) < G.722.1 Annex C-32

G.711 (A or  or both) < G.722.1 Annex C-24

G.711 (A or  or both) < G.723.1

G.711 (A or  or both) < G.728

G.711 (A or  or both) < G.729

1B < 2B < 3B < 4B < 5B < 6B

1H0 < 2H0 < 3H0 < 4H0 < 5H0

H.261-QCIF < H.261-CIF

4/29.97 < 3/29.97 < 2/29.97 < 1/29.97

[End Change]

___________________________

* Contact:	Dave Lindbergh	Phone: +1 978 292 5366

E-mail: lindbergh@92F1.com

It is interpreted as a command to shut off all the output of the I-channel demultiplexer except FAS, BAS and ECS (if relevant). Audio is muted accordingly. Release of this shut off is activated by a fixed rate command (namely by a command other than Var-LSD, Var-MLP). Channels other than I-channel (such as additional channel for 2B communications, or the 2nd through 6th time-slot for H0 communications) remain unchanged.

If video or HSD was set on before this Neutral BAS command is issued, it continues to be on. For example, if video has been on in a 2B communication, and Neutral BAS command is issued, the video is transmitted only in the additional channel. If a fixed rate command for I-channel is then issued, the video also occupies all bit positions of I-channel other than those designated by the fixed rate command, and FAS and BAS positions. In case of 1B communication, video is completely excluded by this Neutral BAS command, but it will recover by, for example, the next 16 kbit/s audio command.

It is noted that no procedures for the use of neutral BAS command have been adopted.

These attribute values designate unframed modes. In the receive direction, reverting to a framed mode can only be achieved by recovering frame and multiframe alignment which might take up to two multiframes (320 ms).