ITU Telecommunication Standardization Sector	Document Q1-D13

Study Group 16

Q.1/16 Rapporteur Meeting

Biel-Bienne, Switzerland 19-20 May 2005

| Source*: | RADVISION |
| --- | --- |
| Title: | Capability Preferences – Suggested behavior |
| Purpose: | Proposal |

___________________

# Introduction

This document proposes to add clarifications to H.324 recommendation which would minimize the probability of occurrence of the problem specified in the “Capability Preferences” proposal.

# Discussion Section(s)

The proposed clarifications would require some additions to H.324 recommendation as follows:

### 6.5.6	Preferred media type

The selection of preferred media type as described in H.245 recommendation section C.4.1.3 or C.5.1.3 is optional, however highly recommended due to high odds for a logical channel conflict. The master which experiences such a conflict shall reject the channel with a masterSlaveConflict reason. The slave which receives the OpenLogicalChannelReject message with the masterSlaveConflict reason should retry to open logical channel with the most preferred media type by the master.

# Conclusion

This document proposes a solution to the media type conflict problem as described in the “Capability Preferences” document. This solution is highly recommended to ensure that a video session will be opened in cases where the media type does not fit both session participants.

___________________

*