ITU Telecommunication Standardization Sector	Document Q1-D13r1

Study Group 16

Q.1/16 Rapporteur Meeting

Biel-Bienne, Switzerland 19-20 May 2005

| Source*: | RADVISION |
| --- | --- |
| Title: | Capability Preferences – Suggested behavior |
| Purpose: | Proposal |

___________________

# Introduction

This document proposes to add clarifications to H.324 recommendation which would minimize the probability of occurrence of the problem specified in the “Capability Preferences” proposal.

# Discussion Section(s)

The proposed clarifications would require some additions to H.324 recommendation as follows:

### 6.5.6  Resolution of Logical Channel Conflicts

In the event of a conflict when requests to open logical channels are initiated at the same time, H.324 terminals should follow the optional recommended procedure in section C.4.1.3/H.245 or C.5.1.3/H.245 as appropriate.

The master which experiences such a conflict shall reject the channel with a masterSlaveConflict reason. The slave which receives the OpenLogicalChannelReject message with the masterSlaveConflict reason should retry to open logical channel with the media type most preferred by the master terminal, unless the master terminal has already opened a logical channel which suits the purpose intended by the slave terminal.

# Conclusion

This document proposes a solution to the media type conflict problem as described in the “Capability Preferences” document. This solution is highly recommended to ensure that a video session will be opened in cases where the media type does not fit both session participants.

___________________

*