| Question(s): | 1 |  | Meeting, date: |  | Geneva, 26 July-5 August 2005 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Study Group: | 16 | Working Party: | 2 | Intended type of document (R-C-D-TD): |  | TD |
| Source: | Editor (P. Luthi) |  |  |  |  |  |
| Title: | Draft new H.242 Amendment 1 “System for establishing communication between audiovisual terminals using digital channels up to 2 Mbit/s” (for Consent) |  |  |  |  |  |
| Contact: | Patrick Luthi Tandberg Norway |  |  |  | Tel: +47 67 125 125 Fax: +47 67 125 234 Email: patrick.luthi@tandberg.net |  |
| Contact: |  |  |  |  | Tel: Fax: Email: |  |
| Please don’t change the structure of this table, just insert the necessary information. |  |  |  |  |  |  |

# Introduction

This document is a new Amendment 1 to H.242. Changes are made against the 03/04 revision of H.242.

The changes were approved at the Biel/Bienne meeting of Q.1 and are summarised below.

- Changes to Table 52/H.242

- Changes to Appendix VI/H.242

# AAP Summary

Videophone, videoconferencing and other conversational multimedia services on fixed bit-rate networks (such as ISDN) depend on the multiplex structure described in ITU-T Rec. H.221. The control channel within this multiplex is the so-called BAS position in each frame. This Recommendation describes all the point-to-point procedures involving the BAS codes: initial capability exchange and mode initialization, including restricted-network operation; in-session mode switching, including audiovisual mode changes and activation of data channels; other procedures, including single/multiple channel interconnection, encryption, mode-preference indication, conveyance of network addresses and use of BAS extension codes.

This Amendment 1 adds the BAS code signalling and H.221 multiplex allocation to support Annex C/G.722.1 (14 kHz audio bandwidth extension of the G.722.1) in H.320 systems. These changes are associated with the changes shown in Amendment 1/H.221.

# 1.  Changes to Table 52/H.242

Table 52/H.242 is updated as follows:

[Begin Change]

Table 52/H.242 – BAS capabilities that can be included in a valid capability set (Note 1)

| Audio | Absent or One or more values (Note 2) from A-law, -law, G.722-48, G.722-64, G.722.1-24, G.722.1-32, G.722.1 Annex C-48, G.722.1 Annex C-32, G.722.1 Annex C-24, G.723.1, G.728, G.729 |
| --- | --- |

[End Change]

# 2.  Changes to Appendix VI/H.242

Appendix VI/H.242 is updated as follows:

[Begin Change]

Appendix VI

Hierarchical capability BAS codes

The following capability codes are hierarchically structured:

G.711 (A or  or both) < G.722-64 < G.722-48

G.711 (A or  or both) < G.722.1-24

G.711 (A or  or both) < G.722.1-32

G.711 (A or  or both) < G.722.1 Annex C-48

G.711 (A or  or both) < G.722.1 Annex C-32

G.711 (A or  or both) < G.722.1 Annex C-24

G.711 (A or  or both) < G.723.1

G.711 (A or  or both) < G.728

G.711 (A or  or both) < G.729

1B < 2B < 3B < 4B < 5B < 6B

1H0 < 2H0 < 3H0 < 4H0 < 5H0

H.261-QCIF < H.261-CIF

4/29.97 < 3/29.97 < 2/29.97 < 1/29.97

[End Change]

___________________________