ITU Telecommunication Standardization Sector	Document Q1-E09r1

Study Group 16

Q.1/16 Rapporteur Meeting	30 Nov. 2005

Geneva, Switzerland 30 November - 2 December 2005

| Source*: | Tandberg |
| --- | --- |
| Title: | Proposed changes to H.241 to support an Additional Computationally Efficient Mode (ACEM) in H.32x systems |
| Purpose: | Proposal |

_______________________________

# 1. Background

In the Q6 meeting in Geneva July 26th to August 5th it was decided to define an additional  computationally efficient mode (ACEM) to H.264 by using H.241.  This work should be done in Q1.  The ACEM solution should consist of:

- Restricting the motion block size to 8x8 or larger.

- Simplified fractional pixel interpolation as already presented in Q6

- Simplified deblocking filter

- Since the other CE tools that had been proposed to Q6 were not to be included, it was mentioned that the deblocking filter could probably be even more simplified to make up for some of complexity reduction that could otherwise have been obtained.

Hence we made some further tests to reduce the deblocking filter complexity a little more.  During that work we also discovered that the filter definition was recursive so that filtering for instance some pixels in a line might depend on all previously filtered pixel on that line.  This was considered to be undesirable, and we made a fix so that there is a brake in recursion near all macroblock edges.  Hence the present definitions consist of the following:

- Simplified fractional pixel interpolation as already presented in Q6

- Simplified deblocking filter with the following small changes to definitions already presented to Q6:

  - Use one decision for every 8 edge lines instead of every 4 edge lines whether to filter or not.  This reduces the overall complexity by about 2%

  - Make a small change in filter definition near macroblock edges to break recursion (see above)

  - Concerning filtering due to motion vector differences, revert from difference of “8 in units of quarter pixels” to “4 in units of quarter pixels” which is used in H.264.

The total results of these minor changes are the 2% reduction in complexity and no measurable change in objective or subjective quality.

# 2. Signaling

Signaling of ACEM needs to be defined in H.241. We propose to do this by defining a new capability parameter, AdditionalModesSupported. Details are given in document Q1-E07, Section 8.3.2.12 in particular.

# 3. New H.241 Annex B

The following text contains the proposed new H.241 Annex B.

Annex B

Additional Computationally Efficient Mode

(Geneva, 2006)

B.1 Scope

This annex describes an Additional Computationally Efficient Mode (ACEM) of Rec. H.264 as a set of modifications to some of the operations described in H.264. The modifications are defined in the following sections.  In this process reference is made to H.264. For variables and functions that are not defined in this annex, the definitions used in H.264 shall be inferred. The modifications in this annex apply only to bitstreams that obey the constraints described in Section A.2.1/H.264 (Baseline Profile). For the purposes of Annex A/H.264, Level requirements are the same in this mode as for the H.264 Baseline Profile.

B.2 Bitstream Conformance

Bitstream conformance with the ACEM shall be identified in the H.264 bitstream as follows:

- By sending a Sequence Parameter Set  with profile_idc=66.

- By sending a User data unregistered SEI message according to D.1.6/H.264 containing the values given in Table B.1 below.

Table B.1/H.241 - User data unregistered SEI message indicating bitstream conformance with this Annex

| UUID value: | xxxxxxxx-xxxx-xxxx-xxxx-xxxxxxxxxxxx [DEFINE IN GENEVA] |
| --- | --- |
| Payload bytes: | Exactly one payload byte shall follow. This shall contain the AdditionalModesSupported booleanArray according to Table 9f/H.241. Bit 1 (value 128) of this BooleanArray shall be set. |

The User data unregistered SEI message shall be sent immediately after the Sequence Parameter Set. Sequence Parameter Sets that are immediately followed by a User data unregistered SEI message as specified in Table B1/H.241 shall only be referred to by bitstreams that conforms to this Annex. If a User data unregistered SEI message according to Table B1/H.241 is received that does not follow immediately after a Sequence Parameter Set, the value of bit 1 in the BooleanArray shall be ignored.

# B.3 Limiting motion compensated prediction block size downward to 8x8

The parameter sub_mb_type as defined in Table 7-17/H.264 shall be equal to 0 for all 8x8 blocks in a macroblock.

# B.4 Use of a different subpixel interpolation scheme

The operations defined in section 8.4.2.2/H.264 “Fractional sample interpolation process” shall be performed with the changes defined in this section.  The changes are:

1) In section 8.4.2.2:

Add frame_num to the list of inputs to the process.

2) In section 8.4.2.2:

Add “(xFracc,yFracc) is in quarter sample units instead of one-eighth sample units stated in 8.4.2.2.”

3) In section 8.4.2.2:

Replace the sentence:

“Let ( xIntC, yIntC ) be a chroma location given in full-sample units and ( xFracC, yFracC ) be an offset given in one-eighth sample units”

with:

“Let ( xIntC, yIntC ) be a chroma location given in full-sample units and ( xFracC, yFracC ) be an offset given in one-quarter sample units”.

4) In section 8.4.2.2:

Equations 8-224 through 8-227 shall be replaced by:

xIntC = ( xAL / SubWidthC ) + ((mvCLX[ 0 ]+(frame_num & 1)) >> 3 ) + xC		(8-224)
yIntC = ( yAL / SubHeightC ) + ((mvCLX[ 1 ]+(frame_num & 1)) >> 3 ) + yC	(8-225)

xFracC = ((mvCLX[ 0 ]+(frame_num & 1)) >>1) & 3	(8-226)
yFracC = ((mvCLX[ 1 ]+(frame_num & 1)) >>1) & 3	(8-227)

5) In section 8.4.2.2.1:

Replace the sentence:

“The luma prediction values at half sample positions shall be derived by applying a 6-tap filter with tap values ( 1, -5, 20, 20, -5, 1 )”

with:

“The luma prediction values at half sample positions b and s shall be derived by applying a 6-tap filter with tap values ( 1, -5, 20, 20, -5, 1 ). The luma prediction values at half sample positions h and m shall be derived by applying a 4-tap filter with tap values ( -1, 5, 5, -1 )”.

6) In section 8.4.2.2.1:

Equations 8-237, 8-238, 8-241 and 8-242 to define b1, h1 and j1 shall not be used since the intermediate values are not used for calculating any of the fractional pixel values.

7) In section 8.4.2.2.1:

Equations 8-239, 8-240, 8-243,  8-244, 8-245, 8-250, 8-251, 8-252 and 8-252 shall be replaced by:

b = Clip1Y( (E – 5 * F + 20 * G + 20 * H – 5* I + J  + 6 ) >> 5 ) 	(8-239)
h = Clip1Y( (– C + 5 * G + 5 * M – R  + 1 ) >> 3 ) 	(8-240)
j = ( H + M ) >> 1) 		(8-243)
s  = Clip1Y( (K – 5 * L + 20 * M + 20 * N – 5* P + Q  + 6 ) >> 5 ) 	(8-244)
m = Clip1Y( (– D + 5 * H + 5 * N – S  + 1 ) >> 3 ) 	(8-245)
f = ( G + m + 1 ) >> 1		(8-250)
i = ( M + b + 1 ) >> 1		(8-251)
k = ( H + s + 1 ) >> 1		(8-252)
q = ( N + h + 1 ) >> 1		(8-253)

8) In section 8.4.2.2.2:

Figure B.1 depicts the position of integer and fractional chroma pixel positions.

A

a

b

c

B

d

e

f

g

h

i

j

k

m

n

p

q

r

C

s

D

Figure B.1/H.241 – Integer samples (shaded blocks with upper-case letters) and fractional sample positions (un-shaded blocks with lower-case letters) for quarter sample chroma interpolation

For further definition of integer and fractional chroma pixel positions, Table 8-12 shall also apply for “Assignment of chroma prediction sample predPartLXC[xC,yC]” by replacing subscript L by subscript C , and by replacing G with A.

To define predPartLXC[ xC, yC ] replace equation 8-266 with the expressions:

b = ( A + B ) >> 1		
h = ( A + C ) >> 1
j = ( B+ C ) >> 1
m = ( B +  D ) >> 1
s = ( C + D ) >> 1

a = ( A + b + 1 ) >> 1
c = ( B + b + 1 ) >> 1
d = ( A + h + 1 ) >> 1
e = ( b + h + 1 ) >> 1
f = ( A + m + 1 ) >> 1
g = ( b + m + 1 ) >> 1
i = ( C + b + 1 ) >> 1
k = ( B + s + 1 ) >> 1
n = ( B + h + 1 ) >> 1
p = ( h + s + 1 ) >> 1
q = ( D + h + 1 ) >> 1
r = ( m + s + 1 ) >> 1

# B.5 Use of a different deblocking filter

Section 8.7/H.264 “Deblocking filter process” shall be replaced by sections B.5.1 and B.5.2 in this annex, except for the definitions of indexA and indexB where Eqs. 8-462 and 8-463 shall be retained.

## B.5.1 Deblocking filter process

A conditional filtering shall be applied to all 4x4 block edges of a picture, except edges at the boundary of the picture and any edges for which the deblocking filter process is disabled by disable_deblocking_filter_idc, as specified below. This filtering process shall be performed on a macroblock basis after completion of the picture construction process prior to the deblocking filter process (as specified in subclauses Error: Reference source not found/H.264 and Error: Reference source not found/H.264) for the entire decoded picture, with all macroblocks in a picture processed in order of increasing macroblock addresses.

The deblocking filter process shall be invoked for the luma and chroma components separately. For each macroblock and each component, vertical edges shall be filtered first, starting with the edge on the left-hand side of the macroblock proceeding through the edges toward the right-hand side of the macroblock in their geometrical order.  Then horizontal edges shall be filtered, starting with the edge on the top of the macroblock proceeding through the edges toward the bottom of the macroblock in their geometrical order.

Macroblocks of type I_PCM shall be treated as Intra macroblocks with qPz = 0. This is in accordance with deblocking filtering in H.264.

Table B.2 - Value of tC and  as a function of index

| index | 0-15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tC | 0 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 |
|  | 0 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 20 | 22 | 24 | 26 | 28 |

To obtain tC use index = index A. To obtain  use index = index B.

Table B.2 (concluded) - Value of tC and  as a function of index

| index | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tC | 3 | 4 | 4 | 4 | 5 | 5 | 6 | 6 | 7 | 8 | 9 | 9 | 11 | 12 | 13 | 13 | 16 | 18 |
|  | 30 | 32 | 34 | 36 | 38 | 40 | 42 | 44 | 46 | 48 | 50 | 52 | 54 | 56 | 58 | 60 | 62 | 64 |

To obtain tC use index = index A. To obtain  use index = index B.

### B.5.2 Filtering process for luma pixels

Filtering of luma pixels shall take place within units of 8x8 luma pixels.  Figure B.2 illustrates how the 8x8 blocks are organized when filtering both across vertical and horizontal edges.

(a)	 (b)

Figure B.2/H.241 – Arrangement of 8x8 blocks
(a) when filtering across vertical edges 
(b) when filtering across horizontal edges

Figure B.2 shall be interpreted as follows:

- A, B, C and D are 4x4 blocks

- pni and qni, n=0,3 and i=0,7 are pixels within the 8x8 block

- The bold line indicates a horizontal or vertical block edge across which deblocking filtering takes place.

- The 8x8 blocks are arranged such that the pixels (p30 to q30 ) or the pixels (p37 to q37 ) are at a macroblock boundary

The parameter d, defined below, shall be used for deciding whether the pixels within an 8x8 block shall be modified or not:

- If the block edge is part of a macroblock edge:

d =  p12 – p02  +  q02 – 2*q12 + q22  +  p15 – p05  +  q05 – 2*q15 + q25 

- Otherwise:

d =  p22 – 2*p12 + p02  +  q02 – 2*q12 + q22  +  p25 – 2*p15 + p05  +  q05 – 2*q15 + q25 

If any of the following conditions are true, no filtering across a block edge shall take place.

- Disable_deblocking_filter_idc for the slice containing block D is equal to 1

- Disable_deblocking_filter_idc for the slice containing block D is equal to 2 and block B and D belong to different slices

- d >= 

Otherwise filtering shall take place if and only if any of the following conditions are true:

- Block A is intra coded or Block C is intra coded

- One or more of the blocks A,B,C, and D contains nonzero transform coefficients.

- The absolute difference between the horizontal or vertical component of the motion vectors used for block A and block C is greater than or equal to 4 in units of quarter luma frame samples.

- Block A and block C are predicted from different reference frames,

For block edges where filtering is performed, all values pni, qni , with n=0,1 and i=0,7, shall be modified in the following way:

If the block edge is not part of a macroblock edge:

 = Clip3(-tC,tC,((q0i +( (p2i + q1i) >> 1)) >> 1) –( (p0i +( (q2i + p1i) >> 1)) >> 1))

Otherwise:

 = Clip3(-tC,tC,((q0i + (q1i >> 1)) >> 1) – ((p0i + (q2i >> 1)) >> 1))

The filtered samples p1i, p0i, q0i, q1i shall be derived by:

p0i = Clip1Y(p0i + )

q0i = Clip1Y(q0i - )

p1i = Clip1Y(p1i + /2)

q1i = Clip1Y(q1i - /2)

### B.5.3 Filtering process for chroma pixels

The chroma filtering process shall take place across an edge between two 4x4 chroma blocks.  Figure B.3 illustrates how two such adjacent blocks are arranged when filtering both across vertical and horizontal edges.

(a)	(b)

Figure B.3/H.241 - Arrangement of 2 4x4 chroma blocks
(a) when filtering across a vertical edge
(b) when filtering across a horizontal edge

- A and B are 4x4.

- pni and qni, n=0,3 and i=0,3 are pixels within the two 4x4 blocks.

- The bold line indicates a horizontal or vertical block edge across which deblocking filtering takes place.

If any of the following conditions are true, no filtering across a block edge shall take place.

- Disable_deblocking_filter_idc for the slice containing block B is equal to 1.

- Disable_deblocking_filter_idc for the slice containing block B is equal to 2 and block A and B belong to different slices.

Otherwise, filtering shall take place if and only if the following condition is  true

- Block A is intra coded or block B is intra coded

For block edges where filtering takes place, all values p0i, q0i with i=0,3 shall be modified in the following way:

 = Clip3(-tC,tC,((((q0i - p0i) << 2) + p1i - q1i + 4)  >> 3))

The filtered result samples  p0i, q0i shall be derived by

p0i = Clip1C(p0i + )

q0i = Clip1C(q0i - )

___________________________

* Contact:	Arild Fuldseth	Phone: +47 67 12 60 08

E-mail: arild.fuldseth@tandberg.net