ITU Telecommunication Standardization Sector	Document Q1-F04

Study Group 16

Q.1,2,3,4,5,21,22,24,25, and 29/16 Rapporteur Meeting

Ottawa, 28 August - 1 September 2006

Question(s):		1

Source*:		AHG Chair

Title:			Report of the H.324 text conversation Ad-hoc Group

Purpose:		Report

1	Introduction

Work in the AHG proceeded by email during the interim period since the last SG16 meeting.

2	AHG discussions

In July,  KTF (Korea) reported that they have discovered, in the process of deploying new H.324/M handsets with a proprietary text conversation feature, that some already existing H.324/M handsets appear to drop video calls when the new H.324/M handset expresses the new and previously unseen capabilities in their terminal capability set.

Specifically, at least one existing H.324/M handset dropped the video called when the new KTF handset expressed a H.245 terminalCapabilitySet message containing the following:

…

{      capabilityTableEntryNumber 4,
      capability receiveUserInputCapability : nonStandard : {
        {
          nonStandardIdentifier h221NonStandard : {
            t35CountryCode 97,
            t35Extension 22,
            manufacturerCode 32768
          },
          data ''H
        }
      }
    },
    {
      capabilityTableEntryNumber 5,
      capability transmitUserInputCapability : nonStandard : {
        {
          nonStandardIdentifier h221NonStandard : {
            t35CountryCode 97,
            t35Extension 22,
            manufacturerCode 32768
          },
          data ''H
        }
      }
    }
  },

…

KTF has addressed this problem by removing these capabilities from their handset and identifying handsets capable of supporting their proprietary text telephony feature via use of the VendorIdentification message.

However, KTF noted that this problem may also affect a standardized GenericCapability defined by ITU for a standardized text conversation feature, and suggests that SG16 take this into consideration for the work on this topic.

3	Editorial status

The draft for H.324 text conversation has not been updated sing the SG16 meeting in April 2006.

_________________