| Question(s): | 1 |  | Meeting, date: |  | Geneva, 14-24 November 2006 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Study Group: | 16 | Working Party: | 2 | Intended type of document (R-C-D-TD): |  | TD |
| Source: | Editor (P. Luthi) |  |  |  |  |  |
| Title: | Proposed draft revised T.120-series Implementors’ Guide |  |  |  |  |  |
| Contact: | Patrick Luthi Tandberg Norway |  |  |  | Tel: +47 67 125 125 Fax: +47 67 125 234 Email: patrick.luthi@tandberg.net |  |
| Contact: |  |  |  |  | Tel: Fax: Email: |  |
| Please don’t change the structure of this table, just insert the necessary information. |  |  |  |  |  |  |

# Introduction

This document is a text revision of the Implementor’s Guide for T.120-series recommendations (version 11), last approved in August 2005.

The changes from the previous published version that appeared in sections 7.1, 7.5 and 7.6 of the IG have been removed since they will be incorporated respectively in the 2007 versions of T.120, T.123 and T.124. This Implementer’s Guide supersedes the ITU-T T.120 Recommendation series Implementor’s Guide approved in August 2005.

This draft is proposed to be approved at the conclusion of this meeting of SG16.

|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | T.120 Implementors' Guide |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (24 November 2006) |
|  | SERIES T: TERMINALS FOR TELEMATIC SERVICES |  |  |  |
|  | Implementors' Guide for ITU-T Rec. T.120 and T.130 sub-series: Data protocols for multimedia conferencing |  |  |  |

Summary

This document is a compilation of reported defects identified with the 1993-1999 editions of the ITU-T T.120-series Recommendations. It is intended to be read in conjunction with the Recommendations to serve as an additional authoritative source of information for implementors. The changes, clarifications and corrections defined herein are expected to be included in future versions of affected T.120-series Recommendations.

This revision contains all updates submitted up to and including those at Study Group 16 meeting, November 2006, in Geneva. This Implementors’ Guide supersedes the ITU-T T.120 Recommendations series Implementors’ Guide approved in August 2005.

Document History

| Version | Date | Description |
| --- | --- | --- |
| 1 | 15 February 1996 | Initial version - completed at the ITU-T Study Group 8 meeting, Geneva, 6-15 February 1996 |
| 2 | 26 April 1996 | Updated at the Q.10/8 Rapporteur's meeting, Boston, 21-26 April 1996 Sections added: 7.5.4 and 7.5.5 |
| 3 | 9 August 1996 | Updated at the Q.10/8 Rapporteur's meeting, Santa Rosa, 5-9 August 1996 Sections added: 7.5.6 |
| 4 | 4 October 1996 | Updated at the Q.10/8 Rapporteur's meeting, Ismaning, 30 September - 4 October 1996 |
| 5 | 17 January 1997 | Updated at the Q.10/8 Rapporteur's meeting, Newport Beach, 13-17 January 1997 |
| 6 | 29 January 1998 | Updated at the ITU-T SG 16 meeting, Geneva, 26 January - 6 February 1998 Sections added: 6.7.1.1 |
| 7 | 20 May 1999 | Updated at the ITU-T SG 16 meeting, Santiago, 17-28 May 1999 Added sections 6.8.6.1 and 6.8.9.1 |
| 8 | 8 June 2001 | Eighth version – Completed at the ITU-T Study Group 16 Rapporteurs meeting. |
| 9 | 15 February 2002 | Ninth version – Completed at ITU-T Study Group 16 Rapporteurs meeting. |
| 10 | 30 January 2004 | Tenth version – Completed at ITU-T Study Group 16 Rapporteurs meeting. |
| 11 | 5 August 2005 | Eleventh version – Completed at ITU-T Study Group 16 Rapporteurs meeting. |
| 12 | 24 November 2006 | Twelfth version – Completed at ITU-T Study Group 16 Rapporteurs meeting. |

Contact information

| ITU-T Study Group 16/Question 1 Rapporteur | Patrick Luthi Tandberg Philip Pedersens vei 22 1366 Lysaker Norway | Tel: +47 67 125 125 Fax: +47 67 125 234 Email: patrick.luthi@tandberg.net |
| --- | --- | --- |
| ITU-T Recommendations T.120-series Editor | TBD – An editor is solicited |  |
| ITU-T Recommendations T.120-series Implementor’s Guide Editor | Patrick Luthi | Tel: +47 67 125 125 Fax: +47 67 125 234 Email: patrick.luthi@tandberg.net |

Table of Contents

1.	Introduction	2

2.	Scope	2

3.	Policies for updating this document	2

4.	Defect Resolution Procedure	2

5.	References	2

6.	Nomenclature	2

7.	Technical and Editorial Corrections	2

7.1	Technical and Editorial Corrections to ITU-T Recommendation T.120	2

7.2	Technical and Editorial Corrections to ITU-T Recommendation T.120, Annex C	2

7.3	Technical and Editorial Corrections to ITU-T Recommendation T.121	2

7.4	Technical and Editorial Corrections to ITU-T Recommendation T.122	2

7.5	Technical and Editorial Corrections to ITU-T Recommendation T.123	2

7.6	Technical and Editorial Corrections to ITU-T Recommendation T.124	2

7.7	Technical and Editorial Corrections to ITU-T Recommendation T.125	2

7.8	Technical and Editorial Corrections to ITU-T Recommendation T.126	2

7.9	Technical and Editorial Corrections to ITU-T Recommendation T.127	2

7.10	Technical and Editorial Corrections to ITU-T Recommendation T.128	2

7.11	Technical and Editorial Corrections to ITU-T Recommendation T.134	2

7.12	Technical and Editorial Corrections to ITU-T Recommendation T.135	2

7.13	Technical and Editorial Corrections to ITU-T Recommendation T.136	2

7.14	Technical and Editorial Corrections to ITU-T Recommendation H.282	2

Annex: T.120 Recommendation Series Defect Report Form	2

Implementor's Guide for the ITU-T T.120 Recommendation series - Data Protocols for Multimedia Conferencing

# 1.	Introduction

This document is a compilation of reported defects identified with the 1993-1999 editions of the ITU-T T.120-series Recommendations. It is intended to be read in conjunction with the Recommendations to serve as an additional authoritative source of information for implementors. The changes, clarifications and corrections defined herein are expected to be included in future versions of affected T.120-series Recommendations.

The first version of the guide was produced following the February 1996 ITU-T Study Group 8 meeting. Wide distribution of this document is expected and encouraged.

# 2.	Scope

This Guide resolves defects in the following categories:

- Editorial errors

- Technical errors such as omissions or inconsistencies

- Ambiguities.

In addition, the Guide may include explanatory text found necessary because of interpretation difficulties apparent from the defect reports.

This Guide will not address proposed additions, deletions, or modifications to the Recommendations that are not strictly related to implementation difficulties in the above categories. Proposals for new features should be made in the normal way through contributions to the ITU-T.

# 3.	Policies for updating this document

This document is managed by the ITU-T Study Group 16 Question 1 Rapporteur's Group. It can be revised at any recognized Q.1/16 Rapporteur's Group meeting provided the proposed revisions are unanimously accepted by the members of the group. A revision history cataloguing the evolution of this document is included.

# 4.	Defect Resolution Procedure

Upon discovering technical defects with any components of the T.120 Recommendations series, please provide a written description directly to the Q.1/16 Rapporteur.

# 5.	References

This document refers to the following T.120-series Recommendations:

- ITU-T Recommendation T.120 (07/96) - Data Protocols for Multimedia Conferencing.

- ITU-T Recommendation T.120, Annex C (02/98) - Lightweight Profiles for the T.120 Architecture.

- ITU-T Recommendation T.121 (07/96) - Generic Application Template.

- ITU-T Recommendation T.122 (02/98) - Multipoint Communication Service - Service Definition.

- ITU-T Recommendation T.123 (05/99) - Network Specific Data Protocol Stacks for Multimedia Conferencing.

- ITU-T Recommendation T.124 (02/98) - Generic Conference Control.

- ITU-T Recommendation T.125 (02/98) - Multipoint Communication Service - Protocol Specification.

- ITU-T Recommendation T.126 (07/97) - Multipoint Still Image and Annotation Protocol.

- ITU-T Recommendation T.127 (08/95) - Multipoint Binary File Transfer Protocol.

- ITU-T Recommendation T.128 (02/98) - Multipoint Application Sharing.

- ITU-T Recommendation T.134 (02/98) - Text Chat Application Entity.

- ITU-T Recommendation T.135 (02/98) - User-To-Reservation System Transactions within T.120 Conferences.

- ITU-T Recommendation T.136 (05/99) - Remote Device Control Application Protocol.

- ITU-T Recommendation H.282 (05/99)- A Far End Camera Control Protocol for Video Conferencing Using H.224.

# 6.	Nomenclature

In addition to traditional revision marks, the following marks and symbols are used to indicate to the reader how changes to the text of a Recommendation should be applied:

| Symbol | Description |
| --- | --- |
| [Begin Correction] | Identifies the start of revision marked text based on extractions from the published Recommendations affected by the correction being described. |
| [End Correction] | Identifies the end of revision marked text based on extractions from the published Recommendations affected by the correction being described. |
| ... | Indicates that the portion of the Recommendation between the text appearing before and after this symbol has remained unaffected by the correction being described and has been omitted for brevity. |
| --- SPECIAL INSTRUCTIONS --- {instructions} | Indicates a set of special editing instructions to be followed. |

# 7.	Technical and Editorial Corrections

## 7.1	Technical and Editorial Corrections to ITU-T Recommendation T.120

Changes from the previous version were incorporated in T.120 (2007).

## 7.2	Technical and Editorial Corrections to ITU-T Recommendation T.120, Annex C

-None-

## 7.3	Technical and Editorial Corrections to ITU-T Recommendation T.121

-None-

## 7.4	Technical and Editorial Corrections to ITU-T Recommendation T.122

-None-

## 7.5	Technical and Editorial Corrections to ITU-T Recommendation T.123

Changes from the previous version were incorporated in T.123 (2007)

## 7.6	Technical and Editorial Corrections to ITU-T Recommendation T.124

Changes from the previous version were incorporated in T.124 (2007)

## 7.7	Technical and Editorial Corrections to ITU-T Recommendation T.125

-None-

## 7.8	Technical and Editorial Corrections to ITU-T Recommendation T.126

### 7.8.1	ASN.1 Errata

Description:	the field “Point” is referred to but undefined in the ASN.1 module for T.126. It should be replaced in its single occurrence by “WorkSpacePoint”.

The ASN.1 text shall be amended as shown below by the underline.

[Begin Correction]

…

--  EditablePlaneCopyDescriptor

--  Paired list of handles for source objects and their copies.

EditablePlaneCopyDescriptor ::= SEQUENCE

{

objectList	SEQUENCE (SIZE (1..65536)) OF SEQUENCE

{

sourceObjectHandle	Handle,

destinationObjectHandle	Handle

--  This handle is used to reference the new copy of the

-- source object in the future.

},

destinationOffset	WorkspacePoint OPTIONAL,

--  This parameter defines an offset to be added to the

--  coordinates of all of the copied objects. If not present,

--  zero offset is assumed.

planeClearFlag	BOOLEAN,

--  When FALSE, the destination objects are appended to

--  the existing set of objects in the destination plane. When

--  TRUE, all existing objects in the destination plane are

--  deleted prior to the copy operation.

…

}

…

[End Correction]

## 7.9	Technical and Editorial Corrections to ITU-T Recommendation T.127

### 7.9.1	ASN.1 Errata

Description:	The following additions to the ASN.1 for the File-OfferPDU were omitted from the published text, as documented in the editor's errata sheet issued at the Paris Rapporteur meeting of 10/95.

The ASN.1 text shown below shall be added.

[Begin Correction]

File-OfferPDU ::= SEQUENCE

{

file-header			FileHeader,

data-channel-id		ChannelID,

file-handle			Handle,

roster-instance		INTEGER (0..65535) OPTIONAL,

file-transmit-token		TokenID OPTIONAL,

file-request-token		TokenID OPTIONAL,

file-request-handle		Handle OPTIONAL,

mbftID			UserID OPTIONAL,

compression-specifier	CompressionSpecifier OPTIONAL,

compressed-filesize		INTEGER OPTIONAL,

ack-flag			BOOLEAN,			-- True if acknowledgements

-- required

...

}

[End Correction]

## 7.10	Technical and Editorial Corrections to ITU-T Recommendation T.128

-None-

## 7.11	Technical and Editorial Corrections to ITU-T Recommendation T.134

-None-

## Technical and Editorial Corrections to ITU-T Recommendation T.135

### 7.12.1	Corrections to Section 9.2 - URST ASN.1 Module

GCC-PROTOCOL is usually not associated with and object identifier (OID) except in T.135. Therefore, to be consistent with the other T.120-series documents, the OID reference in Section 9.2 is removed as shown below.

[Begin Correction]

…

-- Imports definitions from T.124

IMPORTS

TextString, SimpleTextString, DialingString, ExtraDialingString, SubAddressString,

H221NonStandardIdentifier, Key, NonStandardParameter, ChallengeResponse, ChallengeRequest,

Privilege

FROM GCC-PROTOCOL;

[End Correction]

## 7.13	Technical and Editorial Corrections to ITU-T Recommendation T.136

-None-

## 7.14	Technical and Editorial Corrections to ITU-T Recommendation H.282

-None-

# Annex: T.120 Recommendation Series Defect Report Form

| DATE: |  |
| --- | --- |
| CONTACT INFORMATION NAME: COMPANY: ADDRESS: TEL: FAX: EMAIL: |  |
| AFFECTED RECOMMENDATIONS: |  |
| DESCRIPTION OF PROBLEM: |  |
| SUGGESTIONS FOR RESOLUTION: |  |

Note - Attach additional pages if more space is required than is provided above.